Chapter 2:  System Structure

This chapter describes the registers, interrupt scheme, and program levels used in the 3704 and the 3705. The user needs a thorough understanding of these facilities in order to program the controller efficiently.

Note: Except for Extended Addressing, the text of this chapter applies to the 3704, 3705-1, and 3705-II controllers. The Extended Addressing feature is not available in the 3704.

Registers

The controller has two types of registers—general and external. These registers vary in size and location according to how they are used. They can range from one bit to eighteen bits. The following paragraphs briefly describe the types, size, and usage of the registers.

#### General Registers

Thirty-two general registers are available in the controller for program use. These registers are located in a local storage array so as not to occupy usable storage locations. The basic size of each register is one half-word (16 bits). The bits are designated from left to right as byte 0, bits 0-7 and byte 1, bits 0-7. In a 3705 with Extended Addressing (see Chapter 3), each register contains 18 information bits. The 18 information bits are designated from left to right as byte X, bits 6-7; byte 0, bits 0-7; and byte 1, bits 0-7. Without Extended Addressing, byte X is not present, and any reference to it is ignored.

As shown in Figure 2-1, the 32 general registers are divided into four groups of eight registers each. Each group is assigned to a specific program level, except for group 0, which is shared by program levels 1 and 2. (See Program Levels in this chapter.) This allows the control program at one level to be interrupted by another level without the need to save registers. The general registers are numbered 0-7 within each group. Only one group of general registers is active at a time—the group associated with the active program level. The registers within the currently active group are directly addressable with program instructions. The control program can gain access to the general registers in a nonactive group by specifying them as external registers in input and output instructions.

Instruction Address Register

General register 0 in each group is the instruction address register (IAR). This register is an implied base register and contains the address of the next instruction to be executed for the associated program level.

|  |  | Byte X (See note) |  | Byte 0 | Byte 1 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 67 | 01234567 | 01234567 |
| Reg | 0 |  |  |  |  |
|  | 1 |  |  | * | * |
| Group 0 (Program Levels 1 and 2) | 2 |  |  |  |  |
|  | 3 |  |  | * | * |
|  | 4 |  |  |  |  |
|  | 5 |  |  | * | * |
|  | 6 |  |  |  |  |
|  | 7 |  |  | * | * |
| Reg | 0 |  |  |  |  |
|  | 1 |  |  | * | * |
| Group 1 (Program Level 3) | 2 |  |  |  |  |
|  | 3 |  |  | * | * |
|  | 4 |  |  |  |  |
|  | 5 |  |  | * | * |
|  | 6 |  |  |  |  |
|  | 7 |  |  | * | * |
| Reg | 0 |  |  |  |  |
|  | 1 |  |  | * | * |
| Group 2 (Program Level 4) | 2 |  |  |  |  |
|  | 3 |  |  | * | * |
|  | 4 |  |  |  |  |
|  | 5 |  |  | * | * |
|  | 6 |  |  |  |  |
|  | 7 |  |  | * | * |
| Reg | 0 |  |  |  |  |
|  | 1 |  |  | * | * |
| Group 3 (Program Level 5) | 2 |  |  |  |  |
|  | 3 |  |  | * | * |
|  | 4 |  |  |  |  |
|  | 5 |  |  | * | * |
|  | 6 |  |  |  |  |
|  | 7 |  |  | * | * |

* Indicates selectable bytes of general registers

Note: This byte present only in a 3705 with Extended Addressing

Figure 2-1. General Register Groups

Register 0 of the active group is always incremented to point to the next sequential instruction before the current instruction is executed. In most cases, the next halfword is the next instruction to be executed. Sometimes, however, the contents of the instruction address register are changed as the result of the instruction being executed. Execution of a branch instruction, for example, can cause the IAR to be loaded with a storage address other than the one immediately following the current instruction. Refer to the individual instruction definitions in Chapter 4 for the precautions and results of the use of register 0.

IBM 3704 and 3705 Communications Controllers Principles of Operation  2-1

External Registers

Each functional unit of the communications controller (Central Control Unit, communication scanner, channel adapter) has a number of hardware registers that are used to store information required for communication between the control program and hardware circuits. These registers are called external registers.

Many of the external registers contain information pertinent to the operation of the hardware and/or the program. By using an input instruction, the control program can load the contents of an external register into a general register where it can operate on the data. Output instructions load an external register with the contents of the general register specified in the instruction.

The external registers that the control program can use are described in the Central Control Unit chapter    |and in each of the communication scanner and channel adapter chapters in this publication. Appendix A contains a summary of the external register addresses and functions and/or operations associated with each register.

### Program Levels

The controller hardware has five operational program levels. Each program level operates similarly to a subroutine and is responsible for particular phases of the system operation. Figure 2-2 shows (1) the program levels in order of priority, (2) the interrupt requests causing entry, and (3) the general register group associated with each level. Program level 1 has the highest priority, and program level 5 has the lowest. Program levels 1, 2, 3, and 4 (referred to as interrupt program levels) provide the program interface between the hardware functional units and program level 5 (referred to as the background program level).

The following is a brief description of each of the five program levels.

• Background Program Level 5: This level is the lowest priority level and is normally active when none of the other four levels requires program cycles. Functions performed by this level should normally include (1) line management (host command interpretation, control of polling and addressing), (2) data and message handling, and (3) control command decoding and execution. This level cannot interrupt another program level.

• Interrupt Program Level 4: The functions performed by this level should normally include (1) overall management of the system resources, (2) buffer management, (3) queue manipulation, and (4) the dispatching of program level 5 tasks. Certain program-controlled interrupt requests and the supervisor call request (generated when the Exit

instruction is executed at level 5) are assigned to this program level.

Interrupt Program Level 3: Level 3 should be used for most of the host processor/channel adapter interaction. This level handles interrupt requests from the channel adapter(s), the interval timer, the control panel Interrupt push button, and the communication processing that can be deferred from level 2. In addition to hardware interrupts, level 3 can be called by program controlled interrupts (PCI) for initiating I/O and for any other services desired by the user. Level 3 interrupts are less critically time-dependent than those assigned to program level 2.

Interrupt Program Level 2: Because of its high priority, this level services only interrupts from the communication lines for buffer, character or bit service. The control program can request a level 2 interrupt, but for the most part, it is entirely hardware interrupt driven. Normal operational interrupt requests from the communication scanner include (l)Type 1 Scanner bit service interrupts, (2) Type 1 Scanner character service interrupts, (3) Type 2 Scanner character service interrupts and (4) Type 3 Scanner buffer service interrupts. Only critically time-dependent processing should be done at this level. Interrupt Program Level 1: This is the highest-priority program level. It can be masked for channel adapter and scanner checks only when the Central Control Unit is in the test mode. A level 1 interrupt is invoked mainly to service "trouble" indications and is hardware-interrupt driven. Conditions that cause a level 1 request include all critical check conditions, such as (1) program checks, (2) addressing exceptions, (3) Central Control Unit (CCU) checks, and (4) scanner and channel adapter checks. Initial program load (IPL) procedures and address compare interrupts are also handled in this level.

Because the same group of general registers is used for both level 1 and level 2, the level 1 program should take precautions to save the group 0 registers. A Store instruction with register 0 specified in the R field must be the first instruction executed in program level 1. For this special case, the Store instruction is modified (by hardware) to allow the contents of register 0 to be stored at the second operand location.

Interrupts

The communications controller operates in response to requests from either the control program or the hardware. Since these requests may have varying degrees of urgency, a priority system is used. Each program, CCU, and adapter request is assigned a particular priority level. A request for use of the controller by the

2-2

##### Level 1 Interrupt Request

Address Compare L1

IPL L1

Input/Output Check L1

Protection Check L1

Invalid Op Check L1

Type 1, 2, or 3 Scanner L1

Type 2 or 3 Scanner 2 L1

Type 2 or 3 Scanner 3 L1

Type 2 or 3 Scanner 4 L1

Type 2, 3, or 4 CA-2 L1

Remote Program Loader L1

###### Level 2 Interrupt

Type 1 Char. Service L2

Type 1 Scanner Bit Service L2

Type 2 Scanner Char. Service L2

Type 3 Scanner Bufer Service L2

Level 3 Interrupt Request

PCI L3

Type 1, 2, 3 or 4 CA-1 L3

Type 2, 3, or 4 CA-2 L3

Interval Timer L3

Interrupt Push Button

Remote Program Loader L3

Level 4 Interrupt Request

PCI L4

SVC L4

User

Background

Program

Figure 2-2. Program Levels

IBM 37054 and 3705 Communications Controllers Principles of Operation  2-3

control program or hardware functions is called an interrupt request.

Each interrupt request is assigned to a program level. These program levels are numbered from one to five and determine the priority structure. Program level 1 has the highest priority, and the priority level decreases as the program number increases.

The machine priority controls determine when an interrupt can occur. If the interrupt request is to be allowed, the change from the active program level to the interrupting program level takes place immediately after completion of the current instruction. If several interrupt requests having different priorities are present at the same time, the one with the highest priority obtains use of the controller. When an interrupt request is granted use of the controller, it can be interrupted in that use by another request having a higher priority.

When an interrupt occurs, instruction execution at the lower-priority program level is suspended until instruction execution is completed at the higher-priority level. An interrupt to a specific program level prevents future interrupt requests assigned to either that level or to lower-priority program levels from causing another interrupt until the servicing of the first interrupt is complete.

The controller does not allow a particular interrupt if any of the following conditions exist.

• A higher-priority interrupt request is present.

• The program level to be interrupted is already entered ('interrupt entered' latch is on).

• The interrupt request or the program level to be interrupted is masked.

• A cycle-steal request exists for a Type 2 or 3 Channel Adapter or a Type 3 Communication Scanner.

At the time an interrupt is honored, the 'interrupt entered' latch for that program level is turned on. The 'interrupt entered' latch is a hardware latch that signals the controller that the associated program level has been entered. As long as this latch is on, no other interrupt requests to that level are honored. This prohibits interrupts that could destroy necessary information. The 'interrupt entered' latch is not turned off when its associated program level is interrupted by a higher priority level. It is turned off only by an Exit instruction or by a reset condition to the controller.

For an example of the interrupt facility, refer to Figure 2-3. The program at level 4 is being executed, and a level 2 interrupt request occurs (1). The controller hardware forces a branch to the starting address of program level 2 (2), and the program at that level begins servicing the interrupt. A level 3 interrupt request occurs (3), but it is not honored because program level 2 has a higher priority. When the level 2

interrupt has been serviced, the program executes an Exit instruction (4). The controller now allows the next highest-priority interrupt to be serviced. In this example, control is passed to program level 3 at its starting address (5). If, before the level 3 interrupt has been completely serviced another level 3 interrupt request occurs (6), no action is taken because the level 3 'interrupt entered' latch is on. However, as soon as program level 3 executes an Exit instruction (7), signaling the completion of processing and turning off its 'interrupt entered' latch, the controller can honor the second level 3 interrupt request and return control to the starting address of program level 3 (8). When servicing of the latest interrupt is complete and the Exit instruction is executed (9), control is again passed to the highest-priority level that is able to execute. In this case program level 4 is the highest-priority level requiring service (10), so control is returned to it at the instruction following the point of interruption.

At times it may be desirable not to interrupt a particular operation by a higher-priority request. For such cases, a mask can be set to prevent interrupts to a particular program level. See Masking Program Level Priorities in Chapter 5.

When an interrupt occurs, instruction execution at that level begins with the instruction located at the starting address of that level. The starting address of each interrupt level is a permanently assigned storage location. The instruction(s) beginning at these fixed locations must direct the control program to the correct routine(s) to handle the particular interrupt. The remainder of the instructions for a particular program level need not be located in any specific storage area. In addition, some routines may be used by more than one program level. However, in this case, the execution of that routine will be at the priority level of the currently active program level.

The starting addresses for the four program levels that can cause an interrupt are:

| Level | Starting Address (Hex) |
| --- | --- |
| Program Level 1 Program Level 2 Program Level 3 Program Level 4 | X'00010' X'00080' X'00100' X'00800' |

When a program level has completed its interrupt servicing, it must execute an Exit instruction. The Exit instruction causes the 'interrupt entered' latch for that level to be reset and allows control to be passed to the next higher-priority program level requiring service.

When the Exit instruction is executed at program level 5, a supervisor call interrupt request for level 4 (SVC L4) is set. This is the only manner in which program level 5 can generate an interrupt request.

2-4

Fige 2-3. Interrupt Priority Examples

IBM 3704 and 3705 Communications Controllers Principals of Operation  2-5