Chapter 11: Remote Communications Controller

This chapter describes the requirements necessary to support a 3704 or 3705 at a remote location. (See the Introduction to the IBM 3704 and 3705 manual for introductory information about the remote communications controller.) Throughout this chapter, both the controller attached to the host processor and the control program of that controller are referred to as the local communications controller and the local control program respectively. The 3704 or 3705 located at a site away from the host processor and connected to the local communications controller by a local/remote communication link is referred to as the remote communications controller. The control program that resides in the remote communications controller is called the remote control program.

Local/Remote Communication Link

Communication between the local and remote controllers is over a local/remote communication link using the synchronous data link control (SDLC) discipline. Support of SDLC is required in both the local and remote controllers.

For information on the SDLC line discipline, refer to the Synchronous Data Link Control General Information manual.

Data flow through the local controller is the same as described in Chapter 1. However, in support of a remote controller, the data being transmitted to the remote unit exits the local controller via a communication scanner SDLC interface. The local/remote communication link attaches to an SDLC interface of a remote communication scanner; therefore, data enters the remote controller as a received message and the control program must turn this data around and transmit it to the proper communications line for the intended destination. (Refer to Figure 11-1.)

Remote Program Loader

The remote program loader replaces the channel adapter in the remote communications controller with an independent IPL capability and a source of resident diagnostics. The following functions and hardware are provided with the remote program loader.

• A disk storage drive and disk controller that provide a source for the load programs and diagnostic programs.

• A register for storing the type of program load and stimulus.

• A ROS bootstrap program and two load programs that (1) test the instructions used to load programs from the disk, (2) perform a check of the disk con

troller and, (3) control the transfer of a load program from disk storage to main storage.

The disk used in the disk storage drive contains one physical record on each of its 77 tracks and has a minimum access time of 150 milliseconds per track. The disk is the source of load and diagnostic programs for the remote unit and therefore serves as read-only-storage. The ability to write on this disk is normally disabled so as to provide file protection. The only time a write operation can be performed on the disk is when the controller is in the not initialized state (for example, IPL Phase 2 or Phase 3), or when a jumper wire is placed on the write circuit for maintenance purposes.

The following table lists the contents of the tracks.

| Track | Content | Description |
| --- | --- | --- |
| 0 | Load Program 1 (LPG1) | (1) Program that is transferred to main storage by the ROS boot-strap to control future loading of programs. (2) Defines the local /remote communications link for LPG2. |
| 1-5 | Initial Test | A CCU diagnostic exerciser. |
| 6-7 | Load Program 2 (LPG2) | Controls the local/remote communications link for loading and dumping. |
| 8 | IFT Loader/CDS Writer |  |
| 9 | Diagnostic Control Monitor |  |
| 10 | Hardware Configuration Data Set (CDS) for Internal Function Tests(IFT). | Defines the hardware configuration for the internal functional tests. |
| 11 | Load Program 1 | Duplicate of track 0. |
| 12-13 | Load Program 2 | Duplicate of tracks 6 and 7. |

IBM 3704 and 3705 Communications Controllers Principles of Operation   11-1

Figure 11-1. Teleprocessing System with Local and Remote Communciations Controllers.

11-2

| 14 | Reserved |  |
| --- | --- | --- |
| 15-16 | Dump Data | Used to store data from high 8K of main storage. |
| 17 | Dump Routine |  |
| 18-76 | Internal Functional Tests (IFTs) | Reserved for diagnostic tests. |

Remote Program Loader Input Instructions

Four input instructions give the controlling program access to the various remote program loader registers. These instructions are listed below. For a description of each bit used in these instructions, refer to the Remote Program Loader section of Appendix B.

Input X'68' (Level 1 Status): This instruction allows the controlling program to examine the contents of the level 1 status register to determine the cause of a remote program loader level 1 interrupt.

Input X'69' (Level 3 Status): This instruction allows the controlling program to examine the contents of the remote program loader level 3 status register after a level 3 interrupt request or when the disk controller is in a reset state.

Input X'6A' (Parallel Data Register): This instruction is used to transfer data into main storage during a disk data transfer. It is also used for diagnostic purposes.

Input X'6B' (Control Program Load Data Register): This instruction is used to supply information required during IPL and program load phases.

Remote Program Loader Output Instructions

Four output instructions are used to allow the controlling program to initiate disk read, write, and control operations. Listed below are the remote program loader output instructions. The bit positions are described in the Remote Program Loader section of Appendix B.

Output X'68' (Control): This output-is used to prepare the disk controller for subsequent read or write operations by controlling the access mechanism, engaging the read/write head, setting and resetting latches, and setting write current.

Output X'69' (Read/Write): This output commands the disk controller to perform read or write operations. A write operation can be performed only if (1) the communications controller is not initialized, or (2) the 'write enable' back panel jumper is installed.

Output X'6A' (Parallel Data Register): This output is used to load the register which holds data being transferred from the Central Control Unit (CCU) to the disk storage. It is also used for diagnostic purposes.

Output X'6B' (Control Program Load Register): This instruction is used to store information needed during the IPL and program load phases.

Remote Program Loader CCU Instructions

Two additional bits are required within existing CCU input instructions to support the remote program loader. They are byte 0, bit 7 of Input X'76', and byte 1, bit 1 of Input X'77'. See CCU Input Instructions in Appendix B for a description of these bits.

Disk Storage Controller

The disk storage controller is the logical connection between the disk storage drive and the Central Control Unit. This controller, under control of the program residing in main storage, controls all operations (read, write, access, etc.) of the disk storage.

Track Access

The head for the disk can be moved only one track at a time. Therefore, a move of multiple tracks must consist of a multiple of single track moves. For each track that the head is to be moved, the control program must execute an Output X'68' instruction with byte 1, bit 1 set to 1. This bit indicates to the controller that the head is to be moved. In addition, byte 1, bit 2 of Output X'68' must be set to indicate the direction of movement. When byte 1, bit 2 is on, the head moves in a forward direction (the next higher track number), and when this bit is off, the head moves in a reverse direction (toward track 0).

Each time the head is moved from one track to another, a counter in the level 3 status register is updated. By executing an Input X'69' instruction (level 3 status) and then examining byte 0, bits 2-5 of the general register specified, the control program can determine if the access mechanism has moved.Keep-ing track of the head position is the responsibility of the control program.

To recalibrate the disk head, the control program must reset the access counter by turning byte 0, bit 3 of Output X'68' on and then executing 80 single track moves in the reverse direction (Output X'68', byte 1, bit 1 on and bit 2 off). When these moves are com-

IBM 3704 and 3705 Communications Controllers Principles of Operation   11-3

plete, the head is at track 0 (home position), and the access counter is also 0.

To prevent undue wear on the surface of the disk, the head should not be engaged during track to track movement. The head will automatically disengage after a read or write operation within two revolutions of the disk (332 milliseconds) unless one of the operations is re-initiated. Therefore, the control program should ensure that enough time has elapsed to disengage the head prior to a move. Because of this automatic disengage operation, a head engage (Output X'68', byte 1, bit 4 on) should precede each read or write operation.

Track Format

Because the disk used in the remote communications controller is not interchangeable with other machines, the format of the disk records does not follow any previously established standards. The record format is designed to meet the needs of the 3704/3705. The format is as follows:

*--GI-VFO SYNC-DS-BH-DATA--CRC-GF--*

Index                                                         Index

| GI | = Initial Gap - 128 bytes of X'FF' |
| --- | --- |
| VFO SYNC | = 8 bytes of X'00' |
| DS | = Data Sync - 1 half word X'0005' |
| BH | = Block Header - 16 halfwords |
| DATA | = Maximum of 4668 bytes - 1 physical record with three logical records |
| CRC | = Cyclic Redundancy Check Character -2 bytes |
| GF * | = Final Gap to Index - X'FF' |

Block Header Format:

| Halfword | # Description |
| --- | --- |
| 1 | Track ID (X'0000' through X'004C') |
| 2 | Control Information |
| 3 | Reserved |
| 4 | Reserved |
| 5 | Logical Record 1 Byte X Storage Address (Extended Addressing) |
| 6 | Logical Record 1 Bytes 0 & 1 Storage Address |
| 7 | Logical Record 1 Count (Two's complement) |
| 8 | Program Version Level |
| 9 | Logical Record 2 Byte X Storage Address (Extended Addressing) |
| 10 | Logical Record 2 Bytes 0 & 1 Storage Address |
| 11 | Logical Record 2 Count (Two's complement) |

| 12 | ZAP Count (Number Of Temporary Fixes To This Track) |
| --- | --- |
| 13 | Logical Record 3 Byte X Storage Address (Extended Addressing) |
| 14 | Logical Record 3 Bytes 0 & 1 Storage Address |
| 15 | Logical Record 3 Count (Two's complement) |
| 16 | Block Header CRC Character |

The logical record storage address is the main storage address where the logical record is placed during a read operation. Each of the logical records may vary in length which is indicated by the count.

Note: The CRC is compatible with the SDLC CRC. See the Cyclic Redundancy Check section of Chapter 5.

Disk Read and Write Operations

The disk controller performs all read and write operations under the direction of the control program. Only single read or write operations are permitted. Prior to any read or write of the disk, the desired track must be located and the head engaged. The following paragraphs describe the read and write operations.

Read Operation: To initiate a read operation, the control program must execute an Output X'69' instruction with byte 0, bit 2 set to 1. Following this output, the next time the index is detected from the disk (detection of index is a hardware function), the controller begins to shift bits from the disk through the serializer/deserializer circuits and into the controller's parallel data register. When the data sync pattern (X'0005') is recognized by the sync decode circuits, the controller is considered to be in sync with the incoming disk storage data. From this point on, the controller requests a level 3 data service interrupt at each 16-bit (halfword) interval until the index is again detected. The second time the index is passed, the read operation is terminated.

While this read operation is in progress, the control program must service each level 3 interrupt to receive the incoming data and place it in storage. Because the information in disk storage is the IPL and diagnostic programs, the controller will not be executing any other program that would interfere with the processing of incoming data.

Servicing the level 3 interrupt must consist of at least the following sequence of instructions:

• Input X'77'—To determine the cause of the interrupt.

• Input X'69'—To determine the status of the read operation.

11-4

• Input X'6A'—To load contents of the controller parallel data register into a general register for subsequent transfer to storage.

• Output X'68'—To reset the level 3 interrupt request and the controller status registers and to set any other desired control information.

• Some type of store instruction to transfer the half-word just received in the general register to the proper storage location.

The read operation can be terminated by the control program with a controller reset (Output X'68', byte 1, bit 5). This reset is recommended after the CRC characters are read so as not to read the final gap into main storage.

Write Operation: To initiate a write operation the control program must execute an Output X'69' instruction with byte 0, bit 1 set to 1. Following this output, the disk controller immediately requests a level 3 interrupt. The control program can then transfer the first half-word of information to the controller. One halfword is transferred for each level 3 interrupt until the write operation is terminated.

Before executing the Output X'69' to request the write operation, the control program should have loaded a general register with the first halfword of information to be written on the disk. Servicing the level 3 interrupt must consist of at least the following sequence of instructions.

• Input X'77'—To determine the cause of the interrupt.

• Output X'68'—To set the proper control information.

• Output X'6A'—To load the contents of the general register (data to be written) into the controller parallel data register from which it is written on the disk.

• Output X'68'—To reset the level 3 interrupt request.

• Some type of load instruction to place the next half-word to be written into the general register specified in the Output X'6A' instruction.

The controller starts writing data on the disk at the index and continues to write halfwords until the next time the index is encountered. At this point the write operation is terminated.

The control program is responsible for writing the complete track on the disk. This includes the initial gap, VFO sync, data sync, block header, up to 4668 bytes of data, the CRC character, and the all ones field for the final gap. (See Track Format in this chapter.) The controller assures proper serialization of

the data onto the disk as it is received from the control program.

A write operation can be done only when the write circuits are enabled. See Remote Program Loader in this chapter. Attempting to do a write operation when the write circuits are not enabled causes a level 1 interrupt. Normally, the only time a write operation is performed is when a dump is required during an IPL or when the customer engineer has to write for maintenance purposes.

Initial Program Load

The initial program load (IPL) mechanism controls the loading of an initial program into main storage from disk storage. Three phases of the IPL program (IPL Phase 1-3) control the loading operation. IPL is accomplished by successful completion of all three phases. These three phases are similar to the local IPL operation described in Chapter 5. Therefore, this chapter only points out the differences required for remote operation.

IPL phase 1 and phase 2 are identical to phases 1 and 2 of the local controller's IPL program except that the bootstrap program for the remote controller consists of 1024 bytes. Phase 3 of the remote IPL program is divided into three sections.

The first section performs the same checks and saves the same registers as section one of the local IPL program. The second section of the remote phase 3 tests the disk storage drive and the disk controller. Section three of phase 3 controls the loading of load program 1 from disk storage to main storage.

The remainder of the IPL operation is controlled by load program 1 (LPG1) and load program 2 (LPG2). See Figure 11-2 for a flow chart description. LPG1 determines if a dump of the remote controller's storage is required by examining register X'6B' and the ADDRESS/DATA switches (3705) or the Hexadecimal Readout Display (3704). If a dump is required, LPG1:

1. Turns on byte 0, bit 0 of register X'6B'.

2. Transfers contents of high 8K of main storage to tracks 15 and 16 of the disk.

3. Transfers LPG2 from disk tracks 6 and 7 to the high 8K of main storage.

LPG1 then passes control to LPG2.

If no dump is required, LPG1 transfers initial test from disk tracks 1-5 to main storage. LPG1 loads the internal function tests if they have been requested at the control panel. If there is no request to load the internal function tests, load program 2 (LPG2) is loaded and given control.

LPG2 controls loading the control program into main storage from the local/remote communication

IBM 3704 and 3705 Communications Controllers Principles of Operation   11-5

Figure 11-2. IPL-Remote Communications Controller

11-6

link; or, when a dump is required, it controls transferring the contents of main storage and disk tracks 15 and 16 over the local/remote communications link to the local communications controller.

Register X'6B' is available to a remote communications controller to store and pass information which may be required by the IPL and LOAD programs. Refer to Appendix B for a description of the bits of this register.

Remote Power Off

This feature allows the user to power down the remote controller by issuing a command from the host processor. The command issued by the host causes the remote control program to execute an Output X'79' instruction with byte 0, bit 4 set to 1. When power is turned off this way it can only be restored manually at the control panel.

IBM 3704 and 3705 Communications Controllers Principles of Operation   11-7