Chapter 7: Type 2 Communication Scanner

This chapter is intended to give the reader a basic understanding of the operation of the Type 2 Communication Scanner and the requirements necessary to program the scanner.

The Type 2 Communication Scanner (1) scans the interface addresses assigned to the LIB positions it supports, (2) performs character assembly/ disassembly, (3) provides character buffering, and (4) causes program interrupts when character service is required.

Up to four Type 2 Communication Scanners can be installed in the 3705 (Type 2 Scanner-1 through Type 2 Scanner-4). Type 2 Scanner-1 supports attachment of up to four LIBs with 64 half-duplex (HDX) lines. Type 2 Scanner-2, Type 2 Scanner-3, and Type 2 Scanner-4 can each support attachment of up to six LIBs with 96 HDX lines; thus, up to 352 HDX lines can be attached to the 3705 using four Type 2 Scanners. The Type 2 Scanners can be installed with either the Type 1, Type 2, or Type 3 Channel Adapter features.

The 3704 is limited to one Type 2 Scanner, which is operationally equivalent to and program compatible with the 3705 Type 2 Scanner. The 3704 Type 2 Scanner supports one Type Al LIB, which provides a maximum of ten lines. Additional capability is available that enables the scanner to support two LIBs and a maximum of 26 half-duplex lines. These LIBs can be in any combination (except two Type 1 LIBs).

The number of lines supported depends on:

• How lines are used.

• Line disciplines.

• Mix of line speeds.

For information about LIBs and their capacities, refer to the 3704 and 3705 Introduction manual.

Operation and Data Flow

The interface addresses for all installed Type 2 Communication Scanners in the 3705 are generated from a common Type 2 Attachment Base. The 3704 does not have a Type 2 Attachment Base. All necessary hardware functions that would be provided by the Type 2 Attachment Base are integrated within the 3704 Type 2 Scanner. Figure 7-1 shows the basic operation of the Type 2 Scanner. A continuously running scan counter in the attachment base places the generated interface address on an address bus that goes to all scanners simultaneously. This address can be modified, under program control, by the attachment base or the scanner.

The interface address is then used to address an interface control word (ICW), which is loaded into the ICW work register where the scanner hardware determines if any action is to be performed for that interface. If no action is required, the ICW is replaced in local store and the next addressed ICW is loaded into the work register. If the scanner determines that program intervention is required, it requests a level 2 interrupt and loads the interface address into an interrupt priority register.

When the level 2 interrupt actually occurs, the address in the highest-priority interrupt priority register that is active is loaded into the attachment buffer address register (ABAR) and is then available to the control program along with the ICW in the ICW input register.

Type 2 Scanner Registers

The Type 2 Scanner contains various hardware registers that are used to store and pass information and data within the scanner and between the scanner and the control program. Some of these hardware registers are available to the control program as external register addresses through input and output instructions. The external registers required for control program access are described in the following paragraphs.

Local Storage

Each Type 2 Scanner contains a local storage array to store the interface control words when not being used by the scanner hardware or the control program. This storage array holds 96 control words of 48 bits each (46 information bits and 2 parity bits). The 3704 local storage is limited to 16 interface control words (only 10 are used). If the scanner supports two LIBs, the 3704 local storage is increased to 32 interface control words (a maximum of 26 can be used).

ICW Work Register

The Type 2 Scanner control logic uses the ICW work register to access, monitor, and modify an interface control word (ICW). This register is loaded each time an ICW is read out of local storage.

ICW Input Register

The control program uses the ICW input register for access to the interface control words. This register is loaded from the ICW work register and reflects the status of the ICW at the time when it was read out of local storage.

IBM 3704 and 3705 Communications Controllers Principles of Operation  7-1

Figure 7-1. Type 2 Communication Scanner Operation

Attachment Buffer Address Register

The attachment buffer address register (ABAR) is physically located in the Type 2 Attachment Base and supplies the interface addresses to the control program. See I/O Programming Considerations in this chapter for a description of loading the ABAR.

## Programming Note

The ABAR must be initialized by an Output X'40' instruction with an interface address associated with an installed Type 2 Scanner. The Output X'40' must be executed after the controller is powered on and before any other input or output instructions to the scanner are executed.

Display Register

The display register is a temporary storage register that can contain interface control information that the program can use. If bit 38 (display request) of an interface control word is on, control information for that interface is loaded into the display register each time that interface is scanned. The control program can then obtain this information by executing an Input X'46' instruction.

There is only one display register for each Type 2 Scanner; therefore, only one ICW at a time should have its display request bit set to 1. Otherwise, the control program cannot determine which interface was the last one to cause the display register to be loaded.

7-2

Type 2 Scanner Addressing

The Type 2 Communication Scanner scan-addressing and program-addressing mechanism is controlled by the Type 2 Attachment Base. The attachment base generates the basic scan address and places it on a 'line address bus' for availability to all installed Type 2 Communication Scanners. Refer to the Interface Addressing section of Chapter 3 for a detailed discussion of each interface address bit.

Scan Addressing

For scan addressing, an interface in each installed Type 2 Scanner is addressed simultaneously. Each scanner derives the address of the interface it is scanning from the 8-bit address that the Type 2 Attachment Base places on a 'line address bus'. The line address bus is an internal bus that carries the scan address from the attachment base to each of the communication scanners. This address, as modified by each scanner (see upper scan limit), is used not only to select a particular interface but also to address the associated interface control word (ICW) that the scanner maintains in local storage. (See Interface Control Word in this chapter.) The Type 2 Scanner examines this ICW and, when an interface service function is required, performs that function; or, when a character service requires programming action, the Type 2 Scanner signals the attachment base that it needs a program level 2 interrupt.

Scan Counter

The 3705 Type 2 Attachment Base scan counter output provides the basic scan addresses for each Type 2 Scanner. If the scan counter output is not modified, each Type 2 Scanner sequentially scans 96 interface addresses. Under these circumstances, the Type 2 Scanner cannot handle line speeds higher than 4800 bps without having the possibility of undetected bit overrun/underrun conditions. However, the ability to substitute some interface addresses (address substitution) and set a limit on the number of interfaces scanned (scan limit), greatly extends the capability of handling higher-speed lines. These mechanisms cause the scan counter output to be modified to allow certain interface addresses to be scanned at a different rate.

The scan counter in the 3704 continuously steps through 16 different interface addresses at a rate of 1.2 microseconds per address and completes one scan of all addresses in 19.2 usec. However, because the LIB Type Al can support only ten lines, interface addresses 1, 3, C, D, E, and F are not available for use. Addresses 1 and 3 are modified to addresses 0 and 2 respectively. This allows line speeds up to

56,000 bps to be scanned on addresses 0 and 2.1nter-face addresses C, D, E, and F are ignored.

If the Type 2 Scanner supports two LIBs, the scan counter steps through 32 different interface addresses during a 38.4 microsecond scan period. With this expanded capability, all interface addresses (0-F) of both LIBs are scanned. Although 32 different addresses are available, the maximum number of lines supported is 26.

Upper Scan Limit

The Type 2 Scanners have an upper scan limit that can be set and reset under program control by an Output X'42' instruction. Each scanner maintains its own upper scan limit and is independent of the limits set by any of the other installed scanners. Based on the state of its 'upper scan limit' latches, a Type 2 Scanner may modify the 'scan counter' output from the Type 2 Attachment Base in such a way as to limit the number of interface addresses scanned.

The actual modification of the scan address is done by the Type 2 Scanner hardware as the line address bus enters the scanner from the attachment base.Figure 7-2 shows the number of interfaces scanned and the LIB position affected for each setting of the upper scan limit. When the upper scan limit is set to any value other than binary 00, the scanner modifies the addresses above the limit to start at the first address again. For example, if the upper scan limit is set to allow only 16 interface addresses to be scanned, the address is modified to scan the first address again when the scan counter output to that scanner reaches the 17th address. This decreases the period of time between successive scans of the remaining interface addresses to accommodate higher-speed lines. In this case, the scanner with an upper scan limit of binary 11 scans the first 16 interfaces four times in the same period of time as another scanner with no limit scans 96 interfaces. Unless the 3704 scanner can support two LIBs, modification of the upper scan limit is restricted to binary 01 in byte 1, bits 6 and 7 of Output X'42'. A binary 01 sets the limit to eight; any other value sets the scan limit to 16.

If the 3704 scanner supports two LIBs, a binary 11 in bits 6 and 7 causes 16 lines to be scanned twice during the scan period (38.4 microseconds). A binary 01 causes 8 lines to be scanned four times during the same scan period. Values of binary 00 and 10 allow all 32 addresses to be scanned (see Figure 7-2).

IBM 3704 and 3705 Communications Controllers Principles of Operation  7-3

| Upper | Number of | Interface | Interface | Scan |
| --- | --- | --- | --- | --- |
| Scan | Interfaces | Addresses | Addresses | Period |
| Limit* | Scanned | Scanned | Not Scanned | (µsec) |
| 3704 |  |  |  |  |
| 00 | 32** | AddrO-F,LIB 1-2 | — | 38.4 |
| 10 | 32** | AddrO-F,LIB 1-2 | — | 38.4 |
| 11 | 16 | Addr 0-F,LIB 1 | Addr 0-F,LIB2 | 19.2 |
| 01 | 8 | Addr 0-7,LIB 1 | Addr 0-F,LIB2 and | 9.6 |
|  |  |  | Addr 8-F, LIB 1 |  |
| 3705 |  |  |  |  |
| 00 | 96*** | Addr 0-F,LIB 1-6 | — | 153.6 |
| 10 | 48 | AddrO-F,LIB 1-3 | Addr 0-F.LIB 4-6 | 76.8 |
| 11 | 16 | Addr 0-F,LIB 1 | Addr 0-F.LIB 2-6 | 25.6 |
| 01 | 8 | Addr 0-7,LIB 1 | Addr 8-F,LIB 1 and | 12.8 |
|  |  |  | Addr 0-F,LIB 2-6 |  |
| * Set by Output X'42' byte 1, bits 6-7 |  |  |  |  |
| ** Applies only for a 3704 Type 2 Scanner with the capability of supporting two LIBs. |  |  |  |  |
| *** Scanner-1 contains 96 addresses but only uses the first 64. |  |  |  |  |

Figure 7-2. Upper Scan Limit

Address Substitution

The output of the scan counter can be modified to cause certain addresses assigned to LIB position 1 to be substituted on the 'line address bus' in place of normal scan addresses. As a result, those addresses that are substituted are scanned by the Type 2 Scanner more frequently than the other addresses. Address substitution affects all installed scanners in the same manner. (Address substitution is ignored in the 3704 unless the Type 2 Scanner can support two LIBs.) When operating with address substitution, each scanner in the 3705 scans the substituted address or addresses, every 12.8 microseconds (9.6 microseconds for a 3704 Type 2 Scanner with a two-LIB capability), because address substitution occurs every eighth time the scan counter changes state. This allows the substitution address or addresses in each scanner to handle higher line speeds independent of the state of the scan limit.

Address substitution is controlled by a four-bit register called the substitution control register. The bits of this register may be set under program control by Output X'41' byte 1, bits 2, 3, 4, and 5. Each bit of this substitution control register corresponds to one of four substitution addresses assigned to LIB position 1.

## Programming Note

Any combination of the four substitution control register bits may be turned on to produce the desired substitutions. If address substitution is not used, Output

X'41' must be executed with byte 1, bits 2 through 5 off in the register specified by the R operand.

When a given substitution control register bit is on, a corresponding address is substituted on the 'line address bus' every eighth time the scan counter changes state. Combinations of bits on in the substitution control register result in fixed-address substitution for each corresponding bit. Figure 7-3 shows which address is substituted and which addresses are not scanned as a result of that substitution when the different substitution control register bits are on.

| Output | Fixed Address |  |
| --- | --- | --- |
| X'41' | Substituted in | Addresses Not Scanned |
| Byte I, | Each Type 2 Scanner | In Each Type 2 Scanner |
| Bit: | If Substitution Bit ON | If Substitution Bit ON |
| 2 | Adr 0 LIB position 1 | Adr E in LIB positions 1-6 |
|  |  | Adr F in LIB positions 1-6 |
| 3 | Adr 2 LIB position 1 | Adr C in LIB positions 1-6 |
|  |  | Adr D in LIB positions 1-6 |
| 4 | Adr 4 LIB position 1 | Adr A in LIB positions 1-6 |
|  |  | Adr B in LIB positions 1-6 |
| 5 | Adr 6 LIB position 1 | Adr 8 in LIB positions 1-6 |
|  |  | Adr 9 in LIB positions 1-6 |

Figure 7-3. Address Substitution Control

7-4

Program Addressing

Various input and output instructions exist that allow the program to control the operation of the Type 2 Scanners, Type 2 Attachment Base, and the individual interfaces.

However, before the program can examine or modify fields in a interface control word (ICW) associated with a particular interface, the address of that interface, must be placed in the attachment buffer address register (ABAR) of the attachment base. Similarly, before the program can access certain registers in a particular Type 2 Scanner or perform control functions in that Scanner, the interface address in the ABAR must be one of those assigned to that scanner. Two events can cause the contents of the ABAR to be changed: (1) a program level 2 interrupt, and (2) execution of an Output X'40' instruction.

When a program level 2 interrupt occurs, the contents of the ABAR are automatically set by the Type 2 Attachment Base with the interface address from the highest interrupt priority register that is occupied. The control program can determine which interface address is in the ABAR by executing an Input X'40' instruction. The program can then examine and/or modify fields in the ICW associated with this interface. In the other interrupt program levels (1, 3, and 4), the program may find it necessary to gain access to the ICW associated with a specific interface. By executing Output X'40' under such circumstances, the program can set the ABAR according to the interface address in the register specified by the R operand.

To avoid conflicts with the automatic mechanism that sets the ABAR when a program level 2 interrupt occurs, programs executing at program level 3 or 4 should mask program level 2 interrupts before executing Output X'40'. (Program level 1 should save the value of the ABAR, change it to select the desired interface, and then restore the original value to the ABAR.) If more than one program level is likely to execute an Output X'40', additional interlocking conventions must be established within the control program.

Interface Control Word (ICW)

The Interface Control Word (ICW) provides the normal means by which the control program communicates with the Type 2 Scanner and the interface hardware.

The ICW is made up of 46 information bits and 2 parity bits and is physically located in the scanner local storage. Each scanner contains one ICW for each possible interface. However, even though the scanner contains the maximum number of ICWs (32 for a 3704, 96 for a 3705), only those ICWs associated with an attached and active interface are used.

ICW Access

The Type 2 Scanner hardware gains access to an ICW by using the interface address provided by the interrupt priority register in the Type 2 Attachment Base. When the level 2 interrupt occurs, the address from the interrupt priority register is loaded into the attachment buffer address register (ABAR). The program can then execute an Input X'40' instruction to get the storage address associated with the interface. Once the control program obtains the interface address, it has access to the various fields of the ICW through input and output instructions.

Progranuning Note

ICW access at program level 3 or 4 should be performed only when program level 2 interrupts are masked off; otherwise, the result is unpredictable.

ICW Modification

Program access to the various fields in the ICW is through the use of input and output instructions. When the Input X'44', X'45', and X'47' instructions are executed, the ICW bits assigned to those inputs are placed in the register specified in the R operand. See Appendix B for the input/output instruction bit definitions.

The information obtained by the input instructions comes from the ICW input register. There is one ICW input register in each communication scanner. This register is automatically loaded from the ICW work register when a level 2 interrupt occurs or when an Output X'40' instruction is executed in any program level other than level 1 or 2.

The ICW input register does not necessarily reflect the current state of the ICW associated with the interface address in ABAR. The actual ICW may have been modified by the Type 2 Scanner during scan addressing after the ICW input register was set.

Also, the actual ICW may have been altered by the execution of an Output X'43', X'44', X'45', X'46', or X'47' instruction. Subsequent input instructions do not include these alterations because the ICW input register had been set by a previous Output X'40' instruction or Type 2 Scanner level 2 interrupt. In the event that an output instruction and scan addressing both occur during the same scan cycle, the output instruction is executed first; then the scanner performs its modification, if needed. This ensures that the latest modifications to the actual ICW will be included during the next scan addressing operation.

Refer to Interface Control Word Format in this chapter for a complete description of the individual ICW fields and the I/O instructions associated with each field.

IBM 3704 and 3705 Communications Controllers Principles of Operation  7-5

## Programming Note

Since the interface control words are asynchronously interrogated and modified by both the scanners and the control program, caution should be observed to ensure the ICW integrity when the program issues an output instruction. The Type 2 Scanner cannot check whether the control program has modified the ICW correctly. Therefore, errors in the modification itself may be difficult to isolate. To prevent control program modifications to the ICW (via output instructions) from being destroyed by the scanner, program modifications are not permitted during that portion of a scan when the Type 2 Scanner fetches, modifies, and restores the ICW for the line being scanned.

Interface Control Word Format

The following paragraphs describe the ICW fields and their bit meanings. (See Figure 7-4.) For descriptions of ICW fields and their bit meanings when used for Synchronous Data Link Control (SDLC) lines, refer to the SDLC section of this chapter.

ICW Bits 0-7 (Secondary Control Field): The secondary control field (SCF) is used as a sense, status, and operation modifier field between the control program and the communication scanner. Bits 0-4 are set by the Type 2 Scanner hardware according to the conditions described below. Bits 6 and 7 are program controlled. This field may be tested by using the Input X'44' instruction. An Output X'44' instruction is used to reset bits 0-3 and 5 and to set and reset bits 6 and 7. Refer to Appendix B, Input/Output Instruction Bit Definitions.

Bit 0 — Stop Bit Check/Receive Break: For start-stop lines in PCF state X'7', the receive data bit buffer is checked after each character is received. If the bit buffer contains a (space) instead of a (mark), the Type 2 Scanner signals this condition to the control routine by setting this bit.

For start-stop lines during transmit operations (PCF state X'9'), the 'receive data' line is checked for a space (0) condition every time the first bit of a character is placed in the transmit buffer. If a space condition is detected, this bit is set on. When the control program detects that this bit is set for two consecutive characters, it should be interpreted as a receive break signal.

If this bit is 1, the service request interlock (ICW
bit l) will be 0.

For autocall interfaces and binary synchronous line interfaces, this bit is 0.

Bit 1 — Service Request Interlock: This bit is set when the Type 2 Scanner detects that data transfer or

control servicing is required between the control program and the 'parallel data field'. The control program must reset this bit after the interrupt is honored and all bits or bytes of the ICW have been modified. If this bit is already set when the scanner is prepared to set it on, and the PCF state is X'7' through X'A', a character overrun/underrun flag is set (ICW bit 2).

If this bit is 1, the stop bit check/receive break, character overrun/underrun, and modem check bits are 0.

## Programming Note

The control program should reset the service request interlock before setting the PCF state to monitor modem or autocall unit control lines.

Bit 2 — Character Overrun/Underrun: This bit is set when the Type 2 Scanner attempts to set the service request interlock (ICW bit 1) and finds it already set. This error is normally caused by an instantaneous peak overload situation. Errors of this type should not occur in the average installation and should occur only infrequently in high throughput installations.

If a character overrun occurs, the next character received is placed into the PDF field overlaying the character that was to have been serviced. Thus, if an overrun occurs, a character is lost.

In the event of an underrun, the same character is transmitted until the program changes the PDF field to another character or the primary control field is changed from the transmit state.

If this bit is 1, the service request interlock (ICW bit 1) is 0.

Bit 3 — Modem Check: During each bit interval (bit service time), the Type 2 Scanner checks the line interface for the proper modem conditions. This bit is set to 1 to indicate the following conditions:

1. The 'data set ready' line is inactive when the PCF field of the ICW is in states X'5', X'7', X'8', X'9', X'A', X'B' or X'D'.

2. The 'clear to send' line is inactive when the PCF field of the ICW is in states X'9', X'A', X'B', or X'D'.

3. A TTY echo check has been detected.

4. The 'receive line signal detect' line is inactive, the interface is start-stop, ICW bit 7 (pad flag) is on, and the PCF of the ICW is in state X'7' (receive).

If this bit is 1, the service request interlock (ICW bit 1) is 0.

Bit 4 — Received Line Signal Detector: This bit is set when the line interface indicates that the data communication equipment is receiving a carrier signal that

7-6

PRIMARY CONTROL FIELD (PCF)

| HEX | START-STOP | BSC | SDLC | AUTOCALL |
| --- | --- | --- | --- | --- |
| 0 | NO-OP | NO-OP | NO-OP | IDLE |
| 1 | SET MODE | SET MODE | SET MODE | NOT USED |
| 2 | MON DATA SET READY ON | MON DATA SET READY ON | MON DATA SET READY ON | NOT USED |
| 3 | MON RI /DATA SET READY ON | MON RI /DATA SET READY ON | MON RI /DATA SET READY ON | NOT USED |
| 4 | NOT USED | MON PHASE DATA SET READY CHECK OFF | MON FLAG. BLOCK DSR ERR. | MON CALL UNIT ACR COS PND |
| 5 | NOT USED | MON PHASE DATA SET READY CHECK ON | MON FLAG, ALLOW DSR ERR. | MON CALL UNIT ACR COS |
| 6 | NOT USED | NOT USED | RECEIVE INFORMATION, INHIBIT DATA INTERRUPTS | NOT USED |
| 7 | RECEIVE | RECEIVE IN PHASE | RECEIVE INFORMATION, ALLOW DATA INTERRUPTS | NOT USED |
| 8 | TRANSMIT INITIAL | TRANSMIT INITIAL | TRANSMIT INITIAL | DIGIT VALID |
| 9 | TRANSMIT DATA | TRANSMIT DATA | TRANSMIT NORMAL | NOT USED |
| A | TRANSMIT BREAK | TRANSMIT DATA WITH NEW SYNC | TRANSMIT NORMAL WITH NEW SYNC | NOT USED |
| B | PREPARE TO TURN | NOT USED | NOT USED | NOT USED |
| C | TRANSMIT TURNAROUND REQUEST TO SEND OFF | TRANSMIT TURNAROUND REQUEST TO SEND | TRANSMIT TURNAROUND REQUEST TO SEND OFF | NOT USED |
| D | TRANSMIT TURNAROUND REQUEST TO SEND ON | TRANSMIT TURNAROUND REQUEST TO SEND ON | TRANSMIT TURNAROUND REQUEST TO SEND ON | NOT USED |
| E | NOT USED | NOT USED | NOT USED | NOT USED |
| P | DISABLE | DISABLE | DISABLE | DISABLE |

SERIAL DATA  FIELD (SDF)	PARALLEL DATA FIELD (PDF)	LCD

| BIT | MODE SET | AUTOCALL |  |  | AUTOCALL |
| --- | --- | --- | --- | --- | --- |
| 24(0) | NOT USED | INTERRUPT REMEMBER |  | 4 | DIGIT NBR 8 DIGIT NBR 4 DIGIT NBR 2 DIGIT NBR 1 |
| 25(1) | NOT USED | POWER INDICATOR (PWI) |  | 5 | DIGIT NBR 4 |
| 26(2) | NOT USED | CALL REQUEST (CRQ) |  | 6 | DIGIT NBR 2 |
| 27(3) | DIAGNOSTIC WRAP MODE | DATA LINE OCCUPIED (DLO) |  | 7 | DIGIT NBR 1 |
| 28(4) | SET/RESET DATA TERMINAL READY | PRESENT NEXT DIGIT (PND) |  |  |  |
| 29(5) | SYNC BIT CLOCK | DIGIT PRESENT (DPR) |  |  |  |
| 30(61 | EXTERNAL CLOCK | DATA SET STATUS (DSS)/CALL ORIGINATE STATUS (COS) |  |  |  |
| 31(7) | DATA RATE SELECT | ABANDON CALL & RETRY (ACR) |  |  |  |
| 32(8) | OSC SELECT BIT 1 | NOT USED |  |  |  |
| 33(9) | OSC SELECT BIT 2 | NOT USED |  |  |  |
| • ICW BIT 38 SET BY OUTPUT X'43' # ICW BIT 44 SET BY OUTPUT X'46' |  |  |  |  |  |

| 0 | SS 9/6 |
| --- | --- |
| 1 | Not Used |
| 2 | SS 8 / 5 |
| 3 | AUTOCALL |
| 4 | SS 9 / 7 |
| 5 | SS 10 / 7 |
| 6 | SS 10 / 8 |
| 7 | SS 11 / 8 |
| 8 | MONITOR FLAG |
| 9 | SDLC 8-BIT FLAG |
| A | NOT USED |
| B | NOT USED |
| C | BSC EBCDIC |
| D | BSC ASCII |
| E | NOT USED |
| F | FEEDBACK CHECK |

Figure 7-4. Interface Control Word-Type 2 Scanner

meets its requirement for receiving data. The program has no control over this bit.

Bit 5 — This bit is reserved.

Bit 6 — Program Flag: This bit provides a flag in the ICW that can be used by the program.

Bit 7 — Pad Flag: For start-stop transmission, this bit is turned on by the control program when it wants

## IBM 3704 and 3705 Communications Controllers Principles of Operation  7-7

the Type 2 Scanner to hold the output data line at a mark condition for one complete line-transmission character-time. This operation employs the normal transmit character serializing actions except that the start bit is sent as a mark in place of the normal space. The remainder of the character is serialized as normal, and the control program must ensure that the PDF is loaded with X'7F' if the LCD is X'2' or X'FF' if the LCD is not X'2'. Any number of pad characters may be sent by leaving the pad flag on and leaving the PDF set to X'7F' (if LCD is X'2') or X'FF'. When pad transmission is to end, the control program must turn off the pad flag and resume placing normal characters in the PDF. For start-stop receive, this bit is turned on by the control program to cause the Type 2 Scanner to set ICW bit 3 (modem check) when 'receive line signal detect' is inactive. This use of the pad flag provides a higher level of security on switched lines than is attained by monitoring only 'data set ready'. This should only be used on lines with full-duplex facilities.

For autocall and binary synchronous interfaces, this bit is 0.

ICW Bits 8-15 (Parallel Data Field): The parallel data field (PDF) is used as a character buffer. For a transmit operation, the characters to be sent to a terminal are placed in this field by the program with an Output X'44' instruction. Hardware circuits then transfer the character to the serial data field and transmit it to the interface. The format of the character loaded into the PDF depends on the state of the line control definer (LCD).

For receive operations, the character is assembled in the SDF from the line interface and then transferred to the PDF under hardware control. An Input X'44' instruction must then be executed to retrieve the character from this field for program use. The format of the character loaded into the PDF field and how it is used for various line control definer values can be found in the LCD States section in this chapter.

For an autocall interface, the digit number must be placed in the PDF as shown in Figure 7-4.

ICW Bits 16-19 (Line Control Definer): The line control definer (LCD) field defines the type of interface associated with the ICW. The LCD, set under program control by an Output X'45' instruction, must be in agreement with the installed interface type and the common carrier or IBM equipment physically attached to the interface. The LCD field definition must also be in agreement with the basic line control procedures to be executed. An LCD is generally set at IPL time and remains static throughout normal operation. The values of the LCD field are defined in detail

along with its effect on the PDF and SDF fields in the section describing LCD states in this chapter.

The LCD field is also used to indicate that a feedback check has occurred. When the scanner sets the LCD field to X'F', it indicates an improper mode set or a hardware failure in either the Type 2 Scanner or the interface hardware.

## Programming Note

During a diagnostic wrap operation, the LCD of the line or lines in diagnostic receive must agree with the LCD of the line in diagnostic transmit regardless of the common carrier or IBM equipment physically attached to the line.

ICW Bits 20-23 (Primary Control Field): The primary control field (PCF) defines the state of the interface at any particular time. The interpretation of this field depends on the state of the LCD field. Figure 7-4 shows the PCF states for start-stop, BSC, and SDLC line control. See the section on Autocall Interface Operation for a description of the autocall PCF states.

The Output X'45' instruction can be used to set the state of the PCF. Also, an Input X'45' can be executed to test the state of this field. Refer to Appendix B for bit definitions of input/output instructions.

ICW Bits 24-33 (Serial Data Field): The serial data field (SDF) is primarily used as a character deserializer/serializer field. On receive operations, the data coming in from a line is placed in this field bit-by-bit to assemble a character. When a character has been assembled, it is transferred, under hardware control, to the PDF for program access. For transmit operations, a character from the PDF is transferred into the SDF under hardware control and then transferred to the line interface hardware a bit at a time.

The SDF is also used for line interface mode setting and autocall operations. Its format depends on the values in the LCD and PCF fields. See Interface Mode Set and Autocall Interface Operation in this chapter for further definitions.

Program access to this field is through the Input X'45' and Input X'47' instructions and Output X'46' instruction. Refer to Appendix B for bit definitions of input/output instructions.

ICW Bits 34-36 (SDLC Ones Counter): This counter is used for SDLC lines only.

ICW Bit 37 (SDLC Last Line State): This bit is used only by SDLC lines.

ICW Bit 38 (Display Request): This bit allows the state

7-8

of certain latches or signals to be loaded into the Type 2 Scanner display register when the interface associated with this ICW is scanned. The bit may be set and reset by an Output X'43' instruction when the interface address for that ICW is in the attachment buffer address register (ABAR) in the Type 2 Attachment Base.

The contents of the display register in a particular Type 2 Scanner can be accessed by an Input X'46' instruction when that scanner is selected.

Because only one display register is in each Type 2 Scanner, the program should ensure that the display request bit is never on in more than one ICW in each scanner. Only in this way can the information in the display register be meaningful.

Before executing an Input X'46', the program must also ensure that enough time has elapsed to guarantee that the interface has been scanned after setting the display request bit in the ICW associated with the interface.

ICW Bits 39-40: These bits are reserved.

ICW Bit 41 (Level 2 Interrupt Pending): This bit is set when the interrupt priority register (IPR) assigned to this interface is already occupied by another interface. This stacks the new interrupt until the next time the line is scanned and the IPR is not occupied.

ICW Bits 42-43 (Priority Select Bits 1 and 2): These bits assign one of the four interrupt priority registers in the attachment base to the ICW for the interface. All combinations are valid with X'0' designating the lowest priority register and X'3' the highest priority register.

ICW Bit 44 (SDLC NRZI Control): This bit is used only by SDLC lines.

ICW Bit 45: This bit is reserved.

Line Control Definer (LCD) States

This section describes the various LCD (line control definer) states. The LCD field is used during normal transmit and receive operations to define the hardware line control required by the line set type. The variations in the PDF field due to different line set requirements are shown with each LCD state. The first information bit of a transmitted or received character is designated as Xl, the second bit as X2, and the nth and last bit as Xn. For start-stop transmissions, the start and stop bits are not regarded as information bits and are inserted or deleted by the Type 2 Scanner hardware.

LCD State X'0' (Start-Stop 9/6 Bit Control): This state should be set for start-stop transmission with a 9/6 format (that is, one start bit, six data bits, and two stop bits). When a character is sent to an interface, the six data bits must be placed into bits 2-7 of the PDF field as shown below. Characters received from the interface are in the same format when the scanner requests a character service interrupt.

PDF bit positions 0  1  2  3  4  5  6  7

Character bits    0  0  X6 X5 X4 X3 X2 X1

LCD State X1: (Reserved)

LCD State X'2' (Start-Stop 8/5 Bit Control): This state is for start-stop interfaces with an 8/5 format (that is, one start bit, five data bits, and two stop bits). When a character is sent to an interface, the five data bits must be placed into bits 3-7 of the PDF field as shown below. Characters received from the interface are in the same format when the scanner requests a character service interrupt.

PDF bit positions 0  1  2  3  4  5  6  7

Character bits    0  0  0  X5 X4 X3 X2 X1

LCD State X'3' (Autocall): This state is for autocall interfaces. See the Autocall Interface Operation section in this chapter.

LCD State X'4' (Start-Stop 9/7 Bit Control): This state is for start-stop interfaces with a 9/7 format (that is, one start bit, seven data bits, and one stop bit). When a character is sent to an interface, the seven data bits are placed into bits 1-7 of the PDF field as shown below. Characters received from the interface are in the same format when the scanner requests a character service interrupt.

PDF bit positions 0  1  2  3  4  5  6  7

Character bits    0  X7 X6 X5 X4 X3 X2 X1

LCD State X'5' (Start-Stop 10/7 Bit Control): This state is for start-stop interfaces with a 10/7 format (that is, one start bit, seven data bits, and two stop

IBM 3704 and 3705 Communications Controllers Principles of Operation  7-9

bits). When a character is sent to an interface, the seven data bits must be placed into bits 1-7 of the PDF field as shown below. Characters received from the interface are in the same format when the scanner requests a character service interrupt.

PDF bit positions 0  1  2  3  4  5  6  7

Character bits    0  X7 X6 X5 X4 X3 X2 X1

LCD State X'6' (Start-Stop 10/8 Bit Control): This state is for start-stop interfaces with a 10/8 format (that is, one start bit, eight data bits, and one stop bit). When a character is sent to an interface, the eight data bits must be placed into bits 0-7 of the PDF field as shown below. Characters received from the interface are in the same format when the scanner requests a character service interrupt.

PDF bit positions 0  1  2  3  4  5  6  7

Character bits    X8 X7 X6 X5 X4 X3 X2 X1

LCD State X'7' (Start-Stop 11/8 Bit Control): This state is for start-stop interfaces with an 11/8 format (that is, one start bit, eight data bits, and two stop bits). When a character is sent to an interface, the eight data bits must be placed into bits 0-7 of the PDF field as shown below. Characters received from the interface are in the same format when the scanner requests a character service interrupt.

PDF bit positions 0  1  2  3  4  5  6  7

Character bits    X8 X7 X6 X5 X4 X3 X2 X1

LCD States X'8' through X'B' (SDLC Byte Length): These states are for SDLC lines only.

LCD State X'C' (BSC EBCDIC Line Control): This state is for binary synchronous interfaces using the EBCDIC SYN character. When a character is sent to an interface, the data bits must be placed into bits 0-7 of the PDF field as shown below. The SYN character (X'32') provides for automatic detection of the first phase character during a receive operation.

PDF bit positions 0  1  2  3  4  5  6  7

Character bits    X8 X7 X6 X5 X4 X3 X2 X1

LCD State X'D' (BSC USASCII Line Control): This state is for binary synchronous interfaces using the USASCII SYN character. When a character is sent to the interface, the data bits must be placed into bits 0-7 of the PDF field as shown below. The SYN character (X'16') provides for automatic detection of the first phase character during a receive operation.

PDF bit positions 0  1  2  3  4  5  6  7

Character bits    X8 X7 X6 X5 X4 X3 X2 X1

LCD State X'E': (Reserved)

LCD State X'F' (Feedback Check): This state is set by the Type 2 Scanner whenever a feedback check is detected (1) during scan addressing on any of the scanner 'data in' lines from the selected line interface base, or (2) when a bit service reset error is detected on the line from the selected LIB. A set mode to an interface that has been configurated incorrectly also sets this state.

Primary Control Field (PCF) States

This section describes the PCF states for start-stop and BSC line control. See the Autocall Interface Operation section in this chapter for a description of the autocall PCF states. For a description of SDLC line control, see the SDLC section of this chapter.

PCF State X'0' (No-Op): This PCF state causes the Type 2 Scanner to take no action (active or passive) upon subsequent scans. The scanner hardware can request a Type 2 Scanner L2 interrupt and set this PCF state for an interface if it determines that new control information is required from the control program. This PCF state can be set by the control program; however, no interrupts are generated by the interface.

PCF State X'l' (Set Mode): This PCF state causes the scanner to set and reset certain mode latches in the line interface hardware. These latches are specified by the SDF field. When setting this PCF state, the control program must ensure the integrity of the entire ICW. This may be done by first setting the PCF to state X'O' (no-op) so that the ICW will not be modi-

7-10

fied by a possible interrupt. The SDF can then be set to the proper value. Finally, state X'1' (set mode) can be set into the PCF field. Execution of a set mode does not require a bit service request from the addressed interface. However, a bit service request must occur to allow the scanner to request a Type 2 Scanner L2 interrupt to end the set mode operation. The set mode operation ends when the scanner hardware sets the PCF state to X'0' (no-op).

A set mode can be executed to change the state of the data rate selector bit and the oscillator select bits without requiring a disable. However, data terminal ready must remain on.

PCF State X'2' (Monitor Data Set Ready): This PCF state places the interface in a wait-for-incoming-call condition. For switched lines, this state should normally be set by the control program following a PCF state X'F' (disable) and PCF state X'l' (set mode with data terminal ready bit = 1). When an interface is in this state, the Type 2 Scanner tests the 'data set ready' lead from the common carrier or IBM line adapter for an active condition when the ICW is fetched. When data set ready is on, indicating that a call is established, the Type 2 Scanner sets PCF state X'0' (No-Op) for start-stop or PCF state X'4' (monitor phase—data set ready check off) for binary synchronous transmission and requests an L2 interrupt.

Though not necessary, this state can also be used for leased lines. Data set ready should be on at the first bit service request when the interface is scanned.

PCF State X'3' (Monitor Ring Indicator or Data Set Ready): This PCF state, when set by the control program, places the line interface in a wait-for-incoming-call (ring indicator on) or wait-f or-manual-call-out-connection condition (data set ready on). This state must be preceded by setting PCF state X'F' (disable), or a set mode that resets data terminal ready. When the PCP state is set to X'3', the Type 2 Scanner tests the 'ring indicator' and 'data set ready' leads from the common carrier equipment for an active condition of either lead. When 'ring indicator' is active, a call is coming in and a pending connection is to be established. When either of these conditions occurs, the Type 2 Scanner sets PCF state X'O' (no-op) and places the line in a L2 interrupt pending state. This PCF state must be followed by PCF state X'l' (set mode) from the control program to set the 'data terminal ready' latch. After the Type 2 Scanner executes the set mode, it sets PCF state X'O' (no-op) and places the line in a L2 interrupt pending state. The interrupt handling program must then place the line in PCF state X'2' (monitor data set ready on), after which the operation proceeds as described in PCF state X'2'—Monitor Data Set Ready.

PCF State X'4' (Monitor Phase—Data Set Ready Check Off): This PCF state is identical to PCF state X'5' (BSC—monitor phase-data set ready check on) except that the inactive condition of 'data set ready' does not signal a check condition. PCF X'4' is intended to initialize the first receive operation after a switched network call connection has been established.

PCF State X'5' (Monitor Phase—Data Set Ready Check On): This PCF state places a BSC line into a hunt for phase condition. The SDF field is shifted each bit interval time, and the contents are examined by the scanner hardware for a comparison with the bit configuration of the 8-bit SYN character. If a compare is successful, PCF state X'7' is set, and the tag bit is inserted in the SDF. An L2 interrupt request, however, is not generated at this time. The first interrupt request is at the next subsequent character time. When this interrupt request is presented, the control program must examine the character in the PDF to determine if the second SYN character has been received. If so, the PCF is left in state X'7'. If the second SYN has not been received, the program returns the PCF to state X'5'. The Type 2 Scanner also sets PCF state X'5' after completing a transmit turnaround (PCF state X'C' or X'D').

PCF State X'6' (Receive—Inhibit Data Interrupts): This state is used only for SDLC lines.

PCF State X'7' (Receive): Start-Stop: In this PCF state, the Type 2 Scanner monitors for start bits (as described in the Line Control Definer section) according to the setting of LCD X'O', X'2', X'4'-X'7'. This state remains in effect until changed by the control program and is set by the Type 2 Scanner after the completion of a transmit turnaround (PCF state X'C' or X'D').

BSC: In this PCF state, the Type 2 Scanner frames consecutive 8-bit characters (as described in the Line Control Definer section) according to the setting of the LCD X'C' and X'D'. This state remains in effect until changed by the control program and is set by the Type 2 Scanner after one SYN character has been detected in PCF states X'4' or X'5' for BSC.

PCF State X'8' (Transmit Initial): This PCF state is set by the control program. The Type 2 Scanner places the interface hardware into a transmit condition and transmitting begins when the 'clear to send' lead is activated from the carrier equipment. In addition to

IBM 3704 and 3705 Communications Controllers Principles of Operation  7-11

setting this state, the control program must perform the following sequence.

1. Set PCF state X'0' (no-op). This step may have been performed on any previous interrupt.

2. Set the SDF to the first character to be transmitted.

3. Store the second character to be transmitted into the PDF.

4. Set PCF state X'8' (transmit initial).

For start-stop initialization, the first character to be transmitted (set in the SDF) must be X'FF' and the second character (set in the PDF) must be the first information character.

For BSC initialization, when business machine clocking is used, 16 transitions of the 'received data' lead are required to ensure bit synchronization between the transmitting and receiving stations. Therefore, SDF Bits 1-9 must be set to an initial pad of X'355', and the PDF must be set to X'AA'. The next character to be transmitted must be a pad character of X'AA' followed by two SYN characters to enable character synchronization to be established.

BSC specification requires that the first character transmitted be an initial pad character. Therefore, when modem clocking is used, SDF bits 1-9 must be set to X'355'. Since the next two characters must be SYN characters, the PDF must be set to a SYN character. All characters stored into the SDF are assumed to be right-justified.

When the Type 2 Scanner begins transmitting (clear to send on), the hardware changes the PCF control state to X'9' (transmit data).

Note: It may be desirable in certain applications (contention) to test the PCF state in order to determine if a transmit operation should be started. For example, a line may have just set PCF state X'7' (receiving-in-phase), and its subsequent interrupt has not been handled by the control program.

PCF State X'9' (Transmit Data): This PCF state is set by the Type 2 Scanner after completion of PCF state X'8' (transmit initial). Data is transmitted in this state until one of the transmit turnaround states (PCF X'B', X'C' or X'D') is set by the control program. PCF state X'A' (transmit data with new sync) should be used in place of state X'9' for synchronous modem equipment containing a 'new sync' lead.

All control and non-information characters must be supplied by the control program because the scanner does not perform character decoding, encoding, or insertion of any kind during a transmit operation. The scanner detects and signals underruns, but the control program has to take corrective action, (for example, BSC abort sequence).

For BSC transmission on a line with business machine clocking, the first two characters transmitted in the transmit data states (PCF states X'9' and X'A') must be X'AA' followed by two SYN characters. For BSC transmission on a line with modem clocking, the first character transmitted in PCF state 9 (transmit data) must be X'AA' followed by two SYN characters. Note, this may have already been done under PCF state X'8' (transmit initial).

After all information characters (EOB, EOT, ENQ, ACK, check characters, etc.) have been transmitted under the transmit data states (PCF states X'9' or X'A'), the control program must complete the transmit operation by setting one of the transmit turnaround states (PCF state X'B', X'C' or X'D').

PCF State X'A' (Transmit Break—Start-Stop): This state is set by the control program instead of PCF state X'9' (Transmit Data) when transmitting a break signal to the remote destination. The following sequences of PCF states should be set by the control program for transmitting a break signal:

|  | SDF | PDF | Pad Flag |
| --- | --- | --- | --- |
| Transmit Initial | X'FF' | X'FF' | 1 |
| (PCF State X'8') |  |  |  |
| Transmit Break |  | X'00' | 0 |
| Transmit Turnaround |  | X'FF' | 1 |
| (PCF X'B'or X'C') |  |  |  |

After the completion of this sequence, the break signal (continuous spacing) continues for n character times. The stop bits for the spaces (X'00') are inhibited from being transmitted as a mark so that the break signal is continuous spacing.

Note: Two character-time delays exist before the actual break signal is transmitted.

PCF State X'A' (Transmit Data with New Sync): This state is identical to PCF state X'9' (transmit data) except that the 'new sync' line to the modem equipment is active. It must be used only with 4-wire duplex, multipoint leased-line modem equipment where the associated interface is designated as the master station. The control program must change PCF state X'A' to PCF state X'9' (transmit data) in the character service routine that places the last character to be transmitted into the PDF.

PCF State X'B' (Prepare to Turn for Start-Stop): This state is set by the control program on the interrupt following the interrupt that placed the last character (pad or information) to be transmitted into the PDF. While bits are being transmitted, this state is the same as PCF state X'9' (transmit data).

7-12

When the character is completely transmitted, PCF state X'C' (transmit turnaround—request to send off) is set by the scanner. The SDF is set to X'00', and the line interface transmit data buffer is left in the 'mark' state. This action delays completion of the transmit operation to ensure that the stop bit remains on the interface transmit data output at least one bit time before 'request to send' can be turned off. At the next bit interval, if 'clear to send' is off, the line is placed in an interrupt pending condition as the final interrupt of the transmit operation. PCF state X'7' (Receive— Start-Stop) is set by the Type 2 Scanner, and the SDF is left at X'00'. If 'clear to send' is on, the scanner loops, and there is no change in the PCF state or no interrupt generated until 'clear to send' drops.

PCF State X'C' (Transmit Turnaround—Request to Send Off): For BSC line control (LCD=X'C', or X'D') this PCF state is set by the control program on the interrupt following the interrupt that placed the last pad character to be transmitted into the PDF. While bits are being transmitted, this state is the same as state X'9' (transmit data).

When the character is completely transmitted, 'request to send' is reset along with the 'transmit mode' latch in the interface hardware. PCF State X'C' is not changed until 'clear to send' is off. After 'clear to send' is off, PCF state X'5' (monitor phase - data set ready check on) is set, and the line is placed in an interrupt pending state. The control program should make sure 'clear to send' is off before the background time-out elapses. For a description of this state under start-stop line control, see PCF state X'B'.

When the control program wants to close a line that normally transmits with 'request to send' on, it must notify the scanner that 'request to send' is to be turned off by a PCF state X'C'. This must be done by sending a pad message using PCF state X'B' for start-stop, or PCF state X'C' for BSC, instead of PCF state X'D'. The pad message should result in a continuous marking condition on the line (for Start-Stop, see SCF Pad Flag). An alternative is to ensure that final outgoing transmissions use PCF state X'C'.

## Programming Note

Some modems do not turn 'clear to send' off under the above conditions. Therefore, the control program should test this condition and may be required to set the PCF to X'D' and operate with 'request to send' on.

PCF State X'D' (Transmit Turnaround—Request to Send On): This state is set by the control program on the interrupt following the interrupt that placed the last character (pad or information) to be. transmitted into the PDF. While the bits are being serialized in the SDF, this state is the same as PCF state X'9' (transmit data).

When the character is fully serialized, the interface transmit control (not including 'request to send') is reset and the final interrupt request is set for the transmit operation. The PCF is set by the Type 2 Scanner to PCF state X'5' (monitor phase - data set ready check on) for BSC or to PCF state X'7' (receive) for start-stop.

The significance of 'request to send' on (PCF state X'D') is:

Start-Stop: 'Request to send' on is to be used with all common carrier equipment that provides duplex facilities and for IBM line adapter/modem equipment on duplex communication facilities.

BSC: 'Request to send' on is to be used on point-to-point 4-wire duplex and multipoint 4-wire duplex communication facilities where the controller serves as the master station. All BSC switched network communication facilities are half-duplex.

BSC and Start-Stop Local Attachments: Equivalent to 4-wire point-to-point communication facility.

PCF State X'F' (Disable): This state is set by the control program and causes the Type 2 Scanner to turn off data terminal ready. A disable resets all control information in the line that was provided by the last set mode (PCF state X'l'). The scanner hardware then causes the interface to be placed in an interrupt pending state when the 'data set ready' lead and the 'receive line signal detector' lead are deactivated. For auto-dial applications, other conditions on the automatic calling unit must be satisfied before another dial operation can be attempted. Before the interrupt is requested, PCF state X'O' (no-op) is set by the scanner. Because all control information in the line set is reset, the control program must set up the proper control information again by a set mode (PCF state X'l') in the interrupt after the disable.

Interface Mode Set-SDF Values

The interface hardware latches are set and reset according to the value in the SDF field during PCF state X'l'. The following paragraphs define the SDF values for a set mode.

SDF bits 0-2: ICW Bits 24-26 - Not Used

IBM 3704 and 3705 Communications Controllers Principles of Operation  7-13

SDF bit 3: This bit is set to 1 to place the addressed interface in the diagnostic mode.

SDF bit 4: Data Terminal Ready - This bit controls the 'data terminal ready' lead, which must be set to enable the line interfaces provided by all line sets except those used for autocall operation.

SDF bit 5: Synchronous Bit Clock - This bit determines whether synchronous or start-stop clocking is used for the addressed interface when business machine clocking is specified. If SDF bit 6 is 1 (external clock), this bit is ignored.

A feedback check occurs if this bit is on when the program executes a set mode for interfaces provided by all line sets that support start-stop lines only.

SDF bit 6: External Clock - This bit determines whether business machine or modem clocking is used for the addressed interface. A 1 = modem clock, and 0 = business machine clock.

A feedback check occurs if this bit is on when the program executes a set mode for interfaces provided by line sets that allow business machine clock control only.

SDF bit 7: Data Rate Selector - This bit selects a high speed or low speed data rate for the attached modem. A 1 = high data rate, and 0 = low data rate. If modem clocking is specified, this bit selects which of the two clock speeds in the modem is to provide the clock pulses. The low rate usually equals one-half of the high rate. In this case the business machine clock selected by the oscillator select bits must not exceed one-half the clock speed selected in the modem.

A feedback check occurs if this bit is on when the program executes a set mode to a line interface provided by line sets that allow only one data rate.

SDF bits 8 and 9: OSC Select Bits 1 and 2 - The state of these two bits selects the business machine clock to be used by the addressed line interface. At least one business machine clock must be installed in each Type 2 Scanner. See the following section, Business Machine Clocks.

## Programming Note

The oscillator select bits can be changed without causing a switched network connection to be broken if SDF Bit 4 (Data Terminal Ready) is set when the set mode is executed.

Business Machine Clocks

Each Type 2 Scanner must have at least one business machine clock installed and may have as many as four. If modem clocking is used with any of the lines, a business machine clock must be installed with a speed less than one-half that of the lowest speed modem clock. Figure 6-4 lists the business machine clocks available.

For line speeds above 2400 bps, a modem must provide clock pulses. Some line sets can operate with either a business machine clock or a modem clock, and some can operate only with a business machine clock. Refer to the 3 704 and 3 705 Introduction manual for a description of the individual LIB and line set types.

The installed business machine clock used for a given line is selected under program control by executing a set mode (PCF state X'l') with SDF bits 8 and 9 set to indicate the desired clock. Figure 7-5 shows the proper setting of the oscillator select bits to assign an installed oscillator to a given interface.

| SDF 8 | Bits 9 | Selected Business Machine Clock |  |
| --- | --- | --- | --- |
| 0 | 0 | Lowest speed clock | (OSC0) |
| 0 | 1 | Next higher speed clock | (OSC1) |
| 1 | 0 | Next higher speed clock | (OSC2) |
| 1 | 1 | Highest speed clock | (OSC3) |

Figure 7-5. Type 2 Communication Scanner Business Machine Clock Selection.

No business machine clock is selected if the oscillator select bits are set to select an uninstalled oscillator (for example, bits 8 and 9 set to 11 when only two or three oscillators are installed).

Every interface must have a business machine clock assigned whether it is specified to be business machine or modem clocked. For autocall interfaces and for line interfaces that are to use modem clocking, the assigned business machine clock is used to ensure that the interface is periodically accessed. The lowest speed oscillator must always be used for an autocall interface.

The oscillator select bits are set to 0 by a reset to the scanner. Therefore, the lowest speed clock is initially selected, and unless a set mode is executed to select another clock for a given interface, the lowest speed clock is used.

After a power-on-reset occurs, there is a warm-up period associated with the different clocks. (Refer to Figure 6-4.) During this warm-up period, a business machine clock cannot provide service requests.

7-14

Programming Notes

1. The oscillator select bits for a line interface can be changed without causing a switched network connection to be broken, if 'data terminal ready' is up when the set mode is executed.

2. The business machine clock selected for a modem-clocked line interface must be less than one-half the rate of the modem clock.

I/O Programming Considerations

As a general rule, input/output instructions should be issued only when the status of the attachment buffer address register (ABAR) and the particular Type 2 Scanner ICW input register is known. An understanding of how those registers are set or loaded is needed for correct execution.

The ABAR is set under the following conditions:

1. The interface address in the highest priority program level 2 interrupt register located in the attachment base is loaded into ABAR just before program level 2 becomes the current program level.

Therefore, if an Input X'40' is executed as the first instruction in program level 2, the register specified by the R operand contains the interface address for that interrupt.

2. When the program executes an Output X'40', the interface address in the register specified by the R operand is placed in ABAR.

The ICW input register of the selected Type 2 Scanner is loaded with the contents of the ICW associated with the interface address in the ABAR when:

1. The ABAR is loaded after a program level 2 interrupt occurs.

2. The Output X'40' instruction is executed in program level 3 or 4. This enables the level 3 or 4 routines to access any portion of the selected ICW associated with the interface address in the ABAR.

Figure 7-6 summarizes which program levels can set the ABAR in the attachment base and set the ICW input register in the selected scanner.

| Program Level | ABAR | ICW Input Register |
| --- | --- | --- |
| 1 | Output X'40' | Cannot be set |
| 2 | L2 Interrupts | L2 Interrupt |
| 3 or 4 | Output X'40' | Output X'40' |

Figure 7-6. Setting ABAR and ICW Input Register

The following considerations are recommended for executing input/output instructions in the different program levels.

Program Level 1 (Error Routines)

Input X'40' can be executed to obtain the interface address from the attachment buffer address register (ABAR) in the Type 2 Attachment Base. This old interface address should be saved if a different address is required to select the Type 2 Scanner that has its LI interrupt request set.

Output X'40' can be executed to select the appropriate Type 2 Scanner if needed. Only the selected Type 2 Scanner can decode the input/output instructions. However, the scanner input register is not changed if an Output X'40' is executed at program level 1 or 2.

After the Type 2 Scanner is selected, other input and output instructions may be executed as needed. Output instructions may be executed in any order, but all output instructions (Outputs X'42', X'43', X'44', X'45', X'46', and X'47') that set a portion of the ICW must be separated by at least one cycle. This is required because the output register in the Type 2 Scanner buffers the data from the general register and requires time to store the data in the ICW.

Before exiting from program level 1, the program may execute an Output X'40' to place the old interface address back in ABAR if it had been saved. However, one instruction cycle must separate it from any Output X'42'-X'47'. The selected Type 2 Scanner ICW input register is not changed as a result of Output X'40'.

At least one instruction cycle must separate the last output instruction and an Exit instruction.

Program Level 2 (Character Service)

Input X'40' may be executed immediately to obtain the interface address. When Input X'40' is issued while in program level 2, the 'priority register occupied' latch associated with the interface address in ABAR is reset. This indicates that the character service request is being serviced and that the program level 2 interrupt priority register from which the ABAR was loaded is now available for another level 2 interrupt of the same priority. If all the 'priority register occupied' latches are reset, the Type 2 Scanner L2 interrupt request is reset.

Inputs X'44', X'45', or X'47' may be executed whenever necessary to obtain a portion of the ICW from the Type 2 Scanner ICW input register. Outputs X'43', X'44', X'45', X'46', or X'47' may be executed whenever necessary to set a portion of the ICW.

Output instructions may be executed in any order, but all subsequent Output X'43', X'44', X'45', X'46', or X'47' instructions must be separated by at least one cycle.

IBM 3704 and 3705 Communications Controllers Principles of Operation  7-15

At least one instruction cycle must separate the last output instruction and an Exit instruction.

## Programming Note

Subsequent Input X'40' instructions within the same character service interrupt do not reset the 'priority register occupied' latches.

Program Levels 3 and 4 (Lower Level Routines) Output X'7E' may be executed with a 1 in byte 1, bit 2 of the register specified by the R operand. This will 'mask off program level 2 interrupts that could change the contents of the attachment buffer address register (ABAR) in the Type 2 Attachment Base by a character service L2 interrupt.

Output X'40' may be executed to load ABAR with the interface address of a line to be acted upon. The contents of the ICW associated with this interface address are placed in that Type 2 Scanner ICW input register.

After the Type 2 Scanner is selected, (1) Output X'43', X'44', X'45', X'46', or X'47' may be executed (to alter the associated portion of the ICW), followed by some other instruction, or (2) some other instruction must be executed, followed by Input X'44', X'45', X'46', or X'47' (to obtain the associated portion of the ICW that was loaded by the Output X'40' into the ICW input register).

If Output X'43', X'44', X'45', X'46', or X'47' was executed as in (1) above, the ICW content was altered, but the ICW input register still contains the contents of the ICW as it was before the alteration.

Output instructions may be executed in any order, but all subsequent Output X'43', X'44', X'45', X'46', or X'47' instructions must be separated by at least one cycle.

It is recommended that all lines in the addressed Type 2 Scanner be disabled before executing an Output X'42' to change the upper scan limit.

Output X'7F' may be executed with a 1 in byte 1, bit 2 of the register specified by the R operand. This unmasks the program level 2 interrupts. This output instruction must be separated by at least one instruction cycle from the last Output X'43', X'44', X'45', X'46', or X'47' instruction.

Autocall Interface Operation

The Type 2 Communication Scanner supports operation of an autocall interface when the interface is attached to Line Set IE and the line control definer (LCD) field 6f the associated ICW is set to X'3'. The primary control field (PCF) of the ICW is used to control the interface operation. Whenever an autocall

interface that has a service request is scanned, the scanner interprets the PCF to determine what communication should occur between the scanner and the interface hardware. The scanner also determines, from the PCF, whether the interface should be placed in a level 2 interrupt pending state. The lowest speed business machine clock installed in each scanner is used to generate service requests for all autocall interfaces installed in that scanner. Service requests are generated at the same rate as the clock speed.

Interface Control Word for Autocall Operation

The following bits/fields are used for autocall operation.

ICW bit 1	Service Request

ICW bits 12-15	PDF bits 4-7

ICW bits 16-19	LCD

ICW bits 20-23	PCF

ICW bits 24-31	SDF bits 0-7

ICW bit 38	Display Request

ICW bit 41	L2 Interrupt Pending

ICW bits 42-43	Service Priority

## Programming Note

A reset to the scanner sets the PCF state of each ICW to X'0' and resets ICW bits 34-38 and 41 when the interface is scanned. However, when power is turned on in the controller, all other ICW bits are unpredictable. Therefore the program must ensure that during initialization ICW bits 0-3 are reset.

Primary Control Field for Autocall Operation

Five primary control field (PCF) states are available for autocall operation. Setting the PCF to an undefined value may result in improper operation. Each time the control program changes the PCF state, it should also set the LCD to X'3'. Figure 7-4 shows the PCF states for this LCD value. The following PCF states are valid for autocall interfaces.

PCF State X'0': Idle - This state resets the call request and digit present indications in the autocall interface each time the interface is scanned and a bit service is present. If the control program sets this state, no interrupt requests result from that interface until the program changes the PCF to one of the other valid states. If the scanner sets this state as a result of ending a PCF state XT' (disable), the interface is placed in a level 2 interrupt pending state.

PCF State X'4': Monitor Call ACR, COS, PND -When the Type 2 Scanner fetches an ICW for an autocall interface in this PCF state, the autocall interface is monitored for the active state of the following leads:

7-16

ACR—Abandon Call and Retry

COS—Call Originate Status

PND—Present Next Digit

When any of these leads are found to be active, the appropriate SDF bit is set, and the interface is placed in a level 2 interrupt request pending state.

PCF State X'5': Monitor Call ACR, COS - This state is the same as PCF state X'4' except the active condition of present next digit (PND) does not generate a level 2 interrupt request.

This PCF state can only be set by the control program.

## Programming Note

The control program must ensure that the interrupt remember bit (SDF bit 0) is reset when it places the interface in this state. Otherwise no interrupt request can be generated because of active control leads.

PCF State X'8': Digit Valid - This state is set by the control routine after it has placed the next dial digit into the PDF. This digit is continuously presented to the ACU interface until PND falls; the scanner sets the PCF to X'4', and no interrupt is requested.

PCF State X'F': Disable - This state is used to reset the dial interface at the end of the data transfer operation. After all the control leads from the Autocall unit (ACU) have been reset, the Type 2 Scanner sets the PCF to X'O' and places the interface in a level 2 interrupt request pending state.

Serial Data Field for Autocall Operation

The control program can monitor the autocall interface by interrogating the serial data field in the interface control word. Figure 7-4 shows the SDF for ACU interfaces. The serial data field is updated with the current status of the autocall interface each time the interface is scanned and a bit service request is present. SDF bits 1-9 reflect the state of certain autocall control signals and have no effect on the interface operation. Bit 0 is the only SDF bit that affects operation. The following paragraphs describe each SDF bit and its meaning.

SDF Bit 0: Interrupt Remember (IR) - This bit is set by the scanner to indicate a level 2 interrupt pending state. In PCF state X'4' or X'5', the scanner monitors the respective autocall interface leads for an active condition. When one of the monitored leads becomes active, the scanner sets the interrupt remember bit to prevent further interrupts from that interface until the first interrupt has been serviced. This bit must be reset by the control program each time an interrupt is

serviced in order to allow the next interrupt to be recognized.

## Programming Note

The program should not reset the interrupt remember bit before changing the PCF state from X'4' or X'5' to some other state because an unexpected interrupt request may result.

SDF Bit 1: Power Indicator (PWI) - When this bit is 0, the automatic calling equipment is inoperative because of the lack of power.

SDF Bit 2: Call Request (CRQ) - A 1 in this position indicates a request to originate a call to the autocall interface. The scanner sets the CRQ in the autocall interface whenever (1) the interface is scanned, (2) a bit service request is present, and (3) the PCF State is X'4', X'5', or X'8'. If the condition of this bit does not agree with the state defined for the active PCF state, the LCD is set to X'F' to indicate a feedback check.

SDF Bit 3: Data Line Occupied (DLO) - A 1 in this position indicates that the autocall interface is in use. The program should not attempt to originate a call until this lead becomes inactive.

SDF Bit 4: Present Next Digit (PND) - A 1 in this position indicates that the autocall unit is ready to accept the next digit. The PND lead is used by the autocall unit to control the presentation of digits to the unit during a dialing operation.

When the 'present next digit' lead is active and the PCF state is X'4', the scanner sets the interrupt remember bit if it is not already on. If the PND lead is inactive and the PCF state is X'8', the scanner changes the PCF state to X'4'.

SDF Bit 5: Digit Present (DPR) - A 1 in this position indicates that a valid digit is present on the digit leads to the autocall unit. The scanner sets DPR via PCF state X'8' after present next digit comes on and the next dial digit has been placed into the parallel data field, bits 4-7. When the autocall unit turns PND off, the scanner changes the PCF state to X'4' and resets DPR. The scanner resets DPR when the PCF state is changed to X'O', X'4', X'5', or X'F'. If the condition of this bit does not agree with the state defined for the active PCF state, the LCD field is set to X'F' to indicate a feedback check.

SDF Bit 6: Data Set Status (DSS)/Call Originate Status (COS) - A 1 in this position indicates that a connection has been established and that the modem is

IBM 3704 and 3705 Communications Controllers Principles of Operation  7-17

in data mode and can be used for data communications. In PCF state X'4', the scanner sets the interrupt remember bit when the DSS/COS lead becomes active.

SDF Bit 7: Abandon Call and Retry (ACR) - A 1 in this position indicates that a preset time interval in the autocall unit has elapsed since the last change of the present next digit lead. This bit only alerts the control program to the time-out condition; it does not automatically abandon the call and retry. The control program is responsible for abandoning the call and retrying. In PCF state X'4', the scanner sets the interrupt remember bit when the ACR lead becomes active.

SDF Bits 8 and 9: These bits are not used.

Parallel Data Field for Autocall Operation

For autocall operation the parallel data field is used to present the dial digits to the automatic calling unit. When the autocall interface is in PCF state X'8', the parallel data field bits 4-7 must contain a valid digit. In any PCF state other than X'8', the PDF is treated as if it contained all zeros. The valid digits that can be loaded into the PDF are from X'0' to X'9' and X'C' and X'D'. X'O' to X'9' represent the value of the dial digit and X'C' is an end of number bit configuration used to inform the autocall unit that the last digit of the called number has been provided. X'D' is a separator bit configuration used to inform the auto-calling unit to wait for a second dial tone.

Note: The external automatic calling unit (ACU) must have the appropriate features to use X'C' and X'D'.

Input/Output Instructions

The Type 2 Scanner input/output instructions enable the program to communicate between line interface bases (LIBs), program interrupt levels, interface control words (ICWs), and Type 2 Scanner registers. Some of the major functions of the I/O instructions are to:

• Determine the interface address that caused a program level 2 interrupt.

• Determine the cause of a program level 1 interrupt once the scanner causing the interrupt has been identified (Input X'76' has been executed).

• Determine the status of a particular ICW.

• Determine the status of a Type 2 Scanner display register.

• Set the attachment buffer address register (ABAR) with the interface address required for:

a) addressing a particular ICW in program levels 3 or 4.

b) restoring an old ABAR address that had been saved while in the error routines of a program level 1 interrupt. This allows the program to resume normal operation with the same interface address in the ABAR as when the program level 1 interrupt occurred.

c) addressing a particular scanner in program level 1,3, or 4.

• Set and reset bits in a particular ICW.

• Set scan limits in a particular scanner while setting the substitution control register (SCR) in the Type 2 Attachment Base.

• Set control bits in a particular scanner.

## Programming Note

Input/Output instructions are privileged instructions executable only at program level 1, 2, 3 or 4. Any attempt to execute these instructions improperly causes a program level 1 interrupt request by setting the 'input/output check L1' latch. Refer to Input/Output Check description in Chapter 5 for the conditions that cause the check to be set.

Input Instructions

Six input instructions allow the program to obtain the status of the ICW input register, display register, and error register in the Type 2 Scanner, and the interface address in the attachment base ABAR. (Appendix B defines the bits within each input instruction.)

## Programming Notes

1. With Extended Addressing, byte X of all input instructions and external registers is set to zero.

2. When an autocall interface is used, some of the input instructions have different bit definitions. Refer to the individual instruction descriptions for these differences.

Input X'40' (Interface Address): This instruction is used to obtain the line interface address from the ABAR in the attachment base. Conditions that set the ABAR are described in the I/O Programming Considerations section in this chapter.

If Input X'40' is issued during program level 2, the 'priority register occupied' latch associated with the interface address in the ABAR is reset. This indicates that the character service request is being serviced and that the program level 2 interrupt priority register from which the ABAR was loaded is now available for another level 2 interrupt of the same priority. If all the 'priority register occupied' latches are reset, the Type 2 Scanner L2 interrupt request is reset.

7-18

## Programming Note

Subsequent Input X'40' instructions within the same character service interrupt do not reset the 'priority register occupied' latches.

Input X'43' (Check Register): This instruction may be used to obtain the status of the check register of a scanner. Since it is possible for four Type 2 scanners to be installed in the controller, the check register selected is determined by the interface address in the ABAR at time of instruction execution.

Programming Note

If any of the check register bits in the Type 2 Scanner are set to 1, the Type 2 Scanner LI interrupt request is set.

Input X'44' (ICW Input Register - Bits 0-15): This instruction may be used to determine the state of the secondary control field (SCF) and the parallel data field (PDF) in the ICW that is set in the ICW input register. The interface address in the ABAR selects the proper scanner. Refer to I/O Programming Considerations for conditions that set the ICW input register. The SCF and PDF fields and bit definitions are described in the Interface Control Word Format section of this chapter. Synchronous Data Link Control (SDLC) uses certain bits of the SCF differently.

Input X'45' (ICW Input Register - Bits 16-31): This instruction may be used to determine the state of the LCD and PCF fields and SDF bits 0-7 in the ICW that is set in the ICW input register. The interface address in the ABAR selects the proper scanner. Refer to I/O Programming Considerations in this chapter for conditions that set the ICW input register. The LCD, PCF, and SDF fields and bit definitions are described in the Interface Control Word Format section of this chapter. The LCD and PCF fields are used differently for SDLC lines.

Input X'46' (Display Register): This instruction may be used to determine the state of the display register in the Type 2 Scanner selected by the interface address in ABAR.

The program, under control of the display request (ICW bit 38), can cause status information for a particular interface to be placed into the Type 2 Scanner display register when the interface is scanned. Input X'46' can then be used to examine this status information.

Input X'47' (ICW Input Register - Bits 32-45): This instruction may be used to determine the state of SDF bits 8-9, display request bit, L2 interrupt pending bit,

and priority bits 1-2. The ones counter, the last line state, and the NRZI control bit, all of which are used by SDLC, can also be examined by using this instruction. The interface address in the ABAR selects the proper scanner and associated ICW. See I/O Programming Considerations in this chapter for conditions that set the ICW input register. For an interpretation of these bits, see the Interface Control Word Format and the SDLC sections of this chapter.

Output Instructions

Seven output instructions allow the program to set the status of the ICW and to set the upper scan limit and certain other controls in the Type 2 Scanner. The interface address in the ABAR and the substitution control register in the Type 2 Attachment Base may also be set. (Appendix B defines the bits within each output instruction.)

## Programming Note

With Extended Addressing, byte X of all output instructions and external registers has no significance and can be ignored.

Output X'40' (Interface Address): This instruction may be used to set an interface address in the attachment buffer address register (ABAR) of the Type 2 Attachment Base. When this instruction is executed, byte 0, bit 6 through byte 1, bit 6 in the register specified by the R operand are placed in the ABAR.

The interface address placed in ABAR selects the Type 2 Scanner and the ICW associated with that address. Each CCU clock time, the 46 bits of the ICW are placed in the ICW work register. If Output X'40' is executed in program level 3 or 4 the contents of the ICW work register are placed in the ICW input register where it is available for Inputs X'44', X'45', and X'47'.

Output X'41' (Scan Substitution Control): This instruction must be used to set the substitution control register in the Type 2 Attachment Base. See Address Substitution in this chapter for the description and coding of the SCR bits.

Output X'42' (Upper Scan Limit Control): This instruction must be used to set the upper scan limit in the selected Type 2 Scanner. At least one Output X'42' must be executed for each Type 2 Scanner available. The Scanner selected is determined by the interface address in the attachment buffer address register (ABAR) of the attachment base at the time of execution.

IBM 3704 and 3705 Communications Controllers Principles of Operation  7-19

Output X'43' (Control): This instruction may be executed to set or reset various control functions in a Type 2 Scanner. The Type 2 Scanner is selected by the interface address in the attachment buffer address register (ABAR) of the attachment base.

Output X'44' (ICW Bits 0-3 and 5-15): This instruction may be used to reset secondary control field (SDF) bits 0-3 and 5 (5 is for SDLC use only), and to set or reset bits 6-7 of the SCF. It is also used to set or reset the parallel data field (PDF). For a detailed description of these bits, see the Interface Control Word Format section in this chapter and, for Synchronous Data Link Control (SDLC), refer to the SDLC section of this chapter. The PDF field is used as a character buffer. The interface address in the attachment buffer address register (ABAR), located in the Type 2 Attachment Base, selects the Type 2 Scanner and the ICW associated with this address. Refer to the secondary control field of the ICW for an interpretation of the SCF bits (byte 0, bits 0-7). See Interface Control Word Format for the PDF format as it relates to various line control definer states, and for SDLC variations of the PDF format, refer to the SDLC section of this chapter.

Output X'45' (ICW Bits 16-23): This instruction may be used to set the bits of the line control definer (LCD) and the primary control field (PCF) in the ICW. The interface address in the ABAR at execution time selects the proper scanner and the associated ICW. For a detailed interpretation of these bits, see the Interface Control Word Format section in this chapter. See the SDLC section of this chapter for a detailed description of these bits when they are used to define SDLC lines.

Output X'46' (ICW Bits 24-33 and 44): This instruction is used to set the bits (24-33) of the serial data field (SDF) and bit 44, NRZI Control (SDLC only). The interface address in the ABAR at execution time selects the proper scanner and the associated ICW. For a detailed interpretation of the SDF bits, see the Interface Control Word Format section in this chapter. For a description of bit 44, NRZI Control, refer to the SDLC section of this chapter.

Output X'47' (ICW Bits 34-43): This instruction is used to set the state of ICW bits 34-43. The interface address in the ABAR at execution time selects the proper scanner and the associated ICW. For a detailed interpretation of these bits, see the Interface Control Word Format and the SDLC sections of this chapter.

SDLC

The following paragraphs describe the Type 2 Scanner as it applies to Synchronous Data Link Control (SDLC). The differences between SDLC lines and other line types center upon the use of the Interface Control Word (ICW). Some ICW bits are used exclusively for SDLC and some bits, which are common to other line types, are used differently for SDLC. The control program must recognize the line as an SDLC line and interpret the ICW bits accordingly. For a description of the SDLC line discipline, refer to the Synchronous Data Link Control General Information manual.

ICW Bits 0-7 (Secondary Control Field): The secondary control field (SCF) is used as a sense, status, and operation modifier field between the control program and the Type 2 Scanner. Bits 0-5 are set by the Type 2 scanner hardware according to conditions received from the line. Bits 6 and 7 are program controlled.

This field may be examined by using the Input X'44' instruction. An Output X'44' instruction is used to reset bits 0-3 and 5 and to set and reset bits 6 and 7.

Bit 0—Abort: The Type 2 Scanner sets this bit to 1 when it detects seven consecutive 1 bits in the received data stream while the PCF state is X'6' or X'7'. If this bit is 1, the service request interlock (ICW bit 1) is forced to 0. Bit 0 must be reset by the control program using an Output X'44' instruction.

This bit has no significance during a transmit operation.

Note: A transmitted abort sequence consists of eight contiguous 1 bits. However, if the scanner detects seven contiguous I bits while receiving data, it will set the abort bit (ICW bit 0) to 1.

Bit 1—Service Request Interlock: The scanner sets this bit to 1 as described in the ICW Format section of this chapter, except that the scanner is prevented from setting this bit if an SDLC flag or SDLC abort is detected. This bit is reset as previously described in this chapter and also by detection of SDLC abort.

Bit 2—Character Overrun/Underrun: This bit operates as previously described and has one additional function. If the PCF state is X'7' and the flag is detected at other than the predicted position, this bit is set to one.

Bit 3—Modem Check: This bit is the same for all ICWs. See ICW Format in this chapter.

7-20

Bit 4—Receive Line Signal Detector: This bit is the same for all ICWs. See ICW Format in this chapter.

Bit 5—Flag Detection/Disable Zero Insert Remembrance: During a receive operation (LCD X'8' or X'9'), this bit is set to 1 when the scanner detects a flag character in the received data. An interrupt request is not set because ICW bit 5 is set to 1, but an interrupt request may be set by a change of the PCF state due to detection of the flag. This bit must be reset to 0 by the control program when the scanner is in a receive operation.

During a transmit operation, this bit is set to 1 as a character is being transferred from the PDF to the SDF, provided ICW bit 7 is set to 1. ICW bit 5 set to 1 prevents insertion of a 0 after five contiguous 1 bits are transmitted. If ICW bit 7 is 0, ICW bit 5 is set to 0 as a character is transferred from the PDF to the SDF. With ICW bit 5 set to 0, a 0 bit is inserted into the data stream after five contiguous 1 bits are transmitted.

Bit 6—Program Flag: This bit provides a flag in the ICW that can be used by the program.

Bit 7—Disable Zero Insert Control: For a transmit operation, this bit should be turned on by the control program at the same time a flag or an abort character is set into the PDF. When this bit is 0, the scanner inserts a 0 bit into a transmitted character after five contiguous 1 bits are sent. When this bit is 1, the insertion of a 0 bit is blocked after the five 1 bits, and the sixth bit transmitted is not changed.

As a character is transferred from the PDF to the SDF for transmission, the state of ICW bit 7 is transferred to ICW bit 5. This bit has no significance during a receive operation.

ICW Bits 8-33 (Parallel Data Field): These bits form the parallel data field (8-15), the line control definer (16-19), the primary control field (20-23), and the serial data field (24-33). The PDF and the SDF provide the same functions as described in the ICW Format section of this chapter. The changes in the states of the LCD and the PCF for support of Synchronous Data Link Control are described in the following sections.

ICW Bits 34-36 (Ones Counter): These three bits are used as an SDLC ones counter. This counter is stepped and reset by the Type 2 Scanner hardware. During a receive operation, this counter is used to detect:

• Inserted 0's that are to be deleted from the bit stream while in PCF states X'6' and X'7'.

• Flag sequences (X'7E') while in PCF states X'5', X'6', and X'7'.

• Abort sequences (seven consecutive 1 bits) while in PCF states X'6'and X'7'.

During a transmit operation, this counter is used to insert a 0 bit after each five consecutive 1 bits transmitted when ICW bit 5 is set to 0. This applies to PCF states X'8', X'9', X'A', X'C', and X'D' only.

ICW Bit 37 (Last Line State): This bit retains the state of the last bit transacted between the scanner and the LIB during an SDLC operation.

ICW Bits 38-43: These bits have the same definition for SDLC as for other line disciplines.

ICW Bit 44 (NRZI Control): When ICW bit 44 is on and the PCF state is X'9', X'C', or X'D', the data that is transmitted is in NRZI mode (non-return-to-zero-inverted). When this bit is off the data is transmitted in a normal mode (0 and 1 bits). In NRZI mode the line level is complemented when a 0 is transmitted and unchanged when a 1 bit is transmitted. NRZI mode and zero insertion (ICW bit 7) after five consecutive 1 bits ensure a line transition every six bits regardless of the data pattern. NRZI is used to ensure bit synchronization of modems that do not provide received data timing.

Line Control Definer (LCD) States for SDLC

This section describes the LCD (line control definer) states X'8' and X'9' that apply to SDLC lines. These states are added to the states described previously in this chapter, and unlike the other LCD states, the LCD for an SDLC line can be changed by the Type 2 Scanner. The control program sets the desired LCD state by executing an Output X'45' instruction.

When a flag character is detected in the receive data stream and the LCD state is X'8', the Type 2 Scanner hardware resets the existing LCD state and sets an LCD X'9' to receive that character. This is the only case in which the scanner sets the LCD. Returning the state to the proper setting for the next character is the responsibility of the control program.

The variations in the PDF field due to the different LCD states are shown with each state.

LCD State X'8' (SDLC Monitor Flag): This LCD state along with PCF state X'5' is set by the control program to monitor the received information for an SDLC flag sequence. For a description of what happens when a flag is detected, see PCF State X'5' in this section of Chapter 7.

LCD State X'9' (SDLC 8 Bit Byte): This LCD state is

IBM 3704 and 3705 Communications Controllers Principles of Operation  7-21

used for transferring SDLC 8-bit characters. This state must be set by the control program for transmitting on SDLC lines. When a character to be transmitted is sent to the PDF, the eight data bits must be placed into bits 0-7 of the PDF as shown below.

PDF bit positions 0  1  2  3  4  5  6  7

Character bits    Xl X2 X3 X4 X5 X6 X7 X8

Characters received from the interface are in the same format when the scanner requests a character service interrupt. All address, control, and flag characters are 8-bit bytes.

This field is set by scanner hardware when a flag is detected in the received information while LCD state X'8' is set.

This is the only LCD state that can be set by the scanner. It is set by the scanner when a flag is received while LCD state X'8' is set.

Primary Control Field for (PCF) States SDLC This section describes the PCF states for support of SDLC lines in the Type 2 Communication Scanner.

PCF  states X'0', X'1', X'2', X'3', X'A', and XT' are the same as for BSC and start-stop lines, as given elsewhere in this chapter.

PCF State X'4' (Monitor Flag—Block Data Set Ready Error): This PCF state is identical to PCF state X'5' (monitor flag—allow data set ready error) except that the inactive condition of 'data set ready' does not signal a check condition.

PCF State X'5' (Monitor Flag—Allow Data Set Ready Error): This PCF state is used in conjunction with LCD state X'8' to monitor for an SDLC flag after a half-duplex turnaround or after an inactive period on the communication channel. Each bit interval time, the SDF is shifted one bit and the counter located in ICW bits 34-36 is updated. The counter is used to detect the flag character. When a flag character is detected, the following actions are taken by the scanner:

• The contents of the SDF are zeroed out and a tag bit is inserted in the ICW. . The flag detection bit (ICW bit 5) is set on in the SCF.

• The PCF state is set to X'6'.

• The LCD state is set to X'9'.

• A level 2 interrupt is requested.

The flag character is not transferred to the PDF and the service request bit (ICW bit 1) is not turned on.

PCF State X'6' (Receive Information—Inhibit Interrupts): This state is entered when a flag is detected while in state X'4', X'5', or X'7'. During this state the scanner monitors the receive data stream. Inserted zeros are deleted and 8-bit characters are assembled. If contiguous flags are received:

• Flag detection (ICW bit 5) is set in the SCF every character time.

• Transfer from SDF to PDF is inhibited.

• Service request (ICW bit 1) is not set.

• No interrupt is requested.

When a non-flag character is assembled:

• The PCF state is set to X'7'.

• The character is transferred from SDF to PDF.

• Service request (ICW bit 1) is set to initiate data transfer between the control program and the PDF.

PCF State X'7' (Receive Information—Allow Data Interrupts): This state can be entered from PCF state X'6' when a non-flag character is detected. This state is used in conjunction with LCD state X'9' to assemble consecutive bits into 8-bit SDLC characters. The scanner remains in this state until a flag sequence is detected, or until the state is changed by the control program. When a flag sequence is detected, the scanner changes to PCF state X'6' and requests a level 2 interrupt.

PCF State X'8' (Transmit Initial): PCF state X'8' is used to initiate a transmit operation on an SDLC interface when the LCD is set to X'9'. This state disables the NRZI mode and forces the 'send data' lead to a mark state as long as 'clear to send' is not active. When 'clear to send' becomes active, the NRZI mode and the 'send data' lead are allowed to operate normally.

If business machine clocking is used, the first characters transmitted must be X'00' (if using NRZI mode) or X'AA' (if not using NRZI mode) so that the remote clock can get in synchronization. If modem clocking is used, the two leading characters (X'00' or X'AA') are not required, and the first two characters and the tag bit should be set in the SDF and PDF.

When a flag is placed in the PDF for transmission, the disable zero insert control bit (ICW bit 7) must be turned on by the control program. When the Type 2 Scanner begins transmission ('clear to send' on), the scanner hardware changes the PCF state to X'9' (transmit data).

Note: When operating on a half-duplex line, the control program should not set the PCF state to X'8' until ICW bit 4 (receive line signal detector) is turned off.

7-22

PCF State X'9' (Transmit Normal): This PCF state is set by the Type 2 Scanner after completion of PCF state X'8'. Data is transmitted in this state until one of the transmit turnaround states (PCF X'C' or X'D') is set by the control program.

During transmission of characters over the SDLC line, the control program must maintain the proper state of the LCD. For example, when a flag character is placed into the PDF, the LCD must be set to X'9' and ICW bit 7 (disable zero insert control) must be set to 1. This allows transmission of more than five consecutive 1 bits for control purposes. For non-flag characters the disable zero insert control bit must be off.

PCF State X'C' (Transmit Turnaround—Request to Send Off): This PCF state is set by the control program on the interrupt following the interrupt that placed the last flag character to be transmitted into the PDF. While bits are being transmitted, this state is the same as PCF state X'9'.

When the character is completely transmitted, 'request to send' is reset along with the 'transmit mode' latch in the interface hardware.

PCF State X'D' (Transmit Turnaround—Request To Send On): This PCF state is set by the control program when the ending flag character for a message is placed in the PDF and the disable zero-insert control bit (7) is set on. In this state, the scanner transfers the Hag character from the PDF to the SDF and sets the flag detection/disable zero insert remembrance bit (5) to the current state of the disable zero-insert control bit (7) every flag character transfer. Continuous flag characters will be serialized to the LIB without further interrupts until PCF state X'D' is ended by the control program.

The control program normally ends this state by setting PCF state X'9'. When changing from PCF state X'D' to PCF state X'9', the first character to be transmitted in the X'9' state is loaded into the PDF and the disable zero-insert control bit (7) is reset if that character is not a flag. Subsequent characters are supplied by normal data servicing requests.

## Programming Note

When changing from state X'D' to state X'9', the program should check that the zero insert remembrance bit (5) is 1 to ensure that at least one flag character has been sent since state X'D' was set.

Diagnostic Functions

The Type 2 Communication Scanner has two diagnostic functions available to the control program: (1) the diagnostic wrap and (2) the IBM modem wrap test. These two tests are run under the control of the scanner program and provide online testing as described in the following sections. These diagnostics cannot be issued to an autocall interface.

For line interfaces attached through any one scanner, either a diagnostic wrap operation or a modem wrap test can be performed, but not simultaneously. However, these tests can be performed on two different scanners simultaneously.

Diagnostic Wrap Test

The Type 2 Scanner diagnostic wrap provides a means of testing and locating defects in the line control logic and line-interface transmit and receive logic. It also provides a method of online program testing.Diagnos-tic wrap can be performed online without affecting the normal program operation or the lines not in diagnostic mode. The test requires one line interface to act as a transmit line and one or more line interfaces in the same scanner to act as receive lines. Any line in the Type 2 Scanner can be a transmit or a receive line;  however, there may be only one diagnostic transmit line per scanner at any one time.

The diagnostic wrap is initiated under program control by executing Output X'45' and Output X'46' instructions to all lines to be tested. The Output X'45' instruction is executed with byte 1, bits 0-3 set for proper line control and byte 1, bit 7 set to 1 to indicate PCF state X'l' (set mode). The remaining bits of this output are set to 0. See PCF state X'l' in this chapter for further information on set mode.

Output X'46' must be set as follows:

Byte 0, bits 0-7: These bits are 0.

Byte 1, bit 0: This bit is 0.

Byte I, bit 1: Diagnostic Mode (ICW bit 27) - This bit must be set to 1.

Byte 1, bit 2: Data Terminal Ready (ICW bit 28) -This bit must be set to 0.

IBM 3704 and 3705 Communications Controllers Principles of Operation  7-23

Byte 1, bit 3: Synchronous Clock (ICW bit 29) -This bit must be set according to the type of Line Set tested. A 1 is placed in this position for binary synchronous lines, and a 0 for start-stop lines.

Byte 1, bit 4: External Clock (ICW bit 30) - This bit must be set to 0 to select a business machine clock.

Byte 1, bit 5: Data Rate Select (ICW bit 31)-This bit may be either 1 or 0.

Byte 1, bits 6-7: Oscillator Select 1 & 2 (ICW bits 32-33) - These bits are set to select an available line oscillator (business machine clock). For proper setting, see Business Machine Clocks in this chapter. All wrap test lines must select the same oscillator.

After the set modes are issued, the affected line interfaces can be exercised through any sequence of point-to-point or multipoint operations.

During diagnostic wrap operations both the 'data set ready' lead and the 'clear to send' lead are simulated active to prevent the Type 2 Scanner from setting a modem check (ICW bit 3). The 'clear to send' lead is not simulated active if the PCF is set to X'C' and the Type 2 Scanner detects that it has completely serialized the character in the SDF. This is to allow either the PCF state X'B' or X'C' to be used during a diagnostic wrap.

## Programming Note

1. Only one interface per Type 2 Scanner may be in a transmit state at any given time during the operation.

2. The line used for transmit should be the last line to be issued the mode set.

3. Diagnostic wrap cannot be executed on an autocall interface.

4. During a diagnostic wrap operation, the line control definer (LCD) of the line or lines in diagnostic receive must agree with the LCD of the line in diagnostic transmit regardless of the common-carrier or IBM equipment physically attached to the line.

Modem Wrap Test

Modem wrap test tests the scrambler circuits of IBM 3872, 3874, and 3875 modems under program control. The modem test can be performed online without affecting the normal operation of other lines. This test may also be performed simultaneously on any or all interfaces for which the test function is defined.

To execute the modem wrap test, the LCD field of the ICW for the interface to be tested must be X'8', X'9', X'C', or X'D'. The control program must then execute an Output X'46' instruction to set the inter

face control word SDF as follows:

SDF bits 0-2: These bits are not used and should be 0.

SDF bit 3: This bit must be set to 1 to cause the 'diagnostic mode' latch to be set in the line interface hardware, which conditions the modem for testing.

SDF bit 4: This bit must be set to 1 to cause 'data terminal ready'. When this is set, together with the diagnostic mode, it enables a modem wrap test operation to be performed instead of a diagnostic wrap.

SDF bit 5: This bit must be set to 1 to provide synchronous clocking.

SDF bit 6: This bit must be set to 1 if the modem provides the clock and to 0 if the modem does not provide the clock.

SDF bit 7: This bit may be either 0 or 1.

SDF bits 8 and 9: These bits must select an internal oscillator whose speed is less than one-half the clock speed if the IBM modem provides the clock. If the modem does not provide the clock, the bits must select an internal oscillator whose speed matches that of the modem. For the proper setting, see Business Machine Clocks in this chapter.

After the set mode is executed, the interface should be put in PCF state X'8' (transmit initial) to cause 'request to send' to be raised. Before this is done, the PDF and SDF fields should be set to X'FF' and the pad flag (ICW bit 7) turned on to cause continuous marks to be transmitted. When the first interrupt occurs in PCF state X'9', the PCF should be changed to X'D' to cause the interface to turn around with 'request to send' on. When the turnaround is complete, the interface is in PCF state X'5', and a mark is sent.

The program can now test the operation of the modem scrambler circuits by setting the PCF to X'7' (receiving in-phase) and checking the received data for all marks for a period of at least one second.

7-24