### Chapter 8: Type 3 Communication Scanner

This chapter is intended to give the reader a basic understanding of the operation of the Type 3 Communication Scanner and the requirements necessary to program the scanner.

The Type 3 Communication Scanner (1) scans the interface addresses assigned to the LIB positions it supports, (2) performs character assembly/disassembly, (3) provides buffering of up to eight message characters, within local storage, and (4) causes program interrupts when program service is required to obtain another program buffer or to handle an end-of-message condition.

Up to four Type 3 Communication Scanners can be installed in the 3705 (Type 3 Scanner-1 through Type 3 Scanner-4). Type 3 Scanner-1 supports attachment of up to three LIBs with 48 half-duplex (HDX) lines. Type 3 Scanner-2, -3, and -4 can each support attachment of up to four LIBs with 64 HDX lines; thus, up to 240 HDX lines can be attached to the 3705 using four Type 3 Scanners. The Type 3 Scanners can be installed with the Type 2, Type 3, or Type 4 Channel Adapter features.

The number of lines supported by a Type 3 Scanner depends on:

• How lines are used.

• Line disciplines.

• Mix of line speeds.

For information about LIBs and their capacities, refer to the Introduction to the IBM 3704 and 3705 manual.

Operation and Data Flow

The interface addresses for all installed Type 3 Communication Scanners in the 3705 are generated from a common Type 2 Attachment Base. A continuously running scan counter in the attachment base places the generated interface address on an address bus that goes to all scanners simultaneously. This address can be modified, under program control, by the attachment base or the scanner.

The interface address is then used to address an interface control word (ICW), which is loaded into the ICW work register where the scanner hardware determines if any action is to be performed for that interface. If no action is required the ICW is replaced in local store and the next addressed ICW is loaded into the work register. If the scanner determines that program intervention is required, it requests a level 2 interrupt and loads the interface address into an interrupt priority register.

When the level 2 interrupt actually occurs, the address in the highest-priority interrupt priority register that is active is loaded into the attachment buffer address register (ABAR) and is then available to the control program along with the ICW in the ICW input register.

Note: The notation "bit 0.1 ", "bit 1.5", etc., is used throughout this chapter to refer to the bytes and bits of the ICW and registers. For example, "bit 1.5 "refers to byte 1, bit 5.

Type 3 Scanner Registers

The Type 3 Scanner contains various hardware registers that are used to store and pass information and data within the scanner and between the scanner and the control program. Some of these hardware registers are available to the control program as external register addresses through input and output instructions. The external registers required for control program access are described in the following paragraphs.

Local Storage

Each Type 3 Scanner contains a local storage array to store the interface control words when not being used by the scanner hardware or the control program. This storage array holds 64 interface control words of 16 bytes each. (A parity bit is associated with each byte.)

ICW Work Register

The Type 3 Scanner control logic uses the ICW work register to access, monitor, and modify an interface control word (ICW). This register is loaded each time an ICW is read out of local storage.

ICW Input Register

The control program uses the ICW input register for access to the interface control words. This register is loaded from the ICW work register and reflects the status of the ICW at the time when it was read out of local storage.

The ICW is loaded into the input register by level 2 interrupt from the scanner for the associated line or by an Output X'40' instruction in program level 3 or 4 when the address of the line is in the ABAR.

Attachment Buffer Address Register (ABAR)

The attachment buffer address register is physically located in the Type 2 Attachment Base and supplies the interface addresses to the control program. See I/O Programming Considerations in this chapter for a description of loading the ABAR.

Programming Note

The ABAR must be initialized by an Output X'40' instruction with an interface address associated with an installed Type 3 Scanner. The Output X'40' must be executed after the controller is powered on and before any other input or output instructions to the scanner are executed.

IBM 3704 and 3705 Communications Controllers Principles of Operations  8-1

Display Register

The display register is a temporary storage register that can contain interface control information that the program can use. If bit 4.6 (display request) of an interface control word is on, control information for that interface is loaded into the display register each time that interface is scanned. (This bit is turned on via an Output X'43' instruction in which bit 0.2 is on; the instruction must be executed after the address of the line for which the information is to be displayed has been loaded in the ABAR via an Output X'40' instruction.) The control program can then obtain this information by executing an Input X'46' instruction.

There is only one display register for each Type 3 Scanner; therefore, only one ICW at a time should have its display request bit (4.6) set to 1. Otherwise, the control program cannot determine which interface was the last one to cause the display register to be loaded. The display register normally contains the status of the clear to send, ring indicator, data set ready, receive line signal detect, receive data bit buffer, diagnostic wrap mode, bit service request, and ICW diagnostic mode indicators for the line interface (or contains the Autocall unit indicators, for an Autocall interface). Alternatively, the display register contains the status of the scanner-to-line set data out lines (if the scanner is in diagnostic 0 mode [ICW bit 5.5 is on]) or the line set-to-scanner data in lines (if a feedback check has occurred for the line interface [LCD state is X'7' or X'F']).

A feedback check error occurs when the state of any data in line does not match the state of the corresponding data out line (which indicates that the line interface hardware is not in the state intended to be set by the control program). The display register bit corresponding to the non-matching data out/data in line pair is set to 0; all other bits are 1.

Scanner Output Register

The scanner output register buffers the bits received from the CCU outbus until the next CCU time occurs, allowing the ICW bits or control latch settings to be altered. This register is loaded from the CCU outbus via Output X'41'-X'4F' instructions.

Scanner Check Register

The scanner check register comprises 21 check latches, each associated with the detection of a specific error condition in the Type 3 Scanner. The control program may access the check register after an interface address specifying a Type 3 Scanner is placed in the attachment buffer address register (ABAR) via an Output X'40' instruction. The current state of the check register may be determined by an Input X'42' and an Input X'43' instruction.

The Type 3 scanner issues a level 1 interrupt request to the CCU when any of the check register latches is set. These latches may be reset by an Output X'43' instruction. The reset bit (byte 0, bit 1) of the instruction must be on and the Type 3 Scanner request bit (byte 1, bit 5) of the register specified by the R field of the instruction must be on.

Diagnostic Buffer Address Register (DBAR)

Bits 1.0—1.5 of an Input X'42' instruction will contain the failing ICW address (in binary for any ICW, work register parity check, PDF array parity check, or LIB select error). The control program can use this address to determine the line for which the error occurred.

Cycle Steal Buffer Address Register (CBAR)

The scanner loads this register with the current address of main storage into which data is to be stored or from which data is to be fetched with a cycle steal operation. The current address is obtained from ICW bytes 6, 8, and 9.

PDF Array Format

Associated with each ICW is a set of eight addressable buffers called parallel data fields (PDF). Each set is called a PDF array. Each PDF in the array contains eleven bits (plus a parity bit) and is used to hold a data character or control and status information pertaining to the receive or transmit operation in which the scanner is currently engaged.

The format of each PDF is shown in Figure 8-1.

Each PDF array buffers message data between main controller storage and the communication line interface with which the array is associated via the interface control word (ICW). The format and content of the interface control word appears later in this chapter.

Two pointers within the ICW select the PDF to be used in the current receive or transmit operation. The cycle steal PDF array pointer selects the PDF into which a cycle steal operation is to transfer data from main storage (during a transmit operation) or from which a cycle steal operation is to transfer data to main storage (during a receive operation). The PDF array pointer selects the PDF into which the received data from the SDF is placed, or from which data to be transmitted via the SDF is placed.

During a transmit operation, the scanner places data characters from main storage into the PDFs, in succession via a cycle steal operation. Data transfer continues until all eight PDFs in the array are full, or until all the data to be transmitted has been placed in the PDF array. The scanner concurrently removes, in succession, data characters from the PDFs that the cycle steal operation has filled and passes them to the line interface hardware via the serial data field (SDF).

8-2

Figure 8-1. PDF Array Format

IBM 3704 and 3705 Communications Controllers Principles of Operation  8-3

The SDF serializes the bits to the line interface hardware.(The serial data field is located in byte 3 of the ICW.) The scanner increments the cycle steal PDF array pointer as each byte is transferred from main storage, and increments the PDF array pointer as each data byte is transferred from the PDF to the SDF.

The scanner normally transfers data via cycle steal operation two bytes at a time. The scanner initiates a cycle steal operation each time two PDFs associated with the ICW have been emptied (that is, transmitted). The scanner compares the cycle steal PDF array pointer and the PDF array pointer to determine when two bytes are empty.

During a receive operation, the opposite action occurs. The scanner transfers data characters in succession from the line interface (via the SDF, which deserializes the bits into characters) to the PDF array until all data has been received (as indicated by ending characters) or the PDF array is full. Concurrently, the scanner transfers received characters from the PDF to main storage using a cycle steal operation until all PDFs filled during the receive operation are empty or the scanner detects ending characters.

The scanner increments the cycle steal PDF array pointer as each byte is transferred to main storage, and increments the PDF array pointer as each data byte is transferred from the SDF to the PDF.

For BSC receive operations, the scanner initiates a cycle steal operation whenever two or more PDFs in the array have been filled from the line interface (via the SDF). The scanner compares the cycle steal PDF array pointer and the PDF array pointer to determine when at least two PDFs have been filled. For SDLC receive operations, the scanner initiates a cycle steal operation when the content of the cycle steal message counter (ICW bits 13.6,13.7) has been incremented to a non-zero value.

For either a transmit or a receive operation, the PDF array acts as a wraparound eight-byte buffer. During a transmit operation the PDF array pointer (which "empties" the PDF) logically follows the cycle steal PDF array pointer (which "fills" the PDFs) in succession through the array. During a receive operation, on the other hand, the cycle steal PDF array pointer (which "empties" the PDF) follows the PDF array pointer (which "fills" the PDFs). (See Figure 8-1.)

Ending the Cycle Steal Operation

For a transmit operation, the cycle steal operation that transfers data from main storage to the PDF array continues until the 'cycle steal valid' bit in the ICW (bit 6.5) is reset. This occurs when (1) the cycle steal byte count

reaches zero, or (2) the scanner detects an ETB, ETX, or ENQ character in the received data (for BSC line only).

After transmitting all of the data characters to be sent, the scanner transmits the ending character or sequence and then causes a program level 2 interrupt.

For a receive operation, the cycle steal operation that transfers data from the PDF array to main storage continues until the 'cycle steal valid' bit in the ICW (bit 6.5) is reset to 0. This occurs when (1) the cycle steal byte count reaches zero, or (2) the scanner detects a control byte in the PDF array when the 'end of message' bit (ICW bit 0.5) is on.

During a receive operation, the scanner transfers status and control bits to the PDF array following the last message data character received. The specific status and control information placed in the PDF array depends on the current states of the line control definer (LCD), primary control field (PCF), and extended primary control fields of the ICW. These fields are described later in this chapter. Unlike data bytes, which the cycle steal operation transfers to main storage, status bytes are transferred from the PDF to ICW byte 15 (BSC/SDLC control status) and control bits are used to set bits in ICW bytes 0 (secondary control field [SCF]) and 14 (status exception). Storing of status and control bits follows storing of the data bytes.

Upon detecting an ending character or sequence in received data, the Type 3 Scanner causes a program level 2 interrupt.

PDF-Full and PDF-Empty Conditions

When the three low-order bits of both the cycle steal PDF array pointer (ICW bits 12.1-12.3) and the PDF array pointer (ICW bits 12.5-12.7 ) are equal, the PDF array is empty if the high-order bit of each pointer (bits 12.0 and 12.4) are equal; the PDF array is full if those bits are unequal.

Scanner Initialization

The scanner and its associated line interfaces (LIB) are placed in a disable state (1) during a power-on sequence,

(2) during an initial program load (IPL) of the 3705,

(3) when the controller is reset from its control panel, or

(4) during the execution of an Output X'43' instruction (discussed later in this chapter) when the general register specified by the R field of the instruction contains appropriate bits. The control program must enable each scanner in the controller by executing an Output X'43' instruction for which register bits 0.1 and 1.6 are 1 before initializing each ICW and the associated line or autocall interface. This initialization must occur before the interface can operate.

8-4

PDF Array Reset

It is important to note that when 3705 power is turned off, the PDF array is not automatically reset upon restoration of power. Therefore, the programmer, when loading the controller via IPL, must clear the array by issuing, for each line address to be used, four Output X'4D' instructions, followed by one Output X'4E' instruction. Only those addresses to be used by the control program being loaded need have these instructions issued.

Type 3 Scanner Addressing

The Type 3 Communication Scanner scan-addressing and program-addressing mechanism is controlled by the Type 2 Attachment Base. The attachment base generates the basic scan address and places it on a 'line address bus' for availability to all installed Type 3 Communication Scanners. Refer to the Interface Addressing section of Chapter 3 for a detailed discussion of each interface address bit.

Scan Addressing

For scan addressing, an interface in each installed Type 3 Scanner is addressed simultaneously. Each scanner derives the address of the interface it is scanning from the 8-bit address that the Type 2 Attachment Base places on a 'line address bus'. The line address bus is an internal bus that carries the scan address from the attachment base to each of the communication scanners. This address, as modified by each scanner (see "Upper Scan Limit" and "High Speed Select"), is used not only to select a particular interface but also to address the associated interface control word (ICW) that the scanner maintains in local storage. (See Interface Control Word in this chapter.) The Type 3 Scanner examines this ICW and, when an interface service function is required, performs that function; or, when a character service requires programming action, the Type 3 Scanner signals the attachment base that it needs a program level 2 interrupt.

Scan Counter

The 3705 Type 2 Attachment Base scan counter output provides the basic scan addresses for each Type 3 Scanner. If the scan counter output is not modified, each Type 3 Scanner sequentially scans 96 interface addresses. Under these circumstances, the Type 3 Scanner cannot handle line speeds higher than 4800 bps without having the possibility of undetected bit overrun/underrun conditions. However, the ability to substitute some interface addresses (high speed select or address substitution) and set a limit on the number of interfaces scanned (scan limit), greatly extends the capability of handling higher-speed lines. These mechanisms cause the scan counter output to be modified to allow certain interface addresses to be scanned at higher rates than others.

Upper Scan Limit

The Type 3 Scanners have an upper scan limit that can be set and reset under program control by an Output X'42' instruction. Each scanner maintains its own uppei scan limit and is independent of the limits set by any of the other installed scanners. Based on the state of its upper scan limit latches, a Type 3 Scanner may modify the scan counter output from the Type 2 Attachment Base in such a way as to limit the number of interface addresses scanned.

The actual modification of the scan address is done by the Type 3 Scanner hardware as the line address bus enters the scanner from the attachment base. Figure 8-2 shows the number of interfaces scanned and the LIB position affected for each setting of the upper scan limit. When the upper scan limit is set to any value other than binary 00, the scanner modifies the addresses above the limit to start at the first address again. For example, if the upper scan limit is set to allow only 16 interface addresses to be scanned, the address is modified to scan the first address again when the scan counter output to that scanner reaches the 17th address. This decreases the period of time between successive scans of the remaining interface addresses to accommodate higher-speed lines. In this case, the scanner with an upper scan limit of binary 11 scans the first 16 interfaces four times in the same period of time as another scanner with no limit scans 96 addresses.

Address Substitution

The output of the scan counter can be modified to cause certain addresses assigned to LIB position 1 to be substituted on the 'line address bus' in place of normal scan addresses. As a result, those addresses that are substituted are scanned by the Type 3 Scanner more frequently than the other addresses. Address substitution affects all installed scanners in the same manner. When operating with address substitution, each scanner in the 3705 scans the substituted address or addresses, every 12.8 microseconds (3705-I) or 16 microseconds (3705-II), because address substitution occurs every eighth time the scan counter changes state. This allows the substitution address or addresses in each scanner to handle higher line speeds independent of the state of the scan limit.

Address substitution is controlled by a four-bit register called the substitution control register. The bits of this register may be set under program control by Output X'41' byte 1, bits 2,3,4, and 5. Each bit of this substitution control register corresponds to one of four substitution addresses assigned to LIB position 1.

IBM 3704 and 3705 Communications Controllers Principles of Operations  8-5

| Upper Scan Limit* | Number of Interfaces Scanned | Interface Addresses Scanned | Interface Addresses Not Scanned | Effective Scan Period (µsec) |
| --- | --- | --- | --- | --- |
|  |  |  |  | 3705-I 3705-II |
| 00 10 11 01 | 96** 48 16 8 | Addr 0-F,LIBl-6 Addr 0-F, LIB 1-3 Addr 0-F, LIB 1 Addr 0-7,LIB 1 | Addr 0-F, LIB 4-6 Addr 0-F, LIB 2-6 Addr 8-F, LIB 1 and Addr 0-F, LIB 2-6 | 153.6 192 76.8 96 25.6 32 12.8 16 |
| *Set by Output X'42' byte 1, bits 6-7 **Scanner-l contains 64 ICWs in local storage but only uses the first 48. |  |  |  |  |

Figure 8-2. Upper Scan Limit

## Programming Note

Any combination of the four substitution control register bits may be turned on to produce the desired substitutions. If address substitution is not used, Output X'41' must be executed with byte 1, bits 2 through 5 off in the register specified by the R operand.

When a given substitution control register bit is on, a corresponding address is substituted on the 'line address bus' every eighth time the scan counter changes state. Combinations of bits on in the substitution control register result in fixed-address substitution for each corresponding bit. Figure 8-3 shows which address is substituted and which addresses are not scanned as a result of that substitution when the various substitution control register bits are on.

| Output | Fixed Address |  |
| --- | --- | --- |
| X'41' | Substituted in | Addresses Not Scanned |
| Byte 1, | Each Type 3 Scanner | In Each Type 3 Scanner |
| Bit: | If Substitution Bit ON | If Substitution Bit ON |
| 1 | Adr 0 LIB position 1 | Adr E in LIB positions 1-6 |
|  |  | Adr F in LIB positions 1-6 |
| 3 | Adr 2 LIB position 1 | Adr C in LIB positions 1-6 |
|  |  | Adr D in LIB positions 1-6 |
| 4 | Adr 4 LIB position 1 | Adr A in LIB positions 1-6 |
|  |  | Adr B in LIB positions 1-6 |
| 5 | Adr 6 LIB position 1 | Adr 8 in LIB positions 1-6 |
|  |  | Adr 9 in LIB positions 1-6 |

High Speed Select Option

The high speed select option is similar to address substitution in that bit settings within a register alter the scanning pattern so that a predetermined address is scanned several times per scanning cycle, at the expense of not scanning other addresses. This option differs from address substitution, however, in that up to eight addresses serviced by the Type 3 Scanner can receive the increased scanning frequency, and high speed select masks are individually specified for each of the installed Type 3 Scanners. Because the mask specified affects only one scanner, it has no influence on the selection of addresses scanned by other installed scanners. This independence allows more flexibility in selecting addresses to receive the increased scanning. No line interfaces associated with the unscanned addresses can be active while the high speed select option is in effect.

High speed select operation is controlled by an eight-bit scan substitution control register. The bits of this register may be set under program control by Output X'41' byte 0, bits 0-7. Figure 8-4 shows which address is substituted and which addresses are not scanned as a result of that substitution when the various high speed select register bits are on.

Figure 8-3. Address Substitution Control

8-6

| Output X'41' Byte 0, Bit: | .Ffaed Address Selected in Addressed Type 3 Scanner If High Speed Select Bit On | Addresses Not Scanned in Addressed Scanner If High Speed Select Bit On |
| --- | --- | --- |
| 0 | Adr 0 LIB position 1 | Adr 1 in LIB position 1 |
|  |  | Adr 0 & 1 in LIB positions 2-6 |
| 1 | Adr 2 LIB position 1 | Adr 3 in LIB position 1 |
|  |  | Adr 2 & 3 in LIB positions 2-6 |
| 2 | Adr 4 LIB position 1 | Adr 5 in LIB position 1 |
|  |  | Adr 4 & 5 in LIB positions 2-6 |
| 3 | Adr 6 LIB position 1 | Adr 7 in LIB position 1 |
|  |  | Adr 6 & 7 in LIB positions 2-6 |
| 4 | Adr 8 LIB position 1 | Adr 9 in LIB position 1 |
|  |  | Adr 8 & 9 in LIB positions 2-6 |
| 5 | Adr A LIB position 1 | Adr B in LIB position 1 |
|  |  | Adr A & B in LIB positions 2-6 |
| 6 | Adr C LIB position 1 | Adr D in LIB position 1 |
|  |  | Adr C & D in LIB positions 2-6 |
| 7 | Adr E LIB position 1 | Adr F in LIB position 1 |
|  |  | Adr E & F in LIB positions 2-6 |

Figure 8-4. High Speed Select Control

Program Addressing

Various input and output instructions exist that allow the program to control the operation of the Type 3 Scanners, Type 2 Attachment Base, and the individual interfaces.

However, before the program can examine or modify fields in an interface control word (ICW) associated with a particular interface, the address of that interface must be placed in the attachment buffer address register (ABAR) of the attachment base. Similarly, before the program can access certain registers in a particular Type 3 Scanner or perform control functions in that Scanner, the interface address in the ABAR must be one of those assigned to that scanner. Two events can cause the contents of the ABAR to be changed: (1) a program level 2 interrupt, and (2) execution of an Output X'40' instruction.

When a program level 2 interrupt occurs, the contents of the ABAR are automatically set by the Type 2 Attachment Base with the interface address from the highest interrupt priority register that is occupied. The control program can determine which interface address is in the ABAR by executing an Input X'40' instruction. The program can then examine and/or modify fields in the ICW associated with this interface. In the other interrupt program levels (1,3, and 4), the program may find it necessary to gain access to the ICW associated with a specific interface. By executing Output X'40' under such circumstances, the program can set the ABAR according to the interface address in the register specified by the R operand.

To avoid conflicts with the automatic mechanism that sets the ABAR when a program level 2 interrupt occurs, programs executing at program level 3 or 4 should mask program level 2 interrupts before executing Output X'40'. (Program level 1 should save the value of the ABAR, change it to select the desired interface, and then restore the original value to the ABAR.) If more than one program level is likely to execute an Output X'40', additional interlocking conventions must be established within the control program.

Interface Control Word (ICW)

The Interface Control Word (ICW) provides the normal means by which the control program communicates with the Type 3 Scanner and the interface hardware.

Each ICW is 16 bytes long and is located in scanner local storage. Associated with the ICW is an array of eight parallel data fields, each eleven bits (plus parity bit) long. One of the eight PDFs is logically associated with the ICW at any moment via pointers in byte 12 of the ICW. Figure 8-5 shows the format of the ICW and the input/output instructions by which the ICW fields are set or accessed. Seventeen bytes (numbered 0 through 16) are shown. Byte 0 and bytes 2-16 are the sixteen bytes of the ICW proper. Byte 1 is the parallel data field which, though not actually part of the ICW, is shown because it is logically part of the ICW for the purpose of the associated Input/Output instructions. Each scanner contains one ICW for each possible interface. However, even though the scanner contains the maximum number of ICWs (96) only those ICWs associated with an attached and active interface are used.

ICW Access

The Type 3 Scanner hardware gains access to an ICW by using the interface address provided by the interrupt priority register in the Type 3 Attachment Base. When the level 2 interrupt occurs, the address from the interrupt priority register is loaded into the attachment buffer address register (ABAR). The program can then execute an Input X'40' instruction to get the interface address associated with the interface. Once the control program obtains the interface address, it has access to the various fields of the ICW through input and output instructions.

## Programming Note

ICW access at program level 3 or 5 should be performed only when program level 2 interrupts are masked off; otherwise, the result is unpredictable.

IBM 3704 and 3705 Communications Controllers Principles of Operations  8-7

Figure 8-5. Interface Control Word Type 3 Scanner

Editor’s Note: Figure 8-5 originally occupied two pages across a gutter. It has been expanded for readibility to cover six pages.

8-8a

Figure 8-5 (Part 2 of 5) Interface Control Word Type 3 Scanner

IBM 3704 and 3705 Communications Controllers Principles of Operations 8-8b

Figure 8-5 (Part 3 of 5) Interface Control Word Type 3 Scanner

8-8c

Figure 8-5 (Part 4 of 5) Interface Control Word Type 3 Scanner

IBM 3704 and 3705 Communications Controllers Principles of Operations 8-8d

ICW FIELDS AND INPUT/OUTPUT INSTRUCTIONS

Figure 8-5 (Part 5 of 5) Interface Control Word Type 3 Scanner

IBM 3704 and 3705 Communications Controllers Principles of Operations 8-9

# ICW Modification

Program access to the various fields in the ICW is through the use of input and output instructions. When the Input X'44'-X'4F' instructions are executed, the ICW bits assigned to those inputs are placed in the register specified in the R operand. See Appendix B for the input/output instruction bit definitions.

The information obtained by the input instructions comes from the ICW input register. There is one ICW input register in each communication scanner. This register is automatically loaded from the ICW work register when a level 2 interrupt occurs or when an Output X'40' instruction is executed in any program level other than level 1 or 2.

The ICW input register does not necessarily reflect the current state of the ICW associated with the interface address in ABAR. The actual ICW may have been modified by the Type 3 Scanner during scan addressing after the ICW input register was set.

Also, the actual ICW may have been altered by the execution of an Output X'43'-X'4F' instruction. Subsequent input instructions do not include these alterations because the ICW input register had been set by a previous Output X'40' instruction or Type 3 Scanner level 2 interrupt. In the event that an output instruction and scan addressing both occur during the same scan cycle, the output instruction is executed first; then the scanner performs its modification, if needed. This ensures that the latest modifications to the actual ICW will be included during the next scan addressing operation.

Refer to Interface Control Word Format in this chapter for a complete description of the individual ICW fields and the I/O instructions associated with each field.

Programming Note

Since the interface control words are asynchronously interrogated and modified by both the scanners and the control program, caution should be observed to ensure the ICW integrity when the program issues an output instruction. The Type 3 Scanner cannot check whether the control program has modified the ICW correctly. Therefore, errors in the modification itself may be difficult to isolate. To prevent control program modifications to the ICW (via output instructions) from being destroyed by the scanner, program modifications are not permitted during that portion of a scan when the Type 3 Scanner fetches, modifies, and restores the ICW for the line being scanned.

Interface Control Word Format

This section describes the ICW fields and their bit meanings. (See Figure 8.5.)

ICW Bits 0.0-0.7 (Secondary Control Field): The secondary control field (SCF) is used as a sense, status, and operation modifier field between the control program and the communication scanner. Bits 0.0-0.5 are set by the Type 3 Scanner hardware according to the conditions described below. Bits 0.6 and 0.7 are program controlled. This field may be tested by using the Input X'44' instruction. An Output X'44' instruction is used to reset bits 0.0-0.3 and 0.5, to set bit 0.4, and to set and reset bits 0.6 and 0.7. Refer to Appendix B for bit definitions of Input/Output instructions.

Bit 0.0-Abort Detected: The Type 3 Scanner sets this bit to 1 when it detects seven consecutive 1 bits in the received data stream while the PCF state is X'6' or X'7'. If this bit is 1, the service request interlock (ICW bit 0-1) is forced to 0. Bit 0 must be reset by the control program using an Output X'44' instruction.

This bit has no significance during a transmit operation.

Note: A transmitted abort sequence consists of eight consecutive 1 bits. However, if the scanner detects seven consecutive 1 bits while receiving data, it will set the abort bit (ICW bit 0.0) to 1.

Bit 0.1-Normal Service Request Interlock: This bit is set when the Type 3 Scanner detects that buffer servicing or control servicing is required between the control program and the addressed ICW. The control program must reset this bit after the interrupt is honored and all bits or bytes of the ICW have been modified. If this bit is already set when the scanner is prepared to set it on, and the scanner is in a transmit or receive state, a character overrun/underrun flag is set (ICW bit 0.2).

If this bit is 1, ICW bits 0-0, 0.2,0.3,14.1,14.3, 14.4, 14.6, and 14.7 are 0.

Programming Notes

1. The control program should reset the normal service request interlock before setting the PCF state to monitor modem or autocall unit control lines.

2. For level 2 interrupt routines that change the cycle steal address and count, the cycle steal valid bit should be set before the normal service request interlock bit is reset.

3. An Output X'44' instruction that resets the normal service request interlock and/or EOM bits should be the last Output instruction executed for the scanner in program level 2.

8-10

Bit 0.2-Character Overrun/'Underrun: This bit is set when the Type 3 Scanner attempts to set the service request interlock (ICW bit 0.1) and finds it already set. This error is normally caused by an instantaneous peak overload situation. Errors of this type should not occur in the average installation and should occur only infrequently in high-throughput installations.

If a character overrun occurs, the next character received overlays the previously received character in the PDF array. Thus, one or more characters are lost. When this occurs, the flush bit (ICW bit 14.2) is set, the PDF array is reset, and all subsequent data received is discarded until an ending sequence is received. At that point, the EOM bit (0.5) and the character overrun/underrun bit (0.2) are set.

If an underrun occurs (possible only on an SDLC transmit line), an Abort character is transmitted and the EOM and overrun/underrun bits are set.

An underrun condition cannot occur on a BSC line because the scanner automatically transmits SYN characters (non-transparent text) or DLE SYN sequences (transparent text) until the control program sets up another cycle steal operation.

If this bit is 1, the service request interlock (ICW bit 0.1) is 0.

Bit 0.3—Modem Check: During each bit interval (bit service time), the Type 3 Scanner checks the line interface for the proper modem conditions. This bit is set to 1 to indicate the following conditions:

1. The 'data set ready' line is inactive when the PCF field of the ICW is in states X'5', X'6', X'7', X'8', X'9', X'A', or X'B'.

2. The 'clear to send' line is inactive when the PCF field of the ICW is in states X'9' or X'B'.

If this bit is 1, the service request interlock (ICW bit 0.1) is 0.

Bit 0.4-Not Level 2 Bid: This bit is set whenever an Output X'44' instruction is executed, regardless of the contents of the register specified by R. It is reset by the Type 3 Scanner when a level 2 bid is accepted by the interrupt priority register of the Type 2 Attachment Base. When reset, this bit prevents the Type 3 Scanner from making another level 2 interrupt request for the line.

Bit 0.5-End-of-Message Interrupt: This bit is set to 1 by the Type 3 Scanner to indicate the end of a received or transmitted message. If both this bit and bit 0.1 are 1, the receive or transmit operation ended normally. The operation ended abnormally if both this bit and bit 0.0, 0.2,0.3,14.1, 14.3, 14.4, 14.6, or 14.7 are 1.

During an SDLC receive operation, the scanner sets this bit after storing the received data and checking the block check characters (BCC). During an SDLC transmit operation, the scanner sets this bit after sending an ending control character.

During a BSC receive operation, the scanner sets this bit upon detecting an ending sequence after the received data is stored and the BCC is checked. During a BSC transmit operation, the scanner sets this bit after transmitting the ending characters and BCC.

Bit 0.6—Program Flag: This bit provides a flag in the ICW that can be used by the control program for any desired purpose. It is not used by the scanner.

Bit 0.7-Line Trace Active: The control program sets this bit to indicate to the scanner that the line represented by the ICW is to be traced. When this bit is set, the scanner stores the received BCC characters following an ETB or ETX control character. (BCC characters following an ITB character are not stored.)

ICW Bits 1.0-1.7 (Parallel Data Field): The parallel data field (PDF) provides a path for diagnostic programs to alter the contents of the PDF array. An Output X'44' instruction in which bit 0.4 is on will store byte 1 of the register specified by R at the location specified by the PDF array pointer (ICW bits 12.4-12.7). (Bit 0.4 need not be on to write autocall dial digits into the PDF (LCD state X'3' or X'B'.) The PDF array is normally loaded and unloaded automatically by the Type 3 Scanner in a manner that provides up to eight bytes of buffering between a cycle-steal operation to or from storage for receiving or transmitting data over the line.

For an autocall interface, a dial digit must be placed in the PDF as shown in Figure 8-5.

Note: Although referred to as ICW bits 1.0-1.7, the PDF is actually one of the eight elements (PDFs) of the PDF array.

ICW Bits 2.0-2.3 (Line Control Definer): The line control definer (LCD) field defines the type of interface associated with the ICW. The LCD, set under program control by an Output X'45' instruction, must be in agreement with the installed interface type and the common-carrier or IBM equipment physically attached to the interface. The LCD field definition must also agree with the basic line control procedures to be executed. An LCD is generally set at initial program load (IPL) time and remains the same throughout normal operation.

The values of the LCD field are defined in detail, along with the effect of the LCD on the PDF and SDF fields, in the section describing LCD states.

IBM 3704 and 3705 Communications Controllers Principles of Operations  8-11

ICW Bits 2.4-2.7 (Primary Control Field): The primary control field (PCF) defines the state of the interface at any particular time. An extended PCF is located in ICW bits 16.4-16.7; the extended PCF allows further differentiation of the basic PCF states. The interpretation of the PCF and extended PCF Fields depends on the state of the LCD. (See the section Autocall Interface Operation for a description of the PCF states for an autocall interface.)

The Output X'45' instruction can be used to set the state of the PCF and extended PCF. Also, an Input X'45' instruction can be executed to test the state of the PCF fields; an Input X'4B' can be used to test the extended PCF. Refer to Appendix B for bit definitions of Input and Output instructions.

ICW Bits 3.0-3.7, 4.0-4.1 (Serial Data Field): The serial data field (SDF) is primarily used as a character deserializer/seri-alizer field. On receive operations, the data arriving from a line is placed in this field bit by bit to assemble a character. Once assembled, the character is transferred, under hardware control, to the PDF for transfer to 3705 storage via a cycle steal operation. For transmit operations, a character from the PDF is transferred into the SDF under hardware control and then transferred bit by bit to the line interface hardware.

The SDF is also used for autocall operations and for setting certain ICW bits using the Set Mode command. The SDF format depends on the values in the LCD and PCF fields. See Interface Mode Set and Autocall Interface Operation in this chapter for further definitions. Program access to the SDF is through Input X'45' and X'47' instructions. The SDF can be modified by an Output X'46' instruction. Refer to Appendix B for bit definitions of Input and Output instructions.

ICW Bits 4.2-4.4, 4.7, 5.0 (Ones Counter/Interval Timer):

For SDLC, these five bits are used as a ones counter. This counter is incremented and reset by the Type 3 Scanner hardware. During a receive operation, this counter is used to detect:

• Inserted 0's that are to be deleted from the received bit stream while in PCF states X'6' and X'7'.

• Flag sequences (X'7E') occurring while in PCF states X'5' X'6', and X'7'.

• Abort sequences (seven consecutive 1 bits) received while in PCF states X'6' and X'7'.

• Idle sequences (fifteen consecutive 1 bits).

During a transmit operation, this counter is used to:

• Insert a 0 bit after each five consecutive 1 bits transmitted. This applies to PCF states X'9' and X'B' under certain extended PCF states.

• Generate a minimum of fifteen consecutive 1 bits for a line idle state.

For BSC, these five ICW bits are used as an interval timer to provide standard one- and three-second BSC timeouts.

ICW Bit 4.5 (Last Line State/Timeout Control): During an SDLC operation, this ICW bit retains the state of the last data bit transmitted or received between the Type 3 Scanner and the LIB. For BSC, this ICW bit is compared to the timer change latch and, if it is found to be different, causes the interval timer bits of the ICW (bits 4.2,4.3, 4.4, 4.7, 5.0) to be incremented. This ICW bit is set to the same value as that of the timer change latch.

ICW Bit 4.6 (Display Request): This bit allows the state latches in the interface hardware and certain modem (or Autocall unit) signals to be loaded into the Type 3 Scanner display register when the interface associated with this ICW is scanned. The bit may be set and reset by an Output X'43' instruction when the interface address for that ICW is in the attachment buffer address register (ABAR) in the Type 2 Attachment Base.

The contents of the display register in a particular Type 3 Scanner can be accessed by an Input X'46' instruction when that scanner is selected.

Because each Type 3 Scanner has only one display register, the program should ensure that the display request bit is never on in more than one ICW in each scanner. Only in this way can the information in the display register be meaningful.

Before executing an Input X'46', the program must also ensure that enough time has elapsed to guarantee that a bit service has occurred for the interface after setting the display request bit in the ICW associated with the interface. (See also the section Display Register earlier in this chapter.)

ICW Bits 4.7, 5.0: (See ICW Bits 4.2-4.4).

ICW Bit 5.1 (Level 2 Interrupt Pending): This bit is set:

• When the interrupt priority register (IPR) assigned to the interface is already occupied by another interface. This stacks the new interrupt until the next time the-line is scanned and the IPR is not occupied.

• When the Type 3 Scanner wishes to interrupt the program to obtain another data buffer or a control byte is detected in the PDF array.

ICW Bits 5.2-5.3 (Priority Select Bits 1 and 2): These bits assign to the ICW for the interface one of the four interrupt priority registers inthe attachment base. All bit combinations are valid, with X'O' designating the lowest priority register and X'3' the highest.

ICW Bit 5.4 (NRZI Control/Transparent Text): For SDLC, when this bit is on and the PCF state is X'9' or X'B', the data that is transmitted is in NRZI mode (non-return-to-zero-inverted). When this bit is off, the data is transmitted in normal mode (with 0 and 1 bits unchanged by the scanner).

8-12

(In NRZI mode, the line level is complemented when a 0 is transmitted and unchanged when a 1 bit is transmitted. NRZI mode and zero insertion after five consecutive 1 bits ensure the occurrence of a line transition [that is, from 0 to 1 or 1 to 0] at least once every six bits regardless of the data pattern. NRZI mode is used to ensure bit synchronization when using modems that do not provide clocking for received data (or modems that are bit sensitive).

For BSC, this bit is turned on when transparent text transmission begins and is turned off when transparent text transmission ends. While transmission is in transparent text mode, normal BSC control characters are not recognized by the scanner.

This bit is set or reset from bit 0 of the serial data field (SDF) when a set mode operation is executed. SDF bit 0 is set by an Output X'46' instruction in which bit 0.0 is set. The control program should not set this bit for a BSC line.

ICW Bits 5.5-5.6 (Diagnostic Bits 0 and I): These bits are set from SDF bits 1 and 2 when a set mode operation is executed. SDF bits 1 and 2 are set by an Output X'46' instruction in which bits 0.1 and 0.2 are on.

Diagnostic bit 0 is used to place a line interface in scanner wrap mode. Diagnostic bit 1 is used to inhibit the insertion and deletion of control characters for a line address, as follows. When bit 1 is on for a transmit operation, the scanner is inhibited from inserting SYN characters, 0-bits, and flag characters. When bit 1 is on for a receive operation, the scanner is inhibited from deleting all control characters and 0-bits.

These bits are on only for diagnostic operations; for normal data transmission they should be off.

ICW Bit 5.7 (External Clock): This bit is set from SDF bit 6 (ICW bit 3.6) when a set mode operation is executed. The bit in the SDF is set by an Output X'46' instruction in which bit 1.4 is on.

This bit is used by the scanner to control the type of bit synchronization to be used for the modem attached to the line interface associated with the ICW.

ICW Byte 6: (Cycle Steal Control}: This byte contains control and addressing information for cycle steal operations of the Type 3 Scanner.

Bits 6.0-6.3 (Cycle Steal Address Byte X): These four bits contain the four high-order bits of the cycle steal address. (Note: Bits 6.0 and 6.1 (byte X bits 4 and 5) should not be set in an Output X'48' instruction; doing so will cause an address exception.)

Bit 6.4 (ETB/ETX/ENQ in data): This bit is used as an internal control by the Type 3 Scanner. The scanner sets this bit while cycle stealing data from storage upon detecting an ETB, ETX, or ENQ character in transmitted data. If this bit is on when the scanner enters transparent text mode (setting bit 5.4), the cycle steal valid bit (6.5) is set because the ETB, ETX, or ENQ is actually transparent text and not an ending control character. The control program must reset this bit when the next interrupt for the line occurs.

Bit 6.5 (Cycle Steal Valid): This bit is set by the control program to signal the Type 3 Scanner that the cycle steal address, byte count, and transmit data chain bits are valid for use by the scanner. The program must update the cycle steal address bytes before setting this bit. The scanner resets this bit when:

• The byte count reaches 0.

• An end-of-message character has been received and the scanner detects an EOM control byte in the PDF array.

• An end-of-message character is detected in the data stream when fetched from the CCU storage via a cycle-steal operation.

Note: An Output X'48' that sets cycle steal valid should always be followed by an Output X'44' to reset the SCF bits in order to prevent possible double interrupts. This sequence normally occurs just prior to exiting from a program level 2 interrupt service.

Programming Note

Care must be exercised to ensure that this bit is off before any modification is made to the LCD and PCF fields.

Bit 6.6 (Transmit Data Chain Flag): This bit is set by the control program when additional data is to be transmitted from another buffer. If this bit is off and the byte count reaches 0, indicating that the last byte of the message has been transferred to the scanner, the scanner examines the final status byte to determine which ending sequence to initiate. The level 2 interrupt is delayed until the ending sequence is completed.

Bit 6.7 (SDLC Transmit Message Chain Flag}: This bit is set by the control program (for SDLC transmission only) when the buffer currently being transmitted represents the end of the message but another message is in the transmit queue. The scanner transmits the CRC and flag characters after all message data in the parallel data fields (PDF) have been transmitted. A level 2 interrupt is made for the cycle steal address to start transmission of the next message.

IBM 3704 and 3705 Communications Controllers Principles of Operations  8-13

ICW Byte 7 (Cycle Steal Byte Count): This eight-bit field contains the data byte count of the buffer currently allocated to the ICW. In receive mode, the control program allocates a new storage buffer as needed by setting the cycle steal count to the number of character spaces the control program has made available.

In transmit mode, the program must set the cycle steal byte count to the number of data characters to be transmitted from the storage buffer. The maximum byte count is 255 bytes. The byte count should be set equal to the buffer size if the number of characters remaining to be transmitted exceeds the buffer size; the count should be set equal to the remaining number of bytes, if this value is less than the buffer size.

When the count reaches zero, the scanner issues a level 2 interrupt request according to the setting of the data chain and message chain flags. (Bit 7.0 is the high-order bit of the counter.)

During a transmit operation, the byte count is decremented by one for each data character fetched from storage and placed in the PDF array. During a receive operation, the counter is decremented by one for each character transferred from the PDF array to CCU storage.

ICW Bytes 8 and 9 (Cycle Steal Address): These two bytes contain the low-order sixteen bits of the cycle-steal address. This address (together with ICW bits 6.0-6.3) contains the current address for data to be fetched from or stored into main storage. As each byte is stored or fetched, the address is incremented by one. The Type 3 Scanner normally transfers two bytes at a time to or from storage.

ICW Bytes 10 and 11 (Block Check Characters [BCC]): The block check characters are accumulated in these two bytes for SDLC operation or for BSC operation using either the EBCDIC or ASCII codes.

Byte 12 (ICW Controls)

Bits 12.0-12.3 (Cycle Steal PDF Array Address): This field of four bits points to the PDF array address currently associated with the cycle steal data registers. In a transmit PCF state, this address is the next address to be used when transferring data from the cycle steal data register to the PDF array. An Output X'4D' instruction stores byte 0 at the location addressed by the cycle steal PDF counter and byte 1 at that location plus one. The addressed location plus two is stored back in ICW bits 12.0-12.3 for the next operation.

Bits 12.4-12.7 (PDF Address): This field of four bits points to the PDF array address currently associated with the serial data field (SDF). In a receive PCF state, this is the next address to be used when transferring data from the SDF to

the PDF. An Output X'44' instruction in which bit 0.4 is on stores byte 1 at the location specified by the PDF array counter. An Output X'4C' instruction stores eleven bits at the location specified by the PDF array counter. Neither instruction increments the counter.

When a fetch buffer signal is generated (entering level 2 or executing Output X'40' in level 3 or 4), the input register is loaded with the contents of the PDF array address pointed to by the PDF array counter. Input X'44' or X'4C' instructions are used to read byte 1 or all eleven bits of this PDF address, respectively.

ICW Byte 13 (ICW Controls)

Bit 13.0 (Sequence 0): This bit is used by the Type 3 Scanner for additional control within an extended PCF state. This bit is normally set for the first character of a two-character sequence and reset for the second character.

Bit 13.1 (Sequence1I): For SDLC receive operations, the Type 3 Scanner sets this bit to enable the cycle steal interface. This bit is not used for SDLC transmit operations.

For BSC receive operations, this bit is set when an ITB is received as the text ending character. When set, this bit prevents storing of the BCC characters when trace bit (ICW bit 0.7) is on. For BSC transmit operations, this bit is set when an ITB is transmitted as the text ending character. When this bit is set and the ending status byte (ICW byte 15) indicates skip after ITB, the Type 3 Scanner increments the PDF pointer so that the byte following the ITB is not transmitted.

Bit 13.2 (RTS Turnaround Control): The control program sets or resets this bit, which is used by the Type 3 Scanner to control the Request-to-Send (RTS) lead in the modem interface when changing from a transmit state to a receive state. When this bit is off, the RTS lead becomes inactive when the transition occurs. The control program must exercise caution when changing this bit; it should be set or reset only when the line is in a "no-op" state (PCF state 0).

Bit 13.3 (Sequence 2): For SDLC receive operations, the Type 3 Scanner sets the bit upon detecting an ending Flag character for a supervisory or non-sequence frame if no data bytes were received. This action allows the PDF array pointer to be decremented by two so that the BCC characters are not stored. At the next bit service, this bit causes the BCC to be checked and the EOM and BCC check result to be set in the PDF array as a control byte. At this time the scanner resets the bit. This bit is not used for SDLC transmit operations.

8-14

For BSC receive operations, the Type 3 Scanner sets this bit upon receiving the first data character after an SOH or STX character. If the LCD state for the line is for NCP mode this bit causes a level 2 interrupt request for a data buffer when the first non-control character is received. The bit is then reset. If the LCD state is for emulation mode (bit 2.0 is 0), the level 2 interrupt request is not made.

This bit is also set when the last BCC character is received. At the next bit service, the BCC is checked and the EOM and BCC check results are set in the PDF array as a control byte. Then the bit is reset.

This bit is unused for BSC transmit operations.

Bits 13.4-13.5 (Not Used)

Bits 13.6-13.7 (Message Counter): The Type 3 Scanner uses these bits to keep track of multiple SDLC messages that may be queued in the PDF array during a receive operation. The counter is incremented by one when the scanner detects an information or control frame. So long as the counter is not equal to zero, a cycle steal operation can be activated. When an EOM (end-of-message) status is set in the ICW, the counter is decremented by one.

The message counter is not used for SDLC transmit or for BSC operations.

ICW Byte 14 (Status Exception)

Bit 14.0 (Receive Line Signal Detected): The Type 3 Scanner sets this bit to 1 at bit service time when the CF (carrier detect) lead from the modem is active and is reset when the CF lead becomes inactive.

Bit 14.1 (SDLC Idle Detect/BSC Format Exception): For SDLC receive operations the Type 3 Scanner sets this bit upon detecting an idle-line condition while receiving a frame. ICW bit 0.0 (Abort Detect) is also set in this case. Bit 14.1 is also set if the scanner is in PCF/EPCF state X'7'/'7', ICW bit 15.3 is on, and a line-idle condition is detected. A level 2 interrupt request is also generated.

This bit is unused for SDLC transmit operations.

For BSC receive operations, this bit is set when the Type 3 Scanner is in PCF/EPCF state X'5'/'01' and an ITB, ETB, or ETX character is received, or in PCF/EPCF state X'7'/'02' and an ITB, ETB, ETX, SOH, or STX character is received, or in PCF/EPCF state X'7'/'03' and an SOH character is received.

For BSC transmit operations, this bit is set when the Type 3 Scanner is in PCF/EPCF state X'9'/'02' or X'9'/'04' and the initial or final control sequence should be set in the SDF but is not specified by ICW byte 15. If this bit is on, ICW bit 0.1 is reset.

This bit is reset by an Output X'4F' instruction in which bit 0.1 is set.

Bit 14.2 (BSC Flush): This bit is set by the control program to inform the Type 3 Scanner to flush the received data. When this bit is on, the scanner discards data received from the line until it detects an ending control sequence.

Bit 14.3 (Data Check): This bit is set on by the Type 3 Scanner upon detecting a bad BCC character in the received data stream. If this bit is on, ICW bit 0.1 is reset. The data check bit is reset by an Output X'4F' instruction in which bit 0.3 is on.

Bit 14.4 (Bad Pad/Flag Off Boundary): For BSC, this bit is set when the next character received after an ending sequence is not a valid pad character. This bit is not set, however, if the ending sequence includes BCC characters.

For SDLC, this bit is set when a flag byte detected in received data is not on a character boundary. If this bit is on, ICW bit 0.1 is reset. This bit (14.4) is reset by an Output X'4F' instruction in which bit 0.4 is set.

Bit 14.5 (Ack Expected): For SDLC, the control program sets this bit to indicate to the Type 3 Scanner to expect multiple control (C) bytes. When this bit is on, the scanner expects two control bytes to follow the address (A) byte.

For BSC, the control program sets this bit to indicate which BSC Ack response is expected. If the bit is 0, an Ack-0 is expected; if it is 1, an Ack-1 is expected. An Output X'4F' instruction is used to set or reset this bit; the state of bit 0.5 of the instruction determines the state to be placed in bit 14.5.

Bit 14.6 (DLE Sequence Error): The Type 3 Scanner sets this bit upon receiving an invalid control character following a DLE character. This bit is set by the scanner and reset by an Output X'4F' instruction in which bit 0.6 is on. (This bit is not used for SDLC operations.)

Bit 14.7 (Length Check): For SDLC, the Type 3 Scanner sets this bit upon detecting an ending flag after the address and control (AC) characters are received but before two more characters have been received.

For BSC, the Type 3 Scanner sets this bit upon detecting an ETB or ETX character in the transmitted data stream, if the LCD state does not indicate operation in emulation mode. (This bit is not set if the scanner detects ETB or ETX as part of a leading graphics sequence.) This bit is set by the scanner (when set, it forces the scanner to reset ICW bit 0.1) and is reset by an Output X'4F' instruction in which bit 0.7 is on.

ICW Byte 15 (Status Byte): For BSC, the Type 3 Scanner uses this byte during a transmit operation to determine the correct starting and ending control sequence to send, and during a receive operation to indicate to the program the control sequence that has been received. See the section

IBM 3704 and 3705 Communications Controllers Principles of Operations  8-15

ICW Status Byte later in this chapter for the meanings of the status byte states.

For SDLC transmit operation, the control program sets the status byte to specify the action the scanner is to perform at the end of an SDLC frame:

Bits 15.0-15.4: (not used in transmit operations)

Bit 15.5 (Flag): When this bit is set, the scanner transmits consecutive flag characters at the end of the SDLC frame if no line turnaround is to be executed. When the bit is reset, the scanner transmits an ending flag followed by idle characters.

Bit 15.6 (Transmit Pad): When this bit is set, the scanner sends an all-ones character (eight consecutive one bits) before turning the line around. (See also bit 15.7.)

Bit 15.7 (Line Turnaround after Transmission): When this bit is set, the scanner changes from a transmit to a receive state after sending the frame.

For SDLC receive operation, the Type 3 Scanner sets the status byte to inform the control program of unusual conditions:

Bit 15.0 (Control Exception): The scanner sets this bit when a control frame was indicated by the C (control) byte but the flag character was not received three bytes later.

Bit 15.1: (reserved)

Bit 15.2: (reserved)

Bit 15.3 (Idle): The control program sets this bit to request a level 2 interrupt if the receive line enters an idle state or the scanner detects a flag character.

Bits 15.4-15.7: (not used in receive operations)

ICW Byte 16

Bit 16.0 (New Sync): The Type 3 Scanner uses this bit to control the 'new sync' lead in the attached modem. The scanner sets this bit immediately after transmitting the first SYN character (BSC) or a flag character (SDLC) if the PCF state is X'B'. The scanner resets the bit upon transmitting the second BCC character if the PCF state is X'B'. (The program should use this bit only when the scanner is in ICW diagnostic mode.)

Bit 16.1 (Data Terminal Ready [DTR]): This bit is set or reset in accordance with the setting of ICW bit 3.4 (set/reset DTR) when a Set Mode command is executed. When this bit is on, the scanner does not force the clear-to-send condition when the line is in diagnostic mode. This bit can also be set with an Output X'45' instruction if bits 0.0 and 0.1 are set.

Bit 16.2 (OLTT Diagnostic): This bit is set or reset with an Output X'45' instruction according to the state of byte 0, bit 2 of the instruction if byte 0, bit 0 is set. When this bit (16.2) is set, the scanner does not reset the BCC characters before accumulating a new BCC. This action permits the OLTT program to transmit a message having a bad BCC value.

Bit 16.3: (reserved)

Bits 16.4-16.7 (Extended PCF): These bits contain the extended PCF field; the meanings of the states of this field are given under PCF/EPCF States later in this chapter.

Line Control Definer (LCD) States

This section describes the various states of the line control definer (LCD) field. This field is used during normal transmit and receive operations to define the hardware line control required by the type of line set associated with the interface control word.

Where the LCD states are shown in pairs, for example, X'5' and X'D', the first state applies to operation of the interface in emulation mode and the second applies to operation in network control mode.

LCD States X'0', X'1', X'2', X'7', X'8', X'A', X'B', and X'E':  (Reserved)

LCD State X'9': This state causes the Type 3 Scanner to block all characters into eight-bit bytes and to search for SDLC control characters (Flag, Abort, Idle).

LCD State X'3': This state is for Autocall interfaces only. See the Autocall Interface Operation section in this chapter.

LCD States X'4 'and X'C' (BSC EBCDIC Line Control): These states provide for serializing and deserializing eight-bit characters and searching for EBCDIC line control characters. LCD state X'4' is used for emulation mode operation in which initial and final control characters are stored in or fetched from data buffers.

8-16

LCD States X'5' and X'D' (BSC ASCII Line Control): These states have the same purpose as states X'4' and X'C' except that the scanner searches for ASCII control characters.

LCD State X'6' (BSC Transparent ASCII Line Control): This state has the same purpose as state X'5' except that it accommodates transmission of transparent text. The scanner accumulates a cyclic redundancy check (CRC) character instead of the longitudinal redundancy check (LRC) character.

LCD State XT' (Feedback Check): The Type 3 Scanner sets this state in the LCD upon detecting a feedback check error (1) during scan addressing on any of the scanner 'data in' lines from the selected LIB, or (2) when a bit service reset error is detected on the line from the selected LIB. A set mode operation for an interface that has been incorrectly configured also sets this state.

Primary Control Field (PCF) States

This section describes the states of the primary control field (PCF) and extended PCF (EPCF) for BSC and SDLC lines.

PCF State X'0' (No-Op): This PCF state causes the Type 3 Scanner to take no action (active or passive) upon subsequent scans. The scanner hardware can request a Type 3 Scanner L2 interrupt and set this PCF state for an interface if it determines that new control information is required from the control program. This PCF state can be set by the control program; however, no interrupts are generated by the interface.

PCF State X'l' (Set Mode): This PCF state causes the scanner to set and reset certain mode latches in the line interface hardware. These latches are specified by the SDF field. When setting this PCF state, the control program must ensure the integrity of the entire ICW. This may be done by first setting the PCF to state X'O' (no-op) so that the ICW will not be modified by a possible interrupt. The SDF can then be set to the proper value via Output X'46'. Finally, state X'l' (set mode) can be set into the PCF field. Execution of a set mode does not require a bit service request from the addressed interface. However, a bit service request must occur to allow the scanner to request a Type 3 Scanner L2 interrupt to end the set mode operation. The set mode operation ends when the scanner hardware sets the PCF state to X'O' (no-op).

A set mode can be executed to change the state of the data rate selector bit and the oscillator select bits without requiring a disable. However, 'data terminal ready' must remain on.

PCF State X'2' (Monitor Data Set Ready): This PCF state places the interface in a wait-for-incoming-call condition. For switched lines, this state should normally be set by the control program following a PCF state X'F' (disable) and PCF state X'l' (set mode with data terminal ready bit = 1). When an interface is in this state, the Type 3 Scanner tests the 'data set ready' lead from the common carrier or IBM line adapter for an active condition when the ICW is fetched. When data set ready is on, indicating that a call is established, the Type 3 Scanner sets PCF state X'4' (monitor phase—data set ready check off) and requests an L2 interrupt.

Though not necessary, this state can also be used for leased lines. Data set ready should be on at the first bit service request when the interface is scanned.

PCF State X'3' (Monitor Ring Indicator or Data Set Ready): This PCF state, when set by the control program, places the line interface in a wait-for-incoming-call (ring indicator on) or wait-for-manual-call-out-connection condition (data set ready on). This state must be preceded by setting PCF state X'F' (disable), or a set mode that resets data terminal ready. When the PCF state is set to X'3', the Type 3 Scanner tests the 'ring indicator' and 'data set ready' leads from the common carrier equipment for an active condition of either lead. When 'ring indicator' is active, a call is coming in and a pending connection is to be established. When either of these conditions occurs, the Type 3 Scanner sets PCF state X'O' (no-op) and places the line in a L2 interrupt pending state. This PCF state must be followed by PCF state X'l' (set mode) from the control program to set the 'data terminal ready' latch. After the Type 3 Scanner executes the set mode, it sets PCF state X'O' (no-op) and places the line in a L2 interrupt pending state. The interrupt handling program must then place the line in PCF state X'2' (monitor data set ready on), after which the operation proceeds as described in PCF State X'2 '-Monitor Data Set Ready.

PCF State X'4'/EPCF State X'0' (Monitor Phase-Data Set Ready Check Off): This PCF state is identical to PCF state X'5' (BSC—monitor phase-data set ready check on) except that the inactive condition of 'data set ready' does not signal a check condition. PCF X'4' is intended to initialize the first receive operation after a switched network call connection has been established.

PCF/EPCF States-BSC

This section contains the PCF states X'5' through X'B' and associated EPCF states for BSC lines.

IBM 3704 and 3705 Communications Controllers Principles of Operations  8-17

PCF/EPCF State X'5'/'0' (Monitor Phase-Data Set Ready Check On): This PCF/EPCF state places a BSC line into a hunt for phase condition. The SDF field is shifted each bit interval time, and the contents are examined by the scanner hardware for a comparison with the bit configuration of the 8-bit SYN character. If a compare is successful, PCF/EPCF state X'5'/'l' is set, the tag bit is inserted in the SDF, and the sequence 0 bit (ICW bit 13.0) is set.

PCF/EPCF State X'5'/'l' (Character Phase): In this state, with the Sequence 0 bit (ICW bit 13.0) set, character phase is monitored to detect a second SYN character. Upon detecting this character, the scanner hardware resets the Sequence 0 bit. The scanner then decodes the next character assembled in the SDF. Depending upon the character received the PCF/EPCF state changes to X'7'/'2' (Receive Leading Graphics), X'7'/'3' (Receive Control), X'7'/'4' (Receive Data), or (with the Sequence 0 bit off) X'7'/'6' (Receive Ending Pad).

PCF State X'7' (Receive Data): In this state, the Type 3 Scanner is in phase and is receiving control (C) and data characters. The scanner encodes the control characters and stores them in ICW byte 15, and stores data characters in main storage under cycle steal control. If the cycle steal byte count reaches zero or the scanner detects an ending control condition, the scanner generates a level 2 interrupt.

PCF/EPCF State X'7'/'2'(Receive Leading Graphics): This state is entered from state X'5'/'l' when the first character that is not a SYN or control character is assembled in the SDF. ICW bit 15.7 is set to indicate receipt of leading graphics. The scanner remains in this state, transferring characters from the SDF to the PDF at each character transfer time until it detects an ending character sequence. Upon detecting a single ending character, the scanner sets PCF/EPCF state X'7/'6' (Receive Pad) and sets the appropriate final status bits (ICW bits 15.3-15.6). Upon detecting the DLE character of a two-character ending sequence, the scanner sets PCF/EPCF state X'7'/'5' (Received DLE in Leading Graphics) (with Sequence bit 0 on).

PCF/EPCF State X'7'/'3' (Receive Control Sequence): This state is entered from state X'5'/'l' when the first non-SYN character assembled in the PDF is a DLE. The scanner changes states at the next character transfer time unless it receives another DLE. If the second character of an ACK.WACK, RVI, DISC, or Stick sequence is received, the scanner sets PCF/EPCF state X'7'/'6' (Receive Pad) (with Sequence bit 0 off) and sets the appropriate final status bits (ICW bits 15.3-15.6). If the second character is a STX, the scanner sets PCF/EPCF state X'7'/'4' (Receive Text) (with Sequence bit 0 off), resets the BCC, sets Initial status (ICW bits 15.0-15.2), and sets the transparent mode bit (ICW bit 5.4).

PCF/EPCF State X'7'/'4' (Receive Text): In this state, the Type 3 Scanner transfers data characters from the SDF to the PDFs in the PDF array, and accumulates the block check characters (BCC). When the cycle steal byte count reaches zero, the scanner makes a level 2 interrupt request to obtain the next data buffer. This state is used for both normal and transparent text transmission.

If the scanner detects a DLE in normal text, it sets Sequence bit 0 on, transfers the character to the PDF, and accumulates the BCC. If the scanner detects a DLE in transparent text, the scanner sets Sequence bit 0 and deletes the DLE. If the scanner detects an ITB, ETB, or ETX ending sequence, the scanner sets PCF/EPCF state X'7'/'5' (Receive Ending) (with Sequence bit 0 off), sets the appropriate final status (ICW bits 15.3-15.6), and accumulates the BCC. If it receives an ENQ character, the scanner sets PCF/EPCF state X'7/'6' (Receive Pad) (with Sequence bit 0 off), and sets final status. If it detects a time-out condition, the scanner sets PCF/EPCF state X'5'/'0' (Monitor Phase), sets final status, sets End of Message (ICW bit 0.5), and makes a level 2 interrupt request.

PCF/EPCF State X'7'/'5' (Receive Ending): In this state, the Type 3 Scanner is either completing the DLE sequence of a leading graphics sequence or receiving the check character of a text ending sequence.

If Sequence bit 0 is on, a DLE was received as the fast ending character of a leading graphic sequence. If the second character of an ACK, WACK, RVI, DISC, or Stick sequence is received, the scanner sets PCF/EPCF state X'7'/'6' (Receive Pad) (with Sequence bit 0 off) and sets the appropriate final status (ICW bits 15.3-15.6).

If Sequence bit 0 is off, either an LRC check character (for normal ASCII transmission) or the first BCC (for EBCDIC or transparent ASCII transmission) is being received. Upon receipt of the character, the scanner sets PCF/EPCF state X'7'/'6' (Receive BCC) (with sequence bit 0 on). If the LCD state is X'5' or X'D' (BSC ASCII line control), sequence bit 2 is set.

PCF/EPCF State X'7'/'6' (Receive BCC): In this state, the Type 3 Scanner is receiving either the second character of a BCC or (after a non-text ending) an ending pad character.

If sequence bit 0 is on (indicating a text ending), the next PCF/EPCF state entered is determined by the state of the sequence 1 bit. If this bit is on (indicating an ITB character), the next PCF/EPCF state will be X'77'4'. If sequence bit 1 is off (indicating an ETB or ETX ending), the next PCF/EPCF state will be X'5'/'0'. The sequence bit 2 is set when the last BCC character is received. For an ASCII line, this occurs on entering EPCF state X'6'; for an EBCDIC line, this occurs one character time after the last BCC character is received. On the next bit service, the BCC

8-18

is checked, an EOM and BCC result control byte is written into the PDF array, sequence bit 2 is reset, and the new EPCF state is entered.

If sequence bit 0 is off, indicating a non-text ending, the scanner determines if the next four bits are all ones, indicating a Pad character. If so, an EOM control byte is written into the PDF array PCF/EPCF state X'5'/'0' is set, and a level 2 interrupt request is generated. If not, the scanner accepts the next character, sets a bad pad status indication in the PDF array, and sets PCF/EPCF state X'7'/'2'.

PCF/EPCF State X'7'/'8' (Receive Sync Idle): In this state, the Type 3 Scanner detects the SYN SYN or DLE SYN sequence to reset the three-second timeout (for receive operations). In this state the scanner also detects continuous SYN characters or DLE SYN sequences received for more than three seconds. The scanner then sets PCF/EPCF state X'5'/'0' (Monitor Phase), sets the appropriate final status (ICW bits 15.3-15.6), sets the end-of-message bit (ICW bit 0.5), and makes a level 2 interrupt request.

PCF/EPCF State X'7'/'C' (Receive Diagnostic Mode): In this state, the Type 3 Scanner stores all characters received, including SYN and DLE characters. This state is for use by diagnostic routines as a "do-nothing" state to test the scanner hardware. The scanner does not change the PCF/EPCF state while in the diagnostic mode; the control program must make any state changes required.

PCF/EPCF State X'8'/'0' (Transmit Initial): This state is set by the control program to initiate text transmission. The program must initialize the following ICW fields:

• Transmit control (ICW byte 15)

• LCD (ICW byte 2)

• PCF/EPCF (ICW bytes 2 and 16)

• The low order cycle steal address (ICW byte 9)

• The cycle steal control (ICW byte 6) and cycle steal byte count (ICW byte 7). The cycle steal valid bit (ICW bit 6.5) must be set in order for the scanner to initiate a text transmission. (The cycle steal byte count should be set to zero if no data is to be transmitted.)

PCF State X'9' (Transmit Data): This PCF state is used to transmit message data and control characters, the specific kinds of characters being specified by the EPCF states. The scanner enters this state from PCF state X'8'/'0' when the 'clear to send' line of the modem interface becomes active.

PCF/EPCF State X'9'/'0' (Transmit Pad): In this serial state the scanner serializes the leading Pad character from the serial data field (SDF) to the line adapter. After the serializing is completed, the scanner sets the first clock sync or SYN character in the SDF.

PCF/EPCF State X'9'/'l' (Transmit Clock Sync): In this state the scanner serializes the clock sync characters from the SDF to the line adapter. After serializing two such characters, the first SYN character is placed in the SDF and the PCF/EPCF state is changed.

PCF/EPCF State X'9'/'2' (Transmit SYN): In this state the scanner serializes the initial SYN characters from the SDF to the line adapter. After doing so, the scanner places the first character to be transmitted in the SDF. This character is either obtained from the PDF array or generated by the scanner logic according to the current LCD state and ICW byte 15. The PCF/EPCF state is then changed to the appropriate state for the character placed in the SDF.

PCF/EPCF State X'9'/'3' (Transmit Control): In this state the scanner serializes a DLE character from the SDF to the line adapter. After serializing the character, the scanner places the next character to be transmitted in the SDF; the character is either obtained from the PDF array or generated by the scanner logic, as determined by the ICW control bits. The PCF/EPCF state is then set to serialize the character now in the SDF.

PCF/EPCF State X'9'/'4' (Transmit Text): In this state the Type 3 Scanner transmits message text. The scanner transfers each character from the PDF array to the SDF, from which it is serialized to the line adapter. The scanner accumulates the BCC for all characters except SYN or the first DLE character (in transparent mode). After all text characters to be transmitted have been serialized, the scanner sets the ending sequence by setting the appropriate character in the SDF and changing the PCF/EPCF state. Ending characters that the scanner detects in the data stream also cause the PCF/EPCF state to change and the ICW byte 15 to be set to indicate what ending sequence was detected.

PCF/EPCF State X'9'/'5' (Transmit End Sequence): In this state the scanner serializes the ending sequence from the SDF to the line adapter. Upon doing so, the scanner places the first BCC character or a line pad character in the SDF and changes the PCF/EPCF state.

PCF/EPCF State X'9'/'6' (Transmit BCC): In this state the scanner serializes the accumulated BCC character from the SDF to the line adapter. If the ending sequence is ITB, the last BCC character is serialized when the scanner is in PCF/EPCF state (X'9'/'4'). If the ending sequence is not ITB, an all 1's pad character is placed in the SDF after the BCC is serialized and the PCF/EPCF state is changed.

IBM 3704 and 3705 Communications Controllers Principles of Operations  8-19

PCF/EPCF State X'9'/'7' (Transmit Final Pad): In this state the scanner serializes a Pad character comprising all 1's from the SDF to the line adapter. After transmission of this character, the scanner changes the PCF/EPCF state to X'57'0', sets the end-of-message bit (ICW bit 0.5) and makes a level 2 interrupt request.

PCF/EPCF State X'9'/'8' (Transmit SYN Idle): In this state the scanner serializes the first character of a SYN Idle sequence from the SDF to the line adapter. This state is entered when a one-second transmit timeout occurs in text mode. When the SYN character is set in the SDF the PCF/EPCF state is changed to X'9'/'4' and the one-second transmit timeout is started again.

PCF/EPCF State X'9'/'C' (Transmit Diagnostic Mode): In this state the scanner inhibits recognition of characters and insertion of control characters. Data from the PDF array is transmitted to the SDF to be serialized to the line adapter. After all data has been transmitted, the scanner places the line in 'mark' state before changing the PCF/EPCF state to X'9'/'7'.

PCF State X'A' (Transmit Initial with New Sync): This state is identical to PCF state X'8' (transmit initial) except that the 'new sync' interface lead to the modem will be controlled. This state must be used only with four-wire, duplex, multipoint leased-line modems where the associated interface is the master station. All of the EPCF states described for PCF state X'8' are valid for PCF state X'A'.

PCF State X'B' (Transmit Data with New Sync): This state is identical to PCF state X'9' (transmit data) except that the 'new sync' interface lead to the modem is activated when the second SYN character is transmitted and deactivated when the trailing pad character is transmitted. Activation and deactivation are controlled by the scanner's setting of ICW bit 16.0 (new sync). This state must be used only with four-wire, duplex, multipoint leased-line modems where the associated interface is the master station. All of the EPCF states described for PCF state X'9' are valid for PCF state X'B'.

PCF/EPCF States-SDLC

This section contains the PCF states X'5' through X'F' and the associated EPCF states for SDLC lines.

PCF/EPCF State X'5'/'0' (Monitor Flag-Allow Data Set Ready Check): This state is used in conjunction with LCD state X'9' to monitor received data for an SDLC flag character. To do so, the scanner uses the three low-order bits of the ones counter (ICW bits 4.2-4.4). Upon detecting a flag character, the scanner sets a tag bit in the SDF to

block the next character into eight bits and enters PCF/EPCF state X'6'/'l'. If diagnostic bit 1 is set, the PCF/EPCF state is instead changed to X'7'/'C'.

PCF State X'6' (Receive Initiated): The Type 3 Scanner sets this state upon receiving a flag. In this state the scanner receives the address (A), control (C), and first two data characters. The scanner examines the control character to determine whether the frame is information or supervisory.

If the frame is information, the scanner stores the data. (Normally the control program has a two-byte area set up to receive the A and C characters; when these are stored, the scanner makes a level 2 interrupt to obtain a data buffer.) If the scanner does not receive four characters, the length check (ICW bit 14.7) and the EOM (ICW bit 0.5) or Abort Detect (ICW bit 0.0) bit is set and a level 2 interrupt is made.

If the frame is supervisory, on the other hand, the scanner waits for the two BCC characters and the flag that follow the control character before storing the A and C characters. If the Flag or Abort occurs before the fifth character is received in the PDF array, the scanner returns to monitoring for the flag character and does not set any error bits in the ICW. If the fifth character (not including the initial flag) is not a flag, the scanner sets the Control exception bit (ICW bit 15.0) and continues to receive data as if the frame being received were an information frame. If ICW bit 14.5 is on, the scanner adjusts the number of bytes expected to accommodate two control (C) characters.

PCF/EPCF State X'6'/'l' (Receiving Flags): In this state the scanner has received one or more consecutive flag characters. If the character assembled in the SDF is a flag, the scanner remains in this state. The first non-flag character it receives causes the scanner to change the state. Unless an abort condition is detected, the state becomes X'6'^2' (Receive Address) the character in the SDF is transferred to the PDF and the BCC is accumulated.

PCF/EPCF State X'6'/'2' (Received Address): In this state, the scanner has received the address character and the control character is currently being received into the SDF. When the character is assembled in the SDF, the scanner changes to PCF/EPCF state X'6'/'3' (Receive Control), transfers the contents of the SDF to the PDF and accumulates the BCC. The scanner interrogates the control byte and if an information (I) frame is indicated, sets sequence bit 1 (ICW bit 13.1) and increments the cycle steal message counter (ICW bits 13.6-13.7), allowing cycle steal operation to be activated. If a supervisory (S) frame is indicated, the scanner waits for an ending flag character before allowing cycle steal operation to begin. If a flag character or an abort condition is detected, the scanner does not set sequence bit 1 or increment the message counter.

8-20

The scanner decrements the PDF pointer by one, erasing the A character, and sets the PCF/EPCF state to X'6'/'l' (if a flag character was received) or X'5'/'1' (if an abort condition was detected).

PCF/EPCF State X'6'/'3' (Received Control): The scanner enters this state from state X'6'/'2'. In this state the scanner has received the control character and the next character expected is either data or the first BCC character. (If ICW bit 14.5 is on, indicating that two control characters are expected, sequence bit 0 is set upon entering this state and the second control character is received before the PCF/EPCF changes to a new state.) After the character is assembled in the SDF, the scanner changes to state X'6'/'4' (receive data/BCC) with sequence bit 0 on unless a flag character or an abort condition is detected. The scanner transfers the contents of the SDF to the PDF and accumulates the BCC characters.

Upon detecting a flag character, the scanner sets PCF/EPCF state X'6'/'l'. If sequence bit 1 is off (indicating a supervisory frame), the PDF array pointer is decremented to erase the 'AC' characters from the PDF array. If sequence bit 1 is on, the scanner sets end-of-message and length check indicators (ICW bits 0.5 and 14.7) into a control byte in the PDF array for a level 2 EOM interrupt request.

Upon detecting an abort condition, the scanner sets PCF/EPCF state X'7'/'3' (if sequence bit 1 is on) or X'5'/'0' (if sequence bit 1 is off) and adjusts the PDF array pointer to erase the 'AC' characters.

PCF/EPCF State X'6'/'4' (Received Data 1 and 2): The scanner enters this state from state X'6'/'3'. In this state, when Sequence bit 0 is on, the scanner has received the first data or BCC character and expects as the next character the second data or BCC character. Upon receiving that character, sequence bit 0 is reset, the received character is written in the PDF array, and BCC is accumulated. Following this, the next character expected is a flag (for a supervisory frame) or data (for an information frame).

Receipt of a flag character if ICW bit 13.1 is off causes the PDF pointer to be decremented by two (this erases the BCC characters from the PDF array) and sets sequence bit 2 (ICW bit 13.3). This bit causes checking of the BCC accumulation at the next bit service time, writes an EOM control byte with the BCC result into the PDF array, increments the message counter, sets sequence bit 1, and changes the PCF/EPCF state to X'7'/'5'.

Receipt of a data character causes the scanner to write the character into the PDF array, accumulate BCC, and change to state X'7'/'4'.

Upon detecting an abort condition, the scanner enters PCF/EPCF state X'7'/'3' if sequence bit 1 is on. If sequence bit 1 is off, the scanner enters state X'5'/'0' and decrements the PDF pointer to erase the characters received, beginning with the 'A' character.

PCF State X'7' (Received Data): In this state, the Type 3 Scanner is receiving information (I) frames. Data is stored under cycle steal control. If the cycle steal byte count reaches zero or the scanner detects a flag character in the received data, the scanner generates a level 2 interrupt. The scanner must receive an ending flag or detect a line idle condition to leave this PCF state.

PCF/EPCF State X'7'/'3' (Received Abort): In this state, the scanner has received an Abort character between the starting and ending flags of a frame. The scanner remains in this state until it detects either a flag character or a line idle condition. If the scanner detects a flag character, the PCF/EPCF state changes to X'7'/'5' (received ending flag);

if it detects an idle condition, the PCF/EPCF state changes to X'5'/'0' (monitor flag). In either case the scanner also sets end-of-message (ICW bit 0.5) and makes a level 2 interrupt. The abort bit (ICW bit 0.0) is also set and in the case of an idle condition—ICW bit 14.1 is set.

PCF/EPCF State X'7'/'4' (Received Data): In this state, the scanner is receiving message data (other than flag or abort characters). The data is transferred from the SDF to the PDF array and the BCC is accumulated. The PCF/EPCF state does not change while consecutive data characters are being received. If the scanner detects a flag character, the state changes to X'7'/'5' (received ending flags) or X'6'/'l' (received flag). In either case the scanner sets the EOM bit (ICW bit 0.5) and makes a level 2 interrupt request. If the scanner receives an abort character, the PCF/EPCF state changes to X'7'/'3' (received abort).

PCF/EPCF State X'7'/'5' (Receive Ending Flag): In this state the scanner has received an ending flag character. When the character is assembled in the SDF, the scanner changes state to X'6'/'l' (received flag) if a flag has been received or to X'7'/'7' if an abort character has been received. If the received character is neither a flag nor an abort character, the state changes to X'6'/'2' (received address); in this case the contents of the SDF are transferred to the PDF and the BCC is accumulated.

IBM 3704 and 3705 Communications Controllers Principles of Operations  8-21

PCF/EPCF State X'7'/'7' (Receive Idle): This state is entered by the scanner either when the scanner detects an abort character following a good message or when the control program places the scanner in this state. The program uses this state to monitor received data for flag or idle characters. Upon detecting a flag, the state changes to X'6'/'l' (received flag). Upon detecting an idle character (when ICW bit 15.3 is on), the scanner changes state to X'5'/'0' (monitor flag) and requests a level 2 interrupt.

PCF/EPCF State X'8'/'0' (Transmit Initial): This state is set by the control program to initiate text transmission. The program must initialize the following ICW fields:

• Transmit control (ICW byte 15)

• LCD (ICW byte 2)

. PCF/EPCF (ICW bytes 2 and 16)

• The low order cycle steal address (ICW byte 9)

• The cycle steal control (ICW byte 6) and cycle steal byte count (ICW byte 7). The cycle steal valid bit (ICW bit 6.5) must be set in order for the scanner to initiate a text transmission.

This state may also be used in transmitting a supervisory frame by writing the 'AC' characters into the PDF array with an Output X'4D' instruction; the cycle steal address (ICW byte 9) and cycle steal control (byte 6) need not be set.

PCF State X'9' (Transmit Data): This PCF state is used to transmit message data and control characters, the specific kinds of characters being specified by the EPCF states. The scanner enters this state from PCF state X'8'/'0' when the 'clear to send' line of the modem becomes active.

PCF/EPCF State X'9'/'0' (Transmit Pad): In this state the scanner serializes the leading Pad character from the serial data field (SDF) to the line adapter. The address and control bytes are in the PDF array awaiting transmission. Any data to be transmitted is also in the PDF array. At the time the next character is transferred the scanner places a clock sync or flag character in the SDF and changes the PCF/EPCF state.

PCF/EPCF State X'9'/'1' (Transmit Clock Sync): In this state the scanner serializes the clock sync characters from the SDF to the line adapter. After the sync characters are transmitted, the scanner places a flag character in the SDF and changes the PCF/EPCF state.

PCF/EPCF State X'9'/'2' (Transmit Flags): In this state the scanner serializes consecutive flag characters until a data character is available in the PDF for transmission. The scanner then places that character in the SDF, resets the BCC accumulation, accumulates a new BCC for that character, and changes the PCF/EPCF state. Zero bit insertion is inhibited in this state.

PCF/EPCF State X'9'/'3' (Transmit Abort): This state is entered if the scanner is to transmit an abort sequence. After transmitting the abort sequence the scanner places a flag character in the SDF, aborts transmission of the current message, sets appropriate error flags in the ICW, changes the PCF/EPCF state to X'9'/'2' to send consecutive flag characters, and makes a level 2 interrupt request.

The scanner transmits an Abort sequence if the ICW control bits indicate that more data is to be transmitted but the PDF array is empty. This condition also sets the underrun bit (ICW bit 0.2). Zero bit insertion is inhibited in this state.

PCF/EPCF State X'9'/'4' (Transmit Data): In this state the scanner transmits message text. The scanner transfers each character from the PDF array to the SDF, from which it is serialized to the line adapter. When the cycle steal byte count reaches zero and the data chain flag (ICW bit 6.6) is on, the scanner makes a level 2 interrupt request to obtain the next buffer. If the data chain flag is not on (indicating that no more data blocks are to be sent), the scanner continues to transfer characters from the PDF array to the SDF until the PDF array is empty. At the next character transfer time the scanner places the first BCC character in the SDF and changes the PCF/EPCF state. Zero bit insertion is active in this state.

PCF/EPCF State X'9'/'5' (Transmit Ending Flag): This state is entered after the BCC characters are serialized, the ending flag has been placed in the SDF, and the need for a line turnaround is indicated by ICW bit 15.7. When leaving this state, the scanner places a two- or eight-bit pad of 1 bits in the SDF, as indicated by ICW bit 15.6. Zero bit insertion is inhibited in this state.

PCF/EPCF State X'9'/'6' (Transmit BCC): In this state the scanner serializes the two BCC characters from the SDF to the line adapter. After this is completed, an ending flag character is placed in the SDF and the PCF/EPCF state is changed. Zero bit insertion is active in this state.

PCF/EPCF State X'9'/'7' (Transmit Idle): In this state the scanner transmits continuous line idle characters (all one bits) if line turnaround is not specified by ICW bit 15.7 (line turnaround after transmission). If ICW bit 15.7 does specify line turnaround, the scanner transmits two or eight consecutive one bits before changing its state to X'5'/'0'. The two bits serialized from the SDF to the line set ensure that the modem has transmitted an ending flag bit before a line turnaround occurs. Zero bit insertion is inhibited in this state.

8-22

PCF/EPCF State X'9'/'C' (Transmit Diagnostic Mode): In this state the scanner transmits an unmodified bit stream corresponding to the data obtained from storage. When all the data is transmitted, the scanner enters PCF/EPCF state X'9'/'7' before performing a line turnaround. Zero bit insertion is inhibited in this state.

PCF State X'A' (Transmit Initial with New Sync): This state is identical to PCF state X'8' (transmit initial) except that the new sync interface lead to the modem will be controlled according to the setting of ICW bit 16.0. (The scanner sets and resets this bit.) This state must be used only with four-wire, duplex multipoint leased-line modems where the associated interface is the master station. All of the EPCF states described for PCF state X'8' are valid for PCF state X'A'.

PCF State X'B' (Transmit Data with New Sync): This state is identical to PCF state X'9' (transmit data) except that the new sync interface lead to the modem is activated when the address (A) character is transmitted and deactivated when the second BCC character is transmitted. ICW bit 16.0 (set and reset by the scanner) controls the activation of the new sync lead. This state must be used only with four-wire, duplex, multipoint leased-line modems where the associated interface is the master station. All of the EPCF states described for PCF state X'9' are valid for PCF state X'B'.

PCF State X'C' (Reserved)

PCF State X'D' (Reserved)

PCF State X'E' (Transmit Continuous): The control program can use this state to transmit the same character continuously. Before setting this state, the program must set the SDF via an Output X'46' instruction and the PDF via an Output X'44' instruction (bit 0.4 must be on to allow writing in the PDF). The scanner activates the 'request-to-send (RTS) lead to the modem. When the modem signals 'clear to send' (CTS), the scanner sets sequence bit 0 and transfers the SDF content serially by bit to the line set. At each character transfer time, the character in the PDF is transferred to the SDF to be serialized. The PDF pointer is not incremented. The control program must change this state to end the continuous transmission.

PCF State X'F' (Disable): The control program sets this state to cause the Type 3 Scanner to turn off the 'data terminal ready' lead to the modem. A disable command resets all control information that was provided by the last Set Mode instruction (PCF state XT). The scanner then causes the interface to be placed in an interrupt pending state when the 'data set ready' lead and the 'receive line

signal detect' lead are deactivated. For Auto Call applications, other conditions on the automatic calling unit must be satisfied before another dial operation can be attempted following the Disable. The scanner sets PCF state X'O' (no-op) before requesting the interrupt. Because all control information in the line set is reset, the control program must set the proper control information in the line set via a Set Mode instruction (PCF state XT) issued after the Disable command.

ICW Status Byte

The Type 3 Scanner uses the status indicators in byte 15 of the ICW to determine the appropriate control sequences to send during a transmit operation. During a receive operation, the scanner uses these indicators, to inform the control program what control characters were received.

ICW Status Indicators for BSC

The format of the status byte for BSC transmit operations in network control mode is:

| Bit: | 0 1 2 | 3 4 5 6 | 7 |
| --- | --- | --- | --- |
|  | Initial | Final | Leading Graphics Flag |
|  | Control | Control |  |
|  | Sequence | Sequence |  |

For BSC transmit operations in emulation mode, the initial and final control sequences appear in the data stream received from the host processor except for transparent text ending sequences. For these sequences the program sets the final status bits (ICW bits 15.3-15.6) in the same manner as for BSC operations in network control mode.

Initial Control Sequence: These bits determine the control characters to be sent at the beginning of message transmission. If no initial sequence is to be sent, the final control sequence bits determine the response or control characters to be transmitted.

| Bit: | 0 1 2 | Initial Status (Control Sequence) |
| --- | --- | --- |
|  | 0 0 0 | (none) |
|  | 0 0 1 | STX |
|  | 0 1 0 | DLE STX |
|  | 0 1 1 | SOH |
|  | 1 0 0 | Special |
|  | 1 0 1 | (reserved) |
|  | 1 1 0 | (reserved) |
|  | 1 1 1 | (reserved) |

Final Control Sequence: These bits determine the control characters to be sent at the end of a transmission. The meanings of these bits depend on the initial control sequence bits as described above.

IBM 3704 and 3705 Communications Controllers Principles of Operations  8-23

| Bit: 012 | 3456 | Final Status (Control Sequence) |
| --- | --- | --- |
|  | 0011 | Transmit ENQ, then turn line around |
|  | 0110 | Transmit ACK-0, then turn line around |
| 000 | 0111 | Transmit NAK, then turn line around |
|  | 1101 | Transmit RVI, then turn line around |
|  | 1110 | Transmit ACK-1, then turn line around |
|  | 1111 | Transmit WACK, then turn line around |
|  | 0011 | Transmit STX ENQ (TTD), then turn line around; set byte count to 0 |
|  | 1001 | Transmit STX data ETX, then turn line around; do not skip byte for ITB in data |
| 001 | 1010 | Transmit STX data ETB, then turn line around; do not skip byte for ITB in data |
|  | 1101 | Transmit STX data ETX, then turn line around; skip byte if ITB occurs in data |
|  | 1110 | Transmit STX data ETB, then turn line around; skip byte if ITB occurs in data |
|  | 0011 | Transmit DLE STX data DLE ENQ, then turn line around |
|  | 0100 | Transmit DLE STX data DLE ITB (no line turnaround) |
| 010 | 1001 | Transmit DLE STX data DLE ETX, then turn line around |
|  | 1010 | Transmit DLE STX data DLE ETB, then turn line around |
|  | 1101 | Transmit data DLE ETX, then turn line around (ITB sent previously via 010 0100 status byte) |
|  | 1110 | Transmit data DLE ETB, then turn line around: (ITB sent previously via 010 0100 status byte) |
|  | 0011 | Transmit SOH data ENQ, then turn line around |
|  | 1001 | Transmit SOH data ETX, then turn line around; do not skip byte for ITB in data |
| 011 | 1010 | Transmit SOH data ETB, then turn line around; do not skip byte for ITB in data |
|  | 1101 | Transmit SOH data ETX, then turn line around; skip byte if ITB occurs in data (Note: If DLE STX appears in data, scanner ends data with DLE ETX.) |
|  | 1110 | Transmit SOH data ETB, then turn line around; skip byte if ITB occurs in data (Note: If DLE STX appears in data, scanner ends data with DLE ETB.) |
| 100 | 1100 | Transmit EOT, then turn line around |
|  | 1110 | Transmit DLE EOT, then turn line around |

8-24

The format of the status byte for BSC receive operations is:

| Bit: 012 | 3456 | 7 |
| --- | --- | --- |
| Initial | Final |  |
| Status | Status | Leading Graphics Flag |

Initial Status: These bits are set in accordance with the first control character or sequence received from the line interface:

| Bit:012 | Initial Status |
| --- | --- |
| 000 | Control mode status (no text was received) |
| 001 | Text Mode status (first control character is STX) |
| 010 | Transparent text mode status (DLE STX first control characters) |
| 011 | Heading status (first control character is SOH) |
| 100 | Special status (DLE EOT (Disconnect) first control characters) |
| 101 | (reserved) |
| 110 | (reserved) |
| 111 | (reserved) |

Final Status: These bits are set in accordance with the ending control character or sequence received from the line interface:

| Bit 3456 | Final Status/Character received |
| --- | --- |
| 0000 | (Timeout occurred) |
| 0001 | ITB received |
| 0010 | (reserved) |
| 0011 | ENQ received |
| 0100 | EOT received |
| 0101 | DLE x received (x = second character of any valid DLE sequence) |
| 0110 | Wrong ACK received |
| 0111 | NAK received |
| 1000 | (reserved) |
| 1001 | ETX received |
| 1010 | ETB received |
| 1011 | (reserved) |
| 1100 | (reserved) |
| 1101 | RVI received |
| 1110 | Positive ACK (ACK-0 or ACK-1) received |
| 1111 | WACK received |

Leading Graphics bit (bit 7): Bit 7 is set on a BSC receive operation if the first character of the received message is not a control character.

ICW Status Indicators for SDLC

The format of the status byte for SDLC transmit operation is:

Bits: 0 1 2 3 4 5 6 7

Line Turnaround

Line Pad/idle

Flag/idle

Bit 5: When set (1), this bit causes the scanner to transmit continuous flag characters at the end of the frame if bit 7 is not set to cause the line to turn around. When reset (0), this bit causes the scanner to assume a line idle state after sending the ending flag after a frame.

Bit 6: When set (1), this bit causes the scanner to send an all-l's character (8 consecutive 1's) before turning the line around if bit 7 is set to cause line turnaround.

Bit 7: When set (1) this bit causes the scanner to turn the line around (that is, change from a transmit to a receive state) after it transmits the ending flag character.

The format of the status byte for SDLC receive operations is:

Bit: 0  1   2   3   4 5 67

˚ ˚ ˚ ˚ ’’’’‰‰‰‰‰(reserved)

˚ ˚ ˚ ’‰‰ Request L2 interrupt on line idle or

˚ ˚ ˚ flag detect

˚ ’‰’‰‰‰‰(reserved)

’‰Control exception-received data in control frame

Bit 0: The scanner sets this bit when a control (C) frame was indicated by the C byte but the flag character was not received three bytes afterward.

Bits 1 and 2: (reserved)

Bit 3: The control program sets this bit to request that the scanner make a level 2 interrupt request if the receive line assumes an idle state or a flag character is detected.

Interface Mode Set-SDF Values

The interface hardware latches are set and reset according to the value in the SDF field during PCF state X'l'. The SDF values for a set mode instruction are as follows:

SDF bit 0: This bit is set by bit 0.0 of an Output X'46' instruction. This bit is set to 1 to place the addressed line in NRZI mode. ICW bit 5.4 is set or reset according the state of this bit when the set mode instruction is execute! The control program should not set this bit if the ICW is associated with a BSC line.

IBM 3704 and 3705 Communications Controllers Principles of Operation 8-25

SDF bit 1: This bit is set by bit 0.1 of an Output X'46' instruction. This bit is set to 1 to place the addressed line in diagnostic mode 0. ICW bit 5.5 is set or reset according to the state of this bit when the set mode instruction is executed.

SDF bit 2: This bit is set by bit 0.2 of an Output X'46' instruction. This bit is set to 1 to place the addressed line in diagnostic mode 1. ICW bit 5.6 is set or reset according to the state of this bit when the set mode instruction is executed.

SDF bit 3: This bit is set to 1 to place the addressed interface in the line adapter wrap line diagnostic mode.

SDF bit 4: Data Terminal Ready-This bit controls the data terminal ready lead to the modem; this lead must be set to enable the line interfaces provided by all line sets except those used for Auto Call operation (line set IE). ICW bit 16.1 (data terminal ready) is also set when the set mode instruction is executed.

SDF bit 5: Synchronous Bit Clock—This bit determines whether synchronous or start-stop clocking is used for the addressed interface when business machine clocking is specified. If SDF bit 6 is 1 (external clocking), this bit is ignored.

SDF bit 6: External Clocking—This bit determines whether business machine or modem clocking is used for the addressed interface; 1 indicates modem clocking and 0 indicates business machine clocking. ICW bit 5.7 (external modem clocking) is also set when the set mode instruction is executed.

SDF bit 7: Data Rate Selector-This bit selects a high speed or low speed data rate for the attached modem. A 1 = high data rate, and 0 = low data rate. If modem clocking is specified, this bit selects which of the two clock speeds in the modem is to provide the clock pulses. The low rate usually equals one-half of the high rate. In this case the business machine clock selected by the oscillator select bits must not exceed one-half the clock speed selected in the modem.

A feedback check occurs if this bit is on when the program executes a set mode to a line interface provided by line sets that allow only one data rate.

SDF bits 8 and 9: OSC Select Bits 1 and 2-The state of these two bits selects the business machine clock to be used by the addressed line interface. At least one business machine clock must be installed in each Type 3 Scanner. See the following section. Business Machine Clocks.

Programming Note

The oscillator select bits can be changed without causing a switched network connection to be broken if SDF Bit 4 (Data Terminal Ready) is set when the set mode is executed.

Business Machine Clocks

Each Type 3 Scanner must have at least one business machine clock card installed and may have two. The required clock has oscillator bit rates of 150,600, and 1200 bps (designated by oscillator select bit values 0,1, and 2). (See Figure 8-6). The optional clock may have a bit rate of 2000 or 2400 bps (designated by oscillator select value 3).

For line speeds above 2400 bps, a modem must provide clock pulses. Refer to the Introduction to the 3704 and 3705 manual for a description of the individual LIB and line set types.

The installed business machine clock used for a given line is selected under program control by executing a set mode (PCF state XT) with SDF bits 8 and 9 set to indicate the desired bit rate. Figure 8-6 shows the proper setting of the oscillator select bits to assign an installed oscillator to a given interface.

| SDF 8 | Bits 9 | Selected Business Machine | Clock |
| --- | --- | --- | --- |
| 0 0 1 1 | 0 1 0 1 | 150bps 600bps 1200 bps 2000 or 2400 bps | (OSC0) (OSC1) (OSC2) (OSC3) |

Figure 8-6. Type 3 Communication Scanner Business Machine Clock Selection

No business machine clock is selected if the oscillator select bits are set to select an uninstalled oscillator (for example, bits 8 and 9 set to 11 when only the required clock is installed).

Every interface must have a business machine clock assigned whether it is specified to be business machine-or modem-clocked. For autocall interfaces and for line interfaces that are to use modem clocking, the assigned business machine clock is used to ensure that the interface is periodically accessed. The lowest speed oscillator must always be used for an autocall interface.

The oscillator select bits are set to 0 by a reset to the scanner. Therefore, the lowest speed oscillator is initially selected, and unless a set mode is executed to select another oscillator for a given interface, the lowest speed oscillator is used.

After a power-on-reset occurs, there is a warm-up period associated with the different clocks. This period is less than one second for each of the clocks available for the Type 3 Scanner. During this warm-up period, a business machine clock cannot provide service requests.

8-26

Programming Notes

1. The oscillator select bits for a line interface can be changed without causing a switched network connection to be broken, if 'data terminal ready' is active when the Set Mode instruction is executed.

2. The business machine clock selected for a modem-clocked line interface must be less than one-half the rate of the modem clock.

I/O Programming Considerations

As a general rule, input/output instructions should be issued only when the status of the attachment buffer address register (ABAR) and the particular Type 3 Scanner ICW input register is known. An understanding of how those registers are set or loaded is needed for correct execution.

The ABAR is set under the following conditions:

1. The interface address in the highest priority program level 2 interrupt register located in the attachment base is loaded into ABAR just before program level 2 becomes the current program level.

Therefore, if an Input X'40' is executed as the first instruction in program level 2, the register specified by the R operand contains the interface address for that interrupt.

2. When the program executes an Output X'40', the interface address in the register specified by the R operand is placed in ABAR.

The ICW input register of the selected Type 3 Scanner is loaded with the contents of the ICW associated with the interface address in the ABAR when:

1. The ABAR is loaded after a program level 2 interrupt occurs.

2. The Output X'40' instruction is executed in program level 3 or 4. This enables the level 3 or 4 routines to access any portion of the selected ICW associated with the interface address in the ABAR.

Figure 8-7 summarizes which program levels can set the ABAR in the attachment base and set the ICW input register in the selected scanner.

| Program Level | ABAR | ICW Input Register |
| --- | --- | --- |
| 1 | Output X'40' | Cannot be set |
| 2 | L2 Interrupts | L2 Interrupt |
| 3 or 4 | Output X'40' | Output X'40' |

Figure 8-7. Setting ABAR and ICW Input Register

The following considerations are recommended for executing input/output instructions in the different program levels.

Program Level 1 (Error Routines)

Input X'40' can be executed to obtain the interface address from the attachment buffer address register (ABAR) in the Type 2 Attachment Base. This old interface address should be saved if a different address is required to select the Type 3 Scanner that has its LI interrupt request set.

Output X'40' can be executed to select the appropriate Type 3 Scanner if needed. Only the selected Type 3 Scanner can decode the input/output instructions. However, the scanner input register is not changed if an Output X'40' is executed at program level 1 or 2.

After the Type 3 Scanner is selected, other input and output instructions may be executed as needed. Output instructions may be executed in any order, but all output instructions (Outputs X'41' through X'4F' that set a portion of the ICW must be separated by at least one cycle. This is required because the output register in the Type 3 Scanner buffers the data from the general register and requires time to execute the instruction.

Before exiting from program level 1, the program may execute an Output X'40' to place the old interface address back in ABAR if it had been saved. However, one instruction cycle must separate it from any Output X'41'—X'4F'. The selected Type 3 Scanner ICW input register is not changed as a result of Output X'40'.

At least one instruction cycle must separate the last output instruction and an Exit instruction.

Program Level 2 (Character Service)

Input X'40' may be executed immediately to obtain the interface address. When Input X'40' is issued while in program level 2, the 'priority register occupied' latch associated with the interface address in ABAR is reset. This indicates that the character service request is being serviced and that the program level 2 interrupt priority register from which the ABAR was loaded is now available for another level 2 interrupt of the same priority. If all the 'priority register occupied' latches are reset, the Type 2 Scanner L2 interrupt request is reset. Inputs X'41'—X'4F' may be executed whenever necessary to obtain a portion of the ICW from the Type 3 Scanner ICW input register. Outputs X'41'—X'4F' may be executed whenever necessary to set a portion of the ICW.

Output instructions may be executed in any order, but all subsequent Output X'41'—X'4F' instructions must be separated by at least one cycle.

The control program should not issue an Output X'45' instruction to change the LCD and PCF fields if the cycle steal valid bit (ICW bit 6.5) is on, because the results will be unpredictable. The program must issue an Output X'48' instruction to set the cycle steal valid bit before issuing an Output X'44' instruction that resets the SCF field. The Output X'44' instruction should be the last Output instruction issued to the scanner before the Exit instruction is executed.

IBM 3704 and 3705 Communications Controllers Principles of Operations  8-27

At least one instruction cycle must separate the last output instruction and an Exit instruction.

Programming Note

Subsequent Input X'40' instructions within the same character service interrupt do not reset the 'priority register occupied' latches.

Program Levels 3 and 4 (Lower Level Routines)

Output X'7E' may be executed with a 1 in byte 1, bit 2 of the register specified by the R operand. This will 'mask off program level 2 interrupts that could change the contents of the attachment buffer address register (ABAR) in the Type 2 Attachment Base by a character service L2 interrupt.

Output X'40' may be executed to load ABAR with the interface address of a line to be acted upon. The contents of the ICW associated with this interface address are placed in that Type 3 Scanner ICW input register.

After the Type 3 Scanner is selected, (1) Output X'41'-X'4F' may be executed (to alter the associated portion of the ICW), followed by some other instruction, or (2) some other instruction must be executed, followed by Input X'41'-X'4F' (to obtain the associated portion of the ICW that was loaded by Output X'40' into the ICW input register).

If Output X'41'-X'4F' was executed as in (1) above, the ICW content was altered, but the ICW input register still contains the contents of the ICW as it was before the alteration.

Output instructions may be executed in any order, but all subsequent Output X'41'—X'4F' instructions must be separated by at least one cycle.

The control program should not issue an Output X'45' instruction to change the LCD and PCF fields if the cycle steal valid bit (ICW bit 6.5) is on, because the results will be unpredictable. The program must issue an Output X'48' instruction to set the cycle steal valid bit before issuing an Output X'44' instruction that resets the SCF field. The Output X'44' instruction should be the last Output instruction issued to the scanner before the Exit instruction is executed.

It is recommended that all lines in the addressed Type 3 Scanner be disabled before executing an Output X'42' to change the upper scan limit.

Output X'7F' may be executed with a 1 in byte 1, bit 2 of the register specified by the R operand. This unmasks the program level 2 interrupts. This output instruction must be separated by at least one instruction cycle from the last Output X'43', X'44', X'45', X'46', X'47' X'41'- 'X'4F' instruction.

Autocall Interface Operation

The Type 3 Communication Scanner supports operation of an autocall interface when the interface is attached to Line Set IE and the line control definer (LCD) field of the associated ICW is set to X'3'. The primary control field (PCF) of the ICW is used to control the interface operation. Whenever an autocall interface that has a service request is scanned, the scanner interprets the PCF to determine what communication should occur between the scanner and the interface hardware. The scanner also determines, from the PCF, whether the interface should be placed in a level 2 interrupt pending state. The lowest speed business machine clock installed in each scanner (150 bps) is used to generate service requests for all autocall interfaces installed in that scanner. Service requests are generated at the same rate as the clock speed.

Interface Control Word for Autocall Operation

The following bits/fields are used for autocall operation.

| ICW bit 0.1 | Service Request |
| --- | --- |
| ICW bits 1.4-1.7 | PDF bits 4-7 |
| ICW bits 2.0-2.3 | LCD |
| ICW bits 2.4-2.7 | PCF |
| ICW bits 3.0-3.7 | SDF bits 0-7 |
| ICW bit 4.6 | Display Request |
| ICW bit 5.1 | L2 Interrupt Pending |
| ICW bits 5.2-5.3 | Service Priority |

Programming Note

A reset to the scanner sets the PCF state of each ICW to X'0' and sets and resets the following bits:

| Sets: | 0.4 | Resets: | 2.4-2.7 |
| --- | --- | --- | --- |
|  | 4.5 |  | 4.2-4.4 |
|  | 12.0-12.7 |  | 4.6-4.7 |
|  |  |  | 5.0-5.1 |
|  |  |  | 5.5-5.7 |
|  |  |  | 6.5 |

However, when power is turned on in the controller, all other ICW bits are unpredictable. Therefore the program must ensure that during initialization ICW bits 0.0—0.3, 0.5,14.1,14.3,14.4,14.6, and 14.7 are reset.

PCF States for Autocall Operation

Five primary control field (PCF) states are available for autocall operation. Setting the PCF to an undefined value may result in improper operation. Each time the control program changes the PCF state, it should also set the LCD to X'3'. The following PCF states are valid for autocall interfaces.

8-28

PCF State X'0': Idle-This state resets the 'call request' and 'digit present' indications in the autocall interface each time the interface is scanned and a bit service is present. If the control program sets this state, no interrupt requests result from that interface until the program changes the PCF to one of the other valid states. If the scanner sets this state as a result of ending a PCF state XT' (disable), the interface is placed in a level 2 interrupt pending state.

PCF State X'4': Monitor Call ACR, COS, PND- When the Type 3 Scanner fetches an ICW for an autocall interface in this PCF state, the autocall interface is monitored for the active state of the following leads:

ACR-Abandon Call and Retry

COS-Call Originate Status

PND-Present Next Digit

When any of these leads are found to be active, the appropriate SDF bit is set, and the interface is placed in a level 2 interrupt request pending state.

PCF State X'5': Monitor Call ACR, COS-This state is the same as PCF state X'4' except the active condition of 'present next digit' (PND) does not generate a level 2 interrupt request.

This PCF state can only be set by the control program.

Programming Note

The control program must ensure that the interrupt remember bit (SDF bit 0) is reset when it places the interface in this state. Otherwise no interrupt request can be generated because of active control leads.

PCF State X'8': Digit Valid-This state is set by the control routine after it has placed the next dial digit into the PDF. This digit is continuously presented to the ACU interface until PND falls; the scanner sets the PCF to X'4', and no interrupt is requested.

PCF State X'F': Disable-This state is used to reset the dial interface at the end of the data transfer operation. After all the control leads from the Autocall unit (ACU) have been reset, the Type 3 Scanner sets the PCF to X'O' and places the interface in a level 2 interrupt request pending state.

Serial Data Field for Autocall Operation

The control program can monitor the autocall interface by interrogating the serial data field (SDF) in the interface control word. The serial data field is updated with the current status of the autocall interface each time the interface is scanned and a bit service request is present. SDF bits 1 -9 reflect the state of certain autocall control signals and have no effect on the interface operation. Bit 0 is the only SDF bit that affects operation. The following paragraphs describe each SDF bit and its meaning.

SDF Bit 0: Interrupt Remember (IR)-This bit is set by the scanner to indicate a level 2 interrupt pending state. In PCF state X'4' or X'5', the scanner monitors the respective autocall interface leads for an active condition. When one of the monitored leads becomes active, the scanner sets the interrupt remember bit to prevent further interrupts from that interface until the first interrupt has been serviced. This bit must be reset by the control program each time an interrupt is serviced in order to allow the next interrupt to be recognized.

Programming Note

The program should not reset the interrupt remember bit before changing the PCF state from X'4' or X'5' to some other state because an unexpected interrupt request may result.

SDF Bit 1: Power Indicator (PWI)-When this bit is 0 the automatic calling equipment is inoperative because of the lack of power.

SDF Bit 2: Call Request (CRQ)-A 1 in this position indicates a request to originate a call to the autocall interface. The scanner sets the CRQ in the autocall interface whenever (1) the interface is scanned, (2) a bit service request is present, and (3) the PCF state is X'4', X'5', or X'8'. If the condition of this bit does not agree with the state defined for the active PCF state, the LCD is set to XT' to indicate a feedback check.

SDF Bit 3: Data Line Occupied (DLO)-A 1 in this position indicates that the autocall interface is in use. The program should not attempt to originate a call until this lead becomes inactive.

SDF Bit 4: Present Next Digit (PND)-A 1 in this position indicates that the autocall unit is ready to accept the next digit. The PND lead is used by the autocall unit to control the presentation of digits to the unit during a dialing operation.

When the 'present next digit' lead is active and the PCF state is X'4', the scanner sets the interrupt remember bit if it is not already on. If the PND lead is inactive and the PCF state is X'8', the scanner changes the PCF state to X'4'.

SDF Bit 5: Digit Present (DPR)-A 1 in this position indi cates that a valid digit is present on the digit leads to the autocall unit. The scanner sets DPR via PCF state X'8' after present next digit comes on and the next dial digit has been placed into the parallel data field, bits 4-7. When the autocall unit turns PND off, the scanner changes the PCF state to X'4' and resets DPR. The scanner resets DPR when the PCF state is changed to X'0', X'4', X'5', or X F'. If the condition of this bit does not agree with the state defined for the active PCF state, the LCD field is set to 'F' to indicate a feedback check.

IBM 3704 and 3705 Communications Controllers Principles of Operations  8-29

SDF Bit 6: Data Set Status (DSS)/Call Originate Status (COS)—A 1 in this position indicates that a connection has been established and that the modem is in data mode and can be used for data communications. In PCF state X'4', the scanner sets the interrupt remember bit when the DSS/COS lead becomes active.

SDF Bit 7: Abandon Call and Retry (ACR)-A 1 in this position indicates that a preset time interval in the autocall unit has elapsed since the last change of the 'present next digit' lead. This bit only alerts the control program to the time-out condition; it does not automatically abandon the call and retry. The control program is responsible for abandoning the call and retrying. In PCF state X'4', the scanner sets the interrupt remember bit when the ACR lead becomes active.

SDF Bits 8 and 9: These bits are not used.

PDF for Autocall Operation

For autocall operation the parallel data field is used to present the dial digits to the automatic calling unit. When the autocall interface is in PCF state X'8', the parallel data field bits 4-7 must contain a valid digit. In any PCF state other than X'8', the PDF is treated as if it contained all zeros. The valid digits that can be loaded into the PDF are from X'0' to X'9' and X'C' and X'D'. X'0' to X'9' represent the value of the dial digit and X'C' is an end of number bit configuration used to inform the autocall unit that the last digit of the called number has been provided. X'D' is a separator bit configuration used to inform the autocalling unit to wait for a second dial tone.

Note: The external automatic calling unit (ACU) must have the appropriate features to use X'C'and X'D'.

Input/Output Instructions

The Type 3 Scanner input/output instructions enable the program to communicate between line interface bases (LIBs), program interrupt levels, interface control words (ICWs), and Type 3 Scanner registers. Some of the major functions of the I/O instructions are to:

• Determine the interface address that caused a program level 2 interrupt.

• Determine the cause of a program level 1 interrupt once the scanner causing the interrupt has been identified (Input X'76' has been executed).

• Determine the status of a particular ICW.

• Determine the status of a Type 3 Scanner display register.

• Set the attachment buffer address register (ABAR) with the interface address required for:

a) addressing a particular ICW in program levels 3 / 4.

b) restoring an old. ABAR address that had been saved while in the error routines of a program level 1 interrupt. This allows the program to resume normal

operation with the same interface address in the ABAR as when the program level 1 interrupt occurred.

c) addressing a particular scanner in program level 1,3, or 4.

• Set and reset bits in a particular ICW.

• Set scan limits in a particular scanner.

• Set control bits in a particular scanner.

• Set or read the cycle steal controls (address and byte count, ICW bytes 6-9) of a particular ICW.

• Access any of the parallel data field in the PDF array.

• Set the high speed select register in a particular scanner while setting the substitution control register (SCR) in the Type 2 Attachment Base.

Programming Note

Input/Output instructions are privileged instructions executable only at program level 1, 2, 3, or 4. Any attempt to execute these instructions improperly causes a program level 1 interrupt request by setting the 'input/output check LI' latch. Refer to Input {Output Check description in Chapter 5 for the conditions that cause the check to be set.

Input Instructions

Input instructions in the range X'40'—X'4F' allow the program to obtain the status of the ICW input register, display register, and error register in the Type 3 Scanner, and the interface address in the attachment base ABAR. (Appendix B defines the bits within each input instruction.)

Input instructions should be executed only when the status of the attachment buffer address register (ABAR) and the content of the ICW input register are known.

Programming Notes

1. With Extended Addressing, byte X of all input instructions and external registers is set to zero.

2. When an autocall interface is used, some of the input instructions have different bit definitions. Refer to the individual instruction descriptions for these differences.

Input X'40' (Interface Address): This instruction is used to obtain the line interface address from the ABAR in the attachment base. Conditions that set the ABAR are described in the I/O Programming Considerations section in this chapter.

If Input X'40' is issued during program level 2, the 'priority register occupied' latch associated with the interface address in the ABAR is reset. This indicates that the character service request is being serviced and that the program level 2 interrupt priority register from which the ABAR was loaded is now available for another level 2 interrupt of the same priority. If all the 'priority register occupied' latches are reset, the Type 3 Scanner L2 interrupt request is reset.

8-30

Programming Note

Subsequent Input X'40' instructions within the same character service interrupt do not reset the 'priority register occupied' latches.

Input X'41' (High Speed Select): This instruction is used to obtain the settings of the high speed select register bits 0 through 7 for the scanner selected by the address in the ABAR.

Input X'42' (DBAR/Check Register): This instruction is used to obtain the contents or value of the DBAR, the scan limit select bits, the PDF array parity error bit, and the ICW parity error bits for the scanner selected by the address in the ABAR. For any parity error, PDF array error, or LIB select error, the content of the DBAR is the binary address of the ICW selected at the time the error occurred.

Input X'43' (Check Register 1): This instruction may be used to obtain the status of the check register of a Type 3 scanner. Since it is possible for up to four Type 3 scanners to be installed in the controller, the check register selected is determined by the interface address in the ABAR at time of instruction execution.

Programming Note

If any of the check register bits in the Type 3 Scanner are set to 1, the Type 3 Scanner LI interrupt request is set.

Input X'44' (ICW Input Register - Bytes 0-1): This instruction may be used to determine the state of the secondary control field (SCF) and the parallel data field (PDF) in the ICW that is set in the ICW input register. The interface address in the ABAR selects the proper scanner. Refer to I/O Programming Considerations for conditions that set the ICW input register. The SCF and PDF fields and bit definitions are described in the Interface Control Word Format section of this chapter.

Input X'45' (ICW Input Register - Bytes 2-3): This instruction may be used to determine the state of the LCD and PCF fields and SDF bits 0-7 in the ICW that is set in the ICW input register. The interface address in the ABAR selects the proper scanner. Refer to I/O Programming Considerations in this chapter for conditions that set the ICW input register. The LCD, PCF, and SDF fields and bit definitions are described in the Interface Control Word Format section of this chapter.

Input X'46' (Display Register): This instruction may be used to determine the state of the display register in the Type 3 Scanner selected by the interface address in ABAR.

The program, under control of the display request bit (ICW bit 4.6), can cause status information for a particular interface to be placed into the Type 3 Scanner display register when the interface is scanned. Input X'46' can then be used to examine this status information. The display register normally contains the status of the clear to send, ring indicator, data set ready, receive line signal detect, receive data bit buffer, diagnostic wrap mode, bit service request, and ICW diagnostic mode indicators for the line interface (or contains the Autocall unit indicators, for an Autocall interface). Alternatively, the display register contains the status of the scanner-to-line set data out lines (if the scanner is in diagnostic 0 mode [ICW bit 5.5 is on]) or the line set-to-scanner data in lines (if a feedback check has occurred for the line Interface [LCD state is X'7' or X'F'').

Input X'47' (ICW Input Register - Bytes 4-5): This instruction may be used to determine the state of SDF bits 8-9 (ICW bits 4.0-4.1) display request bit (bit 4.6) L2 interrupt pending bit (bit 5.1) and priority select bits 1-2 (bits 5.2-5.3). The ones counter (4.2-4.4,4.7, 5.0) the last line state (4.5) the NRZI control bit (5.4) diagnostic 0-1 bits (5.5-5.6), and external modem clock (5.7) can also be examined by using this instruction. The interface address in the ABAR selects the proper scanner and associated ICW. See I/O Programming Considerations in this chapter for conditions that set the ICW input register. For an interpretation of these bits, see the Interface Control Word Format section of this chapter.

Input X'48' (ICW Input Register-Bytes 6-7): This instruction may be used to determine the state of the Cycle Steal Address byte X and byte count field, and the ETB/ETX/ENQ-in-data, cycle steal valid, data chain flag, and message chain flag bits. The interface address in the ABAR selects the proper scanner and the associated ICW. Refer to I/O Programming Considerations in this chapter for conditions that set the ICW input register. For an interpretation of the bytes and bits mentioned above, see Interface Control Word Format in this chapter.

IBM 3704 and 3705 Communications Controllers Principles of Operation  8-31

Input X'49' (ICW Input Register-Bytes 8-9): This instruction may be used to determine the contents of the cycle steal address bytes 0 and 1. The interface address in the ABAR selects the proper scanner and the associated ICW. Refer to I/O Programming Considerations in this chapter for conditions that set the ICW input register.

Input X'4A' (ICW Input Register-Bytes 10-11): This instruction may be used to determine the state of the block check character bytes 1 and 2 in the ICW. The interface address in the ABAR selects the proper scanner and the associated ICW. Refer to 7/0 Programming Considerations in this chapter for conditions that set the ICW input register.

Input X'4B' (ICW Input Register-Byte 16): This instruction may be used to determine the state of the new sync, data terminal ready, and OLTT diagnostic bits and the extended PCF of the ICW. The interface address in the ABAR selects the proper scanner and the associated ICW. Refer to 110 Programming Considerations in this chapter for conditions that set the ICW input register. For an interpretation of the bits and extended PCF mentioned above, see Interface Control Word Format in this chapter.

Input X'4C' (ICW Input Register-Byte 1 and control bits): This instruction may be used to determine the contents of the PDF in the PDF array that is addressed by the PDF pointer, including the control bits. The interface address in the ABAR selects the proper scanner and the associated ICW. For an interpretation of the control bits of the PDF, see PDF Array Format in this chapter.

Input X'4E'( ICW Input Register-Bytes 12-13): This instruction may be used to determine the contents of the cycle steal-PDF array and PDF array pointers, the sequence and cycle steal message count fields, and the state of the request-to-send turnaround control bit. The interface address in the ABAR selects the proper scanner and the associated ICW. Refer to I/O Programming Considerations in this chapter for conditions that set the ICW input register. For an interpretation of the fields and bits mentioned above, see Interface Control Word Format in this chapter.

Input X'4F' (ICW Input Register-Bytes 14-15): This instruction may be used to determine the state of the status fields and of the following bits: receive line signal detect, format exception/idle detect, BSC flush, data check, bad pad/flag off boundary, ACK expected, leading DLE error, and length check, and leading graphics bits of the ICW. The interface address in the ABAR selects the proper scan

ner and the associated ICW. Refer to I/O Programming Considerations in this chapter for conditions that set the ICW input register. For an interpretation of the fields and bits mentioned above, see Interface Control Word Format in this chapter.

Output Instructions

Output instructions in the range X'40'-X'4F' allow the program to set the status of the ICW and to set the upper scan limit and certain other controls in the Type 3 Scanner. The interface address in the ABAR and the substitution control register in the Type 2 Attachment Base may also be set. (Appendix B defines the bits within each output instruction.)

Programming Note

With Extended Addressing, byte X of all output instructions and external registers has no significance and can be ignored.

Output X'40' (Interface Address): This instruction may be used to set an interface address in the attachment buffer address register (ABAR) of the Type 3 Attachment Base. When this instruction is executed, byte 0, bit 6 through byte 1, bit 6 in the register specified by the R operand are placed in the ABAR.

The interface address placed in ABAR selects the Type 3 Scanner and the ICW associated with that address. Each CCU clock time, the 16 bytes of the ICW are placed in the ICW work register. If Output X'40' is executed in program level 3 or 4 the contents of the ICW work register are placed in the ICW input register where it is available for Inputs X'41'-X'4F'.

Output X'41' (Scan Substitution Control/High Speed Select): This instruction is used to set the substitution control register in the Type 2 Attachment Base and the high speed select registers in the selected Type 3 Scanner. See Address Substitution and High Speed Select Option in this chapter for the description and coding of the address substitution and high speed select mask bits.

Programming Note

Use of address substitution for a 3705 having one or more Type 3 Scanners installed is not recommended, because each substitution control register (SCR) bit used causes one high speed line position on each Type 3 Scanner to be unavailable for use. The high speed select option should be used instead because the high speed select mask affects only one scanner, not all installed scanners. This independence allows more flexibility in selecting addresses to receive the increased scanning.

8-32

Output X'42' (Upper Scan Limit Control): This instruction must be used to set the upper scan limit in the selected Type 3 Scanner. At least one Output X'42' must be executed for each Type 3 Scanner available. The scanner selected is determined by the interface address in the attachment buffer address register (ABAR) of the attachment base at the time of execution. This instruction also sets the diagnostic buffer address register (DBAR).

Output X'43' (Control): This instruction may be executed to set or reset various control functions in a Type 3 Scanner. The Type 3 Scanner is selected by the interface address in the attachment buffer address register (ABAR) of the attachment base.

Output X'44' (ICW Bytes 0 and 1): This instruction may be used to reset secondary control field (SCF) bits 0-3 and 5, to set or reset SCF bits 6-7, and to set SCF bit 4. It is also used to set or reset the parallel data field (PDF) if bit 0.4 is on in the register specified by the Output instruction or if the LCD state is for an Autocall interface. This instruction does not change the PDF array address specified by the PDF pointer (ICW bits 12.4-12.7). For a detailed description of these bits, see the Interface Control Word Format section of this chapter. The interface address in the attachment buffer address register (ABAR), located in the Type 2 Attachment Base, selects the Type 3 Scanner and the ICW associated with this address. Refer to the secondary control field of the ICW for an interpretation of the SCF bits (ICW bits 0.0-0.7). See Interface Control Word Format for the PDF format as it relates to various line control definer states.

Output X'45' (ICW Bytes 2 and 16): This instruction may be used to set the bits of the line control definer (LCD) and the primary control field (PCF) of the ICW. The interface address in the ABAR at execution time selects the proper scanner and the associated ICW. This instruction is also used to set the extended PCF and other control bits in ICW byte 16.

For a detailed interpretation of these bits, see the Interface Control Word Format section of this chapter.

Output X'46' (ICW Byte 3 and Bits 4.0-4.1): This instruction is used to set the bits (0-9) of the serial data field (SDF) and ICW bits 5.4-5.7 and 16.1 from bits in the SDF when a Set Mode PCF is executed. The interface address in the ABAR at execution time selects the proper scanner and the associated ICW. For a detailed interpretation of the SDF bits, see the Interface Control Word Format section of this chapter.

Output X'47 '(ICW'Bits 4.2-4.5, 4.7, 5.0-5.3): This instruction is used to set the state of the ones counter (bits 4.2-4.4, 4.7, 5.0), last line state/timeout control (bit 4.5), display .request (4.6), and priority select bits 1 and 2 (5.2-5.3). The interface address in the ABAR at execution time selects the proper scanner and its associated ICW. For a detailed interpretation of these bits, see the Interface Control Word Format section of this chapter.

Output X'48' (ICW Bytes 6- 7): This instruction may be used to set the bits of the cycle steal address byte X and the byte count, and the ETB/ETX/ENQ-in-data, cycle steal valid, data chain flag, and message chain flag bits. The interface address in the ABAR selects the proper scanner and the associated ICW. For an interpretation of the bytes and bits mentioned above, see Interface Control Word Format in this chapter. (Note: The control program should not set bits 0.0 and 0.1 of the Output X'48' instruction; these bits must be 0.)

Output X'49' (ICW Bytes 8-9): This instruction may be used to change the contents of cycle steal address bytes 0 and 1. The interface address in the ABAR selects the proper scanner and the associated ICW. For an interpretation of the bytes mentioned above, see Interface Control Word Format in this chapter.

Output X'4A' (ICW Bytes 10-11): This instruction may be used to change the block check character bytes 1 and 2 in the ICW. The interface address in the ABAR selects the proper scanner and the associated ICW. For an interpretation of the bytes mentioned above, see Interface Control Word Format in this chapter.

Output X'4C' (ICW Byte 1): This instruction may be used to write a control byte in the PDF array at the address specified by the PDF pointer (ICW bits 12.4-12.7). The PDF pointer is not changed by this instruction. The interface address in the ABAR selects the proper scanner and the associated ICW. For an interpretation of ICW byte 1, see PDF Array Format in this chapter.

Output X'4D' (ICW Byte 1): This instruction may be used to write two bytes in the PDF array at the address specified by the cycle steal PDF array pointer (ICW bits 12.0-12.3) and at that address plus one. (This instruction is normally used to start SDLC address (A) and control (C) characters in the PDF array.) The instruction places byte 0 in the first addrsss and byte 1 in the second address. Execution of this instruction

IBM 3704 and 3705 Communications Controllers Principles of Operation  8-33

causes the cycle steal PDF array pointer to be incremented by two. The interface address in the ABAR selects the proper scanner and the associated ICW. For an interpretation of the bytes mentioned above, see PDF Array Format in this chapter.

Output X'4E'( ICW Bytes 12-13): This instruction may be used to set the contents of the cycle steal PDF array pointer and the PDF array pointer, the sequence and cycle steal message count fields, and the state of the request-to-send turnaround control bit. The interface address in the ABAR selects the proper scanner and the associated ICW. For an interpretation of the fields and bits mentioned above, see Interface Control Word Format in this chapter.

Output X'4F' (ICW Bytes 14-15): This instruction may be used to reset ICW bits 14.0, 14.1, 14.3, 14.4, 14.6, and 14.7. The remaining bits of byte 14 and all bits of byte 15 can be set or reset according to the state of the bits in the register specified by the Output instruction. For an interpretation of these bits, see Interface Control Word Format in this chapter.

### Diagnostic Functions

The Type 3 Communication Scanner has three diagnostic functions available to the control program: (1) the diagnostic wrap test, (2) the IBM modem wrap test, and (3) the ICW diagnostic test. These tests are run under the control of the scanner program and provide online testing as described in the following sections. These diagnostics cannot be executed for an autocall interface.

For line interfaces attached through any one scanner, either a diagnostic wrap test or a modem wrap test can be performed, but not simultaneously. However, these tests can be performed on two different scanners simultaneously.

Diagnostic Wrap Test

The Type 3 Scanner diagnostic wrap test provides a means of locating defects in the line control logic and line-interface transmit and receive logic. It also provides a method of online program testing. Diagnostic wrap tests can be performed online without affecting the normal program operation or the lines not in diagnostic mode. Three variations, or modes, of the diagnostic wrap test are available; these are specified by the settings of the diagnostic bits 0 and 1 (ICW bits 3.1 (5.5) and 3.2 (5.6), and the diagnostic mode bit (ICW bit 3.3).

In normal diagnostic mode (ICW bit 3.3 on), test data is transmitted from the scanner to one line interface

and returned over another line interface. This test requires one installed line interface (line set) to act as the transmit line and one or more other line interfaces (line sets) to act as receive lines. Any line interface installed in the Type 3 Scanner can be a transmit or a receive line; however, only one transmit line at a time in a scanner may be engaged in transmitting diagnostic test data.

In diagnostic 0 mode (diagnostic bit 0 on), the test data is wrapped internally within the scanner, instead of being passed from one line interface to the other. A receive line address and a transmit line address must be specified as in the normal diagnostic mode test, but no line sets need be installed at those addresses. The diagnostic 0 mode test allows the control program to test the Type 3 Scanner data out leads and to transmit data directly from one ICW to another within the same scanner. After the lines have been placed in diagnostic 0 mode, all normal line functions may be simulated.

In diagnostic 1 mode (diagnostic bit 1 on), the scanner is inhibited from modifying or reacting to the transmitted or received data stream. That is, it does not insert SYN characters, 0-bits, or SDLC flag characters into data it is transmitting over the transmit line, does not delete any control characters or 0-bits from data it is receiving over the receive line, and does not perform character translation on either transmitted or received data. Diagnostic 1 mode may be used in combination with either normal diagnostic mode or diagnostic 0 mode.

Test is initiated under program control by executing Output X'45' and Output X'46' instructions to all lines to be tested. The Output X'45' instruction is executed with byte 1, bits 0-3 set for proper line control and byte 1, bit 7 set to 1 to indicate PCF state X'l' (set mode). The remaining bits of this Output instruction are set to 0. See PCF state X'l' in this chapter for further information on set mode.

Output X'46' must be set as follows:

Byte 0, bit 0: NRZI (ICW bit 3.0): When set to 1, this bit causes bit 5.4 to be set. This bit is used to select NRZI operation of an SDLC line.

Byte 0, bit 1: Diagnostic bit 0 (ICW bit 3.1): When set to 1, this bit causes ICW bit 5.5 to be set, which in turn causes data to be transferred (wrapped) from the transmit line to the receive line at the scanner—LIB interface. (When the bit is set to 0, the transfer occurs at the line set if byte 1, bit 1 is set.) This allows the functioning of a line interface to be tested even if no line set is installed at that interface.

8-34

Byte 0, bit 2: Diagnostic bit 1 (ICW bit 3.2): When set to 1, this bit causes ICW bit 5.6 to be set, which in turn causes the scanner to assume a "do-nothing" state for transmit and receive operations. Recognition of control characters and insertion and deletion of zeros are inhibited when the scanner is in this state, thus allowing the program to control the bit stream when the scanner is in transmit mode.

Byte 0, bits 3-7: These bits are 0.

Byte 1, bit 0: This bit is 0.

Byte 1, bit 1: Diagnostic Mode (ICW bit 3.3)-This bit must be set to 1 for the normal diagnostic mode test.

Byte 1, bit 2: Data Terminal Ready (ICW bit 3.4)-This bit must be set to 0.

Byte 1, bit 3: Synchronous Clock (ICW bit 3.5)-This bit must be set to 1.

Byte 1, bit 4: External Clock (ICW bit 3.6)-This bit must be set to 0 to select a business machine clock.

Byte 1, bit 5: Data Rate Select (ICW bit 3.7)-This bit may be either 1 or 0.

Byte 1, bits 6-7: Oscillator Select 1 & 2 (ICW bits 4.0-4.1)—These bits are set to select an available line oscillator (business machine clock). For proper setting, see Business Machine Clocks in this chapter. All wrap test lines must select the same oscillator.

After the set modes are issued, the affected line interfaces can be exercised through any sequence of point-to-point or multipoint operations.

During diagnostic wrap operations both the 'data set ready' lead and the 'clear to send' lead are simulated active to prevent the Type 3 Scanner from setting a modem check (ICW bit 0.3).

The control program should set ICW bit 13.2 (RTS turnaround control) if half-duplex operation is required.

Programming Notes

1. Only one interface per Type 3 Scanner may be in a transmit state at any given time during the operation.

2. The line used for transmit should be the last line to be issued the set mode instruction.

3. Diagnostic wrap tests cannot be executed on an autocall interface.

4. During a diagnostic wrap operation, the line control definer (LCD) of the line or lines in diagnostic receive mode must agree with the LCD of the line in diagnostic transmit mode regardless of the common-carrier or IBM equipment physically attached to the line.

Modem Wrap Test

The modem wrap test checks the scrambler circuits of IBM 3872, 3874, and 3875 modems under program control. The modem test can be performed online without affecting the normal operation of other lines, and may also be performed simultaneously on any or all interfaces for which the test function is defined.

To execute the modem wrap test, the control program must execute an Output X'46' instruction to set the interface control word SDF as follows:

SDF bit 0: When set to 1, this bit causes ICW bit 5.4 (NRZI control) to be set. This bit can be set to 1 to cause the SDLC line to operate in NRZI mode. For non-NRZI operation this bit should be 0.

SDF bit 1: When set to 1, this bit causes ICW bit 5.5 (diagnostic bit 0) to be set. This bit must be 0 to conduct a modem wrap test.

SDF bit 2: When set to 1, this bit causes ICW bit 5.6 (diagnostic bit 1) to be set. This bit should be set to 1 to conduct a normal modem wrap test; it can be set to 0 for a wrap test of a full-duplex modem.

SDF bit 3: This bit must be set to 1 to cause the 'diagnostic mode' latch to be set in the line interface hardware, which conditions the modem for testing.

SDF bit 4: This bit must be set to 1 to activate 'data terminal ready'. When this is set, together with the diagnostic bit, a modem wrap test can be performed instead of a diagnostic wrap test.

SDF bit 5: This bit must be set to 1 to provide synchronous clocking.

SDF bit 6: This bit must be set to 1 if the modem provides the clocking and to 0 if the modem does not provide the clocking.

SDF bit 7: This bit may be either 0 or 1.

SDF bits 8 and 9: These bits must select an internal oscillator whose speed is less than one-half the clock speed if the IBM modem provides the clocking. If the modem does not provide the clocking, the bits must select an internal oscillator whose speed matches that of the modem. For the proper setting, see Business Machine Clocks in this chapter.

IBM 3704 and 3705 Communications Controllers Principles of Operation  8-35

After the set mode instruction is executed, the interface should be put in PCF state X'8' (transmit initial) to cause 'request to send' to be raised. Before this is done, the control program should set the RTS turnaround control (ICW bit 13.2) to allow a line turnaround to occur with the 'clear-to-send' lead active. Upon detecting 'clear to send,' the scanner transmits one pad character and then (assuming that the PDF array is empty) continuous marks, turns the line around (the scanner enters PCF state X'5'), and makes a level 2 interrupt request to the control program.

The program can now test the operation of the modem scrambler circuits by setting the PCF to X'7' (receiving in-phase) and checking the received data for all marks for a period of at least one second.

ICW Diagnostic Mode Test

The ICW diagnostic mode test permits the ICW local store to be exercised as a memory element. In this mode, the ABAR is used an an address register to allow access to the bits of a selected ICW; diagnostic programs may then set and reset the ICW bits as desired.

To place the scanner in diagnostic mode, an Output X'43' instruction in which bits 0.0 and 0.6 of the register specified by the R field are on must be executed. This instruction disables the scanner and places it in diagnostic mode. Individual bits can then be changed by placing the desired ICW address in the ABAR with an Output X'40' instruction and then using appropriate Output instructions to set and reset the bits. Figure 8-5 shows which Output instructions are associated with various ICW fields.

To restore the scanner to normal mode requires an Output X'43' instruction that sets the appropriate bits.

8-36