### Chapter 5: Central Control Unit

This chapter is intended to give the reader a basic understanding of the operation of the Central Control Unit and the requirements necessary to program its operation.

The Central Control Unit (CCU) contains the circuits and data flow paths needed (1) to execute the instruction set, (2) to address storage, (3) to perform arithmetic and logical processing of data, and (4) to control the attached adapters. Operation of the CCU is under control of the programs in storage.

The data flow in the CCU is of a general hardware nature. Data flow for a particular operation is determined by the instruction, cycle steal, or control operation being executed.

### CCU Registers

The CCU contains the 32 general registers used by the control program for instruction execution and data handling. It also contains various hardware registers that are used to store and pass information essential to the controller operation. Some of these hardware registers are available to the control program as external register addresses through the use of input and output instructions. These registers are described in the following paragraphs.

- CRC Register

When an Input X'7B' or X'7C' instruction is executed, this register is loaded with the new Cyclic Redundancy Check character and is loaded into the general register specified by the input instruction. The contents of the general register is a current CRC character that is the combination of the old-CRC register and the CRC data register. See the Cyclic Redundancy Check section of this chapter for a complete description of its use.

- CRC Data Register

The CRC Data register is loaded with the next data character to be included in the calculation of a Cyclic Redundancy Check character. The contents of this register are then combined (by hardware) with the contents of the old-CRC register to form the new CRC character. See the Cyclic Redundancy Check section of this chapter for a complete description of its use.

- Display Register 1 (DR1)

Display Register 1 is used as temporary storage for data to be displayed in the Display A lights on the control panel. The register is displayed on a 3705 whenever the Display/Function Select switch is in the

STORAGE ADDRESS, REGISTER ADDRESS, or any of the FUNCTION 1-6 positions. On a 3704, the Display Register 1 and 2 push button must be active. This register can be loaded by pressing the Store or Set Address/Display push button on the control panel; or it can be loaded with data by executing an Output X'71'instruction. See the 3704 or 3705 Control Panel Guide for information on the operations that set this register.

- Display Register 2 (DR2)

Display Register 2 is used as temporary storage for data to be displayed in the Display B lights on the control panel. The register is displayed on a 3705 whenever the Display/Function Select switch is in the STORAGE ADDRESS, REGISTER ADDRESS, or any of the FUNCTION 1-6 positions. On a 3704, the Display Register 1 and 2 push button must be active. This register can be loaded by pressing the Store or Set Address/Display push button on the control panel; or it can be loaded with data by executing an Output X'72' instruction. See the 3704 or 3705 Control Panel Guide for information on the operations that set this register.

- Lagging Address Register (LAR)

The Lagging Address Register is a "came from" register. It normally contains the address of the last instruction executed prior to the instruction that is currently being executed. The LAR is loaded from the storage address register each time an instruction is executed in program levels 2, 3, 4, and 5. A program level 1 interrupt prevents setting the LAR until the Exit instruction is executed and no other requests for level 1 are outstanding. This action preserves the address of the last instruction executed before a level 1 request. Since level 1 interrupts are caused primarily by the detection of an error condition, this register becomes extremely important for error recovery procedures. Figure 5-1 shows the contents of LAR after a check condition or a control panel operation other than normal instruction execution.

The control program can load the contents of LAR into a general register by executing an Input X'74' instruction. The control program can then either examine the contents of the general register or display the address on the control panel by using the general register as input to the display registers.

IBM 3704 and 3705 Communications Controllers Principles of Operation  5-1

| ##### CONDITION | ##### LAR CONTENTS |
| --- | --- |
| Invalid Op Code Check | Address of last instruction executed before the one that caused the check (see note) |
| Protection Check or Address Exception Check | Address of last instruction executed before the one that caused the check (see note) |
|  | -or- |
|  | Address of the instruction that caused the check |
| In/Out Check at Level 2,3,or 4 | Address of the input or output instruction that caused the check |
| In/Out Check at Level 5 | Address of last instruction executed before the one that caused the check (see note) |
| IPL (including CCU check) | Address of last instruction executed before IPL phase 1 |
| Adapter Check | Unpredictable |
| Control Panel | Operations |
| LOAD ADDRESS COMPARE, PROGRAM STOP or INTERRUPT | Address of last instruction executed before the one whose address is set in switches A to E (3705) or B to E (3704) (see note) |
| LOAD or STORE, ADDRESS COMPARE, PROGRAM STOP or INTERRUPT | Address of instruction that was loaded from or stored into the location set in switches A to E (3705) or B to E (3704) |
| INSTRUCTION STEP | Address of last instruction executed |
| STOP Push Button | Address of last instruction executed |

Note: The last instruction may have been an Exit instruction executing at a higher priority program level than the level executing at the time the condition occurred. Therefore, LAR contains the address of that Exit instruction.

- Old-CRC Register

The Old-CRC accumulation register is used as a temporary storage register in the calculation of a Cyclic Redundancy Check (CRC) character. When an Input X'7B' instruction is executed, the contents of this register are combined with the character in the CRC data register to form a new CRC character. This register is loaded by any Load Halfword instruction. See the Cyclic Redundancy Check section of this chapter for a complete description of its use.

- Operation Register (OP Reg)

The Operation Register is used to hold the first 16 bits of the instruction being executed. This register can be displayed in the Display B lights on the control panel.

- Storage Address Register (SAR)

The Storage Address Register contains the storage address currently used by the program to read or write data to and from storage. This register is loaded from the temporary address register for instruction execution or from the cycle-steal address bus for cycle-steal operations. The contents of SAR are displayed on the 3704 control panel when the SAR and Op Register push button is active.

- Temporary Address Register (TAR)

The Temporary Address Register holds the storage addresses pertaining to instruction execution until the storage address register can be changed. This register normally contains the address of the next instruction to be executed. The contents of TAR are displayed in the DISPLAY A lights on the 3705 control panel when the Display/Function Select switch is in the TAR/OP REGISTER position.

### Setting/Resetting Interrupt Requests

A particular interrupt request latch can be set as a result of a hardware-detected condition or, in some cases, by the program through the execution of an output instruction. The latch can be reset by either an input or an output instruction, depending upon the specific interrupt request. The procedures for setting and resetting individual adapter interrupt requests are described in the adapter sections.

For special service requests, program levels 1, 2, 3, and 4 may issue a program-controlled interrupt (PCI) request to program levels 3 and 4. Output instructions X'7C' (set PCI level 3) and X'7D' (set PCI level 4) set the PCI interrupt requests. (See Appendix B, Output X'7C' and X'7D'.) Certain bits in Output X'77' (miscellaneous control) reset the PCI requests and other Central Control Unit interrupt requests such as the interval timer level 3 request and the SVC level 4 request.

- Determining the Cause of an Interrupt

The priority of simultaneous interrupt requests assigned to the same interrupt program level is resolved by the order in which the program tests the set/reset condition of the CCU and adapter interrupt request latches.

Interrupt requests from the CCU and the adapters are grouped together according to their source for ease of identification. The set/reset condition of a specific interrupt request latch can be determined by checking

5-2

the interrupt request group to which it is assigned. Inputs X'76' and X'77' indicate the interrupt requests that are set by the adapters. A request for level 1 sets a bit in Input X'76', and a request for levels 2 or 3 sets a bit in Input X'77'. The CCU interrupt requests for level 1 or levels 2, 3, and 4 are set in Inputs X'7E' and X'7F' respectively. These inputs load the contents of the appropriate interrupt request group into an active general register. (See Appendix B for input instruction bit definitions of interrupt request groups.) The program may then test the general register to identify the request.

### Masking Program Level Priorities

Programs at program levels 1, 2, 3, or 4 can selectively mask program levels 2, 3, 4, or 5, and level 1 adapter checks. Level 1 adapter checks, however, can be masked only while the CCU is in the test mode. (See CCU Diagnostic Facilities in this chapter.) The normal operational priority structure can be changed by Output instructions X'7E' and X'7F' (set/reset mask register). Refer to Appendix B for the mask register bit assignments.

When a program level is masked, use of machine cycles for instruction execution at that program level is suppressed until it is unmasked. Masking is normally used to prevent a higher-priority program level from interrupting a lower-priority level during execution of a time or data dependent routine. For example, if program level 3 contains a routine that should not be interrupted, the level 3 program can mask level 2 interrupts before entering this routine to ensure contiguous instruction execution. When the routine is completed, level 2 should be unmasked to allow interrupts.

If program level 2, 3, or 4 has already been entered, instruction execution at that level is allowed to finish before the masking of that level is effective. For example, if program level 2 interrupts program level 3 and subsequently masks level 3 interrupts, control can return to the level 3 program at the end of level 2 processing. However, once the level 3 program executes an Exit instruction, interrupts to level 3 cannot occur until it is unmasked. If program level 5 is interrupted by another level that sets the level 5 mask, control cannot be returned to level 5 until it is unmasked.

To selectively mask one or more program levels, one of the active general registers is loaded with the bits corresponding to those program levels to be masked. Output instruction X'7E' (set mask register) is then executed using the general register as input to

the mask register. To selectively unmask one or more program levels, the same procedure is followed except that the Output X'7F' (reset mask register) instruction is executed.

Important: Masking and subsequent unmasking should be handled with extreme caution to avoid disrupting the normal priority structure. If not used carefully, masking could cause overrun conditions or delay of hardware error indications.

### Storage Protect

Storage Protect is a means of notifying the control program whenever the contents of storage are accessed for unauthorized modification. This facility monitors all attempts to modify storage and execute instructions. However, due to hardware restrictions, storage protection is effective only in program level 5 and for cycle-steal operations for Type 2 and Type 3 Channel Adapters and Type 3 Communication Scanners.

Protection is achieved by a hardware comparison of separate keys assigned to the program and to storage. A user's ability to modify storage is identified by a 3-bit protect key. Each program level and cycle-steal mechanism is considered a user, and each has its own protect key. Storage is divided into blocks of 2048 bytes, and each block is assigned a 3-bit storage key.

When a protected area of storage is addressed, the storage key for that location is compared with the protect key associated with the user. Access to the location, for operands and instructions, is granted only when the two keys match. For attempts to execute an instruction, the two 3-bit keys must be equal. If the keys do not match, a protection exception LI interrupt request is set. For attempts to modify a storage location, the keys match when (1) the keys are equal, (2) the protect key is X'O', or (3) the storage key is X'7' (unprotected storage).

The protect keys for program levels 1, 2, 3, and 4 are fixed at 0 and cannot be changed. The protect keys for the remaining users are set by the control program with an Output X'73' (set key) instruction. Byte 1, bit 3 of an Output X'73' instruction must be 0 to indicate protect key selection. The protect key address of the desired user is placed in byte 0, bits 4-6, and the protect key is placed in byte 1, bits 5-7.Figure 5-2 shows the protect key addresses and the users they apply to.

To set any key, byte 1, bit 4 (set key) must also be 1. The set-key bit allows execution of the Output X'73' instruction to access either a storage or protect key without changing the key.

IBM 3704 and 3705 Communications Controllers Principles of Operation  5-3

| Protect Key Address (PKA)* | Applies to |
| --- | --- |
| Bits 01 2 |  |
| 000 | Protect Key - Program Level 5 |
| 00 1 | Protect Key - CA-1 Cycle Steals† |
| 0 1 0 | Protect Key - CA-2 Cycle Steals† |
| 0 1 1 | Reserved |
| 1 00 | Protect Key-Type 3 Scanner Cycle Steals† |
| 1 0 1 | Reserved |
| 1 1 0 | Reserved |
| 1 1 1 | Reserved |
| * Output X'73', byte 0, bits 4-6 † Does not apply to the 3704 |  |

Note: The protect key address is not an actual storage address. This key address is only a reference pointer for assigning a key to a user.

- Figure 5-2. Protect Key Address Bits

The Output X'73' instruction also sets the storage keys. The storage key of an area is determined by dividing storage into 2K blocks. Each block is then given a storage block address, from 0 to 128, relative to its position in storage. Figure 5-3 shows the storage block address that is assigned to the storage blocks. To set a storage key, the storage block address must be placed in byte 0, bits 0-6 of a general register, and the storage key set in byte 1, bits 5-7. Byte 1, bit 3 (key address select) and bit 4 (set key) must also be 1 to select the storage key and allow it to be set when an Output X'73' is executed. See Appendix B for a definition of the bits used in the Output X'73' instruction.

The Input X'73' instruction (insert key) can be used to determine the key (storage or protect) that must be used for storage access. When executed, this input instruction places the key that was addressed by the last Output X'73' instruction into byte 1, bits 5-7 of the register specified by the R operand. Therefore, the program must know what type of key and what address was used in the preceding Output X'73'. When an Output X'73' is executed with byte 1, bit 4 (set key bit) off, no keys are changed. However, this allows the address portion of the register to be used by an Input X'73'. Byte 1, bit 3 (key select bit) of the output instruction must also be known in order to determine the type of key (storage or protect) being accessed. See Appendix B for a definition of the bits used in the Input X'73' instruction.

Resetting the Central Control Unit disables storage protection. Therefore, any instruction fetch is valid, and any attempt to modify storage is permitted. The first Output X'73' instruction to be executed after a reset enables storage protect. This output instruction must set a storage key of 0 at the key address that corresponds to the storage block where the instruction execution is taking place. Otherwise a protection check occurs unless the storage key is already 0.

When the controller is powered on, the bits in all the protect keys except program levels 1-4 and all the storage keys assume a random bit pattern. Therefore, each key must be initialized by an Output X'73' containing its key address and key type (storage or protect). Until each key is fully initialized, caution must be exercised in the control of program levels and I/O activity that may depend on storage protection.

- Interval Timer

The interval timer provides a program level 3 interrupt request (Interval Timer L3) approximately once every 100 milliseconds. The time interval is derived from

SKA Bits 3 to 6

| SKA Bits 0 to 2 | 0000 | 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 | 1000 | 1001 | 1010 | 1011 | 1100 | 1101 | 1110 | 1111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000 | 0000 | 0800 | 1000 | 1800 | 2000 | 2800 | 3000 | 3800 | 4000 | 4800 | 6000 | 5800 | 6000 | 6800 | 7000 | 7800 |
| 001 | 8000 | 8800 | 9000 | 9800 | A000 | A800 | B000 | B800 | C000 | C800 | D000 | D800 | E000 | E800 | F000 | F800 |
| 010 | 10000 | 10800 | 11000 | 11800 | 12000 | 12800 | 13000 | 13800 | 14000 | 14800 | 15000 | 15800 | 16000 | 16800 | 17000 | 17800 |
| 011 | 18000 | 18800 | 19000 | 19800 | 1A000 | 1A800 | 1B000 | 1B800 | 1C000 | 1C800 | 1D000 | 1D800 | 1E000 | 1E800 | 1F000 | 1F800 |
| 100 | 20000 | 20800 | 21000 | 21800 | 22000 | 28000 | 23000 | 23800 | 24000 | 24800 | 25000 | 25800 | 26000 | 26800 | 27000 | 27800 |
| 101 | 28000 | 28800 | 29000 | 29800 | 2A000 | 2A800 | 2B000 | 2B800 | 2C000 | 2C800 | 2D000 | 2D800 | 2E000 | 2E800 | 2F000 | 2F800 |
| 110 | 30000 | 30800 | 31000 | 31800 | 32000 | 32800 | 33000 | 33800 | 34000 | 34800 | 35000 | 35800 | 36000 | 36800 | 37000 | 37800 |
| 111 | 38000 | 38800 | 39000 | 39800 | 3A000 | 3A800 | 38000 | 3B800 |  |  |  |  |  |  |  |  |

Notes:

1. Storage addresses are shown in hexadecimal.

2. Storage addresses shown are beginning addresses of block.

3. Storage Key Address (SKA) bits correspond to Output X'73' instruction byte 0,bits 0 to 6.

- Figure 5-3. Storage Key Addresses

5-4

the 50/60 Hertz power line frequency, and its accuracy depends on the stability of the power source.

The interval timer may be used to maintain a real-time clock in storage, perform long and short I/O time-outs, and perform supervisory functions on a periodic basis. The interval timer interrupt request can be reset by executing an Output X'77' (miscellaneous control) instruction with byte l.bit 1 set to 1.

### Cyclic Redundancy Check (CRC)

Although any cyclic redundancy check (CRC) can be performed by a program algorithm, the communications controller has special hardware to speed the CRC accumulation function required for binary-synchronous (BSC) EBCDIC and Synchronous Data Link Control (SDLC) communications.

For all Load Halfword instructions executed at program levels 1, 2, 3, or 4, the halfword obtained from storage is loaded into both the specified general register and a register called the old-CRC register. For normal operations (non-CRC), the loading of data into the old-CRC register serves no function. However, when a CRC update is to be performed, an additional instruction is not required, because the old CRC accumulation is automatically loaded into the old-CRC register.

To generate a new CRC accumulation, a Load Halfword instruction should be used to load the old-CRC register. The next character to be included in the CRC accumulation should then be placed in the low-order byte of one of the active general registers. Any output instruction can then be executed using the general register as the source of output. This places the character from the general register into a register called the CRC data register.

Note: When updating the CRC accumulation, the output instruction executed also performs its normal functions. Therefore, caution must be taken not to execute an Output instruction that can alter the status or state of either an adapter or an interface.

After a character to be added is in the CRC data register and the old-CRC accumulation is in the old-CRC register, one of the following input instructions should be used to load the new CRC character accumulation to a general register.

• Input X'7B' (BSC CRC) - This instruction is used for interface lines defined as BSC. It stores the new CRC in bytes 0 and 1 of the specified general register.

• Input X'7C' (SDLC CRC) - This instruction is used for interface lines defined as SDLC. It stores the new CRC in the low-order byte (byte 1), bits 0-7 of

the specified general register. During execution of the input instruction, the character to be included in the CRC accumulation and the old-CRC are combined by hardware circuits, and the riew-CRC is stored in bytes 0 and 1 of the general register specified in the input instruction. With Extended Addressing, bits 6 and 7 of byte X are reset to 0. The new CRC accumulation may then be placed in storage by the program.

###### Programming Note

The Load Halfword instruction that loads the old-CRC accumulation, the output instruction that loads the character into the CRC data register, and the input instruction (X'7B' or X'7C') that loads the new CRC into a general register do not have to be consecutive instructions. However, there must not be another Load Halfword instruction (executed at program levels 1, 2, 3, or 4) between loading the old CRC in the old-CRC register and loading the new CRC to a general register. Also, another output instruction must not be executed between loading the CRC data register and storing the new CRC.

- Initial Program Load (IPL)

The initial program load (IPL) mechanism controls the loading of an initial program into the controller via a channel adapter (1) when the system is first powered up, (2) when further processing is not possible due to an error condition, or (3) when the channel adapter decodes a Write IPL command. Three phases of the IPL program (IPL Phase 1-3) control the loading operation. IPL is accomplished by successful completion of all three phases. See Chapter 11 for a description of remote communications controller IPL.

Phase 1 of the IPL operation is a general reset to the controller. During phase 2, a small bootstrap program is automatically loaded into storage from the controller's read-only-storage (ROS) array, and control passes to this program. Execution of the bootstrap program (phase 3) then controls channel operations until the first load module from the host is successfully transferred into storage under a Write IPL channel command (X'05').

After successful transfer of the first program segment into storage, the initial program loading operation performed by the bootstrap program is complete. The controller is initialized, and the bootstrap program passes control to the loaded program segment. This program segment then controls the loading of whatever additional load modules are required to complete the program load operation.

The two lights on the control panel labeled IPL Phase, indicate the three phases of IPL. These lights

IBM 3704 and 3705 Communications Controllers Principles of Operation  5-5

are a binary representation of the phase number. That is, 01 = phase 1,10= phase 2, and 11= phase 3.

The Load light on the control panel comes on when IPL is initiated. It does not go off when IPL Phase 3 is completed. It must be turned off by executing an Output X'79' instruction when the program determines that the controller has been completely loaded.

IPL starts when either of the following occurs:

• The Diagnostic Control switch is in the PROCESS, CCU CHECK HARD STOP or BYPASS CCU CHECK STOP position and either (1) the Load push button is pressed; (2) a power-on occurs; (3) a channel adapter decodes a Write IPL command, and the controller is either not already in IPL phase 1, 2, or 3 or not in a hard stop condition; or (4) the control program executes an Output X'79' instruction with byte 0, bit 2 (set IPL) set to 1 in the register specified by the R operand.

• The Diagnostic Control switch is in the PROCESS position, a Central Control Unit check occurs, and the controller is not in IPL phase 1, 2, or 3.

IPL Phase 1 - Reset: During this phase the Load light is turned on, the CCU is placed in the test mode and a general reset occurs in the controller. Unless the IPL is initiated by a power-on sequence, the phase 1 reset does not affect the state of the channel adapter(s). The reset in the CCU:

• Sets the mask bits for program levels 2-5 and level 1 adapter requests.

• Resets all 'interrupt entered' latches.

• Resets all CCU interrupt requests.

• Resets program stop and hard stop.

• Prohibits storage references and instruction execution.

• Disables the storage protect facility until an Output X'73' instruction is executed.

• Turns on the Test light on the control panel.

IPL Phase 2 - Bootstrap Load: In this phase the bootstrap program is automatically loaded into storage from read-only-storage (ROS). Different ROS arrays are installed for different types or combinations of types of channel adapters installed. The bootstrap program for either channel adapter begins loading at address X'0000' and is 512 bytes long. When two channel adapters are installed, the bootstrap program is 1024 bytes to handle IPL from either channel adapter.

When the bootstrap load operation is complete, the IPL LI interrupt request is set, and the controller enters IPL phase 3 (bootstrap execution).

IPL Phase 3 - Bootstrap Execution: The bootstrap program begins execution at address X'0010' as a result of the IPL level 1 interrupt request set by phase 2 and operates entirely at program level 1.

The bootstrap program is divided into two sections. The first section:

1. Saves the general registers of group 0 starting at storage location X'0780'. Each register location starts on a fullword boundary.

2. Verifies the operation of the controller instructions to be used in the second section of the bootstrap program.

3. If the program is allowed to continue, it saves external registers X'76', X'7D', and X'7E' at storage locations X'0702', X'0704', and X'0706' respectively.

4. Resets the test mode and turns the Test light on the control panel off.

The second section of the bootstrap program controls the channel adapter operations until the first program load module is successfully transferred from the host processor.

If no outstanding command or final status is pending in the channel adapter, it generates an asynchronous status of Device End and Unit Check. If any command other than Write IPL is pending, the channel adapter generates a final status of Channel End, Device End, and Unit Check. In either case, sense bit 6 (Not Initialized) is set for the subsequent Sense command. A Write IPL command normally follows the Sense command. The Not Initialized bit is reset after the first program segment is successfully transferred into storage from the host processor.

The Write IPL command causes the transfer of the first load module from the host processor into controller storage. Under the Write IPL command, the load module is stored in sequential locations starting at storage location X'0400'. The maximum size of this load module cannot exceed 768 bytes. After successful completion of this transfer, the bootstrap program executes an Output X'77' instruction with byte 0, bit 0 (reset IPL LI) on in the register specified by the R operand. This resets the IPL LI interrupt request and turns off the IPL Phase lights on the control panel. The bootstrap program then turns control over to the load module at program level 1 by branching to storage location X'0404'.

The IPL operation is complete when the IPL LI interrupt request is reset and the IPL Phase lights are turned off. However, the Load light on the control panel remains on until reset by byte 1, bit 1 of Output X'79'. The program just loaded from the host should execute the Output X'79' when it is determined that the controller is completely loaded.

5-6

#### Programming Note

The first two halfwords of the load module must contain an IPL source identification (host processor ID) and the number of bytes in the load module (including the source ID and count bytes).

Check Conditions During IPL

Central Control Unit (CCU) checks are prohibited during IPL phase 1 since that phase performs a reset. If a CCU check occurs during IPL phase 2 or IPL phase 3, a hard stop occurs.

If the bootstrap program does not reach completion for any of the following reasons, the controller either comes to a hard stop or enters a loop and attempts to display the cause of the check condition in the control panel display lights.

• CCU check hard stop condition.

• Improper operation of an instruction during instruction verification in the first section of phase 3.

• Program continuity check (Type 2 and Type 3 CA only). This check ensures that all instructions in the bootstrap program have been executed in the correct sequence.

• Channel adapter disabled.

• Channel adapter hardware malfunction.

• Byte count in the second halfword of the load module does not compare with the number of bytes transferred from the host.

Input/Output Instructions

The control program uses input and output instructions to control and monitor the status of the CCU and the installed adapters. Appendix B defines the bits for each of the input and output instructions.

Input Instructions

The Central Control Unit has 16 external registers that can be accessed by executing input instructions. With these input instructions, the control program monitors the status of the CCU, the communication scanners, the channel adapters, and the control panel, and is informed of any error conditions. Three of the input instructions (X'75', X'78', and X'7A') are not used, and if executed, the bits in the general register are set to zero. The other twelve instructions set the bits of the general register according to the particular external register value. (Appendix B defines the bits within each input instruction.)

Input X'70' (Storage Size): This instruction loads a general register with a combination of bits that indicates the amount of storage installed.

Input X'71' (Panel Address/Data Entry): This instruction loads a general register with a combination of bits

to indicate the storage or register address or data to be used in a control panel function. These bits correspond to the Address/Data switches on the 3705 or the Hexadecimal Display switches B-E on the 3704. Through the use of this instruction, the program can accept information from the operator.

Input X'72' (Panel Display/Function Select Control):

This instruction loads a general register with a combination of bits to indicate the position of the control panel Display/Function Select switch. Through the use of this instruction, the program can accept information from the operator.

Note: The STATUS and the TAR & OP REGISTER positions of the 3705 Display/Function Select switch are not included in the register.

Input X'73' (Insert Key): This instruction loads a general register with the storage key or protect key addressed by the last Output X'73' instruction executed. The program must know the bit setting of the the key address bits set in the last Output X'73' instruction before this input is meaningful.

Input X'74' (Lagging Address Register): This instruction loads a general register with the contents of the lagging address register. When this input is executed at program levels 2, 3, or 4, the address transferred into the general register is that of the last instruction executed before the input instruction.

When Input X'74' is executed in program level 1, the address transferred into the general register is that of the last instruction executed at the program level that was active before the level 1 interrupt.

Input X'76' (Adapter Level 1 Interrupt Requests): This instruction loads a general register with information that can be used to determine which channel adapter or communication scanner caused a level 1 interrupt. In a remote communications controller, this instruction is also used to determine if the interrupt was from the remote program loader.

Input X'77' (Adapter Level 2 or 3 Interrupt Requests):

This instruction loads a general register with information that can be used to determine which channel adapter or communication scanner caused a level 2 or level 3 interrupt.

Input X'79' (Utility): This instruction loads a general register with utility information. When it is executed in program level 1, byte 1, bits 0-3 designate the program level that was operating before the level 1 inter-

IBM 3704 and 3705 Communications Controllers Principles of Operation  5-7

rupt. When it is executed in program levels 2, 3, or 4 (or level 1 if level 1 is re-entered immediately after a level 1 Exit), byte 1, bits 0-3 have no significance and are set to zero. When Input X'79' is executed at any level, byte 0, bits 6 and 7 indicate the state of the program level 5 C and Z condition latches.

Input X'7B' (BSC CRC Register): This instruction loads a general register with the new binary synchronous CRC character from the BSC CRC register.

Input X'7C' (SDLC CRC Register): This instruction loads a general register with the new synchronous data link control CRC character from the SDLC CRC register. See the Cyclic Redundancy Check (CRC) section of this chapter.

Input X'7D' (CCU Check Register): This instruction loads a general register with the contents of the CCU check register. The program can test this register to determine the cause of a program level 1 interrupt due to a CCU check condition.

Input X'7E' (CCU Level 1 Interrupt Requests): This instruction loads a general register with a configuration of bits to indicate the cause of a program check or a program level 1 interrupt due to an address compare or IPL. When a program check occurs. Input X'7D', byte 0, bit 3 should also be tested to determine if the check occurred while in program level 1 or in one of the other program levels.

Input X'7F' (CCU Level 2, 3, or4 Interrupt Requests):

This instruction loads a general register with a combination of bits to indicate the cause of various interrupts. Certain interrupt requests from program levels 2-4 set a bit associated with the type of request presented.

Output Instructions

The Central Control Unit has 16 external registers that can be loaded by executing output instructions to control its operation and data flow. However, the Output X'74', X'75', X'76', X'7A', and X'7B' instructions are not used, and if executed, the bit settings of the register specified by the R operand are ignored. The instructions that have an effect on an external register are as follows. (Appendix B defines the bits within each output instruction.)

Output X'70' (Hardstop): This instruction turns the 'hardstop' latch in the CCU on. This stops all machine activity and requires a reset and IPL to continue operation. Since this instruction performs a function and not an operation, the bit settings of the general register are ignored.

Output X'71' (Display Register 1): This instruction loads the contents of the general register into display register 1. Whenever the Display Register 1 and 2 push button on the 3704 control panel or the Display/Function Select switch on the 3705 control panel is in the STORAGE ADDRESS, REGISTER ADDRESS, or any of the FUNCTION 1-6 positions, the bits of the register are displayed in the Display A lights on the control panel.

The Program Display light on the control panel also comes on to inform the operator that information has been placed in the display register.

Output X'72' (Display Register 2): This instruction causes the contents of the general register to be loaded into display register 2. Whenever the Display Register 1 and 2 push button on the 3704 control panel or the Display/Function Select switch on the 3705 control panel is in the STORAGE ADDRESS, REGISTER ADDRESS, or any of the FUNCTION 1-6 positions, the bits of the register are displayed in the Display B lights on the control panel.

The Program Display light on the control panel also comes on to inform the operator that information has been placed in the display register.

Output X'73' (Set Key): This instruction causes the contents of the general register to be used to address and/or set the storage and protect keys.

Output X'77' (Miscellaneous Control): This instruction causes the contents of the specified general register to be used to set or reset various interrupt requests.

Output X'78' (Force CCU Checks): This instruction provides the means for testing the CCU check circuits under diagnostic control. It causes the contents of the general register to be used to force certain error conditions in the CCU data flow. This instruction is valid only while the CCU is in the test mode. Refer to the CCU Diagnostic Facilities section of this chapter for details on forcing check conditions.

Output X'79' (Utility): This instruction causes the contents of the specified general register to set and/or reset various hardware latches and lights.

Output X'7C' (Set PCI L3): This instruction causes a Program Controlled Interrupt request to be set for program level 3 (PCI L3). This allows a program level to transfer a processing requirement to a different priority program level. Since this instruction performs a function, the bit settings of the general register are ignored.

5-8

Output X'7D' (Set PCI L4): This instruction causes a Program Controlled Interrupt request to be set for program level 4 (PCI L4). This allows a program level to transfer a processing requirement to a lower priority level. Since this instruction performs a function, the bit settings of the general register are ignored.

Output X'7E' (Set Mask Bits): This instruction causes the mask bits of the program levels to be set according to the contents of the general register. When a mask bit is set on, interrupts for the program level that corresponds to that bit are not permitted.

Output X'7F' (Reset Mask Bits): This instruction causes the mask bits of the program levels to be reset according to the contents of the general register.

### CCU Checks

The Central Control Unit performs various hardware checks on the program operation and hardware circuits in the controller to ensure proper operation. When a check condition is detected, bits set in the CCU check register indicate the type of check. The control program can then execute an Input X'7D' instruction to load a general register with the information available in the check register.

The first occurrence of a check condition sets the CCU check register. Subsequent checks are not allowed to enter the check register until it is reset. The check register is reset by executing an Output X'77' instruction with byte 0, bit 1 set to 1, or by pressing the CCU Check Reset or Reset push button on the control panel.

When a CCU check condition is detected and the Diagnostic Control switch on the control panel is in the PROCESS position, an automatic IPL is initiated. If the check condition persists after the IPL sequence is initiated, the 'hard stop' latch is set, and the machine stops. Reset and re-IPL are the only means of resetting a hard stop condition.

If the Diagnostic Control switch is not in the PROCESS position, the action taken for a particular switch position is described in the 3704 or 3705 Control Panel Guide.

Program Checks

The Central Control Unit hardware monitors the control program operation for proper instruction execution and indicates to both the control program and the operator when a program check is detected.

Detection of a program check causes a level 1 interrupt request to be set. If the Display/Function Select switch is in the STATUS position, the type of check is indicated in the Display B lights on the control panel. The control program can determine the cause of the

check by executing an Input X'7E' instruction and testing the register bits. Program action may vary according to the type of check detected; see the descriptions of each check (below) for appropriate action.

If error recovery for the program check is successful, the control program can reset the interrupt request by executing an Output X'77' instruction (miscellaneous control) with byte 1, bit 5 set to 1.

A program check while level 1 is active is indicated in the CCU check register (Input X'7D') by byte 0, bit 3 being set to 1, and the type of check is indicated in Input X'7E'. This condition causes an automatic IPL unless the Diagnostic Control switch on the control panel is in one of the check-control positions or the controller is in IPL Phase 3.

Below is a description of each of the four program checks.

In/Out Check

Input and output instructions are privileged instructions that can be executed only in program level 1,2, 3, or 4. Any attempt to execute an input or output instruction at program level 5 causes a level 1 interrupt and sets the In/Out Check bit in the CCU interrupt request group 1 register (Input X'7E').

The In/Out Check bit is set when an input or output instruction is executed at program level 1, 2, 3, or 4 with an external register address that is either not assigned or not recognized by any adapter. For example, issuing an Input X'38' instruction (nonexistent) or an Output X'52' (for Type 2 Channel Adapter only) when a Type 1 Channel Adapter is installed sets the In/Out Check bit. An In/Out check is also set if an Inbus parity check is detected by the CCU during execution of an input instruction.

If an In/Out Check is detected while in program level 1, a no-op is performed, and the Program Check in Level 1 bit in the CCU check register is set along with the In/Out Check LI interrupt request. The In/Out check is then handled as a CCU check.

Protection Exception

Whenever the control program attempts to modify a storage location that is protected and does not have a matching protect key, the result is a protection exception. A protection exception sets the Protection Check LI interrupt request and suppresses storage access.

When a protection exception is detected during a cycle-steal operation, it is signaled to the Type 2 Channel Adapter involved as an addressing error. The adapter immediately terminates its cycle-steal operation and sets its check-interrupt request bit assigned to program level 1.

IBM 3704 and 3705 Communications Controllers Principles of Operation  5-9

If a protection exception occurs when storage is accessed to execute an instruction, an effective no-op is performed, and the instruction address register (IAR) is not updated. If the exception occurs in program level 2, 3, 4, or 5, a level 1 interrupt request is set. The control program can normally determine the address of the last instruction executed by examining the contents of the lagging address register (LAR).

A protection exception caused by an instruction attempting to modify a storage location can occur only when program level 5 is active. (The protect keys for the other program levels are fixed at 0 by the hardware design.) Such a protection violation causes the instruction to be suppressed and sets the level 1 interrupt request. The address of the instruction that caused the exception can be determined by examining the contents of the lagging address register (LAR).

If a protection exception is detected while in program level 1, the result is a no-op, and the Program Check in Level 1 bit in the CCU check register is set along with the Protection Check LI interrupt request. The protection exception is then handled as a CCU check.

Invalid Op-Code Check

The communications controller is limited to the 51 instructions described in Chapter 4. Each instruction has its own operation code bit structure. Whenever an attempt is made to execute an instruction with an op code that does not compare to any of the 51 valid op codes, the Invalid Op Check level 1 interrupt request is set and instruction execution is suppressed.

If an attempt is made at program level 2, 3, 4, or 5 to execute an invalid op code, an interrupt occurs to program level 1.

If an attempt is made at program level 1 to execute an invalid op code, the instruction execution is suppressed, and the Program Check in Level 1 bit in the CCU check register is set along with the Invalid Op Check LI request. The invalid op code check is then handled as a CCU check.

Address Exception

An address exception occurs whenever an attempt is made to gain access to an uninstalled storage location for the given machine. For example, addressing storage location 52,304 when the controller contains only 49,152 bytes of storage causes an address exception.

When an address exception is recognized, an effective no-op is performed for the machine cycle in which it is detected. An Address Exception Check level 1 interrupt is then requested to inform the control program of the error.

An address exception during a cycle-steal operation is signaled to the adapter involved as an address check.

Upon the detection of an address check, the adapter immediately terminates its cycle-steal operation and sets its check interrupt request assigned to program level 1. This informs the control program of the error.

The following checks for address exceptions are made during instruction execution.

a. A check is made in the first cycle of each instruction as the storage address register (SAR) is loaded with the address of the instruction. If an address exception is detected, an effective no-op is performed and the instruction address register (IAR) is not updated. If an address exception occurs during the first cycle of an instruction executing at program level 2, 3, 4, or 5, a level 1 interrupt is taken and the address of the instruction last executed in that program level can normally be determined by examining the contents of the lagging address register (LAR).

b. A check is made in each subsequent cycle of multicycle instructions as the SAR is loaded with the address of the storage location to be accessed. If an address exception is detected, the operation in that cycle is suppressed. If an addressing exception occurs during the second or third cycle of an instruction executing at program level 2, 3, 4, or 5, a level 1 interrupt is taken, and the address of the instruction that caused the error can be determined by examining the contents of the lagging address register.

If an address exception is detected while program level 1 is active, the Program Check in Level 1 bit in the CCU check register and the Address Exception Check LI bits are set to 1. The address exception is then handled as a CCU check.

### CCU Diagnostic Facilities

The communications controller has diagnostic facilities designed to allow the control program to perform test procedures on the controller hardware. The diagnostic test routines can be either part of the online control program or a standalone control program used for testing purposes only. If these test routines are part of the online control program, the communication lines and adapters not being tested are allowed to continue operating.

To use the CCU diagnostic facilities of the control program, the CCU must be in the test mode. This mode of operation allows the use of certain bits and external registers that are not otherwise available to the control program. The test mode is entered by executing an Output X'79' instruction with byte 1, bit 2 (set test mode) set to 1 in the register specified by the R operand. Upon completion of testing, the control

5-10

program should reset the test mode by executing an Output X'79' with byte 1, bit 3 (reset test mode) on.

While in the test mode, the following operations are available for program use:

1. Set and reset diagnostic L2 - Output X'77', byte 0, bit 6 sets a level 2 interrupt request that allows the program to execute a user diagnostic routine during normal program execution. When a level 2 interrupt occurs, the control program should test byte 1, bit 0 of the CCU interrupt request group 2 (Input X'7F') to determine if the interrupt is for diagnostic purposes. On completion of the diagnostic routine, the diagnostic L2 interrupt request must be reset by executing an Output X'77' with byte 0,bit 7 (reset diagnostic L2) set to 1.

2. Set and reset the bypass CCU check stop mode -Output X'79', byte 1, bit 4 set to 1 allows CCU check stops to be bypassed. If the Diagnostic Control switch on the control panel is in the PROCESS or the CLOCK STEP position, the bypass CCU check stop mode forces the check control hardware to operate as if the panel switch were in the BYPASS CCU CHECK STOP position. If the Diagnostic Control switch is in any other position, this operation is ignored. To reset the bypass mode, the control program must execute an Output X'79' instruction with byte 1, bit 5 set to 1.

3. Set and reset L 1 adapter mask - Program level 1 interrupt requests caused by a check condition in an adapter (scanner or CA) can be masked by using an Output X'7E', byte 1, bit 1. This operation performs the same functions for the level 1 adapter checks as those described in the section on Masking Program Level Priorities in this chapter. To unmask the level 1 requests, the control program must execute an Output X'7F' (reset mask bits) with byte 1, bit 1 set to 1.

4. Force CCU checks - While in test mode, the control program can use the Output X'78' instruction. When used in a 3705, the setting of bits in byte 0 of this output instruction complements bits in the bytes being directed to the Arithmetic Logic Unit (ALU) to cause parity checks. The bits in byte 1 are used to complement storage and Z bus parity, and cause A-register or INDATA bus checks.

When used in a 3704, setting bits 5, 6, and 7 of byte 0 selects the instruction cycle in which the selected error occurs. Bits 0-4 of this byte are not used by the 3704. The bits in byte 1 are used to complement storage and Z bus parity, cause A-register checks, and cause B-register, SAR, and Op Reg parity errors.

IBM 3704 and 3705 Communications Controllers Principles of Operations  5-11