Chapter 1: Introduction

The IBM 3704 and 3705 Communications Controllers are transmission control units with processing capabilities that offer advantages not available in other IBM transmission control units. A control program residing in the controller storage performs many of the functions previously performed by the central processing unit. The assumption of these functions by the controller increases the availability of the CPU to process other programs and to perform more involved message-processing functions for the teleprocessing system.

The controller is priority-interrupt driven. This allows the control program to handle service requests at five different priority levels.

The communications controller performs, under program control, the normal transmission control unit functions such as line-control, control character recognition, line time-out, character assembly and disassembly, and redundancy checking. The control program can also (1) handle all polling and addressing of communications lines to determine if a line is ready to send or receive data, (2) take over data link control, (3) add framing characters to the beginning and end of blocks of data, and (4) translate from line code into code recognizable to a message-processing program, and vice versa. Most error recovery procedures can be handled by the communications controller, thus relieving the CPU of a time-consuming and storage-consuming teleprocessing function. The control program can also provide dynamic buffering for incoming data and basic message-processing functions.

Host System Interface

The communications controllers may be attached, via the proper channel adapter, to a System/360, Model 30, 40, 50, 65,67 (in 65 mode), 75, or 195, or System/370 Models 125 through 195. A 3705 with a Type 2 or Type 3 Channel Adapter requires only one subchannel address, and the interface may be connected to a byte multiplexer, block multiplexer, or selector channel. The Type 3 Channel Adapter enables the 3705 to be attached to tightly-coupled multiprocessor systems as a shared symmetrical I/O unit, and to a uniprocessor as an I/O unit with an alternate path capability. The Type 1 Channel Adapter in a 3704 or 3705 operates only on a byte multiplexer channel; a Type 4 Channel Adapter in a 3705 operates on a byte multiplexer, block multiplexer, or selector channel. The channel may require more than one subchannel address, depending on the mode of operation (2701, 2702, or 2703 emulation requires a range of subchannel addresses).

Note: A remote communications controller does not interface with the host system and therefore does not have a channel adapter.

System Support Programs

IBM provides control programs that schedule and control all 3704 or 3705 Communications Controller system resources. Also provided are system support programs. These are host processor programs used primarily to generate or assemble a user's control program and to provide IPL and dump facilities for the controller. The support programs operate under control of the host supervisor.

3704 Description

The 3704 is a low-entry control unit that is upward and downward compatible with a 3705 without Extended Addressing. It is available in four models that are defined by the amount of storage installed. Figure 1-1 lists the models of the 3704 and the storage capacity of each model.

The 3704 consists of a single module that can contain a Central Control Unit, a control panel, 16K bytes of storage, a channel adapter, and a communication scanner. Either a Type 1 or a Type 2 Scanner can be installed, but the channel adapter is limited to a Type 1 Channel Adapter. The maximum number of communication lines is 32 when the Type 1 Scanner is installed and 10 (26 with additional capability) when the Type 2 Scanner is installed. Figure 1-2 shows the maximum configuration of the 3704 hardware.

3705 Description

The 3705-I and 3705-II are available in 20 and 32 models, respectively. The different models provide varying combinations of storage size and maximum line attachment capacity. Figure 1-1 lists all models of the 3705-I and 3705-II with their respective line attachment capacities and storage sizes. The values shown in the middle column represent only the maximum number of lines physically attachable to the controller. The actual number of lines the controller can support depends on such factors as the line speeds (data rates) used and the throughput capacity of the control program.

Each model is represented by a letter and a numeral such as E3. The letter indicates the maximum number of half-duplex lines that can be physically attached to the controller, and the numeral indicates the amount of storage installed in the controller. All 'E' models, for example, can accommodate up to 64 half-duplex lines; all '3' models of the 3705-I have 80K bytes of storage and all '3' models of the 3705-II have 96K bytes of storage.

IBM 3704 and 3705 Communications Controllers Principles of Operation  1-1

The 'A' and 'E' models of the 3705 comprise a standalone base module containing a central control unit (CCU), a control panel, at least 16K bytes (3705-1) or 32K bytes (3705-II) of storage, and provisions for a channel adapter, a two-channel switch (except for a Type 3 Channel Adapter), a communication scanner, and line interface bases (LIB) and line sets to accommodate up to 64 half-duplex lines. Each LIB can have up to eight line sets (depending on line set type), each capable of accommodating 1 or two half-duplex lines.

Each higher letter designation (B, C, D, F, G, H) indicates that the controller contains one or more expansion modules that provide additional line attachment capacity (for a 3705-11) or additional storage and line attachment capacity (for a 3705-I). (All storage in a 3705-11 is contained in the base module, regardless of amount; storage increments in a 3705-I may extend into the first, second, or third expansion module, depending on amount.) All 'B' and 'F' models comprise a base module and a single expansion module; all 'C' and 'G' models comprise a base and two expansion modules;

and all 'D' and 'H' models have a base and three expansion modules.

The base module of a 3705-I contains 16K or 48K bytes of storage and a Type 1 or Type 2 Communication Scanner capable of supporting up to four line interface bases (LIB).

Each expansion module of a 3705-I can contain 0, 32K, or 64K bytes of additional storage and (1) a Type 2 Communication Scanner capable of supporting up to six LIBs, or (2) a Type 3 Communication Scanner and up to four LIBs. The first expansion module can also contain a Type 2, 3, or 4 Channel Adapter and a two-channel switch.

The base module of a 3705- II contains from 32K to 256K bytes of storage and (1) a Type 2 Communication Scanner and up to four LIBs, or (2) a Type 3 Communication Scanner and up to three LIBs.

Each expansion module of a 3705-II can contain either a Type 2 Communication Scanner and up to six LIBs or a Type 3 Communication Scanner and up to four LIBs. The first expansion module can also contain a Type 2, 3, or 4 Channel Adapter and a two-channel switch.

|  | Maximum Number of | Storage |
| --- | --- | --- |
| Model | Lines Attachable | Size (K) |
| IBM 3704 |  |  |
| A1 | 32 | 16 |
| A2 | 32 | 32 |
| A3 | 32 | 48 |
| A4 | 32 | 64 |
| # IBM 3705-I |  |  |
| 0 | 64 | 16 |
| A2 | 64 | 48 |
| B1 | 160 | 16 |
| B2 | 160 | 48 |
| B3 | 160 | 80 |
| B4 | 160 | 112 |
| C1 | 256 | 16 |
| C2 | 256 | 48 |
| C3 | 256 | 80 |
| C4 | 256 | 112 |
| C5 | 256 | 144 |
| C6 | 256 | 176 |
| D1 | 352 | 16 |
| D2 | 352 | 48 |
| D3 | 352 | 80 |
| D4 | 352 | 112 |
| D5 | 352 | 144 |
| D6 | 352 | 176 |
| D7 | 352 | 208 |
| D8 | 352 | 240 |
| IBM 3705-II |  |  |
| E1 | 64 | 32 |
| E2 | 64 | 64 |
| E3 | 64 | 96 |
| E4 | 64 | 128 |
| E5 | 64 | 160 |
| E6 | 64 | 192 |
| E7 | 64 | 224 |
| E8 | 64 | 256 |
| F1 | 160 | 32 |
| F2 | 160 | 64 |
| F3 | 160 | 96 |
| F4 | 160 | 128 |
| F5 | 160 | 160 |
| F6 | 160 | 192 |
| F7 | 160 | 224 |
| F8 | 160 | 256 |
| G1 | 256 | 32 |
| G2 | 256 | 64 |
| G3 | 256 | 96 |
| G4 | 256 | 128 |
| G5 | 256 | 160 |
| G6 | 256 | 192 |
| G7 | 256 | 224 |
| G8 | 256 | 256 |
| H1 | 352 | 32 |
| H2 | 352 | 64 |
| H3 | 352 | 96 |
| H4 | 352 | 128 |
| H5 | 352 | 160 |
| H6 | 352 | 192 |
| H7 | 352 | 224 |
| H8 | 352 | 256 |

Figure 1-1. Line Attachment Capacity and Storage Size by Model

1-2

Storage

The communications controllers, like most central processing units, contain their own internal storage array. This storage provides a residence for the control program and a temporary storage area for data as it is being assembled or disassembled in preparation for transfer to the host processor or a terminal. The numeric part of the model designation (for example, '7' in E7) indicates the amount of storage installed in the controller (see Figure 1-1).

3705-I storage ranges from 16K to 240K bytes, in increments of 32K bytes. The read/write storage cycle time is 1.2 microseconds.

3705-II storage ranges from 32K to 256K bytes, in increments of 32K bytes. The read/write storage cycle time is 1.0 microsecond.

3704 storage ranges from 16K to 64K bytes in increments of 16K bytes. The read/write storage cycle time is 600 nanoseconds.

Bytes of storage are handled separately or grouped together in fields. A halfword is a group of two consecutive bytes and is the basic building block of instructions. A word is a group of four consecutive bytes. The location of any field or group of bytes is specified by the address of its leftmost byte.

Central Control Unit

The Central Control Unit (CCU) contains the circuits and data flow paths to execute the instruction set, and to control storage, the communication scanners, and the channel adapters. The CCU operates under control of the programs residing in storage.

Adapters

Two different adapters are required within the communications controller to connect the Central Control Unit with the host channel and the communication lines. These are the communication scanner (connection between CCU and communication line), and the channel adapter (connection between CCU and host channel). The communication scanners for the communications controllers come in three versions, and the channel adapters come in four versions. The Type 1 Scanner and Types 1 and 4 CA have low functional hardware capabilities and require more program control than the Type 2 and Type 3 Scanner and the Type 2 and Type 3 CAs. The Type 2 and Type 3 Scanner and Type 2 and Type 3 CAs have more functional hardware capabilities and

therefore require less program control while providing increased performance.

Figure 1-3 shows all possible combinations of communication scanners and channel adapters, including the maximum number that can be installed for each model of the 3705 controller. The 3704 supports either a Type 1 or a Type 2 Scanner, but only supports the Type 1 Channel Adapter.

Channel Adapters

Four types of channel adapters (CAs) are available. The Type 1 and Type 4 CA provide for local attachment to a System/360 or System/370 byte multiplexer channel. These adapters contain the hardware circuits necessary to assist the program in emulating an IBM 2701, 2702, or 2703 as well as to allow operation of the controller in native mode. To operate in emulation mode, the controller requires multiple subchannel addresses: one for each line address and one for native mode IPL.

The Type 1 CA transmits data to and from the byte-multiplexer channel in bursts of up to four bytes. The Type 4 CA transmits data in bursts of up to 32 bytes, if operating in extended buffer mode. When not in this mode, the Type 4 CA transmits to and from the channel in bursts of up to four bytes.

The Type 2 and Type 3 CAs, which are available for the 3705 only, provide for local attachment to a System/370 byte multiplexer, block multiplexer, or selector channel. With the Type 2 or Type 3 CA, the 3705 appears as a single control unit on the host processor channel and uses a single subchannel address. The Type 2 and Type 3 CAs operate in the native (one host subchannel address) mode only.

All data transfer between 3705 storage and the Type 2 and Type 3 Channel Adapters is by cycle steal operation. That is, when the CA has data to put in storage, it preempts (under hardware control) the necessary machine cycles to transfer the data. Data transfers from storage to the channel adapter also use the cycle steal technique. A cycle steal operation is accomplished by hardware circuits and does not affect the logical operation of the program.

A second channel adapter (Type 2, 3, or 4) can be installed in the first expansion module of a 3705 Model B, C, I D, F, G, or H. When two channel adapters are installed, they operate independently. The adapter in the base module is called the first adapter (CA-1) and the adapter in the expansion module is called the second adapter (CA-2). Figure 1-3 shows the possible combinations.

IBM 3704 and 3705 Communications Controllers Principles of Operation  1-3

Figure 1-2. Maximum Hardware configuration of the 3704 Communications Cntroller

1-4

| # Channel Adapter Type | Communication Scanner Type | 3705-I Model | 3705-II Model |
| --- | --- | --- | --- |
| 1 2 3 4 | 1 2 3 | A B C D | E F G H |
| 1 | 1 | * x x x | (note 4) |
| 1 | 1 | * x x x | * |
|  | 2 | * x x | * |
|  | 3 | * x | * |
|  | 4 | * | * |
| 1 | 1 | * x x x | * |
|  | 2 | * x x | * |
|  | 3 | * x | * |
|  | 4 | * | * |
| 2 | 1 | * x x | * |
|  | 2 | * x x | * |
|  | 3 | * x | * |
|  | 4 | * | * |
| 1 | 1 |  | * |
|  | 2 |  | * |
|  | 3 | (note 2) | * |
|  | 4 |  | * |
| 2 | 1 |  | * |
|  | 2 | (note 2) | * |
|  | 3 |  | * |
|  | 4 |  | * |
| 1 | 1 1 | * x x | * |
|  | 1 2 | * x | * |
|  | 1 3 | * | * |
|  | 2 1 | * x | * |
|  | 2 2 | * | * |
|  | 3 1 | * | * |
| 2 | 1 1 | * x x | * |
|  | 1 2 | * x | * |
|  | 1 3 | * | * |
|  | 2 1 | * x | * |
|  | 2 2 | * | * |
|  | 3 1 | * | * |
| 1 | 1 | * x x x | * |
|  | 2 | * x x | * |
|  | 3 | * x | * |
|  | 4 | * | * |
| 2 | 1 | * | * |
|  | 2 | * | * |
|  | 3 | * | * |
|  | 4 | * | * |
| 1 | 1 |  | * |
|  | 2 |  | * |
|  | 3 | (note 2) | * |
|  | 4 |  | * |
| 2 | 1 |  | * |
|  | 2 | (note 2) | * |
|  | 3 |  | * |
|  | 4 |  | * |
| 1 | 1 1 | * x x | * |
|  | 1 2 | * x | * |
|  | 1 3 | * | * |
|  | 2 1 | * x | * |
|  | 2 2 | * | * |
|  | 3 1 | * | * |

Figure 1-3. Channel Adapter/Communications Scanner Combinations (Part 1 of 3)

IBM 3705 and 3705 Communications Controller Principles of Operation  1-5

| # Channel Adapter Type | Communication Scanner Type | 3705-I Model | 3705-II Model |
| --- | --- | --- | --- |
| 1 2 3 4 | 1 2 3 | A B C D | E F G H |
| 2 | 1 1 | * x x | * |
|  | 1 2 | * x | * |
|  | 1 3 | * | * |
|  | 2 1 | * x | * |
|  | 2 2 | * | * |
|  | 3 1 | * | * |
| 1 | 1 | * x x | * |
|  | 2 | * x x | * |
|  | 3 | * x | * |
|  | 4 | * | * |
| 2 | 1 | * x x | * |
|  | 2 | * x x | * |
|  | 3 | * x | * |
|  | 4 | * | * |
| 1 | 1 |  | * |
|  | 2 |  | * |
|  | 3 | (note 2) | * |
|  | 4 |  | * |
| 2 | 1 |  | * |
|  | 2 | (note 2) | * |
|  | 3 |  | * |
|  | 4 |  | * |
| 1 | 1 1 | * x x | * |
|  | 1 2 | * x | * |
|  | 1 3 | * | * |
|  | 2 1 | * x | * |
|  | 2 2 | * | * |
|  | 3 1 | * | * |
| 2 | 1 1 | * x x | * |
|  | 1 2 | * x | * |
|  | 1 3 | * | * |
|  | 2 1 | * x | * |
|  | 2 2 | * | * |
|  | 3 1 | * | * |
| 1 1 | 1 | * x x | (note 4) |
| 1 1 | 1 | * x x | * |
|  | 2 | * x x | * |
|  | 3 | * x | * |
|  | 4 | * | * |
| 1 1 | 1 | * x x | (note 4) |
| 1 1 | 1 | * x x | * |
|  | 2 | * x x | * |
|  | 3 | * x | * |
|  | 4 | * | * |
| 1 1 | 1 | * x x | * |
|  | 2 | * x x | * |
|  | 3 | * x | * |
|  | 4 | * | * |
| 1 1 | 1 |  | * |
|  | 2 | (note 2) | * |
|  | 3 |  | * |
|  | 4 |  | * |

Figure 1-3. Channel Adapter/Communications Scanner Combinations (Part 2 of 3)

1-6

| # Channel Adapter Type | Communication Scanner Type | 3705-I Model | 3705-II Model |  |
| --- | --- | --- | --- | --- |
| 1 2 3 4 | 1 2 3 | A B C D | E F G H |  |
| 1 1 | 1 1 | * x x | * |  |
|  | 1 2 | * x | * |  |
|  | 1 3 | * | * |  |
|  | 2 1 | * x | * |  |
|  | 2 2 | * | * |  |
|  | 3 1 | * | * |  |
| 1 1 | 1 | * x x | * |  |
|  | 2 | * x x | * |  |
|  | 3 | * x | * | (note 5) |
|  | 4 | * | * |  |
| 1 1 | 1 |  | * |  |
|  | 2 |  | * | (notes 1 and 5) |
|  | 3 | (note 2) | * |  |
|  | 4 |  | * |  |
| 1 1 | 1 1 | * x x | * |  |
|  | 1 2 | * x | * | (notes 1 and 5) |
|  | 1 3 | * | * |  |
|  | 2 1 | * x | * |  |
|  | 2 2 | * | * |  |
|  | 3 1 | * | * |  |
| 1 1 | 1 | * x x | * |  |
|  | 2 | * x x | * | (notes 1 and 5) |
|  | 3 | * x | * |  |
|  | 4 | * | * |  |
| 1 1 | 1 |  | * |  |
|  | 2 | (note 2) | * | (notes 1 and 5) |
|  | 3 |  | * |  |
|  | 4 |  | * |  |
| 1 1 | 1 1 | * x x | * |  |
|  | 1 2 | * x | * | (notes 1 and 5) |
|  | 1 3 | * | * |  |
|  | 2 1 | * x | * |  |
|  | 2 2 | * | * |  |
|  | 3 1 | * | * |  |

•indicates, for a 3705-II, the model of 3705 required to accommodate the number of channel adapters and communication scanners shown at left.

•indicates, for a 3705-I, the smallest model of 3705 required to accommodate the number of channel adapters and communication scanners shown at left.

x indicates other models of 3705-I that can accommodate the number of channel adapters and communication scanners shown at left; the larger model so indicated may be required if the storage size required exceeds the capacity of the model indicated by *.

Note 1: Communication over both CAs can occur only when the control program is performing emulation subchannel operations over the Type 1 or 4 CA and native subchannel operations over the other CA.

Note 2: A 3705-1 cannot contain only Type 3 Scanners; it can contain Type 3 Scanners in each expansion module if the base module contains a Type 2 Scanner.

Note 3: Communication over both Type 4 CAs can occur only when the control program performs emulation subchannel operations over both CAs or native subchannel operations over one CA and emulation subchannel operations over the other.

Note 4: A 3705-II cannot contain Type 1 Scanners.

Note 5: The Type 4 CA must be installed in the base module when the controller contains a Type 4 and a Type 2 CA or a Type 4 and a Type 3 CA.

Figure 1-3. Channel Adapter/Communications Scanner Combinations (Part 3 of 3)

IBM 3705 and 3705 Communications Controller Principles of Operation  1-7

#### Two-Channel Switch Feature

A two-channel switch can be installed for the Type 1, Type 2 and Type 4 Channel Adapters. With this feature, the communications controller can be attached to two host processor channels through a single channel adapter. (Both channels can be on the same CPU, or they can be on two different CPUs.) However, only one of the channels can be enabled for operation at a time. The enabled channel is selected by means of a manual switch on the control panel.

If two channel adapters are installed (3705 only), both can have a two-channel switch. This allows the controller to be attached to four CPU channels.

Communication Scanners

The communication scanners provide the connection between the communication-line attachment hardware (line interface bases and line sets) and the Central Control Unit via the appropriate attachment base. The primary function of the scanner is to periodically scan the hardware associated with each communication line for service requests. Three types of communication scanner are available.

Type 1 Scanner: When installed in a 3705, the Type 1 Scanner supports four line interfaces (LIB) with a maximum capability of 64 half-duplex lines and can transfer only one bit at a time to the CCU. The LIBs within this scanner are specified as LIB-1, LIB-2, LIB-3, and LIB-4. Only one Type 1 Scanner can be installed in a 3705, which also prohibits installation of a Type 2 Scanner. This limits the communications controller to a maximum of 64 lines. LIBs are described below under Line Interface Bases and Line Sets.

When installed in a 3704, the Type 1 Scanner is capable of supporting only two LIBs (LIB position 1 and LIB position 2). This limits the controller to a maximum of 32 lines.

Type 2 Scanner: The Type 2 Scanner in a 3705 supports either four or six LIBs with a maximum capability of 64 or 96 half-duplex lines, respectively. This scanner transfers a full byte to or from the CCU. Depending on the model of 3705, from one to four Type 2 Scanners can be installed. These scanners are specified as Scanner-1, Scanner-2, Scanner-3, and Scanner-4. Scanner-1 is located in the base module and can support up to four LIBs with 64 lines. Scanners-2, -3, and -4 are located in the expansion modules, and each can support up to six LIBs with 96 lines. The LIBs are specified as LIB-1, LIB-2, LIB-3, LIB-4, LIB-5, and LIB-6 in each scanner, with the exception of Scanner-1, which has only four LIBs.

When installed in a 3704, the Type 2 Scanner is capable of supporting one LIB, Type Al, which limits the controller to a maximum of ten lines. Additional capability is available that enables the scanner to support two LIBs in any combination with the exception of two Type 1 LIBs. With this additional capability, the maximum number of lines supported is increased to 26.

Type 3 Scanner: The Type 3 Scanner in a 3705 supports either three or four LIBs with a maximum capacity of 48 or 64 half-duplex lines, respectively. Depending upon program buffering options used, the Type 3 Scanner transfers up to 255 bytes of data to or from the Central Control Unit (CCU) before interrupting the control program for more data or buffers.

Up to four Type 3 Scanners can be installed in a 3705-II. These are specified as Scanner-1, Scanner-2, Scanner-3, and Scanner-4; the base module contains Scanner-1 and the third expansion module contains Scanner-4.

Up to three Type 3 Scanners can be installed in a 3705-I. These are specified as Scanner-2, Scanner-3, and Scanner-4; installation of Type 3 Scanners in these positions requires that the Scanner-1 position (base module) contain a Type 2 Scanner.

Scanner-1 can support up to three LIBs and 48 lines; each of the remaining scanners can support up to four LIBs and 64 lines. The LIBs within each scanner are designated LIB-1, LIB-2, LIB-3, and LIB-4 (the latter is not available in a Type 3 Scanner in the Scanner-1 position).

Type 1 and Type 2 scanners support both synchronous and asynchronous communication lines. Type 3 scanners support only synchronous lines (SDLC and BCC). All types support lines of different data rates (line speeds). The line type (SDLC, BSC, start-stop), character length, bit clocking mechanism (business machine or modem), installed business machine clock speed, and interrupt priority are selected by the program for each line interface.

Attachment Bases

An attachment base is a required feature for support of the 3705 adapters. Two types of attachment bases are available: the Type 1 Attachment Base and the Type 2 Attachment Base. The 3704 does not require an attachment base, nor does a remote 3705.

The Type 1 Attachment Base provides common controls to the Central Control Unit for the Type 1 Scanner and the Type 1 and Type 4 Channel Adapters. The Type 2 Attachment Base provides common controls to the Central Control Unit and line addressing controls for the Type 2 and Type 3 Scanners.

1-8

One or both of the attachment bases are required, depending on the type of scanner and channel adapter installed in the 3705. The requirements are as follows:

| ## Hardware Installed | ## Attachment Base Required |
| --- | --- |
| Type 1 Scanner/Type 1 or 4 CA | Type 1 Attachment Base |
| Type 2 Scanner/Type 1 CA | Type 1 & Type 2 Attachment Base |
| Type 2 or 3 Scanner/Type 4 CA | Type 1 & Type 2 Attachment Base |
| Type 2 or 3 Scanner/Type 2 or Type 3 CA | Type 2 Attachment Base |

Line Interface Bases and Line Sets

Communication lines to and from the teleprocessing stations are attached to the communications controller through a line interface base (LIB). The primary functions of the LIB are to drive and terminate all signals between the communication scanners and the line sets and to provide bit clocking.

The line interface base is transparent to the data transferred and has no effect on the control program except for bit clock control.

The number and type of LIBs vary according to the controller configuration. As many as 22 LIBs can be installed in a 3705. The 3705-I A models, containing either a Type 1 or Type 2 Communication Scanner, and the 3705-II E models, containing a Type 2 Communication Scanner, are capable of supporting four LIBs. The 3705 B and F, C and G, and D and H models, containing Type 2 Communication Scanners, can support up to 10,16, or 22 LIBs, respectively (four LIBs in the basic module and six LIBs in each of the expansion modules). The 3705-I B, C, and D models can contain up to 8,12, or 16 LIBs, respectively (four LIBs in the base module and four in each expansion module) if equipped with a Type 2 Scanner in the base module and a Type 3 Scanner in each expansion module. The 3705-II F, G.and H models can contain up to 7,11, or 15 LIBs, respectively (three LIBs in the base module and four in each expansion module) if equipped with a Type 3 Scanner in each module.

The 3704 with a Type 1 Scanner supports two LIBs and with a Type 2 Scanner supports one LIB Type Al. A 3704 Type 2 Scanner can optionally support two LIBs. These two LIBs can be any combination, except two Type 1 LIBs; however, a Type 1 LIB can be combined with any other LIB type.

Several different LIB types are available to meet the aeeds of a wide variety of line and terminal types. Each LIB type operates identically and is controlled by the communi-cation scanner to which it is attached. However, the design of the various LIB types differs in order to support the many fae sets and line configurations that can be attached to them. Refer to Introduction to the 3704 and 3705 Communications Controllers (GA27-3051) for a description of the individual LIB and line set types.

The line set is the hardware connection between the LIB and the communication line. A given line set type may support attachment of many different terminals and devices;

therefore, different data sets or modems may be required. A single line set provides the interface for one or two half-duplex communication lines, depending on the type of interface.

Remote Communications Controller

A controller may be attached directly to a host processor channel via a channel adapter, or it may be located many miles distant from the host processor. In the latter case, the controller (called a remote communications controller) requires a remote program loader instead of a channel adapter. The remote controller is connected by a local/remote communication link to another controller that is attached to a host processor channel.

Remote Program Loader

The remote program loader, which replaces the channel adapter in the remote controller, consists of a read-only-storage bootstrap program, and an IBM disk storage drive and disk controller that provides the remote controller with an IPL capability. Chapter 11 describes the remote program loader and its capabilities.

SDLC

Synchronous data link control (SDLC) is the line discipline required for the local/remote communication link. The Type 2 and Type 3 Communication Scanners are capable of handling SDLC communications channels. The Type 1 Scanner, with proper program support, can emulate the Type 2 Scanner's SDLC capability. More information on SDLC can be obtained from Chapter 7 of this manual and from the Synchronous Data Link Control General Information manual.

General Programming Concepts

The control program that resides in 3704/3705 storage controls the transfer of data as it passes through the controller between the stations in the teleprocessing network and the host processor.

The primary functions of the control program are related to transmitting and receiving data. But it can also edit and process the data as it passes through the controller.

IBM 3704 and 3705 Communications Controllers Principles of Operations   1-9

| System Data Flow In performing its functions, the control program interacts with the communication scanners and the channel adapters to control the flow of data through the teleprocessing system. Data entered at a terminal is received by the line set and line interface base. The communication scanner recognizes that service is required and receives data from the LIB. The program places the data in storage, where it is then available to the channel adapter to be sent to the host processor channel. When data is to be transmitted from the host processor to a terminal, the process is reversed. The host channel sends the data to the controller's channel adapter which, under hardware or program control depending on the type of adapter, places the data in storage. An interrupt request to send the data to the LIB and finally to the terminal is then signaled to the control program. Some processing of the data may be accomplished while the data is in storage. Interaction between the controller adapters is through interrupts and input/output instructions. Figure 1-4 illustrates the data flow to and from the terminal and the CPU through the various parts of the controller. |  | Input/Output Instructions The 3704 and 3705 use input and output instructions as the primary link between the hardware and the control program. All control information and data as it enters or exits the controller passes through the CCU and adapter external registers. These external registers are not directly accessible by the control program; therefore, input and output instructions are used to obtain or change the external register contents. When the control program executes an input instruction, the contents of the specified external register are loaded into a general register. The program then has direct access to that information and can act accordingly. In the same manner, the control program can load a general register with control information or data for a particular adapter. When that general register is used with the specified external register and an output instruction is executed, the contents of the general register are transferred into the external register. The subject adapter can then take the action specified by the settings of the register bits. Any data-transfer and adapter-control operations that the control program can perform within the controller are accomplished through input and output instructions. |
| --- | --- | --- |

|  | ## CPU to Station |  | Station to CPU |
| --- | --- | --- | --- |
|  | CPU sends data to the controller |  | Station sends data to the controller |
|  | Channel Adapter notifies Control program as data arrives |  | Communication Scanner notifies Control Program as data arrives. |
|  | Control Program processes data, prepares it for station. |  | Control program processes data, prepares it for CPU. |
|  | Control Program activates communication Scanner when data is ready to be sent to station. |  | Control Program activates Channel Adapter when data is ready to be sent to CPU. |
|  | Data is transmitted across communication line to station. |  | Channel transfers data to CPU. |

Figure 1-4. System Data Flow

1-10