### Appendix B: Input/Output Instruction Bit Definitions

Type 1 Scanner Input Instructions

Input X'41' (Interface Address)

When operating in the bit service mode, this instruction causes a storage address associated with an interface to be loaded into the specified general register. The selected interface is where the scanner has stopped because of an interrupt request. The storage address is a fixed location, determined by the LIB position and the interface address assigned to the line causing the interrupt. Figure 3-5 shows the storage addresses associated with each interface address.

When operating in the character service mode, a fixed address of X'06F0' is loaded into the specified register.

Input X'42' (Control A)

The Input X'42' instruction should be executed only when the scanner is stopped on an interface. This input instruction can be used to check the state of the Output X'42' (control A) instruction. When executed for a particular line, the result in the register specified by the R operand is a bit-f or-bit reflection of the control information set by the Output X'42' instruction for that interface. Refer to the Output X'42' instruction for the definition of each bit position.

Input X'43' (Control B/C)

This instruction cannot be executed immediately following an Output X'43' for feedback checking.How-ever, it can be executed after an Output X'43' to obtain other status indications.

Byte 0, bit 0: Receive Data Bit Buffer—This position indicates a mark or space condition received from the line interface (1 = a mark, 0 = a space).

Byte 0, bit 1: Feedback Check—This bit is set on if a bit to be transmitted is not transferred to the interface correctly or if the interface bit service request fails to reset.

Programming Note

After a feedback check occurs, the bit-service interrupt request cannot be reset and the scanner cannot be restarted until the control program has reset the check indication with an Output X'44' with byte 1, bit 6 set to 1.

Byte 0, bit 2: Interface Check Summary—This bit is set on to indicate (1) the detection of a feedback error, (2) bit overrun/underrun, or (3) 'data set ready'

line not active. It is a summary of these check conditions and is set to 1 when byte 0, bit 1, or byte 1, bit 2 or bit 7 of this input is on.

Byte 0, bit 3: Received Data Lead—This bit indicates the level of the 'receive data' signal from the modem. A 1 indicates a space level and 0 indicates a mark level.

Byte 0, bit 4: Transmit Mode—A 1 in this position indicates that the line interface is set for transmit mode; a 0 indicates that it is set for receive mode.

Byte 0, bit 5: New Sync—This position indicates the state of the 'new sync' line to the modem. A 1 indicates that the line is active. A 0 indicates that the line is inactive.

Byte 0, bit 6: Request to Send (RTS)—A 1 in this position indicates that the 'request to send' line to the modem is active. A 0 indicates that the line is inactive.

Byte 0, bit 7: Send Data Bit Buffer—This position shows the setting of the line interface send data buffer. A 1 indicates a mark, and a 0 indicates a space.

Byte 1, bit 0: Not Clear to Send—A 1 in this position indicates that the 'clear to send' line from the modem is inactive. A 0 indicates that it is active.

Byte 1, bit 1: Ring Indicator—If this bit is 1, the 'ring indicator' line from the modem is active. A 0 indicates that it is inactive.

Byte 1, bit 2: Not Data Set Ready—When this position is A 1, the 'data set ready' line from the modem is inactive. A 0 indicates that it is active.

Byte 1, bit 3: Received Line Signal Detector—When the 'receive line signal detector' line from the modem is active, this bit is 1. A 0 indicates that the line is inactive.

Byte 1, bit 4: Telegraph Interface Echo Check—If this bit position is 1, a TTY echo check has occurred.

Byte 1, bit 5: Diagnostic Mode—A 1 in this position indicates that the line interface is set for the diagnostic wrap mode, and a 0 indicates that it is set for the normal read/write mode. See Type 1 Scanner Diagnostic Wrap in Chapter 6. If this bit is a 1, then byte 1, bit 2 of this input is forced to 0 and byte 1, bit

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-l

Type 1 Communication Scanner

3 of this input is forced to a 1. If this bit is 1 and byte 0, bit 6 is 1, byte 1, bit 0 is forced to 0.

Byte 1, bit 6: Bit Service—A 1 in this position indicates that the scanned interface has requested a bit service.

Byte 1, bit 7: Bit Overrun/Underrun—A 1 in this position indicates that a bit overrun or underrun has occurred.

Autocall Interface Bits for Input X'43'

Byte 0, bit 0: This bit is 0.

Byte 0, bits 1-2: These bits are the same as for line interface.

Byte 0, bit 3: Digit Present (DPR)—A 1 in this position indicates that the 'digit present' signal to the autocall unit is active; a 0 indicates that the signal is inactive.

Byte 0, bits 4-7: These bit positions represent the autocall dial digit in BCD form.

Byte 1, bit 0: Not Abandon Call and Retry—A 1 in this position indicates that the 'abandon call and retry' signal from the autocall unit is inactive. A 0 indicates that it is active.

Byte 1, bit 1: Present Next Digit (PND)—A 1 in this position indicates that the 'present next digit' signal from the autocall unit is active; a 0 indicates that the signal is inactive.

Byte 1, bit 2: Not Data Line Occupied (Not DLO)—A 1 in this position indicates that the 'data line occupied' signal from the autocall unit is inactive. A 0 indicates that it is active.

Byte 1, bit 3: Power Indicator (PWI)—A 1 in this position indicates that the 'power indicator' signal from the autocall unit is active; a 0 indicates that the signal is inactive.

Byte 1, bit 4: Call Request (CRQ)—A 1 in this position indicates that the 'call request' signal to the autocall unit is active; a 0 indicates that the signal is inactive.

Byte 1, bit 5: Call Originating Status (COS)—A 1 in this position indicates that the 'call originating

status' signal from the autocall unit is active; a 0 indicates that the signal is inactive.

Byte 1, bits 6-7: These bits are the same as for a line interface operation.

Input X'44' (Status)

Byte 0, bit 0: Mode Bit Override—When this position is a 1 the 'mode bit override' latch is set. When this latch is set, all interface mode settings, except 01 with high-priority, are overridden. See Interface Modes of Operation in Chapter 6.

Byte 0, bit 1: This bit is 0.

Byte 0, bit 2: Override Remember—When this position is a 1, the 'override remember' latch is set. See Interface Modes of Operation in Chapter 6.

Byte 0, bit 3: Scanner Enabled—When the scanner is enabled, this bit is 1. The bit is 0 when the scanner is disabled.

Byte 0, bit 4: Character Service Pending—This position indicates that the program has a character service request pending.

Byte 0, bits 5-7: These bits are 0. Byte 1, bits 0-1: These bits are 0.

Byte 1, bit 2: LIB 1 Bit Clock Check—This bit is a 1 if a LIB-1 bit clock parity check has occurred.

Byte 1, bit 3: LIB 2 Bit Clock Check—This bit is a 1 if a LIB-2 bit clock parity check has occurred.

Byte 1, bit 4: LIB 3 Bit Clock Check—This bit is a 1 if a LIB-3 bit clock parity check has occurred.

Byte 1, bit 5: LIB 4 Bit Clock Check—This bit is a 1 if a LIB-4 bit clock parity check has occurred.

Byte 1, bit 6: LIB Select Check—This bit is a 1 if more than one LIB or more than one interface is addressed at the same time.

Byte 1, bit 7: CCU Outbus Check—This bit is a 1 when a CCU outbus parity check has been detected.

B-2

Type 1 Scanner Output Instructions

Output X'40' (Set Mode Bit Override and Override Remember)

This output is used to set the 'mode bit override' and the 'override remember' latches. Setting these latches causes the scanner to override all interface mode settings except 01 with high-priority. This instruction performs a function; and, therefore, the bit settings of the register specified by the R operand are ignored. See Interface Modes of Operation in Chapter 6.

Output X'41' (Start Scanner &. Reset Bit Service L2 Request)

The Output X'41' instruction should be executed only when the scanner is stopped on an interface.

This instruction starts the scanner at the completion of the line interface servicing and resets the bit service request for the interface the scanner is addressing. It also resets the program level 2 bit service interrupt request. This instruction performs a function; and, therefore, the bit settings of the register specified by the R operand are ignored.

Output X'42' (Control A)

The Output X'42' instruction should be executed only when the scanner is stopped on an interface.

Byte 0, bits 0-5: These bits are unused.

Byte 0, bits 6- 7: Mode bits—Each interface may be set to one of four level 2 interrupt modes as follows:

00 Disable level 2 interrupts—This mode disables all level 2 interrupts for a given interface.

01 Monitor for ring indicator or data set ready—This mode allows level 2 interrupts if 'ring indicator' or 'data set ready' becomes active for at least one bit time.

10 Monitor for receive data space—This mode allows a level 2 interrupt each time a space bit is received. An interrupt also occurs if 'data set ready' drops for at least one bit time.

11 Enable level 2 interrupts—This mode allows all level 2 interrupts to be enabled for a given interface.

For further discussion on the use of the mode bit setting, refer to Interface Modes of Operation in Chapter 6.

Type 1 Communication Scanner

Byte 1, bit 0: Bit Service Priority—This bit determines the service priority for the interface the scanner is addressing. A 1 in this position sets a lo.w priority; a 0 sets a high priority.

Byte 1, bit 1: Diagnostic Mode—A 1 in this position sets the line interface to the diagnostic mode, and a 0 sets it to the normal read/write mode. See Diagnostic Wrap and Modem Self Test in Chapter 6 for the description of operation in this mode.

Byte 1, bit 2: Data Terminal Ready (DTR)—This bit activates or deactivates the 'data terminal ready' line from the line interface to the modem. A 1 activates the line, and a 0 deactivates it.

Byte 1, bit 3: Synchronous Bit Clock—A 1 in this position sets the line interface for synchronous clocking, and a 0 sets it for start-stop clocking.

Byte 1, bit 4: External Clock—This bit position selects modem (data set) clocking, or business machine (data terminal equipment) clocking. 1 = modem, and 0 = business machine.

Byte 1, bit 5: Data Rate Selector—A 1 in this position selects the high data rate for the attached modem, and a 0 selects the low data rate.

Byte 1, bits 6-7: Oscillator Select Bits—These two bit positions select which one of the four internal bit rates available in the Type 1 Scanner is to be assigned to the line interface.

The internal bit rates available are determined by the bit clocks that have been installed in the Type 1 Scanner in conjunction with the line interface base types and line sets. See the description of Business Machine Clocks in Chapter 6.

Autocall Interface Bits for Output X'42'

The Output X'42' instruction for autocall is the sameas for line interface except that byte 1, bits 1-7 are ignored.

Note: The lowest speed oscillator is always assigned to autocall interfaces.

Output X'43' (Control B)

The Output X'43' instruction should be executed only when the scanner is stopped on an interface.

Byte 0, bits 0-7: These bits are unused.

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-3

Type 1 Communication Scanner

Byte 1, bits 0-3: These bits are unused.

Byte 1, bit 4: Transmit/Receive Mode—A 1 in this position sets the line interface to transmit mode; a 0 sets it to receive mode.

Byte 1, bit 5: New Sync—A 1 in this position activates the 'new sync' line to the modem; a 0 deactivates the line.

Byte 1, bit 6: Request to Send (RTS)—A 1 in this position activates the 'request to send' line to the modem; a 0 deactivates the line.

Byte 1, bit 7: Send Data—A 1 in this position sends a mark to the line interface 'send data buffer'; a 0 sends a space.

Autocall Interface Bits for Output X'43'

Byte 0, bits 0- 7: These bits are unused. Byte 1, bits 0-1: These bits are unused.

Byte 1, bit 2: CRQ—A 1 in this position activates the 'call request' line to the autocall interface; a 0 deactivates the line.

Byte 1, bit 3: DPR—A 1 in this position activates the 'digit present' line to the autocall interface; a 0 deactivates the line.

Byte 1, bits 4- 7: These bit positions represent the dial digit in BCD form.

Output X'44' (General Control)

Byte 0, bits 0- 7: These bits are unused.

Byte 1, bit 0: Diagnostic Bit Service—A 1 in this position sets a latch that causes continuous bit service requests for all 64 line interfaces (used and unused). A 0 resets the latch.

Byte 1, bit 1: This bit is unused.

Byte 1, bit 2: Reset Mode Bit Override—A 1 in this position resets the 'mode bit override' latch as described in the Interface Modes of Operation in Chapter 6.

Byte 1, bit 3: Reset Override Remember—A 1 in this position resets the 'override remember' latch in the

Type 1 Scanner. See Interface Modes of Operation in Chapter 6.

Byte 1, bit 4: Reset Character Service Pending—A 1 in this position resets the 'character service pending' latch set by Output X'46'.

Byte 1, bit 5: Reset Level 1 Checks—A 1 in this position resets all level 1 check conditions resulting from the Type 1 Scanner.

Byte 1, bit 6: Reset Feedback Check—A 1 in this position resets the 'feedback error' latch.

Byte 1, bit 7: Reset Bit Overrun/Underrun—A 1 in this position resets the 'bit overrun/underrun' latch for the interface the scanner is addressing.

Output X'45' (Scanner Control)

This instruction may be used to disable one or more LIBs. A LIB should be disabled only when it is causing solid errors or solid level 1 interrupts.

Byte 0, bit 0: This bit is unused.

Byte 0, bit 1: Set Scanner Enable—A 1 in this position sets the 'scanner enable' latch, allowing normal operation of the scanner.

Byte 0, bit 2: Reset Scanner Enable—A 1 in this position resets the 'scanner enable' latch. This disables the scanner, prevents any further interrupts, forces all interfaces to be reset, and allows no bit services.

A 1 in both bit 1 and bit 2 of this byte is invalid, and the result is unpredictable. The scanner may be either enabled or disabled.

Note: In a 3704, when this bit is on, all LIBs are forced to and held in the enabled state as long as the scanner is disabled, regardless of the setting of the Output X'45' instruction.

Byte 0, bit 3: This bit is unused.

Byte 0, bit 4: Disable LIB 1—A 1 in this position disables LI and L2 interrupts and bit service from LIB 1. A 0 resets the disable condition.

Byte 0, bit 5: Disable LIB 2—A 1 in this position disables LI and L2 interrupts and bit service from LIB 2. A 0 resets the disable condition.

B-4

Byte 0, bit 6: Disable LIB 3—A 1 in this position disables LI and L2 interrupts and bit service from LIB 3. A 0 resets the disable condition.

Byte 0, bit 7: Disable LIB 4—A 1 in this position disables LI and L2 interrupts and bit service from LIB 4. A 0 resets the disable condition.

Byte 1, bits 0- 7: These bits are unused.

Output X'46' (Set Character Service)

The Output X'46' instruction sets the 'character service pending' latch. It also starts the scanner and resets the level 2 bit service interrupt request. This instruction performs a function, and therefore, the bit settings of the register specified by the R operand are ignored.

Output X'47' (Force Bit Service Request)

This instruction is executed to force one level 2 bit service interrupt request. The storage address associated with the interface that is to cause the interrupt

Type 1 Communication Scanner

must be loaded into the register specified by the R operand. When this instruction is executed, the scanner determines the interface address from the storage address and requests a level 2 interrupt for that interface.

Programming Note

Forced bit service cannot be stacked. If an Output X'47' is executed before the previous Output X'47' has been serviced, the second address overlays the first address.

Byte 0, bits 0-5: These bits are unused.

Byte 0, bits 6-7, Byte 1, bits 0-3: Interface Address—These bit positions are loaded with the storage address associated with the interface that is to cause a bit service interrupt.

Byte 1, bits 4-7: These bits are unused.

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-5

Type 2 Communication Scanner

Type 2 Scanner Input Instructions

Input X'40' (Interface Address)

This instruction obtains the line interface address from the ABAR in the attachment base. Conditions that set the ABAR are described in the I/O Programming Considerations section in Chapter 7. When this instruction is executed, the interface address from the ABAR is placed in byte 0, bit 6 through byte 1, bit 6 of the register specified by the R operand. Byte 0, bit 4 is always set to 1. The other register bit positions are set to 0.

Input X'43' (Check Register)

This instruction obtains the status of the check register in the scanner. Since there can be up to four Type 2 scanners, the check register selected is determined by the interface address in the ABAR at the time of instruction execution.

When this instruction is executed, the check register bits are placed in the register specified by the R field.

Programming Note

If any of the check register bits in the scanner are set to 1, the Type 2 Scanner LI interrupt request is set.

Byte 0, bit 0: LIB 1 Bit Clock Check—This bit is set to 1 if a LIB position 1 bit clock control check is detected by the Type 2 Scanner; otherwise, it is set to 0.

Byte 0, bit 1: LIB 2 Bit Clock Check—Same as above.

Byte 0, hit 2: LIB 3 Bit Clock Check—Same as above.

Byte 0, bit 3: LIB 4 Bit Clock Check—Same as above.

Byte 0, bit 4: LIB 5 Bit Clock Check—Same as above.

Byte 0, bit 5: LIB 6 Bit Clock Check—Same as above.

Byte 0, bit 6: LIB Select Check—This bit is set to 1 if the Type 2 Scanner has detected a LIB address parity check on either LIB string 1 (LIB pos 1, 2, or 3) or LIB string 2 (LIB pos 4, 5, or 6); otherwise, it is set to 0.

Byte 0, bit 7: ICW Input Register Check—This bit is set to 1 if the Type 2 Scanner has detected a parity error in the ICW input register; otherwise, it is set to 0.

Byte 1, bit 0: ICW Work Register Check—This bit is set to 1 if the Type 2 Scanner has detected a parity error in the ICW work register; otherwise, it is set to 0.

Byte 1, bit 1: Priority Register Available Check—This bit is set to 1 if the Type 2 Scanner has detected even parity on one of the four priority register available lines; otherwise, it is set to 0. These four lines from the attachment base are parity-checked in the Type 2 Scanner.

Byte 1, bit 2: CCU Outbus Check—This bit is set to 1 if the Type 2 Scanner has detected even parity on the CCU Outbus. Otherwise, it is set to 0.

Byte 1, bit 3: Line Address Bus Check—This bit is set to 1 if the Type 2 Scanner has detected a scanner buffer address register bus parity error during program addressing (refer to the Program Addressing section in Chapter 7); otherwise, it is set to 0.

Byte 1, bits 4-7: These bits are 0.

Input X'44' (ICW Input Register-Bits 0-15)

This instruction determines the state of the 'secondary control field' (SCF) and the 'parallel data field' (PDF) in the ICW that is set in the ICW input register. The interface address in the ABAR selects the proper scanner. When this instruction is executed, the contents of the ICW input register, bit positions 0-15, are placed in the register specified by the R field. Refer to I/O Programming Considerations for conditions that set the ICW input register. The Interface Control Word Format and the SDLC sections of Chapter 7 describe the SCF and PDF fields and their bit definitions.

Input X'45' (ICW Input Register-Bits 16-31)

This instruction determines the state of the LCD and PCF fields and SDF bits 0-7 of the ICW that is set in the ICW input register. The interface address in the ABAR selects the proper scanner. When this instruction is executed, the contents of the ICW input register bit positions 16-31 are placed in the register specified by the R field. Refer to I/O Programming Considerations in Chapter 7 for conditions that set the ICW input register. The LCD, PCF, and SDF fields and bit definitions are described in the Interface Control Word Format and the SDLC sections of Chapter 7.

B-6

Programming Note

This input does not contain the complete serial data field. Only bits 0-7 of the field are available with this input. The remainder of the SDF (bits 8-9) are accessed by the Input X'47' instruction.

Input X'46' (Display Register)

This instruction determines the state of the display register in the Type 2 Scanner selected by the interface address in ABAR. When this instruction is executed, the contents of the Type 2 Scanner display register are placed in the register specified by the R field.

The hardware, because of the display request (ICW bit 38), can cause status information for a particular interface to be placed into the Type 2 Scanner display register when the interface is scanned. Input X'46' can then be used to examine this status information. Refer to Output X'43', which must be used to set/reset the display request bit in the ICW.

Before accessing the display register with an Input X'46', the program must ensure that enough time has elapsed to guarantee that the interface has been scanned at least once after it set the display request bit in the ICW.

Byte 0, bit 0: Clear To Send—This bit is set to 1 if the 'clear to send' line from the modem is on, or if the diagnostic wrap forces 'clear to send' on; otherwise, it is set to 0.

Byte 0, bit 1: Ring Indicator—This bit is set to 1 if the 'ring indicator' line from the modem is on; otherwise, it is set to 0.

Byte 0, bit 2: Data Set Ready—This bit is set to 1 if the 'data set ready' line from the modem is on, or if the diagnostic wrap forces 'data set ready' on; otherwise, it is set to 0.

Byte 0, bit 3: Receive Line Signal Detector—This bit is set to 1 if the 'receive line signal detected' line from the modem is on, or if the diagnostic wrap state forces 'receive line signal detected' on; otherwise, it is set to 0.

Byte 0, bit 4: Receive Data Bit Buffer—This bit is set to 1 if the line interface receive data buffer contains a mark (1). If the buffer contains a space (0), this bit is set to 0.

Type 2 Communication Scanner

Byte 0, bit 5: Diagnostic Wrap Mode—This bit is set to 1 if the line interface is in diagnostic wrap state; otherwise, it is set to 0.

Byte 0, bit 6: Bit Service Request—This bit is set to 1 if the line interface 'bit service request' is on; otherwise, it is set to 0. Normally, bit service must be on before the Type 2 Scanner can access the line associated with the interface address or initiate transmit or receive operations.

Byte 0, bit 7: This bit is 0.

Byte 1, bits 0-7: These bits are 0.

Autocall Interface Bits for Input X'46'

Byte 0, bit 0: Abandon Call and Retry—This bit is set to 1 if the autocall unit 'abandon call and retry' (ACR) is active; otherwise it is set to 0.

Byte 0, bit 1: Present Next Digit—This bit is set to 1 if the autocall unit 'present next digit' (PND) is active; otherwise, it is set to 0.

Byte 0, bit 2: Data Line Occupied—This bit is set to 1 if the autocall unit 'data line occupied' (DLO) is active; otherwise, it is set to 0.

Byte 0, bit 3: Power Indicator—This bit is set to 1 if the autocall unit 'power indicator' (PWI) is active; otherwise, it is set to 0.

Byte 0, bit 4: This bit is 0.

Byte 0, bit 5: Call Originating Status—This bit is set to 1 if the autocall unit 'call originating status' (COS) is active; otherwise, it is set to 0.

Byte 0, bit 6: Bit Service Request—This bit is the same as for line interface.

Byte 0, bit 7: This bit is 0.

Byte 1, bits 0-7: These bits are 0.

Input X'47' (ICW Input Register-Bits 32-45)

This instruction determines the state of SDF bits 8-9, the ones counter (SDLC), the last line state bit (SDLC), the display request bit, the L2 interrupt pending bit, priority bits 1-2, and the NRZI control bit (SDLC). The interface

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-7

Type 2 Communication Scanner

address in the ABAR selects the proper scanner and associated ICW. When this instruction is executed, the contents of the ICW input register, bit positions 32 through 45, are placed in the register specified by the R operand. Byte 0, bit 7 and byte 1, bits 0,6, and 7 are set to 0. See I/O Programming Considerations in Chapter 7 for conditions that cause the ICW input register to be set. For an interpretation of these bits, see the Interface Control Word Format and SDLC sections in Chapter 7.

Type 2 Scanner Output Instructions

Output X'40' (Interface Address)

This instruction sets an interface address in the attachment buffer address register (ABAR) of the Type 2 Attachment Base. When this instruction is executed, byte 0, bit 6 through byte 1, bit 6 in the register specified by the R operand are placed in the ABAR.

The interface address, placed in ABAR, selects the Type 2 Scanner and the ICW associated with that address. When accessed, the ICW is placed in the ICW work register by the scanner hardware. If Output X'40' is executed in program levels 3 or 4, the contents of the ICW work register are placed in the ICW input register where they are available for access by Inputs X'44', X'45', and X'47'.

Output X'41' (Address Substitution Control)

This instruction sets the substitution control register in the Type 2 Attachment Base. See Address Substitution in Chapter 7 for a description and coding of the substitution control bits.

Execution of Output X'41' causes byte 1, bits 2-5 from the register specified by R to be placed into the substitution control register.

Programming Note

If address substitution is not used. Output X'41' must be executed with byte 1, bits 2-5 off in the register specified by R.

Output X'42' (Scan Limit Control)

This instruction sets the 'scan limit' in the selected Type 2 Scanner. At least one Output X'42' must be executed for each Type 2 Scanner available. The scanner selected is determined by the interface address in the attachment buffer address register (ABAR) of the Attachment Base at the time of execution.

When this instruction is executed, byte 1, bits 6 and 7 in the register specified by the R operand are placed in the 'scan limit' latches of the scanner.

Byte 0, bits 0-7: These bits are unused.

Byte 1, bits 0-5: These bits are unused.

Byte 1, bits 6-7: Scan Limit Select Bits 0 and 1—These bits are set as follows to indicate the desired scan limit for each Type 2 Scanner.

| Bit | Scan |
| --- | --- |
| 6 7 | Limit |
| 0 1 | 8 |
| 1 1 | 16 |
| 1 0 | 48 |
| 0 0 | 96 |

Output X'43' (Control)

This instruction sets or resets various control functions in a Type 2 Scanner. The Type 2 Scanner is selected by the interface address in the attachment buffer address register (ABAR) of the Attachment Base. When this instruction is executed, the bit configuration in the register specified by the R field determines which control functions are set or reset.

Byte 0, bit 0: Set Function—A 1 in this position causes the functions of byte 0, bits 2-7 and byte 1, bits 0-7 of this output to be set when the corresponding bit is 1. This bit should not be 1 if byte 0, bit 1 is 1.

Byte 0, bit 1: Reset Function—A 1 in this position causes the functions of byte 0, bits 2-7 and byte 1, bits 0-7 of this output to be reset when the corresponding bit is 1. This bit should not be 1 if byte 0, bit 0 is 1.

Byte 0, bit 2: Display Request—A 1 in this position causes the display request (ICW bit 38) to be set or reset according to byte 0, bits 0 and 1.

Byte 0, bits 3-6: These bits are unused.

Byte 0, bit 7: Disable LIB Position 1—A 1 in this position causes LIB position 1 of the addressed scanner to be disabled or enabled. When this bit is on and byte 0, bit 0 (set function) is on, the LIB position is disabled. When this bit is on and byte 0, bit 1 (reset function) is on, the LIB position is enabled.

Byte 1, bit 0: Disable LIB Position 2—This bit is associated with LIB position 2 and functions the same as byte 0, bit 7.

B-8

Byte 1, bit 1: Disable LIB Position 3—This bit is associated with LIB position 3 and functions the same as byte 0, bit 7.

Byte 1, bit 2: Disable LIB Position 4—This bit is associated with LIB position 4 and functions the same as byte 0, bit 7.

Byte 1, bit 3: Disable LIB Position 5—This bit is associated with LIB position 5 and functions the same as byte 0, bit 7. This bit should be set for a Type 2 Scanner-1.

Byte 1, bit 4: Disable LIB Position 6—This bit is associated with LIB position 6 and functions the same as byte 0, bit 7. This bit should be set for a Type 2 Scanner-1.

Byte 1, bit 5: Type 2 Scanner Level 1 Request—This bit is for diagnostic purposes. A 1 in this position along with the set function (byte 0, bit 0) causes a level 1 interrupt request and sets the following check latches.

•   LIB positions 1-6 Bit Clock Checks.

•   LIB Select Check.

•   ICW Input Register check.

•   ICW Work Register check.

•   Priority Register Available check.

•   CCU Outbus check. .

•   LINEADDBUS check.

A 1 in this position along with the reset function (byte 0, bit 1) resets the level 1 interrupt request and resets the above check latches.

Byte 1, bit 6: Disable Interrupt Requests—This bit is for diagnostic purposes. A 1 in this position along with the set function (byte 0, bit 0) sets the 'power-on-reset' latch and resets the scanners and LIBs. A 1 in this position along with the reset function (byte 0, bit 1) resets the 'power-on-reset' latch, thereby ending the reset state.

Programming Note

During any 'power-on' sequence, the POR latch is set in each Type 2 Scanner. The program must reset this latch with an Output X'43' instruction for each Type 2 Scanner before the scanner can be initialized. Also, a minimum of two scan periods (307.2 microseconds) is required between setting and resetting this bit to ensure completion of the reset function.

Byte 1, bit 7: This bit is unused.

Type 2 Communication Scanner

Output X'44' (ICW Bits 0-15)

This instruction resets secondary control field (SCF) bits 0-3 and 5 (bit 5 is for SDLC use only), and sets or resets bits 6-7 of the SCF. It is also used to set or reset the parallel data field (PDF). The PDF field is used as a character buffer. The interface address in the attachment buffer address register (ABAR), located in the Type 2 Attachment Base, selects the Type 2 Scanner and the ICW associated with this address.

When this instruction is executed, the bit configurations in the register specified by the R field determine what bits are to be set, reset, or left unchanged in ICW bits 0-3 and 5-15. Refer to the Secondary Control Field of the ICW (in Chapter 7) for a description of SCF bits (byte 0, bits 0-7). See ICW Format for the PDF as it relates to various LCD states. See the SDLC section of Chapter 7 for Synchronous Data Link Control descriptions of the SCF bits.

Byte 0, bit 0: When this bit is a 1, ICW bit 0 (stop bit check, receive break, or SDLC abort) is reset to 0; otherwise, it is unchanged.

Byte 0, bit 1: When this bit is a 1, ICW bit 1 (service request) is reset to 0; otherwise, it is unchanged.

Byte 0, bit 2: If this bit is a 1, ICW bit 2 (character overrun/underrun) is reset to 0; otherwise, it is unchanged.

Byte 0, bit 3: If this bit is a 1, ICW bit 3 (modem check) is reset to 0; otherwise, it is unchanged.

Byte 0, bit 4: This bit is unused.

Byte 0, bit 5: If this bit is a 1, ICW bit 5 (SDLC flag detection/disable zero-insert remembrance) is reset to 0; otherwise, it is unchanged.

Byte 0, bit 6: If this bit is a 1, ICW bit 6 (program flag) is set to 1; otherwise, it is reset to 0.

Byte 0, bit 7: If this bit is a 1, ICW bit 7 (pad flag or SDLC disable zero-insert control) is set to 1; otherwise, it is reset to 0.

Byte 1, bits 0-7: The settings of these bits are placed into the ICW bit positions 8-15 (PDF bits 0-7)

IBM 3704 and 3705 Communications Controllers Principles of Operation B-9

Type 2 Communication Scanner

Output X'45' (ICW Bits 16-23)

This instruction sets the bits of the line control definer (LCD) and the primary control field (PCF) in the ICW. When this instruction is executed, byte 1, bits 0-7 in the register specified by the R field are placed in the LCD and PCF fields. The interface address in the ABAR at execution time selects the proper scanner and the associated ICW. For a detailed description of these bits, see the Interface Control Word Format section in Chapter 7. For a description of these bits when used for SDLC, refer to the SDLC section of Chapter 7.

Byte 0, bits 0-7: These bits are unused.

Byte 1, bits 0-3: The settings of these bits are placed into the ICW bit positions 16-19 (LCD bits 0-3).

Byte 1, bits 4-7: The settings of these bits are placed into the ICW bit positions 20-23 (PCF bits 0-3).

Output X'46' (ICW Bits 24-33)

This instruction sets the bits of the serial data field (SDF) in the ICW. When this instruction is executed, byte 0, bits 6-7 and byte 1, bits 0-7 in the register specified by the R field are placed in the SDF of the ICW. The interface address in the ABAR at execution time selects the proper scanner and the associated ICW. For a detailed description of these bits, see the Interface Control Word Format section in Chapter 7.

Byte 0, bit 0: NRZI Control (SDLC only)—The setting of this bit is placed into ICW bit position 44, which is used by SDLC to control the mode of data transmission. See the SDLC section of Chapter 7 for a description of NRZI mode.

Byte 0, bits 1-5: These bits are unused.

Byte 0, bits 6-7: The settings of these bits are placed into the ICW bit positions 24-25 (SDF bits 0 and 1).

Byte 1, bits 0-7: The settings of these bits are placed into the ICW bit positions 26-33 (SDF bits 2-9).

Output X'47' (ICW Bits 34-37 and 39-43)

This instruction sets the state of ICW bits 34-37 and 39-43. Execution of this instruction places bits from the register specified by R into the appropriate ICW bit positions. The interface address in the ABAR at execution time selects the proper scanner and the associated ICW. For a detailed description of these bits, see the Interface Control Word Format section in Chapter 7. For a description of these bits when used for SDLC, refer to the SDLC section of Chapter 7.

Byte 0, bits 0-5: These bits are unused.

Byte 0, bit 6: Ones Counter 0—The setting of this bit is placed in ICW bit position 34. ICW bit positions 34-36 form the SDLC ones counter.

Byte 0, bit 7: Ones Counter 1—The setting of this bit is placed in ICW bit position 35.

Byte 1, bit 0: Ones Counter 2—The setting of this bit is placed in ICW bit position 36.

Byte 1, bit 1: Last Line State (SDLC)—This bit is set by hardware, but it can be set and reset by this output.

Byte 1, bit 2: This bit is unused.

Byte 1, bits 3-4: The settings of these bits are placed into the ICW bit positions 39-40. (These bits are reserved and should be 0.)

Byte 1, bit 5: L2 Interrupt Pending—A 1 in this position sets ICW bit 41, which forces an interrupt for a particular interface without requiring the interface to have a service request set. A 0 leaves the ICW bit unchanged.

Byte 1, bits 6-7: The settings of these bits are placed into the ICW bit positions 42-43 (priority bits 1 and 2).

B-10

Type 3 Communication Scanner

Type 3 Scanner Input Instructions

Input X'40' (Interface Address)

This instruction obtains the line interface address from the ABAR in the attachment base. Conditions that set the ABAR are described in the I/O Programming Considerations section in Chapter 8. When this instruction is executed, the interface address from the ABAR is placed in byte 0, bit 6 through byte 1, bit 6 of the register specified by the R operand. Byte 0, bit 4 is always set to 1. The other register bit positions are set to 0.

Input X'41' (High Speed Select)

This instruction determines the setting of the high speed select register. The contents of this register are placed in the register specified by the R field. Refer to Type 3 Scanner Addressing in Chapter 8 for the meanings of the high speed select bits.

Input X'42' (DBAR/Check Register 0)

This instruction determines the state of check register 0, the diagnostic buffer address register (DBAR) and the upper scan limit bits. The contents of these registers and bits are placed in the register specified by the R field.

Input X'43' (Check Register 1)

This instruction obtains the status of the check register 1 in the scanner. Since there can be up to four Type 3 scanners, the check register selected is determined by the interface address in the ABAR at the time of instruction execution.

When this instruction is executed, the check register bits are placed in the register specified by the R field.

Programming Note

If any of the check register bits in the scanner are set to 1, the Type 3 Scanner LI interrupt request is set.

Byte 0, bit 0: LIB 1 Bit Clock Check-This bit is set to 1 if a LIB position 1 bit clock control check is detected by the Type 3 Scanner; otherwise, it is set to 0.

Byte 0, bit 1: LIB 2 Bit Clock Check-Same as above.

Byte 0, bit 2: LIB 3 Bit Clock Check-Same as above.

Byte 0, bit 3; LIB 4 Bit Clock Check-Same as above.

Byte 0, bits 4 and 5: These bits are 0.

Byte 0, bit 6: LIB Select Check-This bit is set to 1 if more than one LIB was selected or more than one line (or no line) was accessed on the selected LIB, or a line was accessed on a LIB that was not selected.

Byte 0, bit 7: ICW Input Register Check-This bit is set to 1 if the Type 3 Scanner has detected a parity error in the ICW input register; otherwise, it is set to 0.

Byte 1, bit 0: ICW Work Register Check 1-This bit is set to 1 if the Type 3 Scanner has detected a parity error in the ICW work register; otherwise, it is set to 0.

Byte 1, bit 1: Priority Register Available Check—This bit is set to 1 if the Type 3 Scanner has detected even parity on one of the four priority register available lines; otherwise, it is set to 0. These four lines from the attachment base are parity-checked in the Type 3 Scanner.

Byte 1, bit 2: CCU Outbus Check-This bit is set to 1 if the Type 3 Scanner has detected even parity on the CCU Outbus. Otherwise, it is set to 0.

Byte 1, bit 3: Line Address Bus Check—This bit is set to 1 if the Type 3 Scanner has detected a scanner buffer address register bus parity error during program addressing (refer to the Program Addressing section in Chapter 8); otherwise, it is set to 0.

Byte 1, bit 4: Bad Inbound CS Data-This bit is set to 1 if the CCU detects a parity check on the data being transferred to storage via a cycle steal operation.

Byte 1, bit 5: CSAR Check-This bit is set to 1 if the CCU detects an even parity check in the CS address register.

Byte 1, bit 6: Address Exception—This bit is set to 1 if the CCU detects that the address in the CSAR exceeds the CCU storage size. The bit is set to 0 if byte 1, bit 5 is 1.

Byte 1, bit 7: ICW Work Register 0 Check-This bit is set to 1 if the CCU detects an even parity check from the ICW work register for byte 0 or 14.

Input X'44' (ICW Byte 0 and PDF Array)

This instruction determines the state of the secondary control field (SCF) and the parallel data field (PDF) from the PDF array. The interface address in the ABAR selects the proper scanner. When this instruction is executed, the contents of the ICW input register, byte 0, and the PDF input register are placed in the register specified by the R field. Refer to I/O Programming Considerations in Chapter 8 for conditions that set the ICW input register. The Interface Control Word Format section of Chapter 8 describes the SCF and PDF fields and their bit definitions.

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-ll

Type 3 Communication Scanner

Input X'45' (ICW Bytes 2 and 3)

This instruction may be used to determine the state of the LCD, basic PCF, and SDF fields of the ICW that is set in the ICW input register. The interface address in the ABAR selects the proper scanner. When this instruction is executed the contents of the ICW input register bit positions bytes 2 (LCD, PCF) and 3 (SDF) are placed in the register specified by the R field. Refer to I/O Programming Considerations in Chapter 8 for conditions that set the ICW input register. The LCD, PCF, and SDF fields and bit definitions are described in the Interface Control Word Format section of Chapter 8.

Programming Note

This input instruction does not contain the complete serial data field. Only bits 0-7 of the field are available with this input. The remainder of the SDF (bits 8-9) is accessed by the Input X'47' instruction.

Input X'46' (Display Register)

This instruction may be used to determine the state of the display register in the Type 3 Scanner selected by the interface address in ABAR. When this instruction is executed, the contents of the scanner display register are placed in the register specified by the R field.

The hardware, because of the display request (ICW bit 4.6), can cause status information for a particular interface to be placed into the Type 3 Scanner display register when the interface is scanned. Input X'46' can then be used to examine this status information. Refer to Output X'43', which must be used to set/reset the display request bit in the ICW.

Before accessing the display register with an Input X'46', the program must ensure that enough time has elapsed to guarantee that the interface has been scanned at least once after it set the display request bit in the ICW.

Line Interface Bits for Input X'46'

Byte 0, bit 0: Clear to Send-This bit is set to 1 if the 'clear to send' line from the modem is on, or if the diagnostic wrap forces 'clear to send' on; otherwise, it is set toO.

Byte 0, bit 1: Ring Indicator-This bit is set to 1 if the 'ring indicator' line from the modem is on; otherwise, it is set to 0.

Byte 0, bit 2: Data Set Ready-This bit is set to 1 if the 'data set ready'-.line from the modem is on, or if the diagnostic wrap forces 'data set ready' on; otherwise, it is set to 0.

Byte 0, bit 3: Receive Line Signal Detector—This bit is set to 1 if the 'receive line signal detected' line from the modem is on, or if the diagnostic wrap state forces 'receive line signal detected' on; otherwise, it is set to 0.

Byte 0, bit 4: Receive Data Bit Buffer-This bit is set to 1 if the line interface receive data buffer contains a mark (1). If the buffer contains a space (0), this bit is set to 0.

Byte 0, bit 5: Diagnostic Wrap Mode—This bit is set to 1 if the line interface is in diagnostic wrap state; otherwise, it is set to 0.

Byte 0, bit 6: Bit Service Request—This bit is set to 1 if the line interface 'bit service request' is on; otherwise, it is set to 0. Normally, bit service must be on before the Type 3 Scanner can access the line associated with the interface address or initiate transmit or receive operations.

Byte 0, bit 7: ICW Test Mode-This bit is set to 1 if the diagnostic mode latch is set by an Output X'43' instruction or by an 'ICW and array reset.'

Byte 1, bits 0-7: These bits are 0.

Autocall Interface Bits for Input X'46'

Byte 0, bit 0: Abandon Call and Retry-This bit is set to 1 if the autocall unit 'abandon call and retry' (ACR) is active; otherwise it is set to 0.

Byte 0, bit 1: Present Next Digit-This bit is set to 1 if the autocall unit 'present next digit' (PND) is active; otherwise, it is set to 0.

Byte 0, bit 2: Data Line Occupied-This bit is set to 1 if the autocall unit 'data line occupied' (DLO) is active; otherwise, it is set to 0.

Byte 0, bit 3: Power Indicator-This bit is set to 1 if the autocall unit 'power indicator' (PWI) is active; otherwise, it is set to 0.

Byte 0, bit 4: This bit is 0.

Byte 0, bit 5: Call Originating Status-This bit is set to 1 if the autocall unit 'call originating status' (COS) is active; otherwise, it is set to 0.

B-12

Type 3 Communication Scanner

Byte 0, bit 6: Bit Service Request-This bit is the same as for line interface.

Byte 0, bit 7: ICW Test Mode-This bit is the same as for a line interface.

Byte 1, bits 0-7: These bits are 0.

Input X'47' (ICW Bytes 4 and 5)

This instruction is used to determine the state of the bits of ICW bytes 4 and 5: SDF bits 8 and 9, interval timer/ones counter, timeout control/last line state, display request, level 2 interrupt pending, priority select 1 and 2, transparent text/NRZI control, diagnostic 1 and 2, and external modem check bits. The interface address in the ABAR selects the proper scanner and the associated ICW. This instruction places the contents of the ICW input register, bytes 4 and 5, in the register specified by the R field. See I/O Programming Considerations in Chapter 8 for conditions that cause the ICW input register to be set. For an interpretation of these bits, see Interface Control Word Format in Chapter 8.

Input X'48' (ICW Bytes 6 and 7)

This instruction is used to determine (1) the contents of the cycle steal byte count and cycle steal extended address bits X.6 and X.7, (2) the setting of the cycle steal control bits, and (3) whether ETB, ETX, or ENQ characters are present in data. This instruction places the contents of ICW input register bytes 6 and 7 in the register specified by the R field.

Input X'49' (ICW Bytes 8 and 9)

This instruction is used to determine the contents of the cycle steal address register (CSAR). The instruction places the contents of ICW input register bytes 8 and 9 in the register specified by the R field.

Input X'4A' (ICW Bytes 10 and 11)

This instruction is used to determine the contents of the old block check characters (BCC). The instruction places the contents of ICW input register bytes 10 and 11, containing the 16-bit accumulation for SDLC or BSC data, in the register specified by the R field.

Input X'4B' (ICW Byte 16)

This instruction is used to determine the state of the extended PCF and the new sync, data terminal ready, and OLT diagnostic bits. The instruction places the contents of ICW input register byte 16 in the register specified by the R field.

Input X'4C' (PDF Array Bits 0-10)

This instruction is used to determine the contents of bits 0-10 of the PDF currently selected by the PDF array pointer (ICW bits 12.4-12.7). The instruction places the contents of the PDF input register (previously loaded from the PDF) in the register specified by the R field. Refer to PDF Array Format in Chapter 8 for the meanings of the PDF array bits.

Input X'4E' (ICW Bytes 12 and 13)

This instruction is used to determine the PDF array address pointed to by the cycle steal PDF array pointer and the PDF array pointer, and to determine the state of the sequence 0, 1, and 2, RTS turnaround control, and cycle steal message counter bits. The interface address in the ABAR selects the proper scanner and the associated ICW. The instruction places the contents of the ICW input register bytes 12 and 13 in the register specified by the R field.

Byte 0, bits 0-3: Cycle Steal PDF Array Pointer-These bits contain the address of the current PDF from which or to which data is being transferred via cycle steal operation.

Byte 0, bits 4-7: PDF Array Pointer-These bits contain the address of the current PDF from which or to which data is being transferred by the control program.

Byte 1, bits 0-1: Sequence 0 and 1—These bits are used by the scanner in determining the appropriate action to perform upon receiving or transmitting BSC and SDLC control sequences within message data.

Byte 1, bit 2: Request-to-Send Turnaround Control—This bit is set or reset by the control program to determine whether the RTS lead in the modem interface is to remain active or be deactivated when a line turnaround from transmit state to receive state occurs.

Byte 1, bit 3: Sequence 2—This bit is used by the scanner (1) in determining the action to take upon receiving an SDLC flag character under certain conditions, or (2) to indicate that character phase has been entered, for a BSC line.

Byte 1, bits 4-5: These bits are 0.

Byte 1, bits 6-7: Message Counter—These bits are used by the Type 3 Scanner in keeping track of multiple SDLC messages that may be queued in the PDF array during a receive operation.

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-13

Type 3 Communication Scanner

Input X'4F' (ICW Bytes 14 and 15)

This instruction is used to determine the state of the status exception and BSC/SDLC control status fields of the ICW. Refer to Interface Control Word Format in Chapter 8 for the meanings of the bits of these fields. The instruction places the contents of the ICW input register bytes 14 and 15 in the register specified by the R field.

Type 3 Scanner Output Instructions

Output X'40' (Interface Address)

This instruction is used to set an interface address in the attachment buffer address register (ABAR) of the Type 2 Attachment Base. When this instruction is executed, byte 0, bit 6 through byte 1, bit 6 in the register specified by the R operand are placed in the ABAR.

The interface address, placed in ABAR, selects the Type 3 Scanner and the ICW associated with that address. When accessed, the ICW is placed in the ICW work register by the scanner hardware. If Output X'40' is executed in program levels 3 or 4, the contents of the ICW work register are placed in the ICW input register where they are available for access by Inputs X'44', X'45', and X'47'.

Output X'41' (Scan Substitution Control)

This instruction must be used to set the substitution control register in the Type 2 Attachment Base. See Address Substitution in Chapter 8 for a description and coding of the substitution control bits.

Execution of Output X'41' causes byte 1, bits 2-5 from the register specified by R to be placed into the substitution control register.

Programming Note

If address substitution is not used, Output X'41' must be executed with byte 1, bits 2-5 off in the register specified by R.

Output X'42' (DBAR/Scan Limit Control)

This instruction must be used to set the diagnostic buffer address register (DBAR) and the scan limit in the selected Type 3 Scanner. At least one Output X'42' must be executed for each Type 3 Scanner available. The scanner selected is determined by the interface address in the attachment buffer address register (ABAR) of the Attachment Base at the time of execution.

When this instruction is executed, byte 0, bits 1-5 of the register specified by the R field are placed in the DBAR, and byte 1, bits 6 and 7 of that register are placed in the scan limit latches of the scanner.

Byte 0, bits 0-7: These bits are 0.

Byte 1, bits 0-5: DBAR bits—These bits (0-5) are placed in bit positions 5-0, respectively, of the DBAR.

Byte 1, bits 6-7: Scan Limit Select Bits 0 and 1-These bits are set as follows to indicate the desired scan limit for each Type 3 Scanner.

| Bit | Scan |
| --- | --- |
| 6 7 | Limit |
| 0 1 | 8 |
| 1 1 | 16 |
| 1 0 | 48 |
| 0 0 | 96 |

Output X'43' (Control)

This instruction may be executed to set or reset various control functions in a Type 3 Scanner. The Type 3 Scanner is selected by the interface address in the attachment buffer address register (ABAR) of the Attachment Base. When this instruction is executed, the bit configuration in the register specified by the R field determines which control functions are set or reset.

Byte 0, bit 0: Set Function—A 1 in this position causes the functions of byte 0, bits 2-7 and byte 1, bits 0-7 of this output to be set when the corresponding bit is 1. This bit should not be 1 if byte 0, bit 1 is 1.

Byte 0, bit 1: Reset Function—A 1 in this position causes the functions of byte 0, bits 2-7 and byte 1, bits 0-7 of this output to be reset when the corresponding bit is 1. This bit should not be 1 if byte 0, bit 0 is 1.

Byte 0, bit 2: Display Request—A 1 in this position causes the display request (ICW bit 4.6) to be set or reset according to byte 0, bits 0 and 1.

Byte 0, bits 3-5: These bits are unused.

Byte 0, bit 6: Diagnostic Test Mode—A 1 in this position sets the diagnostic latch of the scanner.

Byte 0, bit 7: Disable/Enable LIB Position—A 1 in this position causes LIB position 1 of the addressed scanner to be disabled or enabled. When this bit is on and byte 0, bit 0 (set function) is on, the LIB position is disabled. When this bit is on and byte 0, bit 1 (reset function) is on, the LIB position is enabled.

B-14

Type 3 Communication Scanner

Byte 1, bit 0: Disable/Enable LIB Position 2—This bit is associated with LIB position 2 and functions the same as byte 0, bit 7.

Byte 1, bit 1: Disable/Enable LIB Position 3—This bit is associated with LIB position 3 and functions the same as byte 0, bit 7.

Byte 1, bit 2: Disable/Enable LIB Position 4—This bit is associtated with LIB position 4 and functions the same as byte 0, bit 7. This bit should be set for a Type 3 Scanner in scanner position 1.

Byte 1, bits 3 and 4: These bits are unused.

Byte 1, bit 5: Type 3 Scanner Level 1 Request—This bit is for diagnostic purposes. A 1 in this position along with the set function (byte 0, bit 0) causes a level 1 interrupt request and sets all latches in the control register.

A 1 in this position along with the reset function (byte 0, bit 1) resets the level 1 interrupt request and resets all latches in the control register.

Byte 1, bit 6: Disable Interrupt Requests—This bit is for diagnostic purposes. A 1 in this position along with the set function (byte 0, bit 0) sets the 'power-on-reset' latch and resets the scanners and LIBs. A 1 in this position along with the reset function (byte 0, bit 1) resets the 'power-on-reset' latch, thereby ending the reset state.

Programming Note

During any 'power-on' sequence, the POR latch is set in each Type 3 Scanner. The program must reset this latch with an Output X'43' instruction for each Type 3 Scanner before the scanner can be initialized. Also, a minimum of one scan period (153.6 microseconds) is required between setting and resetting this bit to ensure completion of the reset function.

Byte 1, Bit 7: This bit is unused.

Output X'44' (ICW Bytes 0 and 1)

This instruction (1) resets secondary control field (SCF) bits 0-3 and 5 (5 is for SDLC use only), (2) sets or resets SCF bits 6-7, (3) sets SCF bit 4, and (4) places data in the parallel data field (PDF) of the selected scanner. The PDF field is used as a character buffer. The interface address in the ABAR selects the Type 3 Scanner and the ICW associated with this address.

When this instruction is executed, the bit configuration in the register specified by the R field determine which bits are to be set, reset, or left unchanged in ICW bytes 0 and 1. Refer to the Secondary Control Field of the ICW in Chapter 8 for a description of the SCF bits and Interface Control Word Format in that chapter for a description of the PDF as it relates to various LCD states.

Byte 0, bit 0: When this bit is a 1, ICW bit 0.0 (SDLC abort) is reset to 0; otherwise, it is unchanged.

Byte 0, bit 1: When this bit is a 1, ICW bit 0.1 (normal service request interrupt) is reset to 0; otherwise, it is unchanged.

Byte 0, bit 2: If this bit is a 1, ICW bit 0.2 (character over-run/underrun) is reset to 0; otherwise, it is unchanged.

Byte 0, bit 3: If this bit is a 1, ICW bit 0.3 (modem check) is reset to 0; otherwise, it is unchanged.

Byte 0, bit 4: If this bit is a 1, ICW bit 0.4 (not level 2 bit) is set to 1; otherwise, it is unchanged.

Byte 0, bit 5: If this bit is a 1, ICW bit 0.5 (end of message) is reset to 0; otherwise, it is unchanged.

Byte 0, bit 6: If this bit is a 1, ICW bit 0.6 (program flag) is set to 1; otherwise, it is reset to 0.

Byte 0, bit 7: If this bit is a 1, ICW bit 0.7 (line trace active) is set to 1; otherwise, it is reset to 0.

Byte 1, bits 0-7: The settings of these bits are placed into the ICW positions 1.0-1.7 (PDF bits 0-7).

Output X'45' (ICW Bytes 2 and 16)

This instruction is used to set the bits of the line control definer (LCD), primary control field (PCF), and extended PCF of the ICW, and the new sync, data terminal ready, and OLT diagnostic bits of ICW byte 16. When this instruction is executed, byte 1, bits 0-7 of the register specified by the R field are placed in the LCD and PCF fields, byte 0, bits 0-3 of the register are placed in ICW bits 16.0-16.3, and byte 0, bits 4-7 of the register are placed in the extended PCF (ICW bits 16.4-16.7). The interface address in the ABAR selects the proper scanner and the associated ICW. For a detailed description of these bits, see Interface Control Word Format in Chapter 8.

IBM 3704 and 3705 Communications Controllers Principles of Operation B-15

Type 3 Communication Scanner

Byte 0, bits 0-3: The settings of these bits are placed in ICW bit positions 16.0-16.3.

Byte 0, bits 4-7: The settings of these bits are placed in ICW bit positions 16.4-16.7 (extended PCF).

Byte 1, bits 0-3: The settings of these bits are placed in ICW bit positions 2.0-2.3 (LCD).

Byte 1, bits 4-7: The settings of these bits are placed in ICW bit positions 2.4-2.7 (PCF).

Output X'46' (ICW Bits 3.0-4.1)

This instruction is used to set the bits of the serial data field (SDF) of the ICW and indirectly to set or reset bits 5.4-5.7 of the ICW via a set mode operation subsequently executed by the scanner. When this instruction is executed, byte 0, bits 6-7 and byte 1, bits 0-7 of the register specified by the R field are placed in the SDF. Byte 0, bits 0-3 of the register are placed in ICW bit positions 3.0, 3.1, 3.2, and 3.6, respectively. The interface address in the ABAR at execution time selects the proper scanner and the associated ICW. For a detailed description of these bits, see Interface Control Word Format in Chapter 8.

Byte 0, bits 0-3: The settings of these bits are placed in ICW bit positions 3.0, 3.1, 3.2, and 3.6, respectively.

Byte 0, bits 4-5: These bits are unused.

Byte 0, bits 6-7: The settings of these bits are placed in ICW bit positions 3.0-3.1, respectively (SDF bits 0 and 1).

Byte 1, bits 0-7: The settings of these bits are placed into ICW bit positions 3.2-4.1, respectively (SDF bits 2-9).

Output X'47' (ICW Bits 4.2-4.5, 4.7-5.3)

This instruction sets the state of ICW bits 4.2-4.5 and 4.7-5.3. Execution of this instruction places bits from the register specified by the R field into the corresponding ICW bit positions. The interface address in the ABAR at execution time selects the proper scanner and the associated ICW. For a description of these bits, see Interface Control Word Format in Chapter 8.

Byte 0, bits 0-5: These bits are unused.

Byte 0, bits 6-7, byte 1, bit 0: Ones Counter/Interval Timer Bits 0, 1, and 2—The settings of these bits are placed in ICW bit positions 4.2,4.3, and 4.4, respectively.

Byte 1, bit 1: Last Line State/Timeout Control—This bit is set by the scanner hardware, but it can be set and reset by this instruction.

Byte 1, bit 2: This bit is unused.

Byte 1, bits 3-4: Ones Counter/Interval Timer Bits 3 and 4— The settings of these bits are placed in ICW bit positions 4.7 and 5.0, respectively.

Byte 1, bit 5: Level 2 Interrupt Pending—A 1 in this bit position sets ICW bit 5.1, which forces an interrupt for a particular interface without requiring the interface to have a service request set. A 0 in this bit position leaves the ICW bit unchanged.

Byte 1, bits 6-7: Priority Select Bits 1 and 2-The settings of these bits are placed into ICW bit positions 5.2-5.3, respectively.

Output X'48' (ICW Bytes 6 and 7)

This instruction sets the bits of the cycle steal address, cycle steal control and cycle steal byte count fields. When this instruction is executed, bytes 0 and 1 of the register specified by the R field are placed in bytes 6 and 7, respectively, of the ICW. The interface address in the ABAR at execution time selects the proper scanner and the associated ICW.

Byte 0, bits 0-1: These bits are 0.

Byte 0, bit 2: Cycle Steal Address bit X.6.

Byte 0, bit 3: Cycle Steal Address bit X.7.

Byte 0, bit 4: This bit is 0.

Byte 0, bit 5: Cycle Steal Valid-This bit is set to 1 to allow a cycle steal operation.

Byte 0, bit 6: Data Chain Flag-This bit is set to 1 to indicate data chaining.

B-16

Byte 0, bit 7: Message Chain Flag—This bit is set to 1 to indicate message chaining.

Byte 1, bits 0-7: Cycle Steal Byte Count-These bits contain the byte count for the cycle steal operation.

Output X'49' (ICW Bytes 8 and 9)

This instruction places into the CSAR (ICW bytes 8 and 9) the storage address of the first data byte to be stored or accessed by a cycle steal operation. The storage address is obtained by combining the extended addressing bits (ICW bits 6.2-6.3) and the bits of the CSAR.

Output X'4A' (ICW Bytes 10 and 11)

This instruction sets the contents of the block check characters (BCC) in ICW bytes 10 and 11. This is not a normal action used by the control program, because the scanner hardware accumulates the BCC as it accesses or stores data. The interface address in the ABAR at execution selects the proper scanner and the associated ICW.

Output X'4C' (ICW Byte 1)

This instruction loads the PDF array with the 11-bit data from the register specified by the R field. The PDF pointer is not changed by this instruction. The interface address in the ABAR at execution time selects the proper scanner and the associated ICW. For a description of the PDF bits, see PDF Array Format in Chapter 8.

Output X'4D' (ICW Byte 1)

This instruction loads the cycle steal PDFs with two data bytes from the register specified by the R field. The line address in the ABAR selects the proper scanner and the associated ICW and PDF array for the line interface. The cycle steal pointer selects one of the eight PDFs in the PDF array. Execution of this instruction causes the cycle steal PDF array pointer (ICW bits 12.0-12.3) to be incremented by two.

Output X'4E' (ICW Bytes 12 and 13)

This instruction sets the contents of the ICW control bytes (12 and 13) from the register specified by the R field. The interface address in the ABAR at execution time selects the proper scanner and the associated ICW. For a detailed description of these bits, see Interface Control Word Format in Chapter 8.

Type 3 Communication Scanner

Byte 0, bits 0-3: Cycle Steal PDF Array Pointer-These bytes contain the address of the current PDF from which or to which data is being transferred via cycle steal operatkm-

Byte 0, bits 4-7: PDF Array Pointer-These bits contain the address of the current PDF from which or to which data is being transferred by the control program.

Byte 1, bits 0-1: Sequence 0 and I—These bits are used by the scanner in determining the appropriate action to perform upon receiving or transmitting BSC and SDLC control sequences within message data.

Byte 1, bit 2: Request-to-Send Turnaround Control—This bit is set or reset by the control program to determine whether the RTS lead in the modem interface is to remam active or be deactivated when a line turnaround from tran-mit state to receive state occurs.

Byte 1, bit 3: Sequence 2—This bit is used by the scanner (1) in determining the action to take upon receiving an SDLC flag character under certain conditions, or (2) to indicate that character phase has been entered, for a BSC line.

Byte 1, bits 4-5: These bits are 0.

Byte 1, bits 6-7: Message Counter—These bits are used by the Type 3 Scanner in keeping track of multiple SDLC messages that may be queued in the PDF array during a receive operation.

Output X'4F' (ICW Bytes 14 and 15)

This instruction (1) resets ICW bits 14.0, 14.1, 14.3, 14.4, 14.6, and 14.7 from the corresponding bits of the register specified by the R field, and (2) sets or resets ICW bits 14.2, 14.5, and 15.0-15.7 from the corresponding bits of the register. For a detailed description of these bits, see Interface Control Word Format in Chapter 8.

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-17

Type 1 and Type 4 Channel Adapters

Type 1 and Type 4 CA Input Instructions

Input X'60' (Initial Selection Control)

This instruction loads the register specified by R with the contents of the initial selection control register. The bits of this register are normally set at the completion of initial selection and identify the cause of a Type 1 or 4 CA initial L3 interrupt request.

Programming Note

The control program should not execute an Input X'60' while the channel adapter is enabled unless a Type 1 or 4 CA Initial Selection L3 Interrupt is present. If this input instruction is executed prior to an initial selection interrupt, an I/O parity check may occur because the data can be changing while the register is being accessed.

Byte 0, bit 0: Initial Selection Interrupt—This bit is set by hardware and causes a Type 1 CA initial L3 interrupt request when a Start I/O command is accepted by the adapter and clear initial status is presented to the host channel. This bit is also set when a Test I/O command is received for an emulation subchannel address and a status of X'70' is returned. If this bit is zero, the interrupt request was caused by an unusual condition and can be further defined by the remaining bits of this input. An Output X'62', byte 0, bit 5 or an Output X'60' resets this bit.

Byte 0, bit 1: Interface Disconnect—This bit is set by hardware when an interface disconnect condition (Halt I/O command) is detected during an initial selection sequence. An Output X'62', byte 0, bit 5 or an Output X'60' resets this bit.

Byte 0, bit 2: Selective Reset—This bit is set by hardware when a selective reset condition is detected during an initial selection sequence. An Output X'62', byte 0, bit 5 or an Output X'60' resets this bit.

Byte 0, bit 3: Channel Bus Out Check-This bit is set by hardware when incorrect parity is detected in the channel command byte during initial selection. This causes the CA to automatically respond with Unit Check (UC) initial status. An Output X'62', byte 0, bit 5 or an Output X'60 resets this bit.

Byte 0, bit 4: This bit is 0.

Byte 0, bit 5: Stacked Initial Status—This bit is set by hardware when a stacked status condition is detected during initial selection. An Output X'62', byte 0, bit 5 or an Output X'60' resets this bit.

Byte 0, bit 6: NSC Status Byte Cleared-This bit is set to indicate that a status byte for the native mode subchannel (NSC) has been transferred as the initial status byte in an initial selection. Therefore, the NSC status byte has been cleared, and this resulted in the setting of the CA initial level 3 interrupt request. An Output X'62', byte 0, bit 5 or an Output X'60' resets this bit.

Byte 0, bit 7: System Reset—This bit is set by hardware and causes a level 3 initial selection interrupt when a system reset is detected on the channel interface. An Output X'67', byte 1, bit 3 resets this bit.

Byte 1, bits 0-7: These bits are 0.

Input X'61' (Initial Selection Address and Command)

Programming Note

The control program should not execute an Input X'61' while the channel adapter is enabled unless a CA Initial Selection L3 Interrupt is present. If this input instruction is executed prior to an initial selection interrupt, an I/O parity check may occur because the data can be changing while the register is being accessed.

Byte 0, bits 0-7: Initial Selection Address—These bits are set during an initial 'selection sequence and contain the address of the line selected.

Byte 1, bits 0-7: Initial Selection Command—These bits are set during an initial selection sequence and contain the command as presented to the channel adapter from the channel interface.

Input X'62' (Data/Status Control)

This input loads the register specified by R with the contents of the data/status control register. The bits of this register identify the cause of a CA data/status L3 interrupt request.

Byte 0, bit 0: Outbound Data Transfer Sequence—This bit indicates that the channel adapter hardware is transferring data to the host processor channel. The control program can set or reset this bit with an Output X'62' instruction.

B-18

Byte 0, bit 1: Inbound Data Transfer Sequence—This bit indicates that the channel adapter hardware is transferring data from the host processor channel. The control program can set or reset this bit with an Output X'62' instruction.

Byte 0, bit 2: ESC Final Status Transfer Sequence—This bit indicates that the channel adapter is transferring a 2701/2702/2703 type status byte to the host processor channel. The control program can set or reset this bit with an Output X'62' instruction.

Byte 0, bit 3: NSC Channel End Status Transfer Sequence—This bit indicates that the channel adapter is transferring NSC Channel End status to the host processor channel. The control program can set or reset this bit with an Output X'62' instruction.

Programming Note

If the host channel accepts the NSC Channel End status byte, the channel adapter automatically generates a Busy initial status. This occurs in all subsequent initial selection sequences on the native mode subchannel until the control program initiates an NSC final status transfer by executing Output X'62'.

Byte 0, bit 4: NSC Final Status Transfer Sequence—This bit indicates that the channel adapter is tranferring final status to the host processor channel. The control program can set or reset this bit with an Output X'62' instruction.

Byte 0, bit 5: Channel Stop or Interface Disconnect— This bit indicates that a channel stop or interface disconnect was detected when the channel adapter was in a data transfer sequence. This bit is set by hardware and reset by an Output X'62', byte 0, bit 6.

Byte 0, bit 6: Suppress Out Monitor Interrupt—This bit is set by hardware and causes a CA data/status L3 interrupt request when the 'suppress out monitor' latch (see Output X'67') is on and the 'suppress out' tag line drops. This bit is reset by executing an Output X'62' instruction.

Byte 0, bit 7: Program-Requested Interrupt—This bit indicates that the program has requested a CA data/status L3 interrupt by executing an Output X'67' instruction. Execution of an Output X'62' instruction with any bit combination resets this bit.

Type 1 and Type 4 Channel Adapters

Byte 1, bit 0: Channel Bus Out Check—This bit is set by hardware to indicate incorrect parity on the channel interface during a Write command. This bit is reset by an Output X'62', byte 0, bit 6.

When this check occurs, the byte with incorrect parity is placed in the data buffer, and data transfer is terminated. The transfer byte count (byte 1, bits 5-7) reflects the byte that caused the check.

Byte 1, bit 1: Selective Reset-This bit is set by the CA hardware to indicate the detection of a selective reset when the CA was in a service transfer sequence. This bit is reset by an Output X'62', byte 0, bit 6.

Byte 1, bit 2: Suppress Out—This bit is set to indicate that the 'suppress out' tag line on the channel interface is active.

Byte 1, bit 3: Stacked Ending Status—This bit is set to indicate that the ending status has been stacked by the host processor channel during a status transfer sequence. This bit is reset by an Output X'62', byte 0, bit 6.

Byte 1, bit 4: I/O Command Chaining—This bit indicates command chaining when the CA is in a status transfer sequence. This bit is reset by an Output X'62', byte 0, bit 6.

Byte 1, bits 5-7: Transferred Byte Count Bits 0-2—These bits are set by the CA hardware and reflect the number of bytes transferred across the channel in the current data transfer operation or the last-completed data transfer operation (if none is in progress). Up to four bytes can be transferred in one data transfer sequence.

| 5 | Bit 6 | 7 | Number of Bytes Transferred |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 2 |
| 0 | 1 | 1 | 3 |
| 1 | 0 | 0 | 4 |

Input X'63' (CA Address and ESC Status Bytes)

This instruction loads a general register with the CA line address and the status bits for the line that were set by the last Output X'63' instruction.

Byte 0, bits 0-7: Address Byte—These bits contain the address of the last communication line to be

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-19

Type 1 and Type 4 Channel Adapters

serviced by the channel adapter for data or status transfer.

Byte 1, bit 0: ESC Attention.

Byte 1, bit 1: ESC Status Modifier.

Byte 1, bit 2: ESC Control Unit End.

Byte 1, bit 3: ESC Busy.

Byte 1, bit 4: ESC Channel End.

Byte 1, bit 5: ESC Device End.

Byte 1, bit 6: ESC Unit Check.

Byte 1, bit 7: ESC Unit Exception.

Input X'64' (Data Buffer Bytes 1 and 2)

This instruction loads a general register with the contents of the data buffer bytes 1 and 2 as received from the channel interface. It can also be used to verify the contents of the data buffers following an Output X'63' instruction.

Byte 0, bits 0-7: Data Buffer Byte 1—These bits represent the first byte of data received from the channel during a channel Write command.

Byte 1, bits 0-7: Data Buffer Byte 2—These bits represent the second byte of data received from the channel during a channel Write command.

Input X'65' (Data Buffer Bytes 3 and 4)

This instruction loads a general register with the contents of the data buffer bytes 3 and 4 as received from the channel interface. It can also be used to verify the contents of the data buffers following an Output X'64' instruction.

Byte 0, bits 0-7: Data Buffer Byte 3—T^iese bits represent the third byte of data received from the channel during a channel Write command.

Byte 1, bits 0-7: Data Buffer Byte 4—These bits represent the fourth byte of data received from the channel during a channel Write command.

Input X'66' (NSC Status Byte)

This instruction loads a general register with the contents of the NSC status register. These bits reflect the status bits loaded into ths status register by the last Output X'66' instruction. This instruction should be used only as a diagnostic aid.

Byte 0, bit 0: Attention

Byte 0, bit 1: Status Modifier

Byte 0, bits 2-3: These bits are 0.

Byte 0, bit 4: Channel End.

Byte 0, bit 5: Device End.

Byte 0, bit 6: Unit Check.

Byte 0, bit 7: Unit Exception.

Byte 1, bits 0-7: These bits are 0.

Input X'67' (CA Controls)

This instruction loads a general register with various kinds of CA control information. Byte 1, bits 0-3 (check indications) are reset by an Output X'67', byte 1, bit 2.

Byte 0, bits 0-7: These bits are set to the NSC address for the enabled A or B interface.

Byte 1, bit 0: Channel Bus In Check—This bit indicates that incorrect parity was detected on the channel bus in. When this is detected, the hardware generates good parity and causes a level 1 interrupt.

Byte 1, bit 1: In/Out Instruction Accept Check—This bit indicates that the control program executed an Input or Output X'60', X'61', X'62', X'63, X'64', X'65', or X'66' instruction when the CA was in the process of handling a data/status transfer. Detection of this condition also causes a level 1 interrupt request.

Byte 1, bit 2: CCU Outbus Check—This bit indicates that the CA hardware detected incorrect parity on the CCU outbus. When this is detected, the hardware causes a level 1 interrupt request and prohibits reselection on the channel interface until this bit is reset.

Byte 1, bit 3: Local Store Check—This bit indicates that the CA hardware detected incorrect parity on data bytes gated out of local store. The control program should place good parity in local store by executing an Output X'63', X'64', or X'65' instruction.

.B-20

Byte 1, bit 4: Channel Interface Enabled—This bit indicates that either interface A or interface B is enabled. When no interface is enabled, the bit is 0.

Byte 1, bit 5: NSC Address Active—This bit indicates that the native subchannel has been selected and is active. The bit is reset when the host channel accepts the final status from the CA.

Byte 1, bits 6-7: These bits are unused.

Input X'6C' (EB Mode Control Register) (Type 4 CA)

This instruction loads the contents of the channel adapter's control character recognition latches, the EB mode control bit, and the EB mode byte count into the register specified by the R field.

Byte 0, bit 0: EB Mode-This bit indicates whether the CA is in extended buffer mode (bit=l) or in non-extended buffer mode (bit=0).

Byte 0, bits 1-4: These bits are 0. (Bit 1 is reserved for future use.)

Byte 0, bit 5: DLE Remember Latch—A 1 indicates that the DLE remember latch is set, a 0 indicates that the latch is reset.

Byte 0, bit 6: ASCII Monitor Latch-A 2 indicates that the ASCII monitor latch is set, a 0 indicates that the latch is reset.

Byte 0, bit 7: EBCDIC Monitor Latch-A 1 indicates that the EBCDIC monitor latch is set, a 0 indicates that the latch is reset.

Byte 1, bits 0-7: Register Byte Count (EB Mode)-These bits contain the byte count of data transferred by the Type 4 CA in extended buffer mode.

Input X'6D' (EB Mode Data Buffer) (Type 4 CA)

This instruction loads the contents of the two-byte EB mode buffer into the register specified by the R field. The first (even) data byte is placed in byte 0 of the register; the second (odd) byte is placed in byte 1 of the register.

Type 1 and Type 4 Channel Adapters

Type1 and Type 4 Output Instructions

Output X'60' (Reset Initial Selection)

This instruction resets the CA initial selection latches and the level 3 interrupt request resulting from an initial selection. Since this instruction performs a function, the bit settings of the register specified by the R operand are ignored.

Output X'62' (Data/Status Control)

This instruction unconditionally resets the CA program-requested interrupt and the 'suppress out monitor' latch. It also is used to set the following bits in the data/status control register.

Programming Note

If the suppress out monitor interrupt (Input X'62', byte 0, bit 6) is active, this instruction should be executed when initiating the next transfer sequence.

If the suppress out monitor and the program requested interrupt requests are both set, resetting one of them also resets the other. Therefore, if one of the interrupts is still desired, it must be requested again by an Output X'67' with the appropriate bit set.

Byte 0, bit 0: Outbound Data Transfer Sequence—This bit is set by the control program to initiate an outbound data transfer sequence. For example, a channel Read or Sense command should initiate an outboard data transfer sequence that sends data to the host processor.

Byte 0, bit 1: Inbound Data Transfer Sequence—This bit is set by the control program to initiate an inbound data transfer sequence. For example, a channel Write command should initiate an inbound data transfer sequence that receives data from the host processor.

Byte 0, bit 2: ESC Final Status Transfer Sequence—This bit is set by the control program and signals the hardware to initiate an ESC status transfer using the address and status information previously loaded into the address and status register by an Output X'63'.

Byte 0, bit 3: NSC Channel End Status Transfer—This bit is set by the control program and signals the CA hardware to present Channel End only for the associated NSC address.

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-21

Type 1 and Type 4 Channel Adapters

Byte 0, bit 4: NSC Final Status Transfer—This bit is set by the control program and signals the CA hardware to initiate an NSC final status transfer sequence.

Byte 0, bit 5: Reset Initial Selection-This bit is set by the control program to reset byte 0, bits 0-6 of the initial selection control register (Input X'60'). This reset allows the CA to accept an initial selection. The bit position is reset to 0 at the completion of the Output X'62' instruction.

Note: This bit resets the CA initial L3 interrupt request unless the interrupt was caused by a system reset.

Byte 0, bit 6: Reset Data/Status Interrupt-This bit is set by the control program to reset the CA data/status L3 interrupt request and the following data/status service register bits.

• Interface disconnect

• Selective reset

• Bus out check

• Monitor for circle B and 2848 ETX

• Channel stop

• Stacked status

When this bit is on along with one of the transfer sequence bits (byte 0, bits 0-4), the CA hardware raises the 'request in' tag line on the channel interface, except when ESC Test I/O Status is available (Output X'62', byte 1, bit 4).

This bit position is reset to 0 at the completion of the Output X'62' instruction.

Byte 0, bit 7: Type 4 CA-Reset Extended Buffer Mode. |Type 1 CA—This bit is unused.

Byte 1, bit 0: Set Monitor for an IBM Type I or II Circle B—When this bit is set to 1, the channel adapter hardware circuits monitor the inbound data characters for a circle B (X'3D' or X'BD'). When the circle B is detected, the CA causes a channel stop condition and sets byte 0, bit 5 of Input X'62' to 1 (Channel Stop/Interface Disconnect). This monitor bit is reset by Output X'62', byte 0, bit 6 (Reset data/status interrupt).

Byte 1, bit 1: This bit is unused.

Byte 1, bit 2: Set Monitor for IBM 2848 or 2845 ETX—When this bit is set to 1, the channel adapter hardware circuits monitor the inbound data characters

for a 2848 or 2845 ETX (X'03'). When the ETX is detected, the CA causes a channel stop condition and sets byte 0, bit 5 of Input X'62' to 1 (Channel Stop/Interface Disconnect). This monitor bit is reset by Output X'62', byte 0, bit 6 (Reset data/status interrupt).

Byte 1, bit 3: Set Suppressible Status—This bit should be set when the control program is presenting suppressible status to the host channel when ESC mode is enabled. Status is suppressible if 'stacked status' is received for a particular line or when the line has been issued an interface disconnect. Refer to IBM System/360 and System/370 I/O Interface Channel to Control Unit Original Equipment Manufacturers Information (GA22-6974) for further information on suppressible status.

Byte 1, bit 4: Set ESC Test I/O Status Available— When this bit is set to 1, the controller hardware responds to a Test I/O command given to a 2701/2702/2703 address. The control program must have previously loaded the correct address and status byte into the CA address and emulator status byte register (Output X'63') before setting this bit. This bit is reset by the emulator hardware when it presents this status to the channel.

When this bit is set to 1, byte 0, bit 2 of this instruction must also be set to 1.

Byte 1, bit 5: Type 4 CA-Set Priority Outbound Data Transfer Sequence. Type 1 CA—This bit is unused.

Byte 1, bits 6-7: Request Byte Count 1 and 2—These bits are set according to the number of bytes to be transferred during inbound or outbound data transfer. (Type 4 CA: These bits apply to data transfer in non-EB mode.)

| Bit 6 | 7 | Number of Bytes Transferred |
| --- | --- | --- |
| 0 | 0 | 4 |
| 0 | 1 | 1 |
| 1 | 0 | 2 |
| 1 | 1 | 3 |

B-22

Type 1 and Type 4 Channel Adapten

Output X'63' (CA Address and Emulator Status Bytes)

The control program can set this output to indicate the line address (NSC or ESC) and the status of the line to be serviced next by the CA in emulation mode.

Byte 0, bits 0-7: Address Byte—These bits are set by the control program to indicate the I/O device address to be serviced.

Byte 1, bit 0: Set ESC Attention status.

Byte 1, bit 1: Set ESC Status Modifier.

Byte 1, bit 2: Set ESC Control Unit End status.

Byte 1, bit 3: Set ESC Busy status.

Byte 1, bit 4: Set ESC Channel End status.

Byte 1, bit 5: Set ESC Device End status.

Byte 1, bit 6: Set ESC Unit Check status.

Byte 1, bit 7: Set ESC Unit Exception status.

Output X'64' (Data Buffer Bytes 1 and 2)

This instruction is used only for outbound data transfer. (Type 4 CA: This instruction applies only to data transfer in non-EB mode.)

Byte 0, bits 0-7: Data Buffer Byte 1—These bits represent the first data byte to be sent across the channel during a channel Read command. This buffer must be loaded by the control program.

Byte 1, bits 0-7: Data Buffer Byte 2—These bits represent the second data byte to be sent across the channel during a channel Read command. This buffer must be loaded by the control program.

Output X'65' (Data Buffer Bytes 3 and 4)

This output instruction is used only for outbound data transfer.

Byte 0, bits 0-7: Data Buffer Byte 3—These bits represent the third data byte to be sent across the channel during a channel Read command. This buffer must be loaded by the control program.

Byte 1, bits 0-7: Data Buffer Byte 4—These bits represent the fourth data byte to be sent across the channel during a channel Read command. This buffer must be loaded by the control program.

Output X'66' (CA NSC Status Byte)

This instruction loads the native mode status byte with the bits that are set in the register specified by R. The control program sets these bits to indicate the status to be presented across the channel interface when the CA is in the NSC status transfer sequence. Hardware resets these bits when the status byte is accepted by the channel.

| Byte 0, bits 0-3, 5-7: Type 4 CA-These bits are unused.

| Byte 0, bit 4: Type 4 CA: Set NSC Long Busy.

| Byte 0, bits 0-7: Type 1 CA-These bits are unused.

Byte 0, bits 0- 7: These bits are unused.

Byte 1, bit 0: Set Attention status.

Byte 1, bit 1: Set Status Modifier.

Byte 1, bits 2-3: These bits are unused.

Byte 1, bit 4: Set Channel End.

Byte 1, bit 5: Set Device End.

Byte 1, bit 6: Set Unit Check.

Byte 1, bit 7: Set Unit Exception.

Output X'67' (CA Controls)

This instruction causes various control latches to be set or reset in the channel adapter according to the states of the bits in the register specified by R.

| Byte 0, bits 0-3: Type 1 CA-These bits are unused.

Byte 0, bit 4: Diagnostic Reset—This bit resets the channel adapter when the 'channel interface disable' latch is set. This bit should be set only by a diagnostic program and not by inline code.

Byte 0, bits 5-7: These bits are unused.

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-23

Type 1 and Type 4 Channel Adapters

Byte 1, bit 0: Suppress Out Monitor Interrupt—A 1 in this position causes the CA to monitor the 'suppress out' tag line for the inactive state. When this is detected, the CA data/ status L3 interrupt request is set. After servicing the requesi, the control program should execute an Output X'62' instruction to reset the request.

Programming Note

Following a stacked status condition, the control program can use this bit to cause the channel adapter to signal when the suppress status indication is removed.

Byte 1, bit 1: Set Program-Requested Interrupt—A 1 in this position indicates that a CA data/status L3 interrupt is requested. If a data/status transfer or initial selection is in progress, the interrupt request is held until the sequence is complete.

Byte 1, bit 2: Reset Level 1 Checks—A 1 in this position resets the CA level 1 check latches and interrupt requests.

Byte 1, bit 3: Reset System Reset and NSC Address Active—A 1 in this position resets the level 3 interrupt request caused by a system reset. It also resets the NSC address active indication.

Byte 1, bit 4: Allow Channel Interface Enable—A 1 in this position causes the 'channel interface enable' latch to be set. This enables the CA to communicate with the host processor. The bit must be 0 if byte 1, bit 7 of this output is 1.

Programming Note

The channel interface cannot be enabled following a power-off to power-on-reset until an Output X'67 is executed with this bit on. The IPL Bootstrap program performs this operation in IPL Phase 3.

Byte 1, bit 5: ESC Operational—A 1 in this position sets the emulator subchannel (ESC) addresses to an operational mode. The channel interface must be enabled before the emulator subchannels can become operational.

Byte 1, bit 6: ESC Command Free—A 1 in this position resets the 'ESC command active' latch. The 'ESC command active' latch is set by initial selection.

Programming Note

The CA cannot be disabled until it is free of commands; therefore, the control program must ensure that the 'ESC command active' latch is reset before a disable attempt is made.

Byte 1, bit 7: Allow Channel interface Disable—A 1 in this position sets the 'channel interface disable' latch. This latch overrides the I/O Channel 1 Enable/Disable switch on the control panel and allows the channel to become disabled if (1) the channel adapter is free of commands, (2) commands are not chained, and (3) the adapter is not in initial selection.

The 'channel interface disable' latch is reset by Output X'67', byte l,bit4.

Output X'6C' (EB Mode Control Register) (Type 4 CA)

This instruction sets the contents of the channel adapter's control character recognition latches, the EB mode control bit, and the EB mode byte count from the corresponding bits of the register specified by the R field.

Byte 0, bit 0: Set/Reset EB Mode-A 1 specifies that the CA is to operate in extended buffer mode; a 0 specifies that the CA is to operate in non-extended buffer mode.

Byte 0, bits 1-4: These bits are 0. (Bit 1 is reserved for future use.)

Byte 0, bit 5: DLE Remember Latch-A 1 sets this latch; a 0 resets it.

Byte 0, bit 6: ASCII Monitor Latch-A 1 sets this latch; a 0 resets it.

Byte 0, bit 7: EBCDIC Monitor Latch-A 1 sets this latch; a 0 resets it.

Byte 1, bits 0-7: Register Byte Count (EB Mode)-These bits contain the byte count of data to be transferred by the Type 4 CA in extended buffer mode.

B-24

Type 2 and Type 3 Channel Adapters

Output X'6D' (EB Mode Data Buffer) (Type 4 CA)

This instruction loads two data bytes from the register specified by the R field into the EB mode data buffer of the Type 4 CA. The first data byte (0) of the register is placed in the first (even) byte of the buffer; the second data byte (1) of the register is placed in the second (odd) byte of the buffer.

Type 2 and Type 3 CA Input Instructions

Input X'50' (Inbound Data Control Word Address Register-INCWAR)

This instruction loads a general register with the contents of the INCWAR. The INCWAR contains the storage address of the first control word to be used when the channel adapter receives a Write, Write Break, or Write IPL command. The CA recognizes this instruction only after setting a CA interrupt request.

Byte 0, bits 0-7: INCWAR bits 0-7.

Byte 1, bits 0-7: INCWAR bits 8-15.

Input X'51' (Outbound Data Control Word Address Register-OUTCWAR)

This instruction loads a general register with the contents of the OUTCWAR. The OUTCWAR contains the storage address of the next control word to be used when the channel adapter receives a Read command. The instruction is recognized by the CA only following the setting of a CA interrupt request.

Byte 0, bits 0-7: OUTCWAR bits 0-7.

Byte 1, bits 0-7: OUTCWAR bits 8-15.

Input X'52' (Control Word Byte Count Register-CWCNT)

This instruction loads a general register with the number of bytes remaining to be transferred under the last control word that was fetched. See Control Word Byte Count Register in Chapter 10. The CA recognizes this instruction only after setting a CA interrupt request.

Byte 0, bits 0-5: These bits are 0.

Byte 0, bits 6-7: CWCNT bits 0-1.

Byte 1, bits 0-7: CWCNT bits 2-9.

Input X'53' (Channel Adapter Sense Register-CASNSR)

This instruction loads a general register with the contents of the channel adapter sense register.

The sense register is accessible for input only when the CA is active, selected, and has a level 1 or level 3 interrupt request set, or the CA is in the diagnostic wrap state. If these conditions are not met, an Input X'53' causes a CCU Inbus parity check. If an interrupt request is set and the CA is not active, an Input X'53' causes the general register to be loaded with zeros.

Byte 0, bit 0: Command Reject—This bit is set when the host processor channel command presented to the CA during initial selection is not a valid command for the controller.

Byte 0, bit 1: Intervention Required—This bit indicates programming errors detected by the CCU or CA hardware, or the control program. It is set under hardware control for any one of the following conditions:

1. The CCU 'hard stop' latch is set while the CA is transferring data under a Read, Write, or Write Break command.

2. An addressing exception or a protection check was caused by the address used by the CA for a cycle-steal operation.

3. A TIC command or command chaining to a control word address above 64K has been detected during a CW fetch cycle steal.

4. An OUT or OUT STOP control word was decoded when executing a channel Write, Write Break, or Write IPL command during a CW fetch cycle steal.

5. An IN control word was decoded when executing a channel Read command during a CW fetch cycle steal.

6. An IN, OUT, or OUT STOP control word was decoded with a byte count of zero during a CW fetch cycle steal.

Programming Note

Condition 3 above also sets byte 0, bit 0 (invalid CWAR address) of the channel adapter check register. Conditions 4, 5, and 6 above also set byte 0, bit 1 (invalid control word format) of the channel adapter check register.

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-25

Type 2 and Type 3 Channel Adapters

Byte 0, bit 2: Bus Out Check—This bit is set when a parity error is detected on the channel bus out during the initial selection command transfer or during host processor-to-controller data transfer. The control program cannot set this bit.

Byte 0, bit 3: Equipment Check—This bit is set any time the CA detects an internal hardware error or a parity error on the Inbus or Outbus between the Central Control Unit and the CA.

Byte 0, bit 4: Data Check—This bit can be set on by the control program only during a level 3 interrupt.

Byte 0, bit 5: This bit is 0.

Byte 0, bit 6: Not Initialized—This bit is on when the controller has not been initialized. The channel adapter hardware sets the not-initialized condition when the CA goes offline. This condition is reset by the ROS program via an Output X'77' with byte 0, bit Oon.

Byte 0, bit 7: Abort—This bit indicates that the channel adapter has halted its channel operation abnormally.

Byte 1, bits 0-7: These bits are 0.

Input X'54' (Channel Adapter Status Register-CASTR)

This instruction loads a general register with the contents of the channel adapter status register. The bits of the status register can be set by the control program and/or the CA hardware. See Output X'54' in this appendix for the method of setting each bit.

The status register is accessible for input only when the CA is active, selected, and has a level 1 or level 3 interrupt request set, or the CA is in the diagnostic wrap state. If these conditions are not met, an Input X'54' causes a CCU Inbus parity check. If an interrupt request is set and the CA is not active, an Input X'54' causes the general register to be loaded with zeros.

Byte 0, bit 0: Attention—This bit indicates that Attention has been set.

Byte 0, bit 1: Status Modifier—This bit indicates that the Status Modifier has been set.

Byte 0, bit 2: This bit is 0.

Byte 0, bit 3: Busy—This bit indicates that Busy status has been set.

Byte 0, bit 4: Channel End—This bit indicates that Channel End status has been set.

Byte 0, bit 5: Device End—This bit indicates that Device End status has been set.

Byte 0, bit 6: Unit Check- -This bit indicates that a Unit Check has occurred.

Byte 0, bit 7: Unit Exception—This bit indicates that Unit Exception status has been set.

Byte 1, bits 0-7: These bits are 0.

Input X'55' (Channel Adapter Control Register-CACR)

This instruction loads a general register with the status of various control latches in the CA. This instruction is recognized by the CA only following the setting of a CA select and a CA interrupt request. The CA need not be in the CA active state.

Byte 0, bit 0: Diagnostic Wrap Mode—This bit indicates that the controller is offline and in the diagnostic wrap state.

Byte 0, bit 1: Zero Count Override—This bit indicates the condition of the zero count override flag in the control word just executed. It is reset either when a control word is fetched with zero count override off, or when Channel End is generated for the current command.

Byte 0, bit 2: INCWAR Valid—This bit indicates that the control word address register for inbound data transfer (channel Write command) points to the storage location containing the control word to be used for controlling this type of data transfer.

The control program sets this bit during a CA-requested level 3 interrupt via an Output X'55' instruction. However, once data transfer across the channel begins, this bit is controlled as follows.

After an IN-CW fetch operation, the bit reflects the status of the chain flag of the IN-CW fetched for the CA. During the CW fetch operation, the address in

B-26

Type 2 and Type 3 Channel Adapters

the INCWAR register is incremented by 4 (fullword address). The chain flag in the fetched CW indicates whether or not the updated INCWAR points to a valid CW. If the chain bit is off in the fetched CW, the 'INCWAR valid' latch is reset.

Byte 0, bit 3: OUTCWAR Valid—This bit indicates that the control word address register for outbound data transfer (that is, a channel Read command) points to the storage location containing the control word to be used for controlling this type of data transfer. The control program sets this bit during a CA-requested level 3 interrupt by an Output X'55' instruction. However, once data transfer across the channel has started, this bit is controlled as follows.

After an OUT-CW fetch operation, this bit reflects the status of the chain flag in the OUT or OUT STOP control word fetched for the CA. During the CW fetch operation, the address in the OUTCWAR is incremented by 4 (fullword address). The chain flag in the fetched CW indicates whether or not the updated OUTCWAR points to a valid CW. If the chain bit is off in the fetched CW, the 'OUTCWAR valid' latch is reset.

Byte 0, bit 4: Program Requested Level 3 Interrupt—This bit indicates that the CA L3 interrupt was initiated because the control program set the CA mode register byte 1, bit 0 (set CA L3 request).

Byte 0, bit 5: Program Requested Abort/Level 3 Interrupt—This bit indicates that the current level 3 interrupt was caused by executing an Output X'57' with byte 1, bit 1 on. This bit is reset when Output X'57' is executed with byte 1, bit 3 on (reset L3 request).

Byte 0, bit 6: Program Requested Attention—This bit indicates that the program has requested Attention by executing an Output X'55' instruction with byte 0, bit 6 on. It is reset when the host channel accepts the status byte containing Attention.

Byte 0, bit 7: Channel Adapter Active—This bit indicates that the CA is currently executing a channel command. It is set by completion of the initial selection for the command and is reset when the host channel accepts Device End status for that command.

Byte 1, bit 0: Command Chaining—This bit is set by the Type 2 CA hardware when the 'suppress out' tag line is up at the time the channel accepts ending status from the CA. It is reset at the end of the first level 3 interrupt to occur after the latch has been set each time a valid command is decoded during initial selection, or when suppress out fails and the CA is not active.

Byte 1, bit 1: Write Break Command Remember—This bit is set by the channel adapter when a Write Break command (X'09') is received. It is reset when the host channel accepts Device End status for that command.

Byte 1, bit 2: Channel Stop/Interface Disconnect—This bit is set by the channel adapter when a Channel Stop or an Interface Disconnect is detected on the channel interface. This bit is reset by an Output X'57', byte 1, bit 6.

Byte 1, bit 3: Selective/System Reset—This bit is set by the CA hardware when a system reset or a selective reset is detected on the interface. It is reset by Output X'57', byte 1, bit 5.

Byte 1, bit 4: This bit is 0.

Byte 1, bit 5: Channel Read Command Remember—This bit is set when a Read command is accepted by the CA during initial selection. It is reset when the CPU accepts the Device End status for this command.

Byte 1, bit 6: CA-2 Selected—This bit indicates that the second channel adapter (CA-2) has been selected for operation by an Output X'57'. This bit is always 0 if only one channel adapter is installed.

Byte 1, bit 7: CA-1 Selected—This bit indicates that the first channel adapter (CA-1) has been selected for operation by an Output X'57'.

Input X'56' (Channel Adapter Check Register-CACHKR)

This instruction loads a general register with the contents of the CA check register. All the bits of the check register set a level 1 interrupt request. By executing this input, the level 1 interrupt check routine can determine the exact cause of the CA level 1 check. Except for the bus out checks, all the latches in this register are automatically reset

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-27

Type 2 and Type 3 Channel Adapters

when an Output X'57' is executed to reset the adapter's LI interrupt request. This input instruction is recognized only after a CA interrupt request is set.

Byte 0, bit 0: Invalid CWAR Address—This bit is set when the CWAR associated with the current channel operation points to a storage address above 64K bytes.

Byte 0, bit 1: Invalid Control Word Format—This bit is set when (1) an OUT STOP control word is fetched when executing a channel Write command, (2) an IN control word is fetched when executing a channel Read command, or (3) an IN, OUT, or OUT STOP control word containing a data count of zero is fetched.

Byte 0, bit 2: Cycle Steal Address Check—This bit is set when the CCU signals the channel adapter that an address error has occurred during a cycle steal operation. It indicates that the cycle steal address (1) is beyond the storage capacity of the machine, (2) is of incorrect parity, or (3) points to a protected area of storage.

Byte 0, bit 3: CWAR/Data Buffer Check—This bit indicates that either the INCWAR, OUTCWAR, data 1, or data 2 register contained incorrect parity when access was attempted for either an input instruction or data transfer to the channel during a Read command.

Byte 0, bit 4: CCU Outbus Check—This bit is set (1) when data from an output instruction to the CA has incorrect parity on the CCU Outbus, or (2) when, during a cycle steal, the data from storage contained incorrect parity.

Byte 0, bit 5: CCU Inbus Check—This bit indicates that incorrect parity was present on the Inbus during a CA cycle steal operation or input instruction.

Byte 0, bit 6: Channel Bus Out Check—This bit indicates that a parity check was detected on the channel bus out lines during initial selection or data transfer.

Byte 0, bit 7: This bit is 0.

Byte 1, bits 0-3: These bits are 0.

Byte 1, bit 4: Channel Bus In Check (Interface

A)—This bit indicates that the sense, status, data, or address byte presented to interface A did not have correct parity.

Byte 1, bit 5: Channel Bus In Check (Interface

B)—This bit indicates that the sense, status, data, or address byte presented to interface B did not have the correct parity.

Byte 1, bits 6-7: These bits are 0.

Input X'58' (Channel Bus Out Diagnostic Register-CBODR)

This instruction loads a general register with the current state of the host processor 'bus out' lines. However, byte 0 and byte 1, bit 0 are accessible only in the diagnostic wrap mode, and byte 1, bits 1-7 are accessible when the adapter is selected and in a level 1 or level 3 interrupt state.

Byte 0, bits 0-7: Channel Bus Out bits 0-7.

Byte 1, bit 0: Channel Bus Out Parity Bit—This bit may be used in diagnostic mode to check the CA error-detection circuits.

Byte 1, bit 1: This bit is 0.

Byte 1, bit 2: Transfer Byte 1—This bit indicates that the CA is currently transferring an odd numbered byte across the channel interface.

Byte 1, bit 3: Transfer Byte 2—This bit indicates that the Type 2 CA is currently transferring an even numbered byte across the channel interface.

Byte 1, bit 4: Interface A Enabled—This bit is 1 when channel interface A of the Channel Adapter is currently enabled.

Byte 1, bit 5: Interface B Enabled—This bit is 1 when channel interface B of the channel adapter is currently enabled.

Byte 1, bit 6: CSAR Byte X, Bit 6—This bit is 1 when byte X, bit 6 of the cycle-steal address register (with Extended Addressing) is 1.

Byte 1, bit 7: CSAR Byte X, Bit 7—This bit is 1 when byte X, bit 7 of the cycle-steal address register (with Extended Addressing) is 1.

B-28

Type 2 and Type 3 Channel Adapters

Input X'59' (Cycle-Steal Address Register-CSAR)

This instruction loads a general register with the current storage data address from the cycle-steal address register while data transfer is in progress.

Byte 0, bits 0-7: CSAR byte 0, bits 0-7.

Byte 1, bits 0-7: CSAR byte 1, bits 0-7.

Input X'5A' (CA Data Buffer-CADB)

This instruction loads a general register with incoming data from the channel adapter data buffer.

Byte 0, bits 0-7: Data buffer byte 0, bits 0-7. Any nonstandard channel command is available in this byte for level 3 program interrogation.

Byte 1, bits 0-7: Data buffer byte 1, bits 0-7.

Input X'5B' (Channel Adapter Tag Diagnostic Register)

This instruction loads a general register with a combination of bits to indicate the state of the channel tag lines for diagnostic purposes. This input can be used only when the adapter is in diagnostic mode.

Byte 0, bit 0: Select Out/Hold Out—This bit indicates the state of the diagnostic 'select out/hold out' tag line.

Byte 0, bit 1: Address Out—This bit indicates the state of the diagnostic 'address out' tag line.

Byte 0, bit 2: Command Out—This bit indicates the state of the diagnostic 'command out' tag line.

Byte 0, bit 3: Service Out—This bit indicates the state of the diagnostic 'service out' tag line.

Byte 0, bit 4; Operational Out—This bit indicates the state of the diagnostic 'operational out' tag line.

Byte 0, bit 5: Suppress Out—This bit indicates the state of the diagnostic 'suppress out' tag line.

Byte 0, bits 6-7: These bits are 0.

Byte 1, bit 0: Select Out—This bit indicates the state of the diagnostic 'select out' tag line.

Byte 1, bit 1: Request In—This bit indicates the state of the diagnostic 'request in' tag line.

Byte 1, bit 2: Operational In—This bit indicates the state of the diagnostic 'operational in' tag line.

Byte 1, bit 3: Address In—This bit indicates the state of the diagnostic 'address in' tag line.

Byte 1, bit 4: Status In—This bit indicates the state of the diagnostic 'status in' tag line.

Byte 1, bit 5: Service In—This bit indicates the state of the diagnostic 'service in' tag line.

Byte 1, bit 6: This bit is 0.

Byte 1, bit 7: Generate Busy—This bit indicates that the channel adapter is busy.

Input X'5C' (CA Command Register—CMDR)

This instruction loads a general register with the current channel command being executed. Byte 1 indicates the current or last control word type executed.

Byte 0, bit 0: Test I/O—This bit is 1 when the command received from the host processor was a Test I/O (X'00').

Byte 0, bit 1: Write—This bit is 1 when the command received from the host processor was a channel Write (X'01') command.

Byte 0, bit 2: Read—This bit is 1 when the command received from the host processor was a channel Read (X'02') command.

Byte 0, bit 3: No-Op—This bit is 1 when the command received from the host processor was a No-Op (X'03').

Byte 0, bit 4: Sense—This bit is 1 when the command received from the host processor was a Sense (X'04') command.

Byte 0, bit 5: This bit is 0.

Byte 0, bit 6: Write Break—This bit is 1 when the command received from the host processor was a Write Break (X'09') command.

Byte 0, bit 7: This bit is 0.

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-29

Type 2 and Type 3 Channel Adapters

Byte 1, bit 0: OUT Control Word—This bit is 1 when the current or last control word in use was an OUT control word.

Byte 1, bit 1: OUT STOP Control Word—This bit is 1 when the current or last control word in use was an OUT STOP control word.

Byte 1, bit 2: IN Control Word—This bit is 1 when the current or last control word in use was an IN control word.

Byte 1, bit 3: TIC Control Word—This bit is 1 when the current or last control word in use was a TIC control word.

Byte 1, bit 4: Nonstandard Command—A 1 in this position indicates that a nonstandard command was issued. The command byte is available for examination by the 3705 control program, in byte 0 of register X'5A'.

Byte 1, bit 5: Interface A—A 1 in this position indicates that the Type 3 CA currently is switched to interface A.

Byte 1, bit 6: Interface B—A 1 in this position indicates that the Type 3 CA currently is switched to interface B.

Byte 1, bit 7: Write IPL—This bit is 1 when the command received from the host processor was a Write IPL (X'05') command.

Type 2 and Type 3 CA Output Instructions

Output X'50' (Inbound Data Control Word Address Register-INCWAR)

This instruction loads the INCWAR with the storage address of the control word (CW) to be fetched by the CA cycle-steal hardware when a channel Write, Write Break, or Write IPL command is decoded. The CA recognizes this instruction only after setting a CA interrupt request.

Programming Note

All CWs must begin on a halfword boundary and reside in the lower 64K bytes of storage. CW chaining or a TIC (transfer in channel) to an address above 64K causes a CW error condition resulting in a CA level 1 interrupt.

Byte 0, bits 0-7: INCWAR bits 0-7.

Byte 1, bits 0-7: INCWAR bits 8-15.

Output X'51' (Outbound Data Control Word Address Register-OUTCWAR)

This instruction loads the OUTCWAR with the storage address of the control word (CW) to be fetched by the CA cycle-steal hardware when a channel Read command is decoded. The instruction is recognized by the CA only following the setting of a CA interrupt request.

Programming Note

All CWs must begin on a halfword boundary and reside in the lower 64K bytes of storage. CW chaining or TIC (transfer in channel) to an address above 64K causes a CW error condition resulting in a CA level 1 interrupt.

Byte 0, bits 0-7: OUTCWAR bits 0-7.

Byte 1, bits 0-7: OUTCWAR bits 8-15.

Output X'53' (Channel Adapter Sense Register—CASNSR)

This output instruction is used to set the abort indication in the CA sense register. The CA recognizes this instruction only after setting a CA interrupt request while the CA is in the CA active state. If Output X'53' is issued while in a CA interrupt state and the CA is not active, the output is ignored, and no indication of this is returned to the control program. The control program should always determine the active/inactive state of the CA before issuing this output instruction.

B-30

Type 2 and Type 3 Channel Adapters

Programming Note

The setting of any CASNSR bit causes the Unit Check bit to be set in the CA status register and also causes the termination of any data transfer that may have been in progress. CASNSR is reset during initial selection whenever the CA accepts a command other than Sense, Test I/O, or No-Op.

Byte 0, bit 0: Command Reject—This output allows a level 3 interrupt program to set the Command Reject sense bit. Execution of this output also causes Unit Check to be set in the status register.

Byte 0, bit 1: Intervention Required—This bit should never be set by the control program during normal operation.

Byte 0, bits 2-3: These bits are unused.

Byte 0, bit 4: Set Data Check—This bit is set to 1 by the control program during a level 3 interrupt. The CA must be in the active state or in diagnostic mode.

Byte 0, bits 5-6: These bits are unused.

Byte 0, bit 7: Abort—This bit can be set by the control program during a CA interrupt if the CA active state exists. Abort indicates to the host processor that the control program has halted its channel operation abnormally.

Byte 1, bits 0- 7: These bits are unused.

Output X'54' (Channel Adapter Status Register— CASTR)

This instruction sets the channel adapter status register bits. The instruction is recognized by the CA only following the setting of a CA level 1 or level 3 interrupt request while the CA is in the CA active state or in the diagnostic wrap state.

Byte 0, bit 0: Attention—When the CA is both active and has either its level 1 or level 3 interrupt request latch set, a 1 bit in this position is used to include Attention along with other ending status such as Device End.

Byte 0, bit 1: Status Modifier—This bit may have special applications in controlling channel data transfers. Status Modifier should convey to the host processor a unique indication for the particular command being executed.

Byte 0, bit 2: This bit is unused.

Byte 0, bit 3: Busy—This bit is presented as initial status to all host processor channel commands if (1) initial selection is attempted before Device End status has been signaled to the host processor for the command currently being executed, or (2) the channel adapter has its level 1 or level 3 interrupt request latch set.

If status is available at this time, it is presented along with Busy status. If status has been stacked, that status is also presented along with Busy.

Byte 0, bit 4: Channel End—This bit is set to indicate that the controller has completed the data transfer portion of the channel command in progress. Channel End is set by the CA hardware for each host processor channel command. It may also be set when the CA is in an active state with a level 1 or level 3 request pending and Channel End has not already been presented to the channel for the command being executed.

Byte 0, bit 5: Device End—This bit is set to indicate that the controller has finished with the current host processor channel command and is ready to accept another command from the channel. CE and DE are not always presented together. The conditions under which they are presented separately are discussed under Status Servicing in Chapter 10. When CE and DE are presented separately, the CA requests a level 3 interrupt after the channel accepts CE. When the interrupt request is reset (Output X'57'—channel adapter mode register), the CA sends DE and any other status (Attention, Status Modifier, Unit Exception) that may have been set during execution of the interrupt handling routine.

Byte 0, bit 6: This bit is unused.

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-31

Type 2 and Type 3 Channel Adapters

Byte 0, bit 7: Unit Exception—This bit is set during initial status to notify the channel that the control routine has not set up a CA control word to handle this particular host processor channel command. UE is presented with DE to more efficiently notify the channel that a CW is invalid. Unit Exception can be used in this manner to break host processor channel command chaining without going through another selection sequence when the controller no longer needs service.

When two channel adapters are installed in the controller, and an IPL command is in progress to one of the CAs, Unit Exception is presented as initial status to all commands, except Write IPL, issued to the other CA.

Byte 1, bits 0- 7: These bits are unused.

Output X'55' (Channel Adapter Control Register-CACR)

This instruction sets the CA control register bits. The instruction is recognized by the CA only following the setting of a CA select and a CA interrupt request. The CA need not be in the CA active state.

Byte 0, bits 0-1: These bits are unused.

Byte 0, bit 2: INCWAR Valid—This bit is set by the control program when the control word address register for inbound data transfer (channel Write command) points to the storage location containing the control word to be used for controlling this type of data transfer.

To initialize the CA, the control program sets this bit during a CA-requested level 3 interrupt.

Byte 0, bit 3: OUTCWAR Valid—The control program sets this bit when the control word address register for outbound data transfer (that is, a channel Read command) points to the storage location containing the control word to be used for controlling this type of data transfer. To initialize the CA, the control program sets this bit during a CA-requested level 3 interrupt.

Byte 0, bis 4-5: These bits are unused.

Byte 0, bit 6: Program Requested m-When/Attention—When this bit has been set, the Attention status is presented to the channel as soon as the CA's level 3 interrupt request latch has been reset provided the 3705 is not executing a selection sequence or has not come to a hard stop. The bit is reset when the channel accepts the status byte containing the attention bit. The Type 3 CA presents the Attention status to both channels.

Byte 0, bit 7: This bit is unused.

Byte 1, bits 0-7: These bits are unused.

Output X'56' (Reset Control Register-CACR)

This instruction is executed to reset the TNCWAR valid' and the 'OUTCWAR valid' latches.

Byte 0, bits 0-1: These bits are unused.

Byte 0, bit 2: Reset INCWAR Valid—A 1 in this position causes the 'INCWAR valid' latch to be reset.

Byte 0, bit 3: Reset OUTCWAR Valid—A 1 in this position causes the 'OUTCWAR valid' latch to be reset.

Byte 0, bits 4-7: These bits are unused.

Byte 1, bits 0-5: These bits are unused.

Byte 1, bit 6: This bit being on disables the CA from accepting nonstandard channel commands.

Byte 1, bit 7: This bit being on enables the CA to accept nonstandard channel commands by responding to the channel with a Channel End initial status, and requesting a level 3 interrupt.

Output X'57' (Channel Adapter Mode Register-CAMR)

This instruction loads the channel adapter mode register with the bits set in the general register. Byte 1 of this register can be loaded at any time to initialize or halt the CA. Byte 0, with the exception of bits 3 and 6, is accessible only when the CA is 'not initialized' or is in the diagnostic wrap state.

B-32

Type 2 and Type 3 Channel Adapters

Byte 0, bit 0: Set IPL Attention—A 1 in this position causes the channel adapter 'attention' latch to be set. This is done as soon as the adapter hardware determines that the channel is not examining the status register. The Type 3 CA offers Attention to both channels. Acceptance of the status byte by either channel causes the bit to be reset.

Byte 0, bit 1: Set IPL Channel End—A 1 in this position causes the channel adapter 'channel end status' latch to be set once the adapter hardware determines that the 'channel end remembrance' latch is not on.

Byte 0, bit 2: This bit is unused.

Byte 0, bit 3: Set Asynchronous Device End—A 1 in this position causes the channel adapter 'device end status' latch to be set. The Type 3 CA offers Device End to both channels. Acceptance of the status byte by either channel causes the bit to be reset.

Byte 0, bit 4: Set IPL Unit Check—A 1 in this position causes the channel adapter 'unit check status' latch to be set. This indication should not be set without setting byte 0, bit 7 (IPL not initialized).

Byte 0, bit 5: This bit is unused.

Byte 0, bit 6: Set IPL Unit Exception—A 1 in this position causes the selected CA to present Unit Exception initial status to all Sense commands. This bit is set by the ROS program to prevent two host processors simultaneously trying to IPL the controller when two channel adapters are installed. However, it is the responsibility of the loader program to turn this bit off.

Byte 0, bit 7: Set IPL Not-Initialized—A 1 in this position causes the channel adapter not-initialized sense bit to be set when the channel is not executing a Sense command. The not-initialized sense bit is reset when the ROS program executes an Output X'77' instruction with byte 0, bit 0 set to 1.

Byte 1, bit 0: Set CA Level 3 Interrupt Request—A 1 in this position causes the CA to request a level 3 interrupt once the CA is in the inactive state, with no outstanding status and no channel chaining indicated. The latch is reset at the end of the first level 3 interrupt to occur after it has been set.

Byte 1, bit 1: Set Program Abort—When an Output X'57' is executed with a 1 bit in this position while a channel operation of 'Write', 'Write Break', 'Write IPL', or 'Read' is in progress, the operation is terminated with an ending status of CE, DE, UC, and the abort sense bit is set. Once the status is accepted by the channel, a level 3 interrupt is requested. If other than one of the mentioned channel operations is in progress, the level 3 interrupt is requested at the completion of the operation. If no channel operation is in progress, the level 3 interrupt is requested immediately. Program abort is reset when the level 3 interrupt is reset.

Programming Note

The interrupt program should invalidate the CWARs, if necessary, by resetting the CWAR-valid flags with an Output X'56' instruction. If the program does reset the CWAR-valid flags, they must be restored by an Output X'55' before another channel data transfer can be initiated.

Byte 1, bit 2: Reset CA Level 1 Interrupt Request—A 1 in this position resets the CA 'level 1 interrupt request' latch. This should be the last action the control program takes before leaving a CA-initiated level 1 interrupt. All check latches, except 'bus out check', contained in Input X'55' are reset when this output occurs.

Byte 1, bit 3: Reset CA Level 3 Interrupt Request—A 1 in this position resets the CA 'level 3 interrupt request' latch. However, it does not necessarily reset the cause of the interrupt request.

Byte 1, bit 4: Select CA—A 1 in this position selects CA-1. A 0 in this position selects CA-2. The control program must ensure that the state of this bit is correct any time an Output X'57' is issued.

Byte 1, bit 5: Reset Selective/System Reset—Al in this position resets the selective system reset condition. However, byte 1, bit 3 of this output must also be 1 to reset the resulting level 3 interrupt request.

Byte 1, bit 6: Reset Channel Stop/Interface Disconnect—A 1 in this position resets the Channel Stop or Halt I/O indication set when either of these sequences is received from the channel interface. However, byte 1, bit 3 of this output must also be 1 to reset the resulting level 3 interrupt request.

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-33

Type 2 and Type 3 Channel Adapters

Byte 1, bit 7: Diagnostic Wrap Mode—A 1 in this position forces the channel adapter to go offline and places it in the diagnostic wrap state. A 0 places the CA back online.

Output X'58' (Channel Bus Out Diagnostic Register—CBODR)

The channel bus out diagnostic register (CBODR) is used to simulate I/O 'bus out' when the CA is in diagnostic wrap state. Byte 0, bits 0-7 and byte 1, bit 0 of this output are available only when the adapter is in the diagnostic wrap mode.

Byte 0, bits 0-7: Channel Bus Out bits 0-7.

Byte 1, bit 0: Channel Bus Out Parity bit. This bit may be used in diagnostic state to check the CA error-detection circuits.

Byte 1, bits 1-6: These bits are unused.

Byte 1, bit 7: Reset CA—A 1 in this position causes the CA currently selected to be reset. Its intended use is for diagnostic programs, and it is not necessary for normal channel adapter operation.

Output X'59' (Type 3 Channel Adapter Diagnostic Busy)

This instruction makes either or both Type 3 Channel Adapter interfaces Busy for diagnostic purposes.

Byte 0, bit 0: Set Interface A Busy—This bit is set on to cause Interface A to respond with an initial status of Busy to all channel initiated selection sequences.

Byte 0, bit 1: Set Interface B Busy—A 1 in this position causes Interface B to respond with a Busy status to all channel-initiated sequences.

Byte 0, bits 2-3: These bits are unused.

Byte 0, bit 4: Reset Interface A Busy—A 1 in this position resets Interface A Busy (if previously set by an Output X'59'). If the Interface A Busy condition is already reset, the output performs no operation and is ignored.

Byte 0, bit 5: Reset Interface B Busy—A 1 in this position resets Interface B Busy (if previously set by an Output X'59'). If Interface B Busy condition is already reset, the output performs no operation and is ignored.

Byte 0, bits 6-7: These bits are unused.

Byte 1, bits 0-7: These bits are unused.

Programming Note

Do not execute Output X'59' with both the set and reset bits on for either interface. An Output X'59' with either bits 0 and 3 or 1 and 4 on in byte 0 causes the respective hardware latch to reach an indeterminate state.

Output X'5A' (Channel Adapter Data Buffer-CADB)

This instruction loads the CA data buffer with the data to be sent to the host processor channel for either normal or diagnostic operations.

Byte 0, bits 0-7: Data Buffer byte 0, bits 0-7.

Byte 1, bits 0-7: Data Buffer byte 1, bits 0-7.

Output X'5B' (Channel Adapter Tag Diagnostic Register)

This instruction loads the CA tag diagnostic register with a combination of bits from a general register to raise channel tag lines for diagnostic purposes. The characteristic of this register is such that a 1 sets the corresponding bit, and a 0 resets it. Therefore, care should be taken when issuing this output to ensure the register's integrity.

This output is accessible only when the CA is in the diagnostic wrap mode.

Byte 0, bit 0: Select Out/Hold Out—A 1 in this position raises the inbound 'select out/hold out' tag line.

Byte 0, bit 1: Address Out—A 1 in this position raises the 'address out' tag line.

Byte 0, bit 2: Command Out—A 1 in this position raises the 'command out' tag line.

Byte 0, bit 3: Service Out—A 1 in this position raises the 'service out' tag line.

Byte 0, bit 4: Operational Out—A 1 in this position raises the 'operational out' tag line.

Byte 0, bit 5: Suppress Out—A 1 in this position raises the 'suppress out' tag line.

Byte 0, bits 6- 7: These bits are unused.

Byte 1, bits 0-7: These bits are unused.

B-34

Remote Program Loader

Remote Program Loader Input Instructions

Input X'68' (Level 1 Status)

This instruction allows the control program to examine the contents of the level 1 status register to determine the cause of a remote program loader interrupt.

Byte 0, bit 0:   This bit is set 0.

Byte 0, bit 1: Outbus Parity Error—This bit is set to 1 when a parity error is detected on the Outbus from the CCU to the disk controller. This is an error condition that causes a program level 1 interrupt request.

Byte 0, bits 2-7: These bits are 0.

Byte 1, bits 0-2: These bits are 0.

Byte 1, bit 3: This bit is set to 1 when a Write command is issued to the disk and the write operation is not enabled. This is an error condition that causes a program level 1 interrupt request.

Byte 1, bits 4-7: These bits are 0.

Input X'69' (Level 3 Status)

This register can be accessed only after an interrupt request or when the disk controller is in the reset state.

Byte 0, bit 0: Index—This bit is set to 1 when the index is detected and the interrupt on index bit (Output X'68', byte 1, bit 7) is set.

Byte 0, bit 1: Read Sync—This bit is set on when the disk controller is in sync on a read operation (for diagnostic purposes).

Byte 0, bits 2-5: Head Access Counter—These bits form a counter that controls the head access stepping motor. The bits are named access 0 through access 3 respectively. They are used by the control program to determine if the head has moved after execution of an Output X'68' with byte 1, bit 1 on.

Byte 0, bit 6: Head Engage—This bit is set on when the 'head engage' latch is set on.

Byte 0, bit 7: I/O Data Service Request—This bit is set on when a data service request is present for an input/output operation.

Byte 1, bit 0: Head Disengaged— This bit is set on when the disk head is disengaged and the current is dropped from the disk motor.

Byte 1, bits 1-3: These bits are 0.

Byte 1, bit 4: Overrun—This bit is set on when an I/O overrun is detected during a data transfer from the disk controller to the CCU.

Byte 1, bits 5-7: These bits are 0.

Input X'6A' (Parallel Data Register)

This instruction places the contents of the disk controller's parallel data register on the CCU Inbus. This instruction is used to read data into storage during an I/O data transfer and for diagnostic purposes.

Input X'6B' (Control Program Load Register)

This instruction is used to provide information required by the IPL and program load phases. The bits in this register are reset to 0 when the Reset push button is pressed or when a power-on reset occurs.

Byte 0, bit 0: Preserve Storage—Al in this position indicates that the contents of storage are to be preserved until a storage dump is taken.

Byte 0, bit 1: Control Program IPL—This bit should be on when an IPL is initiated by the control program.

Byte 0, bit 2: This bit is set to 0.

Byte 0, bit 3: Host Initiated IPL—This bit should be on when an IPL is initiated by a host processor command.

The remaining bits of this register are not present in the controller.

Remote Program Loader Output Instructions

Output X'68' (Control)

This instruction prepares for subsequent read or write operations by controlling the access arm.

Byte 0, bit 0: Low Current—This bit sets the current level to the disk for a write operation. When writing on tracks 45 through 76, this bit must be set to 1 (low current). The bit must be 0 (high current) when writing on tracks 0 through 41. Tracks 42, 43, and 44 can use either high or low current.

IBM 3704 and 3705 Communications Controllers Principles of Operation B-35

Remote Program Loader

Byte 0, bit 2: Reset Low Current—A 1 in this position causes the low current indication (byte 0, bit 0 of this output) to be reset.

Byte 0, bit 3: Reset Access Counter—A 1 in this position resets the head access counter (byte 0, bits 2-5 of Input X'69') to Access 0.

Byte 0, bits 4-7: These bits are unused.

Byte 1, bit 0: Reset L3 Interrupt Latch—A 1 in this position resets the 'interrupt on index' latch that is set as a result of byte 1, bit 7 of this output being on.

Byte 1, bit 1: Head Move Enable—A 1 in this position allows the head to be moved. A 0 inhibits all head movement.

Byte 1, bit 2: Head Direction—A 1 in this position indicates that the head is to be moved in a forward direction and a 0 indicates a reverse direction.

Byte 1, bit 3: Set Status Latches—A 1 in this position sets the diagnostic status latches in the disk controller.

Byte 1, bit 4: Head Engage—A 1 in this position sets the 'head engage' latch, which allows the disk head to become active for a read or write operation.

Byte 1, bit 5: Disk Controller Reset—A 1 in this position resets the disk controller. All registers, latches, and counters in the disk controller except the head access counter are reset.

Byte 1, bit 6: LI and L3 Interrupt Reset—A 1 in this position resets the 'level 1 interrupt' and the 'level 3 interrupt' latches in the disk controller and all the controller status latches.

Byte 1, bit 7: Set L3 Interrupt on Index—A 1 in this position causes the disk controller to request a level 3 interrupt on index.

Output X'69' (Read/Write)

Byte 0, bit 0: This bit is unused.

Byte 0, bit 1: I/O Write—A 1 in this position initiates an input/output write operation to the disk.

Byte 0, bit 2: I/O Read—A 1 in this position initiates an input/output read operation to the disk.

Byte 0, bits 3-7: These bits are unused.

Byte 1, bits 0- 7: These bits are unused.

Output X'6A' (Parallel Data Register)

This instruction loads the parallel data register in the disk controller with the information on the CCU Outbus. This instruction is used to write data on the disk during an I/O data transfer or for diagnostic purposes.

Output X'6B' (Control Program Load Register)

This register is used to pass information required by the IPL and program load phases. The bits of this register are reset to 0 when the Reset push button is pressed or when a power-on reset occurs.

Byte 0, bit 0: Preserve Storage—This bit must be set to 1 by the control program if the contents of storage are to be preserved so that the load program can dump the remote storage contents back to the host processor.

Note: This bit does not initiate a dump operation. It only informs the load program to preserve storage until a dump is complete.

Byte 0, bit 1: Control Program IPL—This bit must be set to 1 by the control program to inform the load programs that an IPL operation was initiated by the control program.

Byte 0, bit 2: This bit is unused.

Byte 0, bit 3: Host Initiated IPL—This bit must be set to 1 by the control program to inform the load programs that an IPL operation was initiated by a host processor command.

The remaining bits of this register are not present in the controller.

B-36

Central Control Unit

CCU Input Instructions

Input X'70' (Storage Size Installed)

This instruction causes the register specified by R to be loaded with a bit combination that indicates the amount of storage installed in the controller.

The register bits are set to one of the values shown below to indicate the amount of storage installed in the controller.

| 3705-II Bit Settings |  |  |
| --- | --- | --- |
| Storage | Byte 0, bit | Hex value |
| Size | 3456 | of byte 0 |
| 32K | 0001 | X'02' |
| 64K | 0010 | X'04' |
| 96K | 0011 | X'06' |
| 128K | 0100 | X'08' |
| 160K | 0101 | X'0A' |
| 192K | 0110 | X'0C' |
| 224K | 0111 | X'0E' |
| 256K | 1000 | X'10' |
| 3705-I Bit Settings |  |  |
| Storage | Byte 0, bit | Hex value |
| Size | 0123 | of byte 0 |
| 16K | 0001 | X'10' |
| 48K | 0011 | X'30' |
| 80K | 0101 | X'50' |
| 112K | 0111 | X'70' |
| 144K | 1001 | X'90' |
| 176K | 1011 | X'B0' |
| 208K | 1101 | X'D0' |
| 240K | 1111 | X'F0' |
| 3704 Bit Settings |  |  |
| Storage | Byte 0, bit | Hex value |
| Size | 0123 | of byte 0 |
| 16K | 0001 | X'10' |
| 32K | 0010 | X'20' |
| 48K | 0011 | X'30' |
| 64K | 0100 | X'40' |

All bit positions of byte 1 of this register are 0.

Input X'71' (Panel Address/Data Entry Digits)

This instruction causes the register specified by R to be loaded according to the setting of the Address/Data switches on the control panel.

The action taken for each register bit position is given below.

Byte X, bits 6-7: Address/Data A—On a 3705 with Extended Addressing, these bits are loaded with the information from switch A on the control panel.

Byte 0, bits 0-7: Address/Data B & C—These bits are loaded with the information from switches B and C on the control panel.

Byte 1, bits 0-7: Address/Data D & E—These bits are loaded with the information from switches D and E on the control panel.

Input X'72' (Panel Display/Function Select Switch Controls)

This instruction causes the register specified by R to be loaded with information indicating the position of the Display/Function Select switch on the control panel.

Byte 0, bits 0-2: These bits are 0.

Byte 0, bit 3: Storage Address—This bit indicates that the Display/Function Select switch is in the STORAGE ADDRESS position.

Byte 0, bit 4: Register Address—This bit indicates that the Display/Function Select switch is in the REGISTER ADDRESS position.

Byte 0, bits 5-7: These bits are 0.

Byte 1, bit 0: This bit is 0.

Byte 1, bit 1: Function Select 1—This bit indicates that the Display/Function Select switch is in the FUNCTION 1 position.

Byte 1, bit 2: Function Select 2—This bit indicates that the Display/Function Select switch is in the FUNCTION 2 position.

Byte 1, bit 3: Function Select 3—This bit indicates that the Display/Function Select switch is in the FUNCTION 3 position.

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-37

Central Control Unit

Byte 1, bit 4: Function Select 4—This bit indicates that the Display/Function Select switch is in the FUNCTION 4 position.

Byte 1, bit 5: Function Select 5—This bit indicates that the Display/Function Select switch is in the FUNCTION 5 position.

Byte 1, bit 6: Function Select 6—This bit indicates that the Display/Function Select switch is in the FUNCTION 6 position.

Byte 1, bit 7: This bit is 0.

Input X'73' (Insert Key)

This instruction is associated with storage protection. It causes the key addressed by the last Output X'73' to be inserted into byte 1, bits 5-7 of the register specified by R. All other bits of the register are set to 0. See Storage Protect in Chapter 5.

Input X'74' (Lagging Address Register)

This instruction causes the contents of the lagging address register to be transferred to the register specified by R.

If this instruction is executed at program levels 2, 3, or 4, the address from the LAR is that of the last instruction executed before the input instruction. If this instruction is executed in program level 1, the address from the LAR is that of the last instruction executed before entering level 1.

Input X'76' (Adapter Level 1 Interrupt Requests)

This instruction is associated with program level 1 interrupt requests. It loads the register specified by R with a combination of bits to indicate the origin of an adapter level 1 interrupt request.

Byte 0, bit 0: Type 4 CA-1 or CA-2 L1-This bit indicates that Type 4 CA-1 or CA-2 has requested a program level 1 interrupt.

Byte 0, bit 1: Type 2 or 3 Scanner-1 (or Type 1 Scanner) L1—This bit indicates that a Type 1 Scanner or a Type 2 or 3 Scanner-1 has requested a program level 1 interrupt.

Byte 0, bit 2: Type 2 or 3 Scanner-2 LI-This bit indicates that a Type 2 or 3 Scanner-2 has requested a program level 1 interrupt.

Byte 0, bit 3: Type 2 or 3 Scanner-3 L1-This bit indicates that a Type 2 or 3 Scanner-3 has requested a program level 1 interrupt.

Byte 0, bit 4: Type 2 or 3 Scanner-4 L1-This bit indicates that a Type 2 or 3 Scanner-4 has requested a program level 1 interrupt.

Byte 0, bit 5: Type 1, 2, or 3 CA-1 L1 or Selected Type 4 CA Ll-This bit indicates that a Type 1 CA, a Type 2 or Type 3 CA-1, or a selected Type 4 CA has requested a program level 1 interrupt.

Byte 0, bit 6: Type 2 or 3 CA-2 Ll-This bit indicates that a Type 2 or Type 3 CA-2 has requested a program level 1 interrupt.

Byte 0, bit 7: Remote Loader L1 Request—This bit indicates that the disk controller has requested a program level 1 interrupt.

Byte 1, bits 0-7: These bits are 0.

Input X'77' (Adapter Level 2 or 3 Interrupt Requests)

This instruction is associated with program level 2 and level 3 interrupt requests. It loads the register specified by R with a combination of bits to indicate the origin of an adapter level 2 or 3 interrupt request.

When priority selection is required with two Type 4 CAs, this instruction sets or resets the selected latch in each Type 4 CA according to the state of the "CA4 has priority" latch. It also resets the "prime priority select" latch.

Byte 0, bit 0: This bit is 0.

Byte 0, bit 1: Type 1, 2, or 3 Scanner L2-This bit indicates that a Type 1, Type 2, or Type 3 Scanner has requested a program level 2 interrupt.

Byte 0, bits 2-7: These bits are 0.

Byte 1, bit 0: Type 4 CA L3-This bit indicates that a Type 4 CA-1 or CA-2 has requested a program level 3 interrupt.

Byte 1, bit 1: Remote Loader L3 Request—This bit indicates that the disk controller has requested a program level 3 interrupt.

B-38

Byte 1, bit 2: Type 2 or 3 CA-2 L3-This bit indicates that a Type 2 or Type 3 CA-2 has requested a program level 3 interrupt.

Byte 1, bit 3: Type 1 or Selected Type 4 CA Data/Status L3-This bit indicates that a Type 1 or selected Type 4 CA has requested a program level 3 data service interrupt.

Byte 1, bit 4: Type 1, 2, or 3 CA-1 or selected Type 4 CA L3-This bit indicates that a Type 2 or Type 3 CA-1 has requested a program level 3 interrupt, or a Type 1 or selected Type 4 CA has requested a program level 3 initial selection interrupt.

Byte 1, bit 5: This bit is 0.

Byte 1, bit 6: Type 4 CA Selected-This bit set to 1 indicates that Type 4 CA-2 has been selected. This bit set to 0 indicates that Type 4 CA-1 has been selected.

Byte 2, bit 7: This bit is 0.

Input X'79' (Utility)

This instruction causes the register specified by R to be loaded with information indicating (1) the state of the program level 5 C and Z condition latches, (2) the last program level to be active before a level 1 interrupt, and (3) the state of the IPL escape control.

Byte 0, bits 0-5: These bits are 0.

Byte 0, bit 6: Program Level 5, C Condition—This bit indicates that the 'C condition' latch for program level 5 is on.

Byte 0, bit 7: Program Level 5, Z Condition—This bit indicates that the 'Z condition' latch for program level 5 is on.

Byte 1, bit 0: Program Level 2—This bit indicates that program level 2 was interrupted by level 1. (See note below.)

Byte 1, bit 1: Program Level 3—This bit indicates that program level 3 was interrupted by level 1. (See note below.)

Byte 1, bit 2: Program Level 4—This bit indicates that program level 4 was interrupted by level 1. (See note below.)

Central Control Unit

Byte 1, bit 3: Program Level 5— This bit indicates that program level 5 or the "wait" state was interrupted by level 1. (See note below.)

Byte 1, bit 4: 1 indicates that the 3705 has FET storage (3705-II); 0 indicates that the 3705 has bridge storage (3705-I).

Byte 1, bit 5: Remote 3704—0 indicates 3705; 1 indicates 3704. This bit is required for timing purposes in the test section of the ROS bootstrap program of a remote communications controller.

Byte 1, bit 6: Type 1 or 4 CA Installed-This bit indicates that a Type 1 or 4 CA is installed in the basic module of the controller. When the bit is off, a Type 2 or Type 3 CA is implied. However, this bit will be on only when the program being executed is running in level 1.

Byte 1, bit 7: IPL Escape Control—This bit is 0 as a result of a jumper wire installed by a Customer Engineer. By forcing a branch to storage location X'06FC', this jumper causes a bypass of the part of the bootstrap program that actually controls the initial program load. This is a diagnostic facility for read-only-storage maintenance.

Note to byte 1, bits 0-3: One of these bits is 1 when an Input X'79' is executed in program level 1. The bit that is set indicates the program level that was operating when control was passed to program level 1. All other positions are set to 0. These bits are all 0 if Input X'79' is executed when not in program level 1.

Input X'7B' (BSC CRC Register)

This instruction causes the old CRC character and the data character to be added to the CRC accumulation to be combined and loaded into the BSC CRC register. Then bytes 0 and 1 of the general register specified by R are loaded with the new BSC CRC accumulation character from the BSC CRC register. With Extended Addressing, byte X of the general register is set to 0. See Cyclic Redundancy Check in Chapter 5.

IBM 3704 and 3705 Communications Controllers Principles of Operation B-39

Central Control Unit

Input X'7C' (SDLC CRC Register)

This instruction combines the old CRC character with the data character to be added to the SDLC CRC and loads the new character into the CRC register. The new SDLC CRC is then stored in the specified general register.

Input X'7D' (CCU Check Register)

The bits of this instruction are set when the Central Control Unit detects an error condition. This instruction sets the bits in the general register specified by R to correspond to the CCU check register.

Byte 0, bit 0: Byte X Check—This bit indicates a byte X parity error (for Extended Addressing only).

Byte 0, bit 1: Byte 0 Check—This bit indicates a byte 0 parity error.

Byte 0, bit 2: Byte 1 Check—This bit indicates a byte 1 parity error.

Byte 0, bit 3: L1 Program Check—This bit indicates that a program check occurred while in level 1.

Byte 0, bit 4: SAR Check—This bit indicates a storage address register parity check. Byte 0, bit 0, 1, or 2 is also 1 to indicate which byte of the SAR caused the check.

Byte 0, bit 5: SDR Check—This bit indicates a storage data register parity check. Byte 0, bit 0, 1, or 2 is also 1 to indicate which byte of the SDR caused the check.

Byte 0, bit 6: Op Reg Check—This bit indicates an operation register parity check. Byte 0, bit 0, 1, or 2 is also 1 to indicate which byte of the Op Reg caused the check.

Byte 0, bit 7: Indata Bus Check—This bit indicates an indata bus check. Byte 0, bit 0, 1, or 2 is also 1 to indicate which byte of the indata bus caused the check.

Byte1, bit 0: Cycle Counter Check—This bit indicates that the CCU cycle counter signaled an even number of time slots to the CCU. This check is one of three that set the Clock Check light on the control panel

Byte 1, bits 1-4: These bits are 0.

Byte 1, bit 5: CCU Checks—This bit indicates the presence of a CCU check. The CCU check bit is a summary type bit that is set to 1 whenever any one or more of the CCU check bits of this input are on. When checking for a CCU check condition, this bit should be tested first.

Byte 1, bit 6: Type 2 Attachment Base Clock Check—This bit indicates a Type 2 Attachment Base clock check. This check indicates that the Type 2 Scanner clock has signaled an incorrect number of time slots. The check is one of three checks that set the Clock Check light on the control panel.

Byte 1, bit 7: CCU Clock Check—This bit indicates a Central Control Unit clock check. A CCU clock check indicates that the CCU clock has signaled an incorrect number of time slots. This check is one of three checks that set the Clock Check light on the control panel.

Input X'7E' (CCU Level 1 Interrupt Requests)

This instruction sets the bits in the register specified by R to indicate which level 1 interrupt request is set. Byte 1, bits 1-4 are set as the result of a program check in any level and cause an interrupt to level 1.

Byte 0, bits 0-7: These bits are 0.

Byte 1, bit 0: Address Compare Interrupt L1—This bit indicates a level 1 address compare interrupt.

Byte 1, bit 1: Address Exception L1—This bit indicates that an addressing exception has occurred.

Byte 1, bit 2: In/Out Check L1—This bit indicates that an input/output check has occurred.

Byte 1, bit 3: Protection Check L1—This bit indicates that a protection check has occurred.

Byte 1, bit 4: Invalid Op Check L1—This bit indicates that an invalid op-code check has occurred.

Byte 1, bit 5: This bit is 0.

Byte 1, bit 6: IPL L1—This bit indicates that program level 1 has requested an IPL.

Byte 1, bit 7: This bit is 0.

B-40

Central Control Unit

Input X'7F' (CCU Level 2, 3, or 4 Interrupt Requests)

This instruction is used to set bits in the register specified by R to indicate which level 2, 3, or 4 interrupt request is set.

Byte 0, bit 0: Diagnostic L2—This bit indicates a diagnostic level 2 interrupt request.

Byte 0, bits 1-5: These bits are 0.

Byte 0, bit 6: Panel Interrupt Request L3—This bit indicates an interrupt request from the Interrupt push button on the control panel.

Byte 0, bit 7: PCI L4—This bit indicates a level 4 program-controlled interrupt (PCI L4) request.

Byte 1, bits 0-4: These bits are 0.

Byte I, bit 5: Interval Timer L3—This bit indicates a level 3 interval timer interrupt request.

Byte 1, bit 6: PCI L3—This bit indicates a level 3 program-controlled interrupt (PCI L3) request.

Byte 1, bit 7: SVC L4—This bit indicates a level 4 supervisor call interrupt (SVC L4) request.

CCU Output Instructions

Output X'70' (Hardstop)

This instruction causes the 'hardstop' latch to be set. This causes the controller to come to a complete stop and requires an IPL to continue processing. Since this instruction performs a function, the bit settings of the register are ignored.

Output X'71' (Display Register 1)

This instruction transfers the contents of the register specified by R to display register 1. The Program Display light on the control panel is also turned on.

Output X'72' (Display Register 2)

This instruction transfers the contents of the register specified by R to display register 2. The Program Display light on the control panel is also turned on.

Output X'73' (Set Key)

This instruction is associated with the storage protect mechanism. Refer to Storage Protect in Chapter 5. It is normally used to set either a storage key or protect key with the contents of byte 1, bits 5-7 of the register specified by R.

Byte 1, bit 3 (key select) controls the selection of either a storage key or protect key. If byte 1, bit 4 (set key) of the register is on, the addressed key is set according to byte 1, bits 5-7. If byte 1, bit 4 is off, the addressed key is not set. In either case, the key addressed with the last Output X'73' executed can be accessed with a subsequent Input X'73'.

Byte 0, bits 0-3: Key Address—If byte 1, bit 3 of this output is 1, these bits, along with byte 0, bits 4-6 are set to the storage block number to be referenced when setting a storage key. If byte 1, bit 3 is 0, these bits are ignored.

Byte 0, bits 4-6: Key Address—If byte 1, bit 3 of this output is 1, these bits along with byte 0, bits 0-3 form a storage block number. If byte 1, bit 3 is a 0, these bits are set to the protect key address of the desired user.

Byte 0, bit 7: This bit is unused.

Byte 1, bits 0-2: These bits are unused.

Byte 1, bit 3: Key Address Select—This bit is used to indicate which key this instruction will be used for. If this bit is 1, a storage key is set, and the storage block number should be in byte 0, bits 0-6. If this bit is 0, a protect key is set, and the address of the user should be in byte 0, bits 4-6.

Byte 1, bit 4: Set Key—If this bit is set to 1, the key set in bits 5-7 is inserted for the addressed key. If this bit is 0, no keys are changed.

Byte 1, bits 5-7: Key—These bits contain the storage key or protect key as indicated by byte 1, bit 3 to be assigned to the storage block or user addressed by byte 0, bits 0-6. If byte 1, bit 4 is zero, these bits are ignored.

Output X'77' (Miscellaneous Control)

This instruction contains miscellaneous controls used to set or to reset various interrupt requests.

Byte 0, bit 0: Reset IPL LI—This bit is set to 1 to reset the level 1 IPL request and the CA not-initialized state (Output X'57', byte 0, bit 7).

Byte 0, bit 1: Reset CCU Checks—This bit is set to 1 to reset all CCU checks.

IBM 3704 and 3705 Communications Controllers Principles of Operation  B-41

Central Control Unit

Byte 0, bit 2: Reset Panel Interrupt Request L3—This bit is set to 1 to reset the external interrupt request at program level 3.

Byte 0, bits 3-4: These bits are reserved for diagnostic use. A 0 is the inactive state of these bits. The active state is allowed only in the test mode.

Byte 0 bit 5: This bit is unused.

Byte 0, bit 6: Set Diagnostic L2—If this bit is set to 1 and the CCU is in test mode, the diagnostic interrupt request at program level 2 is set. This bit should not be 1 if the CCU is not in test mode because the level 2 interrupt may be taken and there will be no indication that it is a diagnostic level 2 interrupt request (byte 0, bit 0 of Input X'7F').

Byte 0, bit 7: Reset Diagnostic L2—This bit is set to 1 to reset the level 2 diagnostic interrupt request. It is ignored if the CCU is not in the test mode.

Byte 1, bit 0: This bit is unused.

Byte 1, bit 1: Reset Interval Timer L3—This bit is set to 1 to reset the program level 3 interval timer interrupt request.

Byte 1, bit 2: Reset PCI L3—This bit is set to 1 to reset the level 3 program-controlled interrupt request (PCI L3) set by Output X'7C'.

Byte 1, bit 3: This bit is unused.

Byte 1, bit 4: Reset Address Compare L1—This bit is set to 1 to reset the program level 1 address compare interrupt request.

Byte 1, bit 5: Reset Program Checks L1—This bit is set to 1 to reset all program check interrupt requests to program level 1.

Byte 1, bit 6: Reset PCI L4—This bit is set to 1 to reset the level 4 program-controlled interrupt request (PCI L4) set by Output X'7D'.

Byte 1, bit 7: Reset SVC L4-This bit is set to 1 to reset the level 4 supervisor call (SVC L4) request.

Output X'78' (Force CCU Checks-3705)

This instruction provides a means for testing the CCU check circuits under diagnostic control by forcing checks in the CCU data flow. This output instruction is ignored if the CCU is not in the test mode.

Programming Note

Although the action specified by this instruction is taken only once, the result may be 'permanent' until corrective action is taken to store data with the correct parity into the affected register or storage location.

Byte 0, bits 0- 7: Complement Bits 0-7—A 1 in any of these positions causes the corresponding bit positions of each byte (byte X, 0, and 1) of the input to the ALU check detection circuits to be complemented.

Byte 1, bit 0: Complement Storage Parity—A 1 in this position causes incorrect parity in both byte 0 and byte 1 of the halfword accessed in storage on the next instruction cycle.

Byte 1, bit 1: Complement Z Bus Parity—A 1 in this position causes incorrect parity to be stored into a general register. A load or store instruction should then follow this output to place the data from the general register on the Z bus to exercise parity checking circuits.

Byte 1, bit 2: A-Register Check—A 1 in this position causes incorrect parity in the A-register.

Byte 1, bit 3: Indata Bus Check—A 1 in this position causes the Indata parity bits to be complemented. This results in a B-register parity check when an Input X'70', X'73'-X'75', or X'78'-X'7F' instruction is executed. Input instructions X'71' and X'72' do not give checks.

Byte 1, bits 4- 7: These bits are unused.

Output '78' (Force CCU Checks-3704)

This instruction provides a means of testing the CCU check circuits under diagnostic control by forcing checks in the CCU. This output is ignored if the CCU is not in test mode.

B-42

Programming Note

Although the action specified by this instruction is taken only once, the result may be 'permanent' until corrective action is taken to store data with the correct parity into the affected register or storage location.

Byte 0, bits 0-4: These bits are unused.

Byte 0, bits 5-7: Instruction Cycle Select—A binary decode of these three bits determines the cycle in which the selected error will occur, as follows:

001 = Select next I1 cycle

010 = Select next I2 cycle

011 = Select next I3 cycle

100 = Select next I4 cycle

101 = Select next I5 cycle

Byte 1, bit 0: Complement Storage Parity—This bit causes improper parity to be stored in both main storage bytes addressed during the next selected cycle (selected by byte 0, bits 5-7 of this instruction). This can be used to exercise either SDR or Op Reg parity checks at a later time. For the expected results, the selected cycle must coincide with an actual storage write operation.

Byte 1, bit 1: Complement Z Bus Parity—This bit is used to store even parity into a local store register or an external register.

Byte 1, bit 2: A-Reg Check—This bit causes the A-reg parity bits to be complemented during the next selected cycle.

Byte 1, bit 3: B-Reg Parity Error—This bit causes the B-reg parity bits to be complemented during the next selected cycle.

Byte 1, bit 4: SAR Parity Error—This bit causes the SAR parity bits to be complemented during the next selected cycle.

Byte 1, bit 5: Op Reg Parity Error—This bit causes the Op reg parity bits to be complemented during the next selected cycle. The selected cycle must be II.

Byte 1, bits 6- 7: These bits are unused.

Central Control Unit

Output X'79' (Utility)

This instruction is used to set or reset various CCU latches.

Byte 0, bits 0-1: These bits are unused.

Byte 0, bit 2: Set IPL—This bit is set to 1 to initiate an IPL operation.

Byte 0, bit 3: Set FET Storage Diagnostic Mode-This bit sets the 'allow set memory diagnostic register' latch.

Byte 0, bit 4: Remote Power Off—Using this bit allows the remote communications controller (if equipped with the Remote Power Off feature) to be powered off by the control program. Power must be restored manually.

Byte 0, bit 5: Inhibit Prog Level 5, C & Z Replacement—If this bit is set to 0, the program level 5 C and Z condition latches are set according to byte 0, bits 6-7. If this bit is 1, no action is taken.

Byte 0, bit 6: Program Level 5, C Condition—If byte 0, bit 5 of this output is 1, the 'C condition' latch for program level 5 is set according to the state of this bit.

Byte 0, bit 7: Program Level 5, Z Condition—If byte 0, bit 5 of this output is 1, the 'Z condition' latch for program level 5 is set according to the state of this bit.

Byte 1, bit 0: Reset CCU Check Hard Stop Mode—This bit is set to 1 to reset a CCU check hard stop.

The 'check stop mode' latch is set during IPL phase 1. As long as the Diagnostic Control switch on the control panel is in the PROCESS or CLOCK STEP position, the controller operates as if the switch is in the CCU CHECK HARD STOP position. This state can be overridden by the bypass CCU check stop state.

Byte 1, bit 1: Reset Load Indicator—This bit is set to 1 to reset the Load light on the control panel and to reset the 'load' latch to indicate the completion of IPL. The 'load' latch is set and the Load light is turned on during IPL.

IBM 3704 and 3705 Communications Controllers Principles of Operation B-43

Central Control Unit

Byte 1, bit 2: Set Test Mode—This bit is set to 1 to set the CCU in the test mode. This bit turns on the Test light on the control panel and enables the following diagnostic functions to be performed:

(1) Force CCU checks—Output X'78'.

(2) Set/reset Diagnostic Level 2.

(3) Set/reset Bypass CCU Check Stop.

(4) Set/reset adapter level 1 interrupt request mask.

See the CCU Diagnostic Facilities section in Chapter 5. This bit should not be 1 if byte 1, bit 3 of this output is 1.

Byte 1, bit 3: Reset Test Mode—This bit is set to 1 to reset the test mode. The Test light on the control panel is turned off if the Mode Select and the Diagnostic Control switches are both in the PROCESS position. Any of the test functions allowed by the test mode are also reset. This bit should not be 1 if byte 1, bit 2 of this output is 1.

Byte 1, bit 4: Set Bypass CCU Check Stop Mode—This bit is set to 1 to bypass the CCU check stop. When this bit is set, the operation is the same as if the Diagnostic Control switch on the control panel were in the BYPASS CCU CHECK STOP position. This bit should not be 1 if byte 1, bit 5 of this output is 1. It is ignored if the CCU is not in the test mode.

Byte 1, bit 5: Reset Bypass CCU Check Stop Mode—This bit is set to 1 to reset the bypass CCU check stop. This bit should not be 1 if byte 1, bit 4 of this output is 1. It is ignored if the CCU is not in the test mode.

Byte 1, bit 6: Scope Sync Pulse 1—This bit is set to 1 to generate the scope sync pulse 1. This is an oscilloscope synchronization pulse of 100 ns for maintenance purposes.

Byte 1, bit 7: Scope Sync Pulse 2—This bit is set to 1 to generate the scope sync pulse 1. This is an oscilloscope synchronization pulse of 100 ns for maintenance purposes.

Output X'7C' (Set PCI L3)

This instruction sets the program-controlled interrupt request for level 3. Since this instruction performs a function, the bit settings of the register are ignored.

Output X'7D' (Set PCI L4)

This instruction sets the program-controlled interrupt request for level 4. Since this instruction performs a function, the bit settings of the register are ignored.

Output X'7E' (Set Mask bits)

This instruction sets mask bits that prevent interrupts to a certain program level during processing. See Masking Program Level Priorities in Chapter 5.

Byte 0, bits 0-7: These bits are unused.

Byte 1, bit 0: This bit is unused.

Byte 1, bit 1: Adapter Requests L1—A 1 in this position sets a mask to prevent CA and Scanner interrupts to program level 1 (for diagnostic test mode only).

Byte 1, bit 2: Program Level 2—A 1 in this position sets a mask to prevent interrupts to program level 2.

Byte 1, bit 3: Program Level 3—A 1 in this position sets a mask to prevent interrupts to program level 3.

Byte 1, bit 4: Program Level 4—A 1 in this position sets a mask to prevent interrupts to program level 4.

Byte 1, bit 5: Program Level 5—A 1 in this position sets a mask to prevent instruction execution in program level 5.

Byte 1, bits 6-7: These bits are unused.

Output X'7F' (Reset Mask bits)

This instruction resets the mask bits for program level interrupts. See Masking Program Level Priorities in Chapter 5.

Byte 0, bits 0-7: These bits are unused.

Byte 1, bit 0: This bit is unused.

B-44

Byte 1, bit 1: Adapter Requests LI—A 1 in this position unmasks CA and scanner interrupts to level 1 (for diagnostic test modes only).

Byte 1, bit 2: Program Level 2—A 1 in this position unmasks interrupts to program level 2.

Byte 1, bit 3: Program Level 3—A 1 in this position unmasks interrupts to program level 3.

Central Control Unit

Byte 1, bit 4: Program Level 4—A 1 in this position unmasks interrupts to program level 4.

Byte 1, bit 5: Program Level 5—A 1 in this posi-tion unmasks program level 5 to allow instruction ex-ecution at that level.

Byte 1, bits 6- 7: These bits are unused.

IBM 3704 and 3705 Communications Controllers Principles of Operation B-45