# Chapter 4: Instruction Set

The communications controller contains 51 executable instructions that can be used to tailor a control program to meet the specific needs of the teleprocessing system. The instruction set provides the greatest possible program flexibility within a minimum amount of storage.

This chapter gives the general instruction formats, and describes each individual instruction.

Note: Except for the functions provided by the Extended Addressing feature, the instruction and operand descriptions in this chapter apply to both the 3704 and the 3705.

Figure 4-1 shows the op code bit structures, operand fields, and instruction execution times (in number of storage cycles). A storage cycle for the 3704 (read or write) is 600 nanoseconds, for the 3705-I (read and write) is 1.2 microseconds, and for the 3705-II (read and write) is 1.0 microsecond. The asterisks in the C, Z column designate the instructions that can alter the C and Z condition latches.

Figure 4-2 shows the basic mnemonic designations and assembler operand field designations for the various instructions.

Any attempt at program levels 2, 3, 4, or 5 to execute an operation code other than one of the 51 specified instructions results in a level 1 interrupt with the invalid op check bit set on in the CCU interrupt request group 1 register. An attempt to execute an invalid op code in program level 1 sets the program check in LI bit in the CCU check register along with the invalid op check bit and is handled as a CCU check. In all cases, instruction execution is suppressed. See CCU Checks in Chapter 5.

Instruction Format

The instruction length can be either one or two half-words. All instructions must be located in storage on integral half word boundaries.

The eight basic instruction formats are denoted by the format codes RR, RS, RT, RA, RSA, RE, RI and EXIT. The format codes express, in general terms, the operation to be performed. RR denotes a register-to-register operation; RS, a register-storage operation; RSA, a register-to-storage with addition operation; RT, a branch operation; RA, a register-to-immediate address operation; RE, a register-to-external register operation; RI, a register-to-immediate operand operation; and EXIT, a program level exit operation.

To help describe the execution of instructions, operands are designated as either first or second operands. For RR format instructions, the first and

second operands are denoted by the number following the name of the field (for example RI, R2).

Instruction Operand Fields

Instruction operands are in four classes: (1) immediate operands in the instructions themselves, (2) operands in external registers, (3) operands in the active group of general registers, and (4) explicitly addressed operands in storage.

The following fields represent the operands in the instruction format.

Immediate Operands

I Field: The I field in RI format instructions contains an 8-bit immediate data field.

A Field: With Extended Addressing, the A field in RA format instructions contains an 18-bit immediate data field. Without Extended Addressing, the A field is treated as a 16-bit immediate data field, and bits 14 and 15 of the instruction are ignored.

External Register Operands

E Field: The E field in RE format instructions specifies the hexadecimal address of an external register operand. This field is used only for input and output instructions. Throughout this text, many references are made to input and output instructions. These references specify the value of the E field in the form X'nn', where nn is the hexadecimal address of the external register. Appendix A shows the external register addresses and functions.

##### General Register Operands

R Field: The R field in RI, RS, RE, RR, RA, RSA and some RT format instructions specifies the general register in the active group that contains the first operand. For byte operations, the register specified by this field must be an odd-numbered register. For all other operations, any one of the eight general registers in the active group may be specified.

For STH and ST instructions, if the R field is zero, a constant of all zeros is selected as the operand, rather than the contents of register 0. z

IBM 3704 and 3705 Communications Controllers Principles of Operation .4-1

|  |  | C, | 3704 | 3705 | FORMAT |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Instruction | Z | Cycles | Cycles | 0 | 1 2 | 3 | 4 | 5 6 | 7 | 8 | 9 - 11 | 12-14 |  |  | 15 |  |
| B | Branch |  | 2 | 1 | 1 | 0 1 | 0 | 1 |  |  |  |  |  |  |  |  |  |
| BCL | Branch on C Latch |  | 2 | 1 | 1 | 0 0 | 1 | 1 |  |  |  | T |  |  |  | ± |  |
| BZL | Branch on Z Latch |  | 2 | 1 | 1 | 0 0 | 0 | 1 |  |  |  |  |  |  |  |  |  |
| BCT | Branch on Count |  | 3 | 1 | 1 | 0 1 | 1 | 1 |  |  | 1 | T |  |  |  | ± |  |
| BB | Branch on Bit |  | 3 | 1 | 1 | 1 M | M | 1 |  |  | M |  |  |  |  |  |  |
| LRI | Load Register Immediate | * | 3 | 1 | 1 | 0 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |
| ARI | Add Register Immediate | * | 3 | 1 | 1 | 0 0 | 1 | 0 |  |  |  |  |  |  |  |  |  |
| SRI | Subtract Register Immediate | * | 3 | 1 | 1 | 0 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |
| CRI | Compare Register Immediate | * | 3 | 1 | 1 | 0 1 | 1 | 0 | R | N |  | I |  |  |  |  |  |
| XRI | Exclusive OR Register Immediate | * | 3 | 1 | 1 | 1 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |
| ORI | OR Register Immediate | * | 3 | 1 | 1 | 1 0 | 1 | 0 |  |  |  |  |  |  |  |  |  |
| NRI | And Register Immediate | * | 3 | 1 | 1 | 1 1 | 0 | 0 |  |  |  |  |  |  |  |  |  |
| TRM | Test Register under Mask | * | 3 | 1 | 1 1 1 1 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| LCR | Load Character Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 0 | 0 0 0 | 1 0 0 |  |  | 0 |  |
| ACR | Add Character Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 0 | 0 0 1 | 1 0 0 |  |  | 0 |  |
| SCR | Subtract Character Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 0 | 0 1 0 | 1 0 0 |  |  | 0 |  |
| CCR | Compare Character Register | * | 3 | 1 | 0 | R2 | N | 0 | R2 | N | 0 | 0 1 1 | 1 0 0 |  |  | 0 |  |
| XCR | Exclusive OR Character Register | * | 3 | 1 | 0 |  | 2 | 0 |  | 1 | 0 | 1 0 0 | 1 0 0 |  |  | 0 |  |
| OCR | OR Character Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 0 | 1 0 1 | 1 0 0 |  |  | 0 |  |
| NCR | And Character Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 0 | 1 1 0 | 1 0 0 |  |  | 0 |  |
| LCOR | Load Character with Offset Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 0 | 1 1 1 | 1 0 0 |  |  | 0 |  |
| ICT | Insert Character and Count |  | 5 | 1 | 0 |  |  | 0 |  |  | 0 | 0 0 1 | 0 0 0 |  |  | 0 |  |
| STCT | Store Character and Count |  | 5 | 1 | 0 |  |  | 0 | R | N | 0 | 0 1 1 | 0 0 0 |  |  | 0 |  |
| IC | Insert Character | * | 4 | 2 | 0 |  |  | 1 |  |  | 0 | D |  |  |  |  |  |
| STC | Store Character |  | 4 | 2 | 0 | B |  | 1 |  |  | 1 |  |  |  |  |  |  |
| LH | Load Halfword | * | 4 | 2 | 0 |  |  | 0 |  |  | 0 | D |  |  | 1 |  |  |
| STH | Store Halfword |  | 4 | 2 | 0 |  |  | 0 | R |  | 1 |  |  |  | 1 |  |  |
| L | Load | * | 5 | 2# | 0 |  |  | 0 |  |  | 0 | D |  | 1 0 |  |  |  |
| ST | Store |  | 5 | 2# | 0 |  |  | 0 |  |  | 1 |  |  | 1 0 |  |  |  |
| LHR | Load Halfword Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 1 | 0 0 0 | 0 0 0 |  | 0 |  |  |
| AHR | Add Halfword Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 1 | 0 0 1 | 0 0 0 |  | 0 |  |  |
| SHR | Subtract Halfword Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 1 | 0 1 0 | 0 0 0 |  | 0 |  |  |
| CHR | Compare Halfword Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 1 | 0 1 1 | 0 0 0 |  | 0 |  |  |
| XHR | Exclusive OR Halfword Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 1 | 1 0 0 | 0 0 0 |  | 0 |  |  |
| OHR | OR Halfword Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 1 | 1 0 1 | 0 0 0 |  | 0 |  |  |
| NHR | And Halfword Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 1 | 1 1 0 | 0 0 0 |  | 0 |  |  |
| LHOR | Load Halfword with Offset Regsiter | * | 3 | 1 | 0 | R2 |  | 0 | R1 |  | 1 | 1 1 1 | 0 0 0 |  | 0 |  |  |
| LR | Load Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 1 | 0 0 0 | 1 0 0 |  | 0 |  |  |
| AR | Add Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 1 | 0 0 1 | 1 0 0 |  | 0 |  |  |
| SR | Subtract Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 1 | 0 1 0 | 1 0 0 |  | 0 |  |  |
| CR | Compare Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 1 | 0 1 1 | 1 0 0 |  | 0 |  |  |
| XR | Exclusive OR Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 1 | 1 0 0 | 1 0 0 |  | 0 |  |  |
| OR | OR Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 1 | 1 0 1 | 1 0 0 |  | 0 |  |  |
| NR | And Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 1 | 1 1 0 | 1 0 0 |  | 0 |  |  |
| LOR | Load with Offset Register | * | 3 | 1 | 0 |  |  | 0 |  |  | 1 | 1 1 1 | 1 0 0 |  | 0 |  |  |
| BALR | Branch & Link Register |  | 4 | 2 | 0 |  |  | 0 |  |  | 0 | 0 0 0 | 0 0 0 |  | 0 |  |  |
| IN | Input |  | 2 | 1 | 0 | E |  | 0 | R |  | E |  | 1 1 0 |  | 0 |  |  |
| OUT | Output |  | 2 | 1 | 0 |  |  | 0 |  |  |  |  | 0 1 0 |  | 0 |  |  |
| EXIT | Exit |  | 2 | 1 | 1 | 0 1 | 1 | 1 | 0 0 | 0 | 0 | 1 0 0 | 0 0 0 |  | 0 |  | 16 31 |
| BAL | Branch & Link |  | 3 | 2 | 1 | 0 1 | 1 | 1 | R |  | 0 | 0 0 0 0 0 |  | A |  |  |  |
| LA | Load Address |  | 3 | 2 | 1 | 0 1 | 1 | 1 |  |  | 0 | 0 1 0 0 0 |  |  |  |  |  |

*  Instructions that can alter condition latches.

±	1 = -	0 = +

#  = 3 Cycles with Extended Addressing

Figure 4-1. Instruction Bit Structure

4-2

| Instruction | Format Code | Mnemonic | Operand Field Format |
| --- | --- | --- | --- |
| Add Character Register | RR | ACR | R1(N1),R2(N2) |
| Add Halfword Register | RR | AHR | R1,R2 |
| Add Register | RR | AR | R1,R2 |
| Add Register Immediate | RI | ARI | R(N),I |
| AND Character Register | RR | NCR | R1(N1),R2(N2) |
| AND Halfword Register | RR | NHR | R1,R2 |
| AND Register | RR | NR | R1,R2 |
| AND Register Immediate | RI | NRI | R(N),I |
| Branch | RT | B | T |
| Branch and Link | RA | BAL | R,A |
| Branch and Link Register | RR | BALR | R1,R2 |
| Branch on Bit | RT | BB | R(N,M),T |
| Branch on Count | RT | BCT | R(N),T |
| Branch on C Latch | RT | BCL | T |
| Branch on Z Latch | RT | BZL | T |
| Compare Character Register | RR | CCR | R1(N1),R2(N2) |
| Compare Halfword Register | RR | CHR | R1,R2 |
| Compare Register | RR | CR | R1,R2 |
| Compare Register Immediate | RI | CRI | R(N),I |
| Exclusive OR Character Regerister | RR | XCR | R1(N1),R2(N2) |
| Exclusive OR Halfword Register | RR | XHR | R1,R2 |
| Exclusive OR Register | RR | XR | R1,R2 |
| Exclusive OR Register Immediate | RI | XRI | R(N),I |
| Exit | EXIT | EXIT | --------- |
| Input | RE | IN | R,E |
| Insert Character | RS | IC | R(N),D(B) |
| Insert Character and Count | RSA | ICT | R(N),B |
| Load | RS | L | R,D(B) |
| Load Address | RA | LA | R,A |
| Load Character Register | RR | LCR | R1,R2 |
| Load Character Register with Offset Reg. | RR | LCOR | R1,R2 |
| Load Halfword | RS | LH | R,D(B) |
| Load Halfword Register | RR | LHR | R1,R2 |
| Load Halfword with Offset Reg. | RR | LHOR | R1,R2 |
| Load Register | RR | LR | R1,R2 |
| Load Register Immediate | RI | LRI | R(N),I |
| Load with Offset Register | RR | LOR | R1,R2 |
| OR Character Register | RR | OCR | R1(N1),R2(N2) |
| OR Halfword Register | RR | OHR | R1,R2 |
| OR Register | RR | OR | R1,R2 |
| OR Register Immediate | RI | ORI | R(N),I |
| Output | RE | OUT | R,E |
| Store | RS | ST | R,D(B) |
| Store Character | RS | STC | R(N),D(B) |
| Store Character and Count | RSA | STCT | R(N),B |
| Store Halfword | RS | STH | R,D(B) |
| Subtract Character Register | RR | SCR | R1(N1),R2(N2) |
| Subtract Halfword Register | RR | SHR | R1,R2 |
| Subtract Register | RR | SR | R1,R2 |
| Subtract Register Immediate | RI | SRI | R(N),I |
| Test Register Under Mask | RI | TRM | R(N),I |

Figure 4-2. Instruction Format

IBM 3704 and 3705 Communications Controllers Principles of Operation  4-3

Rl Field: The Rl field in RR format instructions specifies the general register containing the first operand. For byte operations, the register specified by this field must be an odd-numbered register. For all other operations, any one of the eight general registers in the active group may be specified.

R2 Field: The R2 field in the RR format instructions specifies the general register containing the second operand. For byte operations, the register specified by this field must be an odd-numbered register. For all other operations, any one of the eight general registers in the active group may be specified.

N Field: Except for ARI, SRI, and BCT instructions, the N field in RS, Rl, and RT format instructions specifies whether byte 0 (N=0) or byte 1 (N=1) of the register specified in the R field is to be treated as the eight-bit first operand. For ARI, SRI and BCT instructions, the N field specifies whether byte 0 (N=0), or both byte 0 and byte 1 (N= 1) are to be treated as the operand.

Programming Note

With Extended Addressing, byte X is also part of the first operand in an ARI or SRI instruction.

N1 Field: Except for ACR and SCR instructions, the N1 field in RR format instructions specifies whether byte 0 (N1=0) or byte 1 (N1=1) of the register specified in the Rl field is to be treated as the eight-bit operand. For ACR and SCR instructions, the N1 field specifies whether byte 0 (N1 =0), or both byte 0 and byte 1 (N1=1) are to be treated as the operand.

Programming Note

With Extended Addressing, byte X is also part of the

first operand in an ACR or SCR instruction.

N2 Field: The N2 field in RR format instructions specifies whether byte 0 (N2=0) or byte 1 (N2=1) of the register specified in the R2 field is to be treated as the eight-bit operand.

M Field: The binary value of the three-bit M field in the Branch on Bit instruction (BB) specifies the bit (bit 0-7) to be tested in the byte operand selected by the R and N fields of the instruction.

##### Explicitly Addressed Operands in Storage

B Field: The B field in RS and RSA format instructions specifies a general register in the active group that contains a base address. The base address is used to address second operand locations in storage. For RS format instructions, the storage address is formed by the addition of the base address (contained in the register specified by the B field) and a positive displacement specified in the D field of the instruction.

For ICT and STCT (RSA format) instructions, the base address contained in the register specified by the B field is used without modification for the storage address.

For RS format instructions, if the 3-bit B field is zero, a fixed address constant is used for the base address instead of the contents of register 0. This permits direct access to system parameters at preassigned storage locations. The address constants for the various instructions are:

| Instruction | Address Constant |
| --- | --- |
| IC and STC | X'0680' |
| LH and STH | X'0700' |
| L and ST | X'0780' |

D Field: The D field in RS format instructions contains the displacement that is added to the base address to form the storage address of the second operand. The D field is treated as a positive binary number that represents a byte displacement for 1C and STC instructions, a halfword displacement for LH and STH instructions, and a fullword displacement for L and ST instructions. The displacement ranges that can be specified are:

| Instruction | Displacement |
| --- | --- |
| 1C and STC | 0 to 127 bytes |
| LH and STH | 0 to 126 bytes in multiples of two |
| L and ST | 0 to 124 bytes in multiples of four |

T Field: The T field in the RT format instructions is treated as a signed binary number. The number represents a halfword displacement from the address in register 0 of the active group of registers. Before the RT format instructions are executed, register 0 is incremented to point to the next sequential instruction (as is done for all instructions). This means that the displacement is with respect to the address of the next sequential instruction after the branch instruction. The low-order bit of the T field (instruction bit 15) is used as the sign bit. When this bit is zero, the displacement is positive. If the bit is a one, the displacement is negative. Thus, the following displacement

4-4

ranges are allowed in the formation of the branch address:

| Instruction | T Field range in halfwords | Displacement from Branch instruction in halfwords |
| --- | --- | --- |
| BCL,BZL,B | +1023 to-1023 | +1024 to-1022 |
| BB, BCT | +63 to -63 | +64 to -62 |

Condition Latches

A condition latch is a hardware latch that may be set or reset by instruction execution. Each of the five program levels has its own set of two condition latches. These condition latches are designated C and Z. The results of many instructions set the C and Z latches of the active group to 1 or 0. The states of these latches are described in the following instruction description sections. These condition latches can be inspected for decision making by Branch on C Latch (BCL) and Branch on Z Latch (BZL) instructions. A branch instruction never alters a condition latch.However, the program level 5 condition latches can be altered by executing an Output X'79' instruction. The state of these latches may be used as input via an Input X'79' instruction.

Programming Note

Since there is a separate set of C and Z condition latches for each of the five program levels, the state of the condition latches used by an interrupted program is not affected by other interrupting programs.

General Register Usage

Any register in the active group may be specified as containing an operand. However, in a byte operation, only the odd-numbered registers (that is, 1, 3, 5 and 7) in the active group may be specified as containing the byte operand. This is because only a two-bit field is allocated within the machine bit structure of the instruction to specify the register. Therefore, when the hardware decodes an instruction that contains a byte operation, a low-order one bit is added to access an odd-numbered register.

When a general register is specified for a halfword operation, byte 0 and byte 1 of the register are treated as a contiguous 16-bit binary operand. With Extended Addressing, byte X of the register is not used in a half-word operation and does not affect the setting of the condition latches.

When a general register is specified for an 18-bit operation (with Extended Addressing), byte X, byte 0,

and byte 1 of the register are treated as a contiguous 18-bit binary operand.

When a general register is specified for a single byte operand, either byte 0 or byte 1 of the register may be specified as the operand. The other bytes in the register do not affect the setting of the condition latches. For ARI, SRI, ACR, SCR, and BCT instructions, byte 0 or both byte 0 and byte 1 can be specified as the first operand in the general register to contain the result of the arithmetic operation. For example, for the Add Character Register (ACR) instruction, the operand in R2 is added to the operand in Rl, and the result is placed in Rl. Since Rl is to contain the result, byte 0 or both bytes 0 and 1 of Rl could be specified as an operand, whereas only a single byte (0 or 1) of R2 can be specified as an operand.

Programming Note

With Extended Addressing, byte X is included with the selected byte of the first operand in an ACR, ARI, SCR, or SRI instruction but does not affect the setting of the condition latches.

Instruction Descriptions

The following is a description of each of the 3704/3705 instructions.

BRANCH

B	T	[RT]

| 1 0 1 0 1 |  | T |  | ± |
| --- | --- | --- | --- | --- |
| 0 | 4 | 5 | 15 |  |

This instruction causes an unconditional branch to the branch address. The branch address is formed by adding the displacement value in the T field to the address of the next sequential instruction. Therefore, the branch address is always calculated relative to the next sequential instruction. The T field allows a displacement of +1023 to -1023 halfwords.

The low-order bit position (bit 15) of this instruction indicates whether the displacement is positive or negative. A 0 in bit 15 indicates that the displacement is in a positive direction, and a 1 indicates a negative direction.

Resulting Condition Latches: Unchanged

BRANCH ON C LATCH

BRANCH ON Z LATCH

BCL	T	[RT]

| 1 0 0 1 1 |  | T |  | ± |
| --- | --- | --- | --- | --- |
| 0 | 4 | 5 | 15 |  |

IBM 3704 and 3705 Communications Controllers Principles of Operation  4-5

BZL	T	[RT]

| 1 0 0 0 1 |  | T |  | ± |
| --- | --- | --- | --- | --- |
| 0 | 4 | 5 | 15 |  |

These instructions test the state of the C and Z condition latches associated with the active group of registers. If the tested latch is not set (0), the next sequential instruction is executed. If the tested latch is set (1), the next instruction to be executed is at the branch address. The branch address is formed by adding the displacement value in the T field to the address of next sequential instruction. Therefore, the branch address is always calculated relative to the next sequential instruction. The T field allows a displacement of +1023 to -1023 halfwords.

The low-order bit position (bit 15) of this instruction indicates whether the displacement is positive or negative. A 0 in bit 15 indicates that the displacement is in a positive direction, and a 1 indicates a negative direction.

Resulting Condition Latches: Unchanged

BRANCH ON BIT

BB	R(N,M),T	[RT]

| 1 1 M M 1 | R | N | M | T |  | ± |
| --- | --- | --- | --- | --- | --- | --- |
| 0 4 | 5 6 | 7 | 8 | 9 | 14 15 |  |

This instruction tests the state of a specified bit in a general register. If the bit tested is a zero, the next sequential instruction is executed. If the bit tested is a one, then the next instruction to be executed is at the branch address. The branch address is formed by adding the displacement value in the T field to the address of the next sequential instruction. Therefore, the branch address is always calculated relative to the next sequential instruction. The T field allows a displacement of -63 to +63 halfwords. The M field specifies which one of the eight bits of byte 0 (if N=0) or byte 1 (if N=1) of R is to be tested. The register specified by R must be an odd-numbered register.

The low-order bit position (bit 15) of this instruction indicates whether the displacement is positive or negative. A 0 in bit 15 indicates that the displacement is in a positive direction, and a 1 indicates a negative direction.

Resulting Condition Latches: Unchanged

BRANCH ON COUNT

BCT	R(N),T	[RT]

| 1 0 1 1 1 | R | N | 1 | T |  | ± |
| --- | --- | --- | --- | --- | --- | --- |
| 0 4 | 5 6 | 7 | 8 | 9 | 14 15 |  |

The count value in the register specified by R is decremented by one and then tested for zero. If the result is zero, the next sequential instruction is executed. If the result is nonzero, the next instruction to be executed is at the branch address. The count is contained in byte 0 (if N=0) or in both bytes 0 and 1 (if N= 1) of the register. The branch address is formed by adding the displacement value in the T field to the address of the next sequential instruction. Therefore, the branch address is always calculated relative to the next sequential instruction. The T field allows for a displacement of —63 to +63 halfwords. The register specified by R must be an odd-numbered register.

The low-order bit position (bit 15) of this instruction indicates whether the displacement is positive or negative. A 0 in bit 15 indicates that the displacement is in a positive direction, and a 1 indicates a negative direction.

Resulting Condition Latches: Unchanged

Programming Note

If before execution of this instruction, the count value (byte 0 or bytes 0 and 1) in the register is zero, the effective count value is 256 or 65,536, respectively.

BRANCH AND LINK

BAL	  R,A	  [RA]

| 1 0 1 1 1 | R | 0 0 0 0 0 0 | A |
| --- | --- | --- | --- |
| 0 4 | 5 7 | 8 13 | 14 31 |

This instruction is a 32-bit instruction that causes an unconditional branch. The address of the next sequential instruction is stored as link information in the register specified by R. Subsequently, the instruction address in register 0 is replaced by the branch address (address contained in the A field), and the branch is executed.

Resulting Condition Latches: Unchanged

4-6

#### Programming Notes

1. Since register 0 is the IAR, no linkage is provided if it is specified in the R field.

2. Bits 14 and 15 in the A field are used only with Extended Addressing.

BRANCH AND LINK REGISTER

BALR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | 0 1 0 0 0 0 0 0 |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The address of the next sequential instruction is stored as link information in the register specified by Rl. Subsequently, the instruction address in register 0 is replaced by the branch address (address in the register specified by R2), and the branch is executed. The branch address is obtained from R2 before the link information is stored in Rl.

Resulting Condition Latches: Unchanged

#### Programming Note

Since register 0 is the IAR, no linkage is provided if it is specified in the Rl field, and no branch occurs if it is specified in the R2 field.

ADD REGISTER

AR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’98’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The second operand (R2) is added to the first operand (Rl), and the sum is placed in the first operand location. Addition of all bits in the register operands is performed logically without regard to a sign, and the appropriate condition latches are set.

Resulting Condition Latches:

C An overflow occurred from Rl

Z The result in Rl = 0

##### Programming Notes

1. If register 0 is specified by Rl, a branch to the address formed in register 0 results, and the condition latches remain unchanged.

2. With Extended Addressing, this instruction operates on all 18 bit positions (bytes X, 0, and 1) of the registers. Without Extended Addressing, operation is the same as for the Add Halfword Register instruction.

ADD HALFWORD REGISTER

AHR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’90’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The second operand (R2, bytes 0 and 1) is added to the first operand (Rl, bytes 0 and 1), and the sum is placed in the first operand location. Addition of the register operands is performed logically without regard to a sign, and the appropriate condition latches are set.

Resulting Condition Latches:

C An overflow occurred from byte 0 of Rl

Z The result in bytes 0 and 1 of Rl = 0

Programming Note

If register 0 is specified as Rl, a branch results to the address formed in register 0, and the condition latches remain unchanged.

ADD CHARACTER REGISTER

ACR	 R1(N1),R2(N2)	  [RR]

| 0 | R2 | N2 |  | 0 | R1 | N1 |  | X’18’ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 2 |  | 3 | 4 | 5 6 |  | 7 | 8 15 |

Resulting Condition Latches:

N1=0

C An overflow occurred from byte 0 of Rl

Z The result in byte 0 of Rl = 0

N1 = 1

C An overflow occurred from bytes 0 and 1 of Rl

Z The result in bytes 0 and 1 of Rl =0

Programming Note

With Extended Addressing, the first operand includes byte X of the register specified by Rl. However, byte X does not affect the setting of the condition latches.

IBM 3704 and 3705 Communications Controllers Principles of Operation  4-7

ADD REGISTER IMMEDIATE

ARI	 R(N),I	  [RR]

| 1 0 0 1 0 | R | N | I |
| --- | --- | --- | --- |
| 0 4 | 5 6 | 7 | 8 15 |

The second operand (I field) is added to the first operand (byte 0 if N=0 or bytes 0 and 1 if N= 1 of the register specified by R). The sum is then placed in the first operand location. The register specified by R must be an odd-numbered register. If N=0, byte 1 of R remains unchanged.

Resulting Condition Latches:

N = 0

C An overflow occurred from byte 0 of R

Z The result in byte 0 of R = 0

N = 1

C An overflow occurred from bytes 0 and 1 of R

Z The result in bytes 0 and 1 of R = 0

Programming Note

With Extended Addressing, the first operand includes byte X of the register specified by R. However, byte X does not affect the setting of the condition latches.

SUBTRACT REGISTER

SR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’A8’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The second operand (R2) is subtracted from the first operand (Rl), and the result is placed in the first operand location.

Subtraction is performed by adding the two's complement of the second operand to the first operand and setting the appropriate condition latch. If the difference is less than zero, the result is in the two's complement form.

Resulting Condition Latches:

C The result in Rl < 0

Z The result in Rl = 0

#### Programming Notes

1. If register 0 is specified by Rl, a branch results to the address formed in register 0, and the condition latches remain unchanged.

2. With Extended Addressing, this instruction operates on all 18 bit positions (bytes X, 0, and 1) of the registers.

Without Extended Addressing, the opera tion is the same as for the Subtract Halfword Register instruction

SUBTRACT HALFWORD REGISTER

SHR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’A0’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The second operand (R2, bytes 0 and 1) is subtracted from the first operand (Rl, bytes 0 and 1), and the result is placed in the first operand location.

Subtraction is performed by adding the two's complement of the second operand to the first operand and setting the appropriate condition latch. If the difference is less than zero, the result is in the two's complement form.

Resulting Condition Latches:

C The result in bytes 0 and 1 of Rl < 0

Z The result in bytes 0 and 1 of Rl = 0

Programming Note

If register 0 is specified by Rl, a branch results to the address formed in register 0, and the condition latches remain unchanged.

SUBTRACT CHARACTER REGISTER

SCR	  R1(N1),R2(N2)	  [RR]

| 0 | R2 | N2 |  | 0 | R1 | N1 |  | X’28’ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 2 |  | 3 | 4 | 5 6 |  | 7 | 8 15 |

The second operand (R2, byte 0 if N2=0 or byte 1 if N2=1) is subtracted from the first operand (Rl, byte 0 if N1=0 or bytes 0 and 1 if N1 = 1). The result is placed into the first operand location. The registers specified by Rl and R2 must be odd-numbered registers.

Before the subtraction is performed, the second operand is expanded with high-order zeros to equal the size of the first operand. Subtraction is performed by adding the two's complement of the second operand to the first operand and setting the appropriate condition latch. If the difference is less than zero, the result is in the two's complement form.

Resulting Condition Latches:

N1=0

C The result in byte 0 of Rl < 0

Z The result in byte 0 of Rl = 0

4-8

N1 = 1

C The result in bytes 0 and 1 of Rl < 0

Z The result in bytes 0 and 1 of Rl = 0

Programming Note

With Extended Addressing, the first operand includes byte X of the register specified by Rl. However, byte X does not affect the setting of the condition latches.

SUBTRACT REGISTER IMMEDIATE

SRI	  R(N),I	  [RI]

| 1 0 1 0 0 | R | N | I |
| --- | --- | --- | --- |
| 0 4 | 5 6 | 7 | 8 15 |

The second operand (I field) is subtracted from the first operand (byte 0 if N=0 or bytes 0 and 1 if N= 1 of the register specified by R). The result is placed in the first operand location. The register specified by R must be an odd-numbered register.

Before the subtraction is performed, the second operand is expanded with high-order zeros to equal the size of the first operand. Subtraction is performed by adding the two's complement of the second operand to the first operand and setting the appropriate condition latch. If the difference is less than zero, the result is in the two's complement form.

Resulting Condition Latches:

N = 0

C The result in byte 0 of R < 0

Z The result in byte 0 of R = 0

N = 1

C The result in bytes 0 and 1 of R < 0

Z The result in bytes 0 and 1 of R = 0

Programming Note

With Extended Addressing, the first operand includes byte X of the register specified by R. However, byte X does not affect the setting of the condition latches.

INSERT CHARACTER

IC	 R(N),D(B)	  [RS]

| 0 | B | 1 | R | N | 0 | D |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 6 | 7 | 8 | 9 15 |

The eight-bit character at the second operand address is placed into byte 0 (if N=0) or byte 1 (if N= 1) of the register specified by R. The remaining bits of the register are unchanged. The storage address of the

second operand is formed by adding the displacement value in the D field to the contents of the base register specified by the B field. The D field allows for a displacement of 0 to +127 bytes. The register specified by R must be an odd-numbered register.

Resulting Condition Latches:

C The selected byte of R contains an even number of 1 bits

Z The selected byte of R = 0

#### Programming Note

If the B field is 0, address X'0680' is used as the base address instead of the contents of register 0. This permits direct addressing of the 128 bytes starting at address X'0680' without having to load a base register.

INSERT CHARACTER and COUNT

ICT	 R(N),B	  [RS]

| 0 | B | 0 | R | N | X’10’ |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 6 | 7 | 9 15 |

The eight-bit character at the second operand address is placed into byte 0 (if N=0) or byte 1 (if N=1) of the register specified by R. After the storage address has been obtained from the base register (B), the contents of the base register are incremented by 1 .Therefore, after the execution of this instruction, the register specified by the B field normally contains an address one byte greater than before execution. (See note 2 below.) The register specified by R must be an odd-numbered register.

Resulting Condition Latches: Unchanged

Programming Notes

1. Register 0 should not normally be specified in the B field, because it contains the instruction address.

2. If R and B specify the same (odd) register, its contents are incremented by one before the character is inserted into the selected byte of the register.

LOAD

L	 R,D(B)	  [RS]

| 0 | B | 0 | R | 0 | D | 1 0 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 | 9 13 | 14 15 |

IBM 3704 and 3705 Communications Controllers Principles of peration  4-9

This instruction loads the data (second operand) from a four-byte field in storage into the first operand (register specified by R). The four-byte field containing the second operand must be on a halfword boundary. Since the general registers are not a fullword (32 bits) in length, only the low-order bits of that storage location are used. (See note 4 below.) The storage address is formed by adding the displacement value in the D field to the contents of the base register specified by B. The D field allows for a displacement of 0 to 124 bytes in multiples of 4 (31 fullwords).

Resulting Condition Latches:

C The result in R ≠ 0

Z The result in R = 0

Programming Notes

1. The low-order bit of the storage address is ignored since storage is addressed on halfword boundaries with,this instruction.

2. If register 0 (IAR) is specified in the R field, this instruction results in an unconditional branch to the address loaded in register 0, and the condition latches remain unchanged.

3. If the B field is 0, address X'0780' is used as the base address instead of the contents of register 0. This permits direct addressing of the 32 fullwords starting at address X'0780' without having to load a base register.

4. With Extended Addressing, this instruction loads the 18 low-order bits from the four-byte field addressed by the second operand into bytes X, 0, and 1 of R. The 14 high-order bits in storage are ignored. Without Extended Addressing, the low-order 16 bits of the addressed four-byte field are loaded into bytes 0 and 1 of R, and the 16 high-order bits in storage are ignored.

LOAD HALFWORD REGISTER

LHR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’80’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

This instruction loads a halfword from storage into bytes 0 and 1 of the register specified by R. The storage address is formed by adding the displacement value in the D field to the contents of the base register (B). The D field allows for a displacement of 0 to 126 bytes in multiples of 2 (63 halfwords).

Resulting Condition Latches:

C The result in bytes 0 and 1 of R ≠ 0

Z The result in bytes 0 and 1 of R = 0

Programming Notes

1. For all Load Halfword instructions, the halfword obtained from storage is loaded into both the specified general register and the old-CRC register. See Cyclic Redundancy Check in Chapter 5. For normal operations (non-CRC), the loading of data into the old-CRC register serves no function.

2. The low-order bit of the storage address is ignored since storage is addressed on halfword boundaries with this instruction.

3. If register 0 (IAR) is specified in the R field, this instruction results in an unconditional branch to the address formed in register 0, and the condition latches remain unchanged.

4. If the B field is 0, address X'0700' is used as the base address instead of the contents of register 0. This permits direct addressing of the 64 halfwords starting at address X'0700' without having to load a base register.

5. With Extended Addressing, byte X of the register specified by R is set to zero during the load operation.

LOAD REGISTER

LR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’88’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The second operand (R2) is loaded into the first operand (Rl). All bits of the register specified by R2 are moved into the register specified by Rl and are not changed. Condition latches are set according to the result in the first operand.

Resulting Condition Latches:

C The result in R1 ≠ 0

Z The result in Rl = 0

Programming Notes

1. If register 0 is specified by Rl, a branch results to the address formed in register 0, and the condition latches remain unchanged.

2. With Extended Addressing, this instruction operates on all 18 bit positions (bytes X, 0, and 1) of the registers. Without Extended Addressing, operation

4-10

is the same as for the Load Halfword Register instruction.

LOAD HALFWORD REGISTER

LHR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’80’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The second operand (R2, bytes 0 and 1) is loaded into the first operand (Rl, bytes 0 and 1). The second operand is not changed, and the condition latches are set according to the result of the first operand.

Resulting Condition Latches:

C The result in bytes 0 and 1 of R1 ≠ 0

Z The result in bytes 0 and 1 of Rl = 0

Programming Note

If register 0 is specified by Rl, a branch results to the address formed in register 0, and the condition latches remain unchanged.

LOAD CHARACTER REGISTER

LCR	  R1(N1),R2(N2)	  [RR]

| 0 | R2 | N2 |  | 0 | R1 | N1 |  | X’08’ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 2 |  | 3 | 4 | 5 6 |  | 7 | 8 15 |

The second operand (R2, byte 0 if N2=0 or byte 1 if N2=1) is loaded into the first operand (Rl, byte 0 if N1=0 or byte 1 if N1=1). The registers specified by Rl and R2 must be odd-numbered registers.

Resulting Condition Latches:

C The selected byte of Rl contains an even number of 1 bits

Z The selected byte of Rl = 0

LOAD REGISTER IMMEDIATE

LRI	  R(N),I	  [RI]

| 1 0 0 0 0 | R | N | I |
| --- | --- | --- | --- |
| 0 4 | 5 6 | 7 | 8 15 |

The second operand (I field) is loaded into the first operand (byte 0 if N=0 or byte 1 if N= 1 of the register specified by R). The non-selected byte(s) of the register remain unchanged. The register specified by R must be an odd-numbered register.

Resulting Condition Latches:

C The result in the selected byte of R ≠ 0

Z The result in the selected byte of R = 0

LOAD ADDRESS

LA	R,A	[RA]

| 1 0 1 1 1 | R | 0 0 1 0 0 0 | A |
| --- | --- | --- | --- |
| 0 4 | 5 7 | 8 13 | 14 31 |

This instruction is a 32-bit instruction. The second operand (A field) is treated as an immediate operand and is loaded into the first operand (R).

Resulting Condition Latches: Unchanged

Programming Notes

1. With Extended Addressing, instruction bits 14 and 15 in the A field are loaded into byte X of R. Without Extended Addressing, these bits are ignored.

2. If register 0 is specified by R, a branch results to the address contained in the A field.

LOAD with OFFSET REGISTER

LOR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’F8’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The second operand (R2) is shifted right one bit position, and the result is loaded into the first operand (Rl). A zero bit is inserted in the high-order bit position of Rl.

Resulting Condition Latches:

C A 1 bit shifted out of byte 1, bit 7

Z The result in Rl = 0

Programming Notes

1. If register 0 is specified by Rl, a branch results to the address formed in register 0, and the condition latches remain unchanged.

2. With Extended Addressing, this instruction operates on all 18 bit positions (bytes X, 0, and 1) of the registers. Without Extended Addressing, the operation is the same as the Load Halfword with Offset Register instruction.

IBM 3704 and 3705 Communications Controllers Principles of Operation  4-11

LOAD HALFWORD with OFFSET REGISTER

LHOR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’F0’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The second operand (R2, bytes 0 and 1) is shifted right one bit position, and the result is loaded into the first operand (Rl, bytes 0 and 1). A zero bit is inserted into the high-order bit position of Rl, byte 0.

Resulting Condition Latches:

C A 1 bit shifted out of byte 1, bit 7

Z The result in bytes 0 and 1 of Rl = 0

Programming Note

If register 0 is specified by Rl, a branch results to the address formed in register 0, and the condition latches remain unchanged.

LOAD CHARACTER with OFFSET REGISTER

LCR	  R1(N1),R2(N2)	  [RR]

| 0 | R2 | N2 |  | 0 | R1 | N1 |  | X’78’ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 2 |  | 3 | 4 | 5 6 |  | 7 | 8 15 |

The second operand (R2, byte 0 if N2=0 or byte 1 if N2=1) is shifted right one bit position, and the result is loaded into the first operand (Rl, byte 0 if N=0 or byte 1 if N= 1). A zero bit is inserted into the high-order bit position of the selected byte of Rl. The non-selected byte(s) of Rl remain unchanged. The registers specified by Rl and R2 must be odd-numbered registers.

Resulting Condition Latches:

C A 1 bit shifted out of the selected byte

Z The result in the selected byte of Rl =0

STORE

ST	 R,D(B)	  [RS]

| 0 | B | 0 | R | 1 | D | 1 0 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 | 9 13 | 14 15 |

This instruction stores the contents of the first operand (the register specified by R) into the second operand in storage. The address of the second operand must be on a halfword boundary. Since the general registers are not a fullword (32 bits) in length, only the low-

fected. (See note 5 below.) The storage address is formed by adding the displacement contained in the D field to the contents of the base register specified by the B field. The D field allows for a displacement of 0 to 124 bytes in multiples of 4 (31 fullwords).

Resulting Condition Latches: Unchanged

Programming Notes

1. The low-order bit of the storage address is ignored because storage is addressed on halfword boundaries with this instruction.

2. If the B field is 0, address X'0780' is used as the base address instead of the contents of register 0. This permits direct addressing of the 32 fullwords starting at address X'0780' without having to load a base register.

3. If the R field is 0, zeros are stored, instead of the contents of register 0 (with the exception of note 4 below).

4. A Store instruction must be located at storage location X'0010' so that when a program level 1 interrupt occurs, this instruction is the first to be executed at that level. The normal function of the store instruction is modified in this special case to permit storing the contents of register 0. See the programming note under Program Levels in Chapter 2.

5. With Extended Addressing, the contents of the register (bytes X, 0, and 1) are stored into the low-order 18 bits of the four-byte field addressed in storage. The 14 high-order bits remain unchanged. Without Extended Addressing, bytes 0 and 1 of the register are stored into the low-order 16 bits of the four-byte field in storage, and the 16 high-order bits remain unchanged.

STORE HALFWORD

STH	 R,D(B)	  [RS]

| 0 | B | 0 | R | 1 | D | 1 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 | 9 14 | 15 |

This instruction stores bytes 0 and 1 of the register specified by R into the second operand in storage. The storage address of the second operand is formed by adding the displacement value in the D field to the contents of the base register specified by B. The D field allows for a displacement of 0 to 126 bytes in multiples of 2 (63 halfwords).

Resulting Condition Latches: Unchanged

4-12

Programming Notes

1. The low-order bit of the storage address is ignored because storage is addressed on halfword boundaries with this instruction.

2. If the R field is zero, X'0000' is stored at the storage address instead of the contents of register 0.

3. If the B field is 0, address X'0700' is used as the base address instead of the contents of register 0. This permits direct addressing of the 64 halfwords starting at address X'0700' without having to load a base register.

STORE CHARACTER

STC	 R(N),D(B)	  [RS]

| 0 | B | 1 | R | N | 1 | D |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 6 | 7 | 8 | 9 15 |

The first operand (byte 0 if N=0 or byte 1 if N=1 of the register specified by R) is stored into the second operand in storage. The storage address of the second operand is formed by adding the displacement value specified by the D field to the contents of the base register specified by B. The D field allows for a displacement of 0 to 127 bytes. The register specified by R must be an odd-numbered register.

Resulting Condition Latches: Unchanged

Programming Note

If the B field is 0, address X'0680' is used as the base address instead of the contents of register 0. This permits direct addressing of the 128 bytes starting at address X'0680' without having to load a base register.

STORE CHARACTER and COUNT

STCT	 R(N),B	  [RS]

| 0 | B | 0 | R | N | X’30’ |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 6 | 7 | 9 15 |

The first operand is placed in the second operand in storage. The storage address of the second operand is contained in the register specified by the B field. The byte stored is byte 0 (if N=0) or byte 1 (if N= 1) of the register specified by R. After the storage address has been obtained from the base register (B field), the contents of the register are incremented by 1 .Therefore, at the completion of the execution of this instruction, the base register contains an address one byte greater than before execution. The register specified by R must be an odd-numbered register.

Resulting Condition Latches: Unchanged

#### Programming Notes

1. Register 0 should not normally be specified in the B field, because it contains the instruction address.

2. If R and B specify the same (odd) register, its contents are incremented by one before the selected byte of that register is stored.

COMPARE REGISTER

CR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’B8’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The second operand (R2) is compared to the first operand (Rl), and the result sets the appropriate condition latch. This instruction performs a logical compare without regard to a sign bit. All bits of each operand participate in the comparison. The contents of the registers are not changed.

Resulting Condition Latches:

C Value in Rl < value in R2

Z Value in Rl = value in R2

#### Programming Note

With Extended Addressing, this instruction operates on all 18 bit positions (bytes X, 0, and 1) of the registers. Without Extended Addressing, operation is the same as the Compare Halfword Register instruction.

COMPARE HALFWORD REGISTER

CHR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’B0’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The second operand (R2, bytes 0 and 1) is compared to the first operand (Rl, bytes 0 and 1), and the result sets the appropriate condition latch. This instruction performs a logical compare without regard to a sign bit. All bits of each operand participate in the comparison. The contents of the registers are not changed.

Resulting Condition Latches:

C Bytes 0 and 1 of Rl < bytes 0 and 1 of R2

Z Bytes 0 and 1 of Rl = bytes 0 and 1 of R2

COMPARE CHARACTER REGISTER

CCR	 R1(N1),R2(N2)	  [RR]

| 0 | R2 | N2 |  | 0 | R1 | N1 |  | X’38’ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 2 |  | 3 | 4 | 5 6 |  | 7 | 8 15 |

IBM 3704 and 3705 Communications Controllers Principles of Operation  4-13

The second operand (R2, byte 0 if N2=0 or byte 1 if N2=1), is compared with the first operand (Rl, byte 0 if N1=0 or byte 1 if N1=1), and the appropriate condition latch is set. This instruction performs a logical compare without regard to a sign bit. All bits of each operand participate in the comparison. The registers specified by Rl and R2 must be odd-numbered registers. The contents of the registers are not changed.

Resulting Condition Latches:

C The selected byte of Rl < the selected byte of R2

Z The selected byte of Rl = the selected byte of R2

COMPARE REGISTER IMMEDIATE

CRI	 R(N),I	  [RR]

| 1 0 1 1 0 | R | N | I |
| --- | --- | --- | --- |
| 0 4 | 5 6 | 7 | 8 15 |

The second operand (I field) is compared with the first operand (byte 0 if N=0 or byte 1 if N= 1 of the register specified by R), and the appropriate condition latch is set. This instruction performs a logical compare without regard to a sign bit. All bits of each operand participate in the comparison. The register specified by R must be an odd-numbered register, and its contents are not changed.

Resulting Condition Latches:

C The value in the selected byte of R < I

Z The value in the selected byte of R = I

AND REGISTER

NR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’E8’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The second operand (R2) is ANDed with the first operand (Rl), and the result is placed in the first operand location.

Operands are treated as unstructured logical quantities, and the connective AND is applied bit by bit. A bit position in the result is set to one if the corresponding bit positions in both operands contain a one; otherwise, the result bit is set to zero. All bits of each operand participate in the operation. Any value in the operands or result is valid.

Resulting Condition Latches:

C The result in Rl ≠ 0

Z The result in Rl =0

Programming Notes

1. If register 0 is specified by Rl, a branch results to the address formed in register 0, and the condition latches remain unchanged.

2. With Extended Addressing, this instruction operates on all 18 bit positions (bytes X, 0, and 1) of the registers. Without Extended Addressing, the operation is the same as for the And Halfword Register instruction.

AND HALFWORD REGISTER

NHR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’E0’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The second operand (R2, bytes 0 and 1) is ANDed with first operand (Rl, bytes 0 and 1), and the result is placed in the first operand location.

The operation is performed in the same manner as the AND Register instruction except that only the low-order 16 bits (bytes 0 and 1) of the registers are used.

Resulting Condition Latches:

C The result in bytes 0 and 1 of Rl ≠ 0

Z The result in bytes 0 and 1 of Rl = 0

Programming Note

If register 0 is specified by Rl, a branch results to the address formed in register 0, and the condition latches remain unchanged.

AND CHARACTER REGISTER

NR	 R1(N1),R2(N2)	  [RR]

| 0 | R2 | N2 |  | 0 | R1 | N1 |  | X’68’ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 2 |  | 3 | 4 | 5 6 |  | 7 | 8 15 |

The second operand (R2, byte 0 if N2=0 or byte 1 if N2= 1) is ANDed with the first operand (Rl, byte 0 if N1=0 or byte 1 if N1=1). The operation is performed in the same manner as the AND Register instruction, and the result is placed in the first operand location. The registers specified by Rl and R2 must be odd-numbered registers. The nonselected byte(s) of Rl remain unchanged.

4-14

Resulting Condition Latches:

C The result in the selected byte of R1 ≠ 0

Z The result in the selected byte of Rl =0

AND REGISTER IMMEDIATE

NRI	 R(N),I	  [RR]

| 1 1 1 0 0 | R | N | I |
| --- | --- | --- | --- |
| 0 4 | 5 6 | 7 | 8 15 |

The second operand (I field) is ANDed with the first operand (byte 0 if N=0 or byte 1 if N=1 of the register specified by R). The result is placed into the first operand location with the remaining byte(s) of the register unchanged. The operation is performed in the same manner as the AND Register instruction. The register specified by R must be an odd-numbered register.

Resulting Condition Latches:

C The result in the selected byte of R ≠ 0

Z The result in the selected byte of R = 0

OR REGISTER

OR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’D8’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The second operand (R2) is ORed with the first operand (Rl), and the result is placed in the first operand location. All bits of each operand participate in the operation.

Operands are treated as unstructured logical quantities, and the connective inclusive OR is applied bit by bit. A bit position in the result is set to one if the corresponding bit position in either one or both of the operands contains a one. Otherwise, the result bit is set to zero. Any value in the operands or result is valid.

Resulting Condition Latches:

C The result in Rl ≠ 0

Z The result in Rl = 0

Programming Notes

1. If register 0 is specified by Rl, a branch to the address formed in register 0 results, and the condition latches remain unchanged.

2. With Extended Addressing, this instruction operates on all 18 bit positions (bytes X, 0, and 1) of the registers. Without Extended Addressing, the operation is the same as the Or Halfword Register instruction.

OR HALFWORD REGISTER

OHR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’D0’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The second operand (R2, bytes 0 and 1) is ORed with the first operand (Rl, bytes 0 and 1), and the result is placed in the first operand location.

The operation is performed in the same manner as the OR Register instruction with the exception that only the low-order 16 bits (bytes 0 and 1) of the registers are used.

Resulting Condition Latches:

C The result in bytes 0 and 1 of Rl ≠ 0

Z The result in bytes 0 and 1 of Rl = 0

#### Programming Note

If register 0 is specified by Rl, a branch results to the address formed in register 0, and the condition latches remain unchanged.

OR CHARACTER REGISTER

OCR	 R1(N1),R2(N2)	  [RR]

| 0 | R2 | N2 |  | 0 | R1 | N1 |  | X’58’ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 2 |  | 3 | 4 | 5 6 |  | 7 | 8 15 |

The second operand (R2, byte 0 if N2=0 or byte 1 if N2= 1) is ORed with the first operand (Rl, byte 0 if N1=0 or byte 1 if N1=1). The operation is performed in the same manner as the OR Register instruction, and the result is placed in the first operand location. The registers specified by Rl and R2 must be odd-numbered registers. The nonselected byte(s) of Rl remain unchanged.

Resulting Condition Latches:

C The result in the selected byte of R1 ≠  0

Z The result in the selected byte of Rl = 0

IBM 3704 and 3705 Communications Controllers Principles of Operation  4-15

OR REGISTER IMMEDIATE

ORI	 R(N),I	  [RR]

| 1 1 0 1 0 | R | N | I |
| --- | --- | --- | --- |
| 0 4 | 5 6 | 7 | 8 15 |

The second operand (I field) is ORed with the first operand (byte 0 if N=0 or byte 1 if N=1 of the register specified by R). The result is placed into the first operand location with the remaining byte(s) of R unchanged. The operation is performed in the same manner as the OR Register instruction. The register specified by R must be an odd-numbered register.

Resulting Condition Latches:

C The result in the selected byte of R ≠ 0

Z The result in the selected byte of R = 0

EXCLUSIVE OR REGISTER

XR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’C8’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The second operand (R2) is exclusive ORed with the first operand (R1), and the result is placed in the first operand location. All bits of each operand participate in the operation.

Operands are treated as unstructured logical quantities, and the connective exclusive OR is applied bit by bit. A bit position in the result is set to one if the corresponding bit positions in the two operands are unlike; otherwise, the resulting bit is set to zero.

Resulting Condition Latches:

C The result in R1 ≠ 0

Z The result in R1 ≠  0

Programming Notes

1. If register 0 is specified by R1, a branch results to the address formed in register 0, and the condition latches remain unchanged.

2. With Extended Addressing, this instruction operates on all 18 bit positions (bytes X, 0, and 1) of the registers. Without Extended Addressing, the operation is the same as the Exclusive OR Halfword Register instruction.

EXCLUSIVE OR HALFWORD REGISTER

XHR	  R1,R2	  [RR]

| 0 | R2 | 0 | R1 | X’C0’ |
| --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 15 |

The second operand (R2, bytes 0 and 1) is exclusive ORed with the first operand (R1, bytes 0 and 1), and the result is placed in the first operand location.

The operation is performed in the same manner as the Exclusive OR Register instruction except that only the low-order 16 bits (bytes 0 and 1) of the registers are used.

Resulting Condition Latches:

C The result in bytes 0 and 1 of R1 ≠ 0

Z The result in bytes 0 and 1 of R1 = 0

##### Programming Note

If register 0 is specified by R1, a branch results to the address formed in register 0, and the condition latches remain unchanged.

EXCLUSIVE OR CHARACTER REGISTER

XCR	 R1(N1),R2(N2)	  [RR]

| 0 | R2 | N2 |  | 0 | R1 | N1 |  | X’48’ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 2 |  | 3 | 4 | 5 6 |  | 7 | 8 15 |

The second operand (R2, byte 0 if N1=0 or byte 1 if N2=1) is exclusive ORed with the first operand (R1, byte 0 if N1=0 or byte 1 if N1=1). The result is placed in the first operand location, and the appropriate condition latch is set. The operation is performed in the same manner as the Exclusive OR Register instruction. The registers specified by RI and R2 must be odd-numbered registers.

Resulting Condition Latches:

C The result in the selected byte of R1 ≠ 0

Z The result in the selected byte of R1 = 0

EXCLUSIVE OR REGISTER IMMEDIATE

XRI	 R(N),I	  [RR]

| 1 1 0 0 0 | R | N | I |
| --- | --- | --- | --- |
| 0 4 | 5 6 | 7 | 8 15 |

The second operand (I field) is exclusive ORed with the first operand (byte 0 if N=0 or byte 1 if N=1 of the register specified by R). The resulting byte is placed in the first operand location, and the appropriate condition latch is set. The operation is performed in the same manner as the Exclusive OR Register instruction. The register specified by R must be an odd-numbered register.

4-16

Resulting Condition Latches:

C The result in the selected byte of R ≠ 0

Z The result in the selected byte of R = 0

TEST REGISTER UNDER MASK

XRI	 R(N),I	  [RR]

| 1 1 1 1 0 | R | N | I |
| --- | --- | --- | --- |
| 0 4 | 5 6 | 7 | 8 15 |

The state of the first operand bits selected by a mask is used to set the appropriate condition latch. The byte of immediate data (I field) is used as an eight-bit mask. The bits of the mask are made to correspond one for one with the bits of the first operand (byte 0 if N=0 or byte 1 if N= 1 of the register specified by R). A mask bit of one indicates that the register bit is to be tested. When the mask bit is zero, the register bit is ignored. Testing is done by ANDing the selected byte of the register with the immediate operand. The contents of R are not altered. The register specified by R must be an odd-numbered register.

Resulting Condition Latches:

C The result ≠ 0

Z The result = 0

EXIT

EXIT	[EXIT]

| 1 0 1 1 1 0 0 0 | 0 1 0 0 0 0 0 0 |
| --- | --- |
| 0 7 | 8 15 |

The Exit instruction is used to exit from the active program level. The interrupt-priority logic then determines which group of general registers to select as the active group for the next program operation. If executed at program level 5, the level 4 supervisor call interrupt request (SVC L4) is set. Then the next instruction executed is normally the instruction at the starting address for program level 4. However, if other interrupt requests are present, the next instruction executed is the instruction at the starting address of the highest priority program level requesting an interrupt.

This instruction also resets the 'interrupt entered' latch for the program level that executes it.

Resulting Condition Latches: Unchanged

INPUT

IN	R,E	[RE]

| 0 | E | 0 | R | E | 1 1 0 0 |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 11 | 12 15 |

This instruction loads the register specified by R with the contents of one of 128 input-addressable external registers, specified by the E field. Throughout this text the input instructions are referred to in the form: Input X'nn' where nn is the hexadecimal address of the external register. Appendix A shows the hexadecimal addresses of the external registers. The 32 general registers can also be addressed as external registers. Appendixes B and C show the bit definitions for the external registers.

Resulting Condition Latches: Unchanged

Programming Notes

1. If register 0 is specified by R, this instruction results in a branch to the address formed in register 0.

2. This instruction is a privileged instruction executable only at program levels 1, 2, 3 or 4. Any attempt to execute this instruction at program level 5 causes the level 1 input/output check interrupt request (In/Out Check L1) to be set.

3. The input/output check request is set when the instruction is executed at program level 1, 2, 3, or 4 if the external register address either is not assigned or is not recognized by any adapter or the CCU.

4. The input/output check request is set if incorrect parity is detected on the CCU inbus when an input instruction is executed. This can occur for some input instructions if they are executed at an improper time. See the individual input instruction descriptions for details.

5. With Extended Addressing, byte X of the register specified by R is set to zeros with the following exceptions: (1) the E field is X'OO' through X'lF' (signifying a general register), (2) the E field is X'74' (lagging address register), or (3) the E field is X'71' (panel address/data entry digits).

OUTPUT

OUT	R,E	[RE]

| 0 | E | 0 | R | E | 0 1 0 0 |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 3 | 4 | 5 7 | 8 11 | 12 15 |

IBM 3704 and 3705 Communications Controllers Principles of Operation  4-17

This instruction loads one of 128 output-addressable external registers specified by the E field with the contents of the register specified by R. Throughout this text the output instructions are referred to in the form:

Output X’nn’, where nn is the hexadecimal address of the external register. Appendix A shows the hexadecimal addresses of the external registers. The 32 general registers can also be addressed as external registers. Appendixes B and C show the bit definitions for the external registers.

Resulting Condition Latches: Unchanged

Programming Notes

1. If register 0 of the active group of general registers is addressed as the external register, this instruction results in a branch to the address formed in register 0.

2. This instruction is a privileged instruction executable at program levels 1, 2, 3 or 4 only. Any attempt to execute this instruction at program level 5 causes the level 1 input/output check interrupt request (In/Out Check L1) to be set.

3. The input/output check request is set when this instruction is executed at program levels 1, 2, 3 or 4 and if the external register address either is not assigned or is not recognized by the CCU or any adapter.

4. If the R field is 0, and an external register from X'00' through X'lF' is specified in the E field, the parity bits of the external register are regenerated, but the data is not changed.

5. If any output instruction is executed at program levels 1, 2, 3, or 4, the output also causes the CRC data register in the CCU to be loaded with the contents of byte 1 of the register specified by R. See Cyclic Redundancy Check in Chapter 5.

6. With Extended Addressing, byte X of the register specified by R is ignored with the following exceptions: (1) the E field specifies an external register from X'00' through X'lF' (signifying a general register) or (2) the E field is X'71' or X'72' (display register 1 or 2).

4-18