# Preface

This reference publication contains the hardware operation and programming requirements of the IBM 3704 and the IBM 3705 Communications Controllers. The information applies to the IBM 3704, IBM 3705-I and 3705-II except where specifically noted.

The publication is intended for any user of a communications controller who is attempting to write or modify a 3704/3705 control program. The reader should have an understanding of basic teleprocessing operations and a thorough knowledge of IBM System/360 and System/370 channel operations. The Introduction to the IBM 3704 and 3705 Communications Controllers (GA27-3051) is a prerequisite publication. Related publications are the IBM 3705 Communications Controller Assembler Language manual (GC30-3003), the Synchronous Data Link Control General Information manual (GA27-3093), the Guide to Using the IBM 3704 Communications Controller Control Panel (GA27-3086), and the Guide to Using the IBM 3705 Communications Controller Control Panel (GA27-3087). Other IBM publications concerning the communications controllers are identified and described in the IBM System/360 and System/370 Bibliography, GA22-6822.

This manual is divided into eleven chapters and three appendixes.

Chapter 1: Introduction

Provides a general description of the 3704/3705 and their functional units

Chapter 2: System Structure

Describes the registers, interrupt scheme, and levels

of the control program.

Chapter 3: Storage and Line Addressing

Describes the basic storage addressing procedure

and the format for addressing the individual communication lines.

Chapter 4: Instruction Set

Describes each of the 3704/3705 machine instructions with their format and condition codes

Chapters 5 through 10:

Describe the operation and programming requirements of the Central Control Unit, the Types 1,2,

and 3 Communication Scanners, and the Types 1,

2, 3, and 4 Channel Adapters.

Chapter 11: Remote Communications Controller

Describes the hardware and programming required

to support a communications controller at a remote location.

The appendixes contain (A) External register functions, (B) Input/Output instruction bit definitions, and (C) Input/Output instruction summary.

Fifth Edition (June 1976)

This is a major revision of and makes obsolete GC30-3004-3. The Summary of Amendments section describes the changes made in this edition.

Changes are periodically made to the information herein; before using this publication in connection with the operation of IBM systems, consult the latest IBM System! 370 Bibliography (GC20-0001) and associated Technical Newsletters for the editions that are applicable and current.

Requests for copies of IBM publications should be made to your IBM representative or to the IBM branch office serving your locality.

This manual has been prepared by the IBM System Communications Division, Publications Center, Department E01, P.O. Box 12195, Research Triangle Park, North Carolina 27709. A  form for reader's comments is provided at the back of this publication. If the form has been removed, comments may be sent to the above address. Comments become the property of IBM.

© Copyright International Business Machines Corporation 1972, 1973, 1974, 1976

# Contents

Chapter 1: Introduction ...........	1-1

Host System Interface ............	1-1

System Support Programs ...........	1-1

3704 Description .............	1-1

3705 Description .............	1-1

Storage ................	1-3

Central Control Unit ............	1-3

Adapters ................	1-3

Channel Adapters ............	1-3

Two-Channel Switch Feature .........	1-8

Communication Scanners ..........	1-8

Attachment Bases ............	1-8

Line Interface Bases and Line Sets ........	1-9

Remote Communications Controller ........	1-9

Remote Program Loader ..........	1-9

SDLC ................	1-9

General Programming Concepts .........	1-9

System Data Flow ............	1-10

Input/Output Instructions ..........	1-10

Chapter 2: System Structure	2-1

Registers ................	2-1

General Registers ............	2-1

Instruction Address Register .........	2-1

External Registers ............	2-2

Program Levels ..............	2-2

Interrupts ...............	2-2

Chapter 3: Storage and Line Addressing	3-1

Storage Addressing .............	3-1

Extended Addressing ...........	3-1

Interface Addressing ............	3-2

LIB and Line Addressing ...........	3-2

Chapter 4: Instruction Set ..........	4-1

Instruction Format .............	4-1

Instruction Operand Fields ..........	4-1

Immediate Operands ...........	4-1

External Register Operands .........	4-1

General Register Operands ..........	4-1

Explicitly Addressed Operands in Storage .....	4-4

Condition Latches .............	4-5

General Register Usage ............	4-5

Instruction Descriptions ...........	4-5

Chapter 5: Central Control Unit .........	5-1

CCU Registers ..............	5-1

CRC Register .............	5-1

CRC Data Register ............	5-1

Display Register 1 (DR1) ..........	5-1

Display Register 2 (DR2) ..........	5-1

Lagging Address Register (LAR) ........	5-1

Old-CRC Register ............	5-2

Operation Register (Op Reg) .........	5-2

Stroage Address Register (SAR) ........	5-2

Temporary Address Register (TAR) .......	5-2

Setting/Resetting Interrupt Requests ........	5-2

Determining the Cause of an Interrupt ...'.....	5-2

Masking Program Level Priorities .........	5-3

Storage Protect ..............	5-3

Interval Timer ..............	5-4

Cyclic Redundancy Check (CRC) .........	5-5

Initial Program Load (IPL) . ..........	5-5

Check Conditions During IPL .........	5-7

Input/Output Instructions ...........	5-7

Input Instructions ............	5-7

Output Instructions ............	5-8

CCU Checks ...............	5-9

Program Checks ..............	5-9

In/Out Check .............	5-9

Protection Exception ...........	5-9

Invalid Op-Code Check . ..........	5-10

Address Exception ............	5-10

CCU Diagnostic Facilities ...........	5-10

Chapter 6: Type 1 Communication Scanner .....	6-1

Operation and Data Flow ...........	6-1

Interface Scanning .............	6-2

Bit Service Priority ............	6-2

Interrupt Requests .............	6-3

Bit Service Interrupt Request .........	6-3

Character Service Interrupt Request .......	6-3

Scanner and Interface Initialization ........	6-5

Programming Considerations ..........	6-5

Interface Modes of Operation ..........	6-5

Business Machine Clocks ...........	6-6

Autocall Interface Operation ..........	6-7

Input/Output Instructions ...........	6-7

Input Instructions ............	6-7

Output Instructions............	6-8

Error Indications .............	6-9

Level 1 Errors .............	6-9

Interface Errors .............	6-9

Diagnostic Functions ............	6-10

Diagnostic Bit Service ...........	6-10

Diagnostic Wrap Mode ...........	6-10

Modem Wrap Test ............	6-10

Chapter 7: Type 2 Communication Scanner	7-1

Operation and Data Flow ...........	7-1

Type 2 Scanner Registers ...........	7-1

Local Storage .............	7-1

ICW Work Register ............	7-1

ICW Input Register ............	7-1

Attachment Buffer Address Register .......	7-2

Display Register .............	7-2

Type 2 Scanner Addressing ..........	7-3

Scan Addressing .............	7-3

Scan Counter .............	7-3

Upper Scan Limit ............	7-3

Address Substitution ...........	7-4

Program Addressing ............	7-5

IBM 3704 and 3705 Communications Controllers Principles of Operation   iii

Interface Control Word ............  	7-5

ICW Access .............. 	7-5

ICW Modification  ............ 	7-5

Interface Control Word Format ........ 	7-6

Line Control Definer (LCD) States  ....... 	7-9

Primary Control Field (PCF) States ....... 	7-10

Interface Mode Set-SDF Values ........ 	7-13

Business Machine Clocks ........... 	7-14

I/O Programming Considerations .........	 7-15

Program Level 1 (Error Routines) ........ 	7-15

Program Level 2 (Character Service) ....... 	7-15

Program Levels 3 and 4 (Lower Level Routines)   . . .	7-16

Autocall Interface Operation .......... 	7-16

Interface Control Word for Autocall Operation .  .  .  . 	7-16

Primary Control Field for Autocall Operation .  .  .  .	7-16

Serial Data Field for Autocall Operation .  .  .  .  .  .	7-17

Parallel Data Field for Autocall Operation .  .  .  .  .	7-18

Input/Output Instructions ...........	 7-18

Input Instructions ............ 	7-18

Output Instructions ............ 	7-19

SDLC ................. 	7-20

Line Control Definer (LCD) States for SDLC  .... 	7-21

Primary Control Field (PCF) States for SDLC .... 	7-22

Diagnostic Functions  ............ 	7-23

Diagnostic Wrap Test  ........... 	7-23

Modem Wrap Test ............ 	7-24

Chapter 8: Type 3 Communication Scanner   ..... 	8-1

Operation and Data Flow ........... 	8-1

Type 3 Scanner Registers ...........  	8-1

Local Storage   .............  	8-1

ICW Work Register ............	8-1

ICW Input Register ............ 	8-1

Attachment Buffer Address Register (ABAR) .... 	8-1

Display Register .............   	8-2

Scanner Output Register  ......... 	8-2

Scanner Check Register   ......... 	8-2

Diagnostic Buffer Address Register (DBAR) .... 	8-2

Cycle Steal Buffer Address Register (CBAR)   ... 	8-2

PDF Array Format ............ 	8-2

Ending the Cycle Steal Operation .......  	8-4

PDF-Full and PDF Empty Conditions  ..... 	8-4

Scanner Initialization  ..........  	8-4

PDF Array Reset   ........... 	8-5

Type 3 Scanner Addressing   .......... 	8-5

Scan Addressing .............  	8-5

Scan Counter   .............  	8-5

Upper Scan Limit ............ 	8-5

Address Substitution   ........... 	 8-5

High Speed Select Option..	8-6

Program Addressing............  	8-7

Interface Control Word (ICW) . ......... 	8-7

ICW Access .............. 	8-7

ICW Modification ............ 	8-10

Interface Control Word Format ........ 	8-10

Line Control Definer (LCD) States  ....... 	8-16

Primary Control Field (PCF) States ....... 	8-17

PCF/EPCF States-BSC   ......... 	8-17

PCF/EPCF States-SDLC ......... 	8-20

ICW Status Byte ............. 	8-23

ICW Status Indicators for BSC   ....... 	8-23

ICW Status Indicators for SDLC ....... 	8-25

Interface Mode Set-SDF Values ........ 	8-25

Business Machine Clocks 	 8-26

I/O Programming Considerations ......... 	8-27

Program Level 1 (Error Routines)    ....... 	8-27

Program Level 2 (Character Service) ....... 	8-27

Program Levels 3 and 4 (Lower Level Routines)   . . . 	8-28

Autocall Interface Operation .......... 	8-28

Interface Control Word for Autocall Operation .... 	8-28

PCF States for Autocall Operation   ....... 	8-28

SDF for Autocall Operation ......... 	8-29

PDF for Autocall Operation ......... 	8-30

Input/Output Instructions ........... 	8-30

Input Instructions ............ 	8-30

Output Instructions............ 	8-32

Diagnostic Functions  ............ 	8-34

Diagnostic Wrap Test  ........... 	8-34

Modem Wrap Test ............ 	8-35

ICW Diagnostic Mode Test   ......	8-36

Chapter 9: Type 1 and Type 4 Channel Adapters .... 	9-1

Operation and Data Flow ...........	 9-1

Type 1 and Type 4 CA Modes of Operation ......	 9-1

Channel Adapter States   ........... 	9-1

Initial Selection State ...........  	 9-1

Data Transfer State ............  	9-2

Status Transfer State   ...........  	9-2

Channel Adapter Registers ........... 	9-2

Initial Selection Control Register (ISCR)......  	9-2

Data/Status Control Register (DSCR)   ...... 	9-2

Initial Selection Address and Command Register

(IASCR)...	9-2

Initial Selection Status Register   ........   	9-2

NSC Status Register   ........... 	9-2

Error/Control/Condition Register ........  	9-2

Local Store Registers  ...........  	9-3

Extended-Buffer Mode Local Store (Type 4 CAOnly) ....	 9-3

Extended-Buffer Mode Control Register (Type 4
CA Only)  ......... 	9-3

Extended-Buffer Mode Counter Register ...... 	9-3

I/O Device Addresses  ............ 	9-3

Initial Selection Address   ..........  	9-4

Transfer Sequence Addresses ......... 	9-4

Channel Adapter Interrupt Requests ........ 	9-4

Level 1 Interrupt Request .......... 	9-4

Level 3 Interrupt Request .......... 	9-5

Channel Commands .............. 	9-6

Channel Adapter Status Presentation ........ 	9-7

Initial Selection Status ...........   	9-7

Final Status ..............  	9-7

Emulation Mode Test I/O .......... 	9-7

Native Mode Asynchronous Status  ....... 	9-8

Stacked Initial Status ...........  	9-8

Stacked Final Status   ...........  	9-9

Status Transfer Sequences ........... 	9-10

ESC Final Status   ............ 	9-10

NSC Final Status   ............ 	9-10

NSC Channel End Status .......... 	9-10

Data Transfer Sequences-Non-EB Mode ....... 	9-10

Outbound Data Transfer .......... 	9-10

Inbound Data Transfer    .......... 	9-11

Data Transfer Sequences-EB Mode (Type 4 CA) . . . .	9-11

Outbound Data Transfer .......... 	9-11

Inbound Data Transfer ........... 	9-11

iv

Input/Output Instructions ........... 	9-12

Input Instructions ............ 	9-12

Output Instructions .  ........... 	9-13

Error Indications   ............. 	9-15

Channel Bus In Check ........... 	9-15

Channel Bus Out Check in EB Mode ....... 	9-15

In/Out Instruction Accept Check ........ 	9-15

I/O Check on Input or Output X'6D' Instruction in

EBMode   .............. 	9-15

CCU Bus Out Check   ........... 	9-15

CCU Bus Out Check on Output X'6D' Instruction

in EBMode. ............. 	9-16

Local Store Parity Check .......... 	9-16

Chapter 10: Type 2 and Type 3 Channel Adapters   .	10-1

Operation and Data Flow ........... 	10-1

Cycle Steal Operation ............ 	10-2

Channel Adapter States   ........... 	10-2

Channel Adapter Registers ........... 	10-2

Inbound Data Control Word Address

Register (INCWAR) .....	10-3

Outbound Data Control Word Address

Register (OUTCWAR)  .......... 	10-3

Control Word Byte Count Register (CWCNTR) .  .	10-3

Channel Adapter Sense Register (CASNSR) . . . . .	10-3

Channel Adapter Status Register (CASTR) . . . . .	10-3

Channel Adapter Control Register (CACR) . . . . .	10-4

Channel Adapter Check Register (CACHKR)  . . . .	10-4

Channel Adapter Mode Register (CAMR)   ..... 	10-4

Channel Bus Out Diagnostic Register (CBODR) ... 	10-4

Cycle Steal Address Register (CSAR) ....... 	10-4

Channel Adapter Data Buffer (CADB)  ...... 	10-4

Channel Tag Diagnostic Register (CTDR)   ..... 	10-4

Channel Adapter Command Register (CMDR) .... 	10-4

Channel Adapter Control Word  ......... 	10-5

Control Word Command Codes ........ 	10-5

Control Word Conventions. ......... 	10-7

Channel Adapter Interrupt Requests ........ 	10-7

Channel Commands . ............ 	10-7

Write (X'O1)    .............. 	10-7

Write Break (X'09') ............ 	10-7

Write IPL(X'05')   ............ 	10-8

Read (X'02') .............. 	10-8

Sense (X'04')   ............. 	10-8

No-Op(X'03') ............. 	10-8

Test I/O (X'OO') ............. 	10-8

Nonstandard Commands .......... 	10-8

Status Servicing .............. 	10-9

Initial Status .............. 	10-9

Ending Status   ............. 	10-9

Stacked Status ............. 	10-9

Asynchronous Status .......... .	10-10

Multiple-Channel Operation  .........  .	10-10

Type 3 Channel Adapter-Channel Interface States . . .	10-10

Type 3 CA Response to Resets   ....... .	10-11

Input/Output Instructions ..........  .	10-11

Input Instructions .  .  .  .  .  .  .  .  .'. .  .  .	10-12

Output Instructions.  ..........  .	10-12

Error Indications   ......   ......  .	10-12

Chapter 11: Remote Communications Controller .... 	11-1

Local/Remote Communication Link ........ 	11-1

Remote Program Loader ........... 	11-1

Remote Program Loader Input Instructions . .  . . .	11-3

Remote Program Loader Output Instructions . . . .	11-3

Remote Program Loader CCU Instructions . . . . .	11-3

Disk Storage Controller   ........... 	11-3

Track Access.  ............. 	11-3

Track Format   ............. 	11-4

Disk Read and Write Operations .  .  .  .  .  .  .  .	11-4

Initial Program Load   ............ 	11-5

Remote Power Off ............. 	11-7

Appendix A-External Register Addresses  . 	A-l

Appendix B-Input/Output Instruction Bit Definitions   .  . 	B-l

Type 1 Scanner Input Instructions   ........ 	B-l

Type 1 Scanner Output Instructions ........ 	B-3

Type 2 Scanner Input Instructions   ........ 	B-6

Type 2 Scanner Output Instructions ........ 	B-8

Type 3 Scanner Input Instructions   ........ 	B-l 1

Type 3 Scanner Output Instructions ........ 	B-14

Type 1 and Type 4 CA Input Instructions ...... 	B-18

Type 1 and Type 4 CA Output Instructions . . . .. . 	B-21

Type 2 and Type 3 CA Input Instructions ...... 	B-25

Type 2 and Type 3 CA Output Instructions ...... 	B-30

Remote Program Loader Input Instructions ...... 	B-35

Remote Program Loader Output Instructions ..... 	B-35

CCU Input Instructions   ........... 	B-37

CCU Output Instructions ........... 	B-41

Appendix C - Input/Output Instruction Summary Charts .. 	C-l

Glossary ............... 	Glos-1

Index ................. 	X-l

IBM 3704 and 3705 Communications Controllers Principles of Operation  v

Figures

1-1	Line Attachment Capacity and Storage Size by Model .	1-2

1-2	Maximum Hardware Configuration of the 3704

Communications Controller ........ 	1-4

1-3	Channel Adapter/Communication Scanner

Combinations ............ 	1-5

1-4	System Data Flow ...........	1-10

2-1	General Register Groups  ......... 	2-1

2-2	Program Levels ............ 	2-3

2-3	Interrupt Priority Example   ........ 	2-5

3-1	Storage Address Bit Positions
(Without Extended Addressing) ....... 	3-1

3-2	Storage Address Bit Positions
(With Extended Addressing) ........ 	3-1

3-3	Type 1 Scanner Interface Address Bits ..... 	3-3

3-4	Types 2 and 3 Scanner Interface Address Bits ... 	3-3

3-5	Storage Address Assignments ........ 	3-4

4-1	Instruction Bit Structure ......... 	4-2

4-2	Instruction Format ...........	4-3

5-1	Operations that Set the Lagging Address Register .  . 	5-2

5-2	Protect Key Address Bits ......... 	5-4

5-3	Storage Key Addresses .......... 	5-4

6-1	Type 1 Communication Scanner Operation ....	6-1

6-2	Level 2 Program Flow (Type 1 Communication 
Scanner)   ............. 	6-2

6-3	Interfaces Serviced According to Priority   .... 	6-4

6-4	Business Machine Clocks and Warm Up Period .  .  . 	6-6

6-5	Type 1 Scanner Business Machine 
Clock Selection   ........... 	6-7

7-1	Type 2 Communication Scanner Operation ....	7-2

7-2	Upper Scan Limit ........... 	7-4

7-3	Address Substitution Control ........ 	7-4

7-4	Interface Control Word-Type 2 Scanner   ....	7-7

7-5	Type 2 Communication Scanner Business Machine 
Clock Selection   ...........	7-14

7-6	Setting ABAR and ICW Input Register .....	7-15

8-1	PDF Array Format ...........	8-3

8-2	Upper Scan Limit ...........	8-6

8-3	Address Substitution Control ........ 	8-6

8-4	High Speed Select Control ......... 	8-7

8-5	Interface Control Word-Type 3 Scanner   ....	8-8

8-6	Type 3 Communication Scanner Business Machine 
Clock Selection   ...........	8-26

8-7	Setting ABAR and ICW Input Register .....	8-27

10-1	Type 2 and Type 3 Channel Adapter Control Word   .	10-6

11-1	Teleprocessing System with Local and Remote 
Communications Controllers . ....... 	11-2

11-2	IPL-Remote Communications Controller . . . .	11-6

vi

Abbreviations

ABAR	attachment buffer address register	HIO	halt I/O

ACR	add character register instruction	IAR	instruction address register

ACR	abandon call and retry	1C	insert character instruction

ACU	automatic calling unit	ICT	insert character and count instruction

AHR	add halfword register instruction	ICW	interface control word

ALU	arithmetic logic unit	IN	input instruction

AR	add register instruction	INCWAR 	in-bound control word address register

ARI	add register immediate instruction	I/O	input/output

ATT	attention	IPL	initial program load

B	branch instruction	IPR	interrupt priority register

BAL	branch and link instruction	L	load instruction

BALR	branch and link register instruction	LA	load address instruction

BB	branch on bit instruction	LAR	lagging address register

BCL	branch on C latch instruction	LCD	line control identifier

BCT	branch on count instruction	LCOR	load character with offset

BSC	binary synchronous communication		register instruction

BZL	branch on Z latch instruction	LCR	load character register instruction

CA	channel adapter	LH	load halfword instruction

CACHKR	channel adapter check register	LHOR	load halfword with offset

CACR	channel adapter control register		register instruction

CADB	channel adapter data buffer or	LHR	load halfword register instruction

channel adapter diagnostic busy	LIB	line interface base

CAMR	channel adapter mode register	LOR	load with offset register instruction

CASNSR	channel adapter sense register	LR	load register instruction

CASTR	channel adapter status register	LRI	load register immediate instruction

CBODR	channel bus out diagnostic register	NCR	AND character register instruction

CCR	compare character register instruction	NHR	AND halfword register instruction

CCU	central control unit	NR	AND register instruction

CE	channel end	NRI	AND register immediate instruction

CHR	compare halfword register instruction	NSC	native subchannel

CMDR	channel adapter command register	ORC	OR character register instruction

COS	call originate status	OHR	OR halfword register instruction

CPU	central processing unit	OR	OR register instruction

CR	compare register instruction	ORI	OR register immediate instruction

CRC	cyclic redundancy check	OUT	output instruction

CRI	compare register immediate instruction 	OUTCWAR 	out-bound control word address register

CRQ	call request	PCF	primary control field

CSAR	cycle steal address register	PCI	program controlled interrupt

CTDR	channel tag diagnostic register	PDF	parallel data field

CTRL	control	PND	present next digit

CW	control word	RA	register and immediate address operation

CWAR	control word address register	RE	register and external register operation

CWCNTR	control word byte count register	RI	register and immediate operand operation

DBAR	diagnostic buffer address register	ROS	read-only-storage

DE	device end	RR	register to register operation

DLO	data line occupied	RS	register and storage operation

DPR	digit present	RSA	register and storage with count operation

DTR	data terminal ready	RT	branch operation

EB	extended-buffer (mode)	RTS	request to send

ESC	emulator subchannel	SAR	storage address register

EXIT	exit instruction	SCF	secondary control field

FDX	full duplex	SCR	subtract character register instruction

HDX	half duplex	SDF	serial data field

HEX	hexadecimal	SDLC	synchronous data link control

IBM 3704 and 3705 Communications Controllers Principles of Operation vii

SDR	storage data register 	TAR 	temporary address register

SHR	subtract halfword register instruction 	TIC	transfer in channel

SIO	start I/O 	TIO 	test I/O

SM	status modifier 	TRM	 test register under  mask instruction

SR	subtract register instruction 	UC 	unit check

SRI	subtract register immediate instruction 	UE	unit exception

ST	store instruction 	XCR 	exclusive OR  character register instruction

STC	store character instruction 	XHR 	exclusive OR  halfword register instruction

STCT	store character and count instruction 	XR 	exclusive OR  register instruction

STH	store halfword instruction 	XRI 	exclusive OR register immediate instruction

SVC	supervisor call

viii

Summary of Amendments—Fifth Edition

The fifth edition contains the following changes:

• The publication now applies to the 3705-II as well as the IBM 3704 and 3705-I. Distinctions between the three machines appear as appropriate.

• A new chapter—Chapter 8—describes the Type 3 Communication Scanner. Former Chapters 8,9, and 10 are renumbered to Chapters 9,10, and 11, respectively.

• Chapter 9 (formerly Chapter 8) now applies to the Type 4 Channel Adapter as well as the Type 1 Channel Adapter. Distinctions between the two types appear as appropriate.

• Appendixes B and C are updated to reflect the new Input and Output instructions and bit definitions for the Type 3 Communication Scanner and the Type 4 Channel Adapter.

In addition to the foregoing, several minor corrections and clarifications appear in this publication. Technical changes are denoted by a vertical line at the left of the revised text or illustration.

Summary of Amendments—Fourth Edition

The fourth edition contained the following changes.

Hardware Changes

The following changes to the manual are the result of new 3704 and 3705 hardware features and capabilities.

Remote Communications Controller

Information is now included on communications controllers that are at a location remote from the host processor. Chapter 10 deals exclusively with the remote communications controllers, and Appendixes A, B, and C describe the Input/Output instructions allocated to the remote program loader.

SDLC

Information on a new line control discipline. Synchronous Data Link Control (SDLC), is included in Chapter 7, and Input/Output instructions for SDLC are described in the appendixes.

3704 Type 2 Scanner Additional Capability

The 3704 with a Type 2 Communication Scanner can now support two line interface bases and up to 26 communication lines if the additional capability is selected. The additional capability is described in Chapter 7 and at other applicable places where the Type 2 Scanner is described.

## Manual Changes

The following additional changes have been made to the manual, but they do not reflect changes to the product.

Panel Information Removed

All information on the 3704 and 3705 control panels has been removed from this manual. Refer to the Guide to Using the IBM 3704 Communications Controller Control Panel and the Guide to Using the IBM 3705 Communications Controller Control Panel for information that was formerly in Chapter 10.

LIB and Line Set Information Removed

Appendix D, which described the line interface bases and line sets, has been removed. Refer to the Introduction to the IBM 3704 and 3705 Communications Controllers for this information.

TNL Incorporated

Technical Newsletter GN30-3024, which added information on the Type 3 Channel Adapter, has been incorporated into this edition.

Miscellaneous Changes

Numerous minor editorial and technical changes have been made in this revision. The index and table of contents have been updated to reflect all changes.

IBM 3704 and 3705 Communications Controllers Principles of Operation   ix