Chapter 3: Storage and Line Addressing

This chapter describes the storage addressing scheme and the format for addressing the individual communication lines.

### Storage Addressing

Byte locations in storage are consecutively numbered starting with 0; each number is considered the address of the corresponding byte. A group of bytes in storage is addressed by the high-order byte of the group. The number of bytes in the group is either implied or explicitly defined by the operation.

The basic addressing scheme uses a 16-bit binary address to accommodate a maximum byte address of 65,535. The two bytes of the halfword used for addressing are specified from left to right as byte 0 and byte 1. The bits within these two bytes are numbered left to right from 0 to 7. All general registers and the CCU external registers involved in addressing storage are two bytes in length and the bit positions correspond to the basic addressing scheme.

Figure 3-1 shows the storage address bit positions as they are used in the basic addressing scheme.

|  | Byte 0 | Byte 1 |
| --- | --- | --- |
| Bit pos | 01234567 | 01234567 |

Figure 3-1. Storage Address Bit Positions

(Without Extended Addressing)

Storage addressing wraps at the maximum byte address of 65,535. This means that if, in the formation of a storage address, the binary representation of the address is greater than 16 bits in length, the actual address used will be only the address formed in the low-order 16 bits. For example, if the formation of a storage address uses a base address of X'A080' and a displacement of X'6010', the combined address does not include the carry of the high-order bit and therefore generates an effective address of X'0090'.

An addressing exception is recognized if the storage address is greater than the number of installed storage locations but less than the point of storage wrap. For example, in the 3705-I controller, any attempt to address storage between 16,384 (or 49,152, whichever model is installed) and 65,535 results in an addressing exception. See Program Checks in Chapter 5.

Instructions and halfword operands must be located on integral halfword boundaries in storage. A boundary is called integral for a unit of information when its storage address is a multiple of the length of the unit in bytes. For example, a word (four bytes)

must be located in storage so that its address is a multiple of the number 4. A halfword (two bytes) must have an address that is a multiple of the number 2.

Storage addresses are expressed in binary form. In binary, integral boundaries for halfwords and words can be specified only by an address in which one or two low-order bits, respectively, are zero. For example, the integral boundary for a word is a binary address in which the two low-order positions are zero.

Extended Addressing

When a controller contains more than 64K bytes of storage, the basic 16-bit address structure is not sufficient. To address the storage positions above 64K, two additional address bits are required. These two bits, designated as byte X, bits 6 and 7, allow address generation up to the maximum of 245,760 bytes, for a 3705-I, or 262,144 bytes, for a 3705-II. This addition of byte X is referred to as Extended Addressing. Figure 3-2 illustrates the storage address bit positions used by the affected registers with Extended Addressing.

| Byte X |  |  | Byte 0 | Byte 1 |
| --- | --- | --- | --- | --- |
| Bit Pos | 67 |  | 01234567 | 01234567 |

Figure 3-2. Storage Address Bit Positions 
(With Extended Addressing)

With Extended Addressing, the Central Control Unit data flow registers and all general registers are expanded to 18 bits. The two additional bits (byte X, bits 6-7) must be handled as an integral part of the register regardless of the address being operated on. The only exceptions are: (1) byte X is ignored for output instructions not involved in addressing storage and can be set to either 1 or 0, and (2) byte X is set to zeros for input instructions not involved in addressing storage.

Storage wrap and addressing exception also apply to Extended Addressing. However, with Extended Addressing, the point of storage wrap is 262,144, and a carry from this value wraps back to zero. The range of addresses that cause an addressing exception is from the maximum number of storage locations installed to the point of storage wrap. For example, if the storage installed in a 3705-1 is 180,224 bytes, then any address generated between 180,224 and 262,144 causes an addressing exception. Addressing exceptions also apply to the 3705-II.

IBM 3704 and 3705 Communications Controllers Principles of Operation  3-1

Note: This manual is based primarily on the basic 16-bit addressing scheme. However, any exceptions in operation due to Extended Addressing are pointed out and explained by programming notes. The 3704 does not have the Extended Addressing capability.

Interface Addressing

The lines attached to a communication scanner are assigned interface addresses when the machine is installed. The interface address assigned to a given line is determined by the physical location of the line interface hardware and by the type of scanner (Type 1, Type 2, or Type 3) installed in the controller.

Sixteen interface addresses are assigned to each Line Interface Base position, permitting a total of 352 lines on the 3705 with the maximum configuration. To uniquely address each of these lines requires nine address bits. Figures 3-3 and 3-4 show the address bits and their interpretation. The S field specifies which scanner the interface is attached to and is used when one or more Type 2 or Type 3 Scanners are installed. The S field is not used with a Type 1 Scanner, since only one Type 1 Scanner can be installed in a controller. The L field specifies the Line Interface Base number within the specified scanner. Only two bits of the L field (bits 3 and 4) are used with a Type 1 Scanner since four is the maximum number of LIBs that can be installed in a Type 1 Scanner. The I field is the position of the interface attachment in the specified LIB.

Whether all 16 interface addresses assigned to a given LIB position are usable depends on the type of LIB installed in that LIB position, and the type and number of line sets installed in that LIB.

With a Type 1 Scanner: When an interface address is passed to the program by an Input X'41' instruction, the interface address bits 3-8 are placed in byte 0, bits 6-7, and byte 1, bits 0-3, of the general register specified in the instruction. Byte 0, bit 4 is set to 1, and the remaining bits are set to 0.

This alignment associates a specific 2-byte storage address with each interface address. These storage addresses point to consecutive 16-byte blocks in storage. This area of storage can be directly addressed and contains a control block for control information and data handling routine addresses

required for servicing an interface. Figure 3-5 shows the specific storage address associated with each interface position.

For consistency, when the program provides an interface address to the Type 1 Scanner via an Output X'47' instruction, the same alignment must be maintained.

With a Type 2 or 3 Scanner: When an interface address is passed to the program by an Input X'40' instruction, the interface address bits 0-8 are placed in byte 0, bits 6-7, and byte 1, bits 0-6, of the specified general register. Byte 0, bit 4 is set to 1 and the remaining bits of the register are set to 0.

This alignment associates a specific 2-byte storage address with each interface address. These storage addresses are aligned on halfword boundaries and are arranged so that this area in storage can be used as a vector table to direct the control program to a routine that is designed to service the particular interface type. Figure 3-5 shows the storage address for each specific interface position.

For consistency, when the program provides an interface address to the Type 2 or 3 Scanner via an Output X'40' instruction, the same alignment must be maintained.

Note: Interface addressing in the 3704 with a Type 2 Scanner follows the same addressing scheme as a 3705 Type 2 Scanner-1, LIB position 1. Interface address bits 4, 5, 6, 7, and 8 specify lines 0-F in LIB Type A 1. However, addresses 1, 3,C, D, E, and F are not used. If the scanner supports two LIBs (LIB positions 1 and 2), all interface addresses are used.

LIB and Line Addressing

Each line or autocall interface must be addressable for the following:

1. Scan addressing

2. Program addressing

Scan addressing and program addressing are performed differently for the Type 1, Type 2, and Type 3 Communication Scanners. These functions are described in detail in the communication scanner chapters.

3-2

| Byte 0 | Byte 1 |
| --- | --- |

External

|  | Register X’41’ | 0 | 1 | 2 |  | 3 |  |  | 4 |  | 5 |  | 6 |  | 7 |  | 0 |  | 1 |  | 2 |  | 3 |  | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit Positions | 0 | 0 | 0 |  | 0 |  |  | 1 |  | 0 |  | X |  | X |  | X |  | X |  | X |  | X |  | 0 | 0 | 0 | 0 |
| Field |  |  |  |  |  |  | S |  |  |  |  | L |  |  |  |  |  |  |  | I |  |  |  |  |  |  |  |  |
| Address Bits |  |  |  |  | 0 |  |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  |  |  |  |  |  |

00=LIB pos 1 01=LIB pos 2 10=LIB pos 3 11=LIB pos 4

Interface X'0' - X’F’

Not Used

Figure 3-3. Type 1 Scanner Interface Address Bits

| Byte 0 | Byte 1 |
| --- | --- |

External

|  | Register X’40’ | 0 | 1 | 2 | 3 | 4 | 5 | 6 |  | 7 | 0 | 1 | 2 |  | 3 | 4 |  | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit Positions | 0 | 0 | 0 | 0 | 1 | 0 | X |  | X | X | X | X |  | X | X |  | X | X | 0 |
| Field |  |  |  |  |  |  |  |  | S |  |  | L |  |  |  |  | I |  |  |  |
| Address Bits |  |  |  |  |  |  |  | 0 |  | 1 | 2 | 3 |  | 4 | 5 | 6 | 7 |  | 8 |  |

000=Not Used 001=Not Used 010=LIB pos 1 011=LIB pos2 100=LIB pos 3 101=LIB pos 4* 110= LIB pos 5** 111= LIB pos 6**

00=Scanner-1 01=Scanner-2 10=Scanner-3 11=Scanner-4

Interface X'0' - X’F’

*Invalid for Type 3 Scanner 1

**Invalid for any Type 3 Scanner

Figure 3-4. Types 2 and 3 Scanner Interface3 Address Bits

IBM 3704 and 3705 Communications Controller Principles of Operation  3-3

|  |  | INTERFACE ADOBESS ASSIGNMENTS (HEX) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | S/L |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | (HEX) I -» 0 |  | I | 2 | 3 | 4 | 5 | (, | 7 | Q | 9 | A | B | C | D | E | F |
|  |  | y |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | STORAGE ADDRESSES (HEX) • |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type 1 | LIB position 1 | 00 — | 800 | 810 | 820 | 830 | 840 | 850 | 860 | 870 | 880 | 890 | 8AO | 8BO | 8CO | 8DO | 8EO | 8FO |
| scanner | 2 | 01 — | 900 | 910 | 920 | 930 | 940 | 950 | 960 | 970 | 980 | 990 | 9AO | 9BO | 9CO | 9DO | 9EO | 9FO |
|  | 3 | 02 — | AOO | A10 | A20 | A30 | A40 | A50 | A60 | A70 | A80 | A90 | AAO | ABO | ACO | ADO | AEO | AFO |
|  | 4 | 03 — | BOO | B10 | B20 | B30 | B40 | 850 | B60 | 870 | 880 | 890 | BAO | 880 | 8CO | BDO | BEO | BFO |
| Type 2 | 3705 BASE MODULE |  | STORAGE ADDRESSES (HEX) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| or | L16 position 1 | 02 — | 840 | 842 | 844 | 846 | 848 | 84A | 84C | 84E | 850 | 852 | 854 | 856 | 858 | 85A | 85C | 85E |
| Type 3 | 2 | 03 — | 860 | 862 | 864 | 866 | 868 | 86A | 86C | 86E | 870 | 872 | 874 | 876 | 878 | 87A | 87C | 87E |
| Scanner-1 | 3 | 04 — | 880 | 882 | 884 | 886 | 888 | 88A | 88C | 88E | 890 | 892 | 894 | 896 | 898 | 89A | 89C | 89E |
|  | 4** | 05 — | 8AO | 8A2 | 8A4 | 8A6 | 8A8 | 8AA | 8AC | 8AE | 880 | 882 | 884 | 886 | 888 | 8BA | 8BC | 88E |
|  | 3705 EXPANSION MODULE 1 |  | STORAGE ADDRESSES (HEX) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| or | LIB position 1 | OA — | 940 | 942 | 944 | 946 | 948 | 94A | 94C | 94 E | 950 | 952 | 954 | 956 | 958 | 95A | 95C | 95E |
| Type 3 | 2 | 0B — | 960 | 962 | 964 | 966 | 968 | 96A | 96C | 96E | 970 | 972 | 974 | 976 | 978 | 97A | 97C | 97E |
| Scanner-2 | 3 | OC — | 980 | 982 | 984 | 986 | 988 | 98A | 98C | 98E | 990 | 992 | 994 | 996 | 998 | 99A | 99C | 99E |
|  | 4 | OD — | 9AO | 9A2 | 9A4 | 9A6 | 9A8 | 9AA | 9AC | 9AE | 980 | 982 | 984 | 9B6 | 988 | 98A | 98C | 98E |
|  | 5'* | OE — | 9CO | 9C2 | 9C4 | 9C6 | 9C8 | 9CA | 9CC | 9CE | 9DO | 9D2 | 9D4 | 9D6 | 9D8 | 9DA | 9DC | 9DE |
|  | 6** | OF — | 9EO | 9E2 | 9E4 | 9E6 | 9E8 | 9EA | 9EC | 9EE | 9FO | 9F2 | 9F4 | 9F6 | 9F8 | 9FA | 9FC | 9FE |
|  | 3705 EXPANSION MODULE 2 |  | STORAGE ADDRESSES (HEX) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type 2 | LIB position 1 | 12 — | A40 | A42 | A44 | A46 | A48 | A4A | A4C | A4E | A50 | A52 | A54 | A56 | A58 | A5A | A5C | A5E |
|  | 2 | 13 — | A60 | A62 | A44 | A66 | A68 | A6A | A6C | A6E | A70 | A72 | A74 | A76 | A78 | A7A | A7C | A7E |
| Type3 | 3 | 14 — | A80 | A82 | A84 | A86 | A88 | A8A | A8C | A8E | A90 | A92 | A94 | A96 | A98 | A9A | A9C | A9E |
| Scanner-3 | 4 | 15 — | AAO | AA2 | AA4 | AA6 | AA8 | AAA | AAC | AAE | ABO | A82 | AB4 | AB6 | AB8 | A8A | ABC | ABE |
|  | 5** | 16 — | AGO | AC2 | AC4 | AC6 | AC8 | ACA | ACC | ACE | ADO | AD2 | AD4 | AD6 | AD8 | ADA | ADC | ADE |
|  | 6** | 17 — | AEO | AE2 | AE4 | AE6 | AE8 | AEA | A EC | AEE | AFO | AF2 | AF4 | AF6 | AF8 | AFA | AFC | AFE |
|  | 3705 EXPANSION MODULE 3 |  | STORAGE ADDRESSES (HEX) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Type 2 | LIB position 1 | 1A — | B40 | 842 | B44 | B46 | 848 | 84A | B4C | B4E | B50 | 852 | B54 | 856 | 858 | B5A | B5C | B5E |
|  | 2 | 1B — | B60 | B62 | B64 | B66 | B68 | B6A | B6C | 86E | B70 | B72 | 874 | 876 | B78 | B7A | B7C | B7E |
| Type 3 | 3 | 1C — | 880 | B82 | B84 | 886 | 888 | 88A | B8C | 88E | 890 | 892 | B94 | B96 | 898 | B9A | 89C | B9E |
| Scanner-4 | 4 | ID — | BAD | BA2 | 8A4 | 8A6 | BA8 | BAA | 8AC | 8AE | BBO | B82 | 8B4 | BB6 | 888 | B8A | 8BC | BBE |
|  | 5** | IE — | BCO | 8C2 | BC4 | 8C6 | BC8 | 8CA | BCC | 8CE | 8 DO | BD2 | BD4 | 8D6 | 8D8 | BDA | 8DC | BDE |
|  | 6** | IF — | BEO | BE2 | 8E4 | BE6 | 8E8 | BEA | BEC | 8EE | BFO | BF2 | BF4 | 8F6 | BF8 | BFA | BFC | 8FE |

* Storage address X'6FO' is used for character service **Type 2 Scanner only

Figure 3-5. Storage Address Assignments

3-4