### Chapter 9: Type 1 and Type 4 Channel Adapters

This chapter gives the reader a basic understanding of the operation of the Type 1 and Type 4 Channel Adapters and the requirements necessary to program the adapter.

With the Type 1 or Type 4 Channel Adapter (CA), the communications controller establishes and maintains communications with the byte-multiplexer channel of an IBM System/360 or System/370. With the proper programming support, the Type 1 or Type 4 CA allows the controller to operate in a native 3704/3705 mode and/or in an emulation mode, emulating the 2701, 2702, or 2703 transmission control units.

The Type 1 and Type 4 CA rely heavily on the program to perform control operations. Data transfers across the channel interface occur in multibyte bursts with program intervention required before and after each burst. The Type 1 CA allows bursts of up to 4 bytes; the Type 4 CA allows bursts of up to 32 bytes (with extended-buffer mode of operation, as described below). Channel I/O command decoding and interpretation, data transfers between the channel adapter and storage, ending status generation, and various other functions must generally be performed by the program.

Because the Type 1 and Type 4 Channel Adapters are similar in operation in most respects, only the differences are identified as pertaining to one type or the other. Otherwise, the term channel adapter (or CA) refers to both types of adapter.

Operation and Data Flow

The channel adapter receives an address from the host processor and determines whether the host wants to communicate with the controller in native mode or emulation mode. The mode of operation (native or emulation) is then set and initial selection completed. The CA then requests an initial level 3 interrupt to make this information available to the control program via input instructions.

Data from the host channel interface is received into the data buffers where the control program must retrieve it by executing input instructions and placing the bytes in storage. Channel End and Device End are generated by the control program when the complete message or block of data is received.

When data transfer is from the channel adapter to the host channel, the control program may request an attention interrupt from the host processor. The host processor must initiate an initial selection sequence, and the control program must load the data buffers with the byte to be transferred. Data from the data buffers can then be transferred across the channel interface. Channel End and Device End are generated by the control program when the complete message or block of data has been sent.

Type 1 and Type 4 CA Modes of Operation

The CA has two modes of operation—Native Subchannel Mode (NSC) or Emulation Subchannel Mode (ESC). With the proper programming support, the CA allows the controller to operate in NSC and ESC modes concurrently or separately.

Native mode permits servicing any number of lines up to 352 (32 for a 3704), using only one host subchannel address. The line address decoding is handled by the control program. Initial Program Load must always be handled in NSC mode.

Emulation mode allows the controller to emulate the 2701,2702, and 2703 transmission control units using existing host programs and subchannel addresses.

Note: Most operations of the CA are identical whether in native or emulation mode. Throughout this chapter, the exceptions and/or differences in operation due to native or emulation mode are noted by an NSC or ESC heading on the paragraph that describes the particular operation. All text that is not designated as either NSC or ESC can be assumed to apply equally to both modes of operation.

Extended-Buffer Mode of Operation: The Type 4 Channel Adapter has an additional choice of operating modes:

extended-buffer (EB) mode or non-extended buffer (non-EB) mode.

Operation of the Type 4 CA in non-EB mode is identical to that of the Type 1 CA.

When operating in EB mode, the Type 4 CA can (with proper control program support) transfer from one to 32 bytes over the channel to or from the host processor before interrupting the control program to obtain more data. This contrasts with a maximum transfer of four bytes when the CA is operating in non-EB mode. A bit in the EB mode control register determines whether the CA is to operate in EB mode. An Output X'6C' instruction must be used to set or reset this bit.

Channel Adapter States

The CA operates in one of three active states under direction of the control program: (1) initial selection state, (2) data transfer state, or (3) status transfer state. When not active, the CA is in a ready mode; that is, the adapter can accept instructions but is not in one of the active states.

##### Initial Selection State

The initial selection state is entered when the host processor begins an initial selection sequence. The CA continually monitors the channel interface for one of its

IBM 3704 and 3705 Communications Controllers Principles of Operation  9-1

assigned addresses. If one of these addresses is detected, the CA traps this address and proceeds with the initial selection. If the command is received without error (correct parity), an initial status of all zeros is returned unless the command is a Test I/O or a No-Op. (See Channel Adapter Status Presentation in this chapter for Test I/O and No-Op exceptions.) If the command has bad parity, the CA returns Unit Check status to the host channel.

When the command from the host is a No-Op, the CA returns Channel End, Device End, and performs no further action.

During initial selection, the I/O device address and the command are stored in their respective channel adapter registers (see below). A level 3 interrupt is then requested by the initial selection hardware, and control is passed to the level 3 interrupt program.

##### Data Transfer State

The data transfer state is entered when the control program initiates a data transfer sequence. Data transfer can be from the host processor to the controller or from the controller to the host processor. The data is transferred across the channel under hardware control. When the data transfer ends, the channel adapter hardware signals the control program with a level 3 interrupt request.

##### Status Transfer State

The status transfer state is entered when the control program initiates a status transfer sequence. The status information byte is transferred to the host during this sequence.

Channel Adapter Registers

Various hardware registers are available to the control program for interface between the CA hardware and the levels 1 and 3 program routines. The following sections briefly describe these registers and the method of program access. See Appendix B for the bit definitions of the I/O instructions and the registers.

##### Initial Selection Control Register (ISCR)

This one-byte register contains information identifying the event that caused the CA Initial level 3 interrupt request. The register is available to the control program with an Input X'60' instruction.

##### Data/Status Control Register (DSCR)

This two-byte register contains control information used when passing data and status bytes across the

channel interface. The bits in this register are set by the control program (Output X'62') and the CA hardware as a result of either a channel data or status transfer. This register is available to the control program with an Input X'62' instruction.

##### Initial Selection Address and Command Register (ISACR)

This two-byte register contains the I/O device address and the command byte presented to the channel adapter during initial selection. This register is available to the control program with an Input X'61' instruction.

##### Initial Selection Status Register

This one-byte register contains the status byte generated and presented by the channel adapter hardware during initial selection sequences except under the following conditions.

NSC: The NSC status byte from the NSC status register is presented instead of the hardware generated status when (1) an initial selection sequence occurs for the native mode subchannel and (2) the NSC status byte provided by the control program has not been accepted by the host (as in the case of stacked status).

ESC: The ESC status byte provided by the control program is presented instead of the hardware generated status when (1) an initial selection sequence occurs for an emulation address (Test I/O initial selection and addresses compare) and (2) the control program has signaled that both an ESC status transfer sequence is required and that ESC Test I/O status is available.

##### NSC Status Register

This one-byte register contains the current status of the NSC and is gated over the channel interface during NSC status transfer sequences. The control program should set the NSC status by executing an Output X'66' instruction. This register can be accessed by executing an Input X'66' instruction.

##### Error/Control/Condition Register

This one-byte register contains detected hardware errors and asynchronous hardware control information. The register can be accessed by an Input X'67' instruction.

9-2

##### Local Store Registers

These two-byte registers provide buffering for the I/O device address used in all data and status transfer sequences initiated by the controller, and up to four bytes of data for inbound/outbound data transfer sequences in non-EB mode (up to 32 bytes of data for a Type 4 CA in EB mode). In EB mode a separate register stack is used to contain the data. When the controller is in emulation mode, the ESC final status is included along with the transfer address. These registers are as follows.

Transfer Address/ESC Status Register: This register can be accessed by executing an Input X'63' and loaded by executing an Output X'63'. The transfer address portion (byte 0) contains the I/O device address to be presented to the multiplexer channel when the controller initiates any data/status transfer sequence. The ESC status byte portion (byte 1) contains the status byte to be presented to the multiplexer channel on completion of an ESC command.

Data Buffer Byte 1/Data Buffer Byte 2 Register: This two-byte register contains the first and second data bytes transfered across the channel interface during an inbound or outbound data transfer sequence (in non-EB mode). The register can be loaded by executing an Output X'64' instruction or accessed by executing an Input X'64' instruction.

Data Buffer Byte 3/Data Buffer Byte 4 Register: This two-byte register contains the third and fourth data bytes transferred across the channel interface during an inbound or outbound data transfer sequence in non-EB mode. The register can be loaded by executing an Output X'65' instruction or accessed by executing an Input X'65' instruction.

Extended-Buffer Mode Local Store (Type 4 CA Only) The local store register for operation of a Type 4 Channel Adapter in extended-buffer (EB) mode provides buffering for up to 32 bytes of data for both outbound and inbound data transfer sequences in either NSC or ESC mode.

This register can be loaded by first executing an Output X'6C' instruction to place the CA in EB mode and reset the EB mode address register, then executing a sequence of Output X'6D' instructions, each of which loads two bytes into the register. Thus 16 Output X'6D' instructions (preceded by an Output X'6C') are needed to load the maximum of 32 bytes into the register. (A delay of at least one instruction time is required between the Output X'6C' and the Output X'6D' instructions, and between successive Output X'6D' instructions.)

The EB mode local store can be accessed by executing an Input X'6C' instruction to reset the EB mode address register and then executing a sequence of Input X'6D' instructions, each of which loads two bytes from the local store into the register specified by the instruction. (The first and second bytes are moved into bytes 0 and 1 of the specified register. Thus 16 Input X'6D' instructions (preceded by an Input X'6C') are required to read 32 bytes from the EB mode local store. (A delay of at least one instruction time is required between the Input X'6C' and the first Input X'6D' instructions, and between successive Input X'6D' instructions.)

Note: Powering on or resetting of the 3705 (via the control panel) does not ensure that the EB mode local store has correct parity. The control program must do this before accessing the local store by setting EB mode operation in the CA with an Output X'6C' instruction, executing 16 Output X'6D' instructions, then resetting the EB mode with an Output X'6C' instruction.

##### Extended-Buffer Mode Control Register (Type 4 CA Only

This register contains information that controls the operation of the Type 4 CA while it is operating in extended-buffer mode. The register can be accessed by an Input X'6C' instruction while the CA is in this mode. When  servicing a Type 4 CA L3 interrupt, the control program can execute Output X'6C' instructions to set and reset individual bits in the register.

##### Extended-Buffer Mode Counter Register

This register is a six-bit counter used to address the 32-byte EB mode local store. The counter is incremented by two bytes while the CA is in EB mode when an Output X'6D' or Input X'6D' instruction is executed. It is reset to a Output X'6C' instruction or (when the CA is in EB mode) by an Output X'62' or Input X'6C' instruction. An Input X'6C' instruction initially resets the counter and then increments it by two. Each subsequent Input X'6D' or Output X'6D' instruction increments the counter by two.

I/O Device Addresses

The channel adapter must determine whether to recognize the I/O device address presented when initial selection occurs. It must also determine which I/O device address to present when the control program signals the channel adapter to perform a data/status sequence. The following sections describe the methods of determining initial selection and data transfer addresses.

IBM 3704 and 3705 Communications Controllers Principles of Operation  9-3

##### Initial Selection Address

The address byte presented during initial selection must have correct parity, or the channel adapter will not decode the address. The initial selection addresses that the channel adapter recognizes are determined by plug options wired by the customer engineer from information supplied by the user.

The address assigned for the native subchannel (NSC) of a Type 1 CA (but not a Type 4 CA) can be within the range of addresses assigned for ESC but cannot also be used for emulation. The assigned NSC address can be checked by executing an Input X'67' when the channel is enabled.

When power is turned on, the channel adapter is placed in such a state that it will not recognize any ESC addresses until after the interface is enabled, the CCU is initialized, and the control program executes an Output X'67' instruction with byte 1, bit 5 = 1 (Set ESC Operational).

When a CA Initial L3 interrupt request is caused by an initial selection, the program can determine the I/O device address by executing an Input X'61'. This interrupt request can be caused by an initial selection sequence for any operational ESC address or for the native mode subchannel address. The program should be prepared to handle initial selection sequences for all operational addresses.

NSC: The NSC address can be assigned any value in the range of 0-255. If the Two Channel Switch Feature is installed, the NSC addresses for interfaces A and B are assigned separately and can be either two different addresses or the same address used for both interfaces.

Because the native mode uses only one subchannel address, the terminal address must be transferred from the host as data. The location and format of these addresses must be coordinated between the host program and the control program in the controller.

ESC: The ESC addresses must be a group of contiguous addresses. For the 3705, the lowest address (L) in the group can be set to 0 or any multiple of 16 from 16 to 240. The highest address (H) in the group can be set to any value greater than (L) that is one less than a multiple of 4, from 3 to 255. The range of recognizable ESC addresses can be set to include a minimum of 4 or a maximum of 256 addresses. For the 3704, the lowest address can be any even number and the highest address can be any odd number. The range of addresses can be a minimum of 2 or a maximum of 32. This range must be the same for both interface A and interface B if the Two.Channel Switch feature is installed on the 3704 or 3705.

Note: If emulation is not required, the machine can be wired (by the customer engineer) so that no ESC addresses are recognized.

##### Transfer Sequence Addresses

When the control unit initiates a data/status transfer sequence, it must present the I/O device address associated with the transfer. Since this address is variable, it must be provided by the control program. This is done by executing an Output X'63'.

Since Output X'63' allows the control program to provide any I/O device address in the range of 0-255, and the channel adapter cannot determine if this is the address that should be presented, the control program must ensure that the address is correct. An incorrect address results in improper channel operation.

Channel Adapter Interrupt Requests

The Type 1 or Type 4 Channel Adapter can initiate interrupt requests for either level 1 or level 3 service. Level 1 requests are caused when the CA detects an error or check condition. Two different level 3 interrupt requests can be set for the purpose of controlling channel adapter operations: the CA Initial L3 and the CA Data/Status L3 interrupt request.

##### Level 1 Interrupt Request

When one of the following checks occurs, a level 1 interrupt request is set and the corresponding but is set in error/control/condition register. This register is made available to the control program by executing an Input X'67' instruction. See Appendix B for the placement of the bits within this register.

1. Channel Bus In Check: This error is set when the CA hardware detects bad parity in the byte to be sent to the host.

2. In/Out Instruction Accept Check: This error is set when the CA hardware detects the execution of a valid input/output instruction at an invalid time (that is, the CA is actively handling a data or status transfer sequence and any Input or Output instructions except X'67' is executed).

3. CCU Outbus Check: This error is set when the CA hardware detects bad parity on the CCU Outbus.

4. Local Store Parity Check: This error is set when the CA hardware detects bad parity on data being read out of the local store registers.

See the Error Indications section in this chapter for information on how to handle these errors.

9-4

##### Level 3 Interrupt Request

Two different level 3 interrupt requests can be set: an initial L3 and a data/status L3. These are identified to the control program when Input X'77' is executed. Either one or both of these requests can be set. Several different events can cause either to be set.

If the CA initial L3 is the only interrupt request set, it can be serviced without being concerned that a CA data/status L3 interrupt request will occur. Any pending data/status transfer sequence is inhibited until the CA initial L3 request is reset. The level 3 interrupt can be reset by either an Output X'60' or Output X'62' instruction except that when a system reset occurs, an Output X'67' must be executed to reset the system reset. If an Output X'62' is executed to reset the CA initial L3 request, the control program must be prepared to re-initiate a data/status transfer if one was pending at the time the initial L3 interrupt was reset.

If just the CA data/status L3 interrupt request is set, the program must be aware that a system reset sequence can occur during servicing of the data/status interrupt request. If the system reset occurs before the Input X'62' is executed, the conditions that caused the interrupt are lost to the program. A system reset causes an initial L3 interrupt request and resets all hardware latches except the 'system reset' latch.

If both interrupt requests are set, the program must service both requests before executing the output instruction to reset them. If both are set and the initial L3 interrupt request was caused by a system reset, the conditions that caused the data/status L3 will be lost due to the system reset.

Initial L3 Interrupt Request: A CA initial L3 interrupt request is made when (1) an initial selection sequence occurs, (2) a system reset sequence occurs, or (3) NSC status is cleared because of an initial selection sequence. When an initial L3 interrupt occurs, Input X'60' can be executed to determine the cause.

Once the CA initial L3 interrupt request is set, the channel adapter automatically responds with a short control-unit-busy status (Control Unit End, Status Modifier, Busy) to all initial selection attempts. This status is indicated until the control program signals the channel adapter to reset the condition that caused the interrupt request to be set. During this period, no channel command can be accepted in either emulation or native mode. The control program should therefore signal the channel adapter as soon as possible to reset the cause of the interrupt request.

Once the CA initial L3 interrupt request is set, subsequent data/status transfer sequences are inhibited until the interrupt request is reset. This means that the initial

L3 interrupt request and the data/status L3 interrupt request can both be set only if the data/status request occurs first, and only if the initial L3 interrupt request is not the result of a system reset.

The CA initial L3 interrupt request can' be reset by executing an Output X'60', X'62', or X'67', depending on the cause of the interrupt. (See the preceding section, Level 3 Interrupt Request, for restrictions on resetting level 3 interrupts.)

Data/Status L3 Interrupt Request: A CA data/status L3 interrupt request can be set by

• any of the five data/status transfer sequences

• a suppress-out monitor interrupt

• a program-requested interrupt

• a selective service reset

• A CCU Outbus check when the adapter is in one of the five data/status transfer sequences (only during an Output X'62' for a 3704)

• a channel stop or interface disconnect

• recognition of an ETB or ETX character when a Type 4 CA is in EB mode and the ESC mode is enabled

• recognition by a Type 4 CA of a number of consecutive SYN characters when (1) the scanner is in EB mode, (2) the ESC mode is enabled, and (3) the 'monitor SYN' latch (Input X'6C' instruction bit 0.4) is on.

Automatic Priority Selection for Dual Type 4 CAs: Communications controller hardware can automatically select which of two Type 4 CAs having level 3 interrupt requests pending is to be selected for handling of its interrupt request. (This function applies only to two Type 4 CAs installed in the controller. It is not available for other combinations of channel adapters.)

The selection is based on which of the dual CAs has the higher-priority interrupt request pending. The priority sequence is:

Priority outbound data transfer sequence (highest)
Outbound data transfer sequence
Initial selection interrupt
Inbound data transfer sequence
Other data/status interrupts (lowest)

(Byte 1, bit 5 of the Output X'62' instruction specifies whether the outbound data transfer sequence has priority status or not.)

The automatic selection occurs only when requested by the control program. To do so, it must execute an Output X'67' instruction in which all bits are 0. The next Input X'77' instruction executed will then automatically select and access the states of the CA currently having the higher-priority interrupt request.

IBM 3704 and 3705 Communications Controllers Principles of Operation  9-5

The control program can cause a data/status L3 interrupt by executing an Output X'67' instruction with the proper bits set for the operation desired. See Appendix B for the output instruction bit definitions. The program should not initiate another transfer sequence or monitor for the inactive condition of the 'suppress out' line unless the initial L3 or the data/status L3 interrupt has been set. The control program can signal the channel adapter at any time to cause a program requested interrupt when conditions permit. When a data/status L3 interrupt occurs, Input X'62' can be executed to determine the cause.

The data/status L3 interrupt request can be reset by executing an Output X'62' with the appropriate bits set. Refer to Appendix B, Output X'62' for bit definitions.

When a channel-stop or interface-disconnect condition is detected during an outbound or inbound data transfer, in EB mode, the number of bytes successfully transferred can be obtained by executing an Input X'6C" instruction and examining bits 2-7 of byte 1 of the register loaded by the instruction.

Channel Commands

The channel adapter recognizes any I/O command byte combination as a valid command if odd parity is maintained on the I/O channel interface and there is no pending activity on the channel. Except for those commands listed below, the channel adapter hardware accepts the command, returns an initial selection status of X'00' and causes a CA initial L3 interrupt request. The channel adapter hardware initially accepts the full range of commands (X'00' through X'FF'). It is the responsibility of the control program to decode the command, determine its validity and respond accordingly.

The hardware recognizes the following commands and takes the action listed.

##### NSC:

I/O No-Op (X'03'}—When this command is presented to the NSC address, an immediate initial status of CE, DE is presented by the hardware if the channel adapter is free of commands. If the NSC has a pending status available (a previous NSC status byte has been stacked), the hardware presents it with the 'Busy' status bit as initial status to the No-Op command.

Test I/O (X'00')—When this command is present-ed to the NSC address, the hardware presents the current status of the NSC. If the NSC is free of commands, the hardware responds with a X'00' status byte during the initial status presentation to the Test I/O. If the NSC hardware has a pending status available in the NSC status register, this status is presented during the initial status presentation

to the Test I/O. The busy bit does not accompany this status. If the NSC is currently executing a command, but no status is available, the Busy status bit is presented as initial status.

Test I/O and No-Op must be recognized by the control program if these immediate commands are stacked.

Write IPL (X'05')— When this command is presented to the NSC address, the hardware accepts the command, returns the initial status byte of X'00', and causes a CA initial L3 interrupt request. The hardware also notifies the CCU (Central Control Unit) of the reception of this command by raising an internal interface line to the CCU. See Initial Program Load in Chapter 5 for an explanation of the IPL procedures.

##### ESC:

I/O No-Op (X'03')—When this command is presented to a valid ESC address, the hardware presents an immediate CE, DE initial selection status.

Test I/O (X'00')—Refer to Emulation Mode Test I/O in this chapter for an explanation of this operation.

The following commands are standard channel I/O commands issued by the host access method for either native or emulation mode. The control program has the responsibility to decode the command and initiate the appropriate action. Refer to Inbound Data Transfer and Outbound Data Transfer in this chapter for the sequence of instructions that the control program must execute for proper data transfer.

Write (X'01') - The channel adapter hardware accepts this command and returns an initial selection status of X'00'. The purpose of the command is to transfer data from the host processor to the communications controller. The data may be either user data or control information to inform the control program of a function to be performed.

Read (X'02') - The purpose of this command is to transfer data from the communications controller to the host processor. The channel adapter hardware accepts the command and returns an initial selection status of X00'

Sense (X'04') - This channel command should result in the transfer of one byte of sense data from the communications controller to the host. The channel adapter hardware

9-6

accepts the command and returns an initial status of X'00'. Normal ending status is CE, DE unless a Halt I/O is detected when the adapter is not initialized. In this case, a CE, DE, UC is returned to the channel. It is the responsibility of the control program to recognize the command, load the sense byte into a data register, and send it to the host. The transfer is the same as a data transfer with one byte of data. The sense byte for the Type 1 and Type 4 Channel Adapters are the same as for the Type 2 Channel Adapter. See Input X'53' in Appendix B for a definition of each bit.

Channel Adapter Status Presentation

##### Initial Selection Status

Initial Selection status is generated by hardware and presented to the channel during the initial selection sequence. The host processor must examine the status byte to determine whether the command was accepted by the channel adapter. If the command was not accepted, the status byte contains bits indicating the reason.

The following paragraphs describe the initial selection status bytes that can be returned to the host processor channel and their meanings.

##### X'00' (Zero Status):

NSC: The hardware accepts a command byte, other than a No-Op, and the NSC is free of commands.

ESC: The hardware accepts a command byte other than a Test I/O or No-Op command, and there is no initial select L3 interrupt request or no program requested L3 interrupt request pending.

X'02' (Unit Check): The hardware detects even parity in the command byte.

X'0C' (Channel End, Device End): This status is returned by the hardware as an immediate response to a No-Op command.

X'10' (Busy):

NSC: The NSC hardware is active with another command and has not presented final status to that command or, for the Type 4 CA only, an Output X'66' with bit 0.4 set to 1 was executed to place the CA in the 'long busy' state.

X'70' (Status Modifier, Control Unit End, Busy): This status is returned as a short control-unit-busy sequence to any initial selection sequence when any of the following conditions are present:

• The CA has an initial selection L3 interrupt request pending due to the acceptance of a previous command, and the control program has not yet reset the interrupt request.

• The CA hardware has previously detected a system reset indication and caused a CA initial L3 interrupt request, and the control program has not reset the initial select L3 interrupt request.

• The CA hardware has previously detected a selective reset during a service transfer and caused a CA data/status L3 interrupt request, and the control program had not reset this request.

• A program requested interrupt is pending, and the control program has not reset the request.

ESC: This is the normal initial selection status returned by the hardware when a Test I/O is issued to an ESC address.

##### Busy Bit and Status Information:

NSC: When an initial selection occurs for the NSC and status status is pending in the NSC status register, the Busy bit, in addition to the pending status, is returned.

##### Pending Status:

NSC: When a Test I/O command is issued to the NSC and status is pending in the NSC status register, the pending status is returned without the Busy bit.

##### Final Status

The final status is a control program generated status and should be presented to the host processor upon completion of a command. Bit definitions for the final status byte must be coordinated between the control program and the host program for proper operation. Upon completion of a command, the control program should load the appropriate register with the status byte to be presented and initiate a final status transfer sequence. Refer to Status Transfer Sequence in this chapter for additional information.

##### Emulation Mode Test I/O

When a Test I/O command is issued to an ESC address, the hardware responds with initial selection status and also

IBM 3704 and 3705 Communications Controllers Principles of Operation  9-7

causes CA initial L3 interrupt request. Although the initial selection status presented to the host is X'70' (Status Modifier, Control Unit End, Busy), byte 0, bit 0 of the CA initial selection control register (X'60') is set to 1 (normal initial selection). The control program should recognize the command and address and execute:

1. Output X'63'-To load the ESC address and the status byte to be presented to this Test I/O command.

2. Output X'62'-To set ESC Final Status Transfer Sequence, Reset Initial Selection, Reset Data/Status Control, and Set ESC Test I/O Status Available.

When the next initial selection sequence occurs, the hardware compares the address presented to the adapter with the address that had received the Test I/O command.

If these addresses compare and the subsequent command is a Test I/O command, the hardware presents the status byte that was loaded by the above instructions. The hardware then causes a CA data/status L3 interrupt request. When the control program executes an Input X'62', the ESC Final Status Transfer Sequence bit is on, indicating a successful completion of the Test I/O sequence.

If the addresses do not compare during the initial selection sequence, or if the command is not a Test I/O, the hardware resets out of the ESC Test I/O mode and handles this sequence as a standard initial selection. If this occurs, the control program must reset the ESC final status transfer sequence by executing an Output X'62' instruction.

Between the time the Test I/O command is first issued and the time the control program executes the above Output X'62' and X'63' instructions, the hardware responds with an initial status of X'70' to any initial selection sequence from the host processor. In this case, an Output X'60' should not be used to reset initial selection because the address compare uses the address in the initial selection address control register. Refer to Initial Selection Status in this chapter for an explanation of X'70' status.

##### Native Mode Asynchronous Status

The control program initiates the presenting of asynchronous status to notify the host processor of an unusual condition or a required action.

Note: The control program should not attempt to present asynchronous status when the interface is disabled (that is, byte 1, bit 4 of an Input X'67' instruction is 0).

The control program initiates the action by requesting a CA program interrupt. This is done by executing an Output X'67' instruction. When the requested interrupt is allowed, the hardware sets a CA data/status L3 interrupt request.

The control program should then prepare to present the asynchronous status by:

1. Loading the address of the NSC (Output X'63').

2. Loading the NSC status byte to be presented (Output X'66').

3. Executing an Output X'62' with the NSC Final Status Transfer set to 1. Any Output X'62' instruction resets the program requested interrupt.

After the control program executes the Output X'62' instruction to present asynchronous status, it must execute an Input X'67' instruction to determine if the interface is enabled. If it is not, the program should then execute, in sequence:

1. Output X'67' (bit 1.7 must be on) to allow the interface to be disabled.

2. Output X'67' (bit 0.4 must be on) to reset the CA.

3. Output X'67' (bit 1.4 must be on) to allow the interface to be enabled.

Following these instructions, the control program must then reexecute the asynchronous status presentation.

At the completion of the status transfer, the hardware causes a CA data/status L3 interrupt to inform the control program of the results of the transfer. The control program should then execute an Input X'62' instruction- If the status byte is accepted, the NSC final status transfer sequence bit is one.

##### Stacked Initial Status

The host processor can stack all initial status responses except zero status (X'00') to a Start I/O. When an initial status is stacked, the hardware causes a CA initial L3 interrupt request. The control program must determine what status was stacked by analyzing the contents of the initial selection control register and the initial selection address and command byte register. This information is obtained by executing an Input X'60' and X'61'. The status and address of the stacked device must be presented later in a final status transfer. The control program must be able to distinguish between the NSC address and the ESC addresses to be able to load and transfer the proper status.

The following initial status is presented by the CA hardware and is capable of being stacked.

Channel End/Device End (X'0C') - The initial status is presented to an No-Op command. If the initial status is stacked, the initial status byte stacked bit is on in the initial selection control register (Input X'60'). The address and command byte can be obtained by executing an Input X'61' instruction.

9-8

Unit Check (X'02') - Unit Check is presented if the hardware detects even parity in the command byte. If this initial status is presented, the initial status byte stacked bit and the channel out bus check bit is on in the initial select control register (Input X'60'). The device address may be obtained by executing an Input X'61' instruction.

NSC: Zero or any pending status to a Test I/O addressed to the NSC.

If a Test I/O command is issued to the NSC and the status is stacked, the initial status byte stacked bit is on in the initial select control register (Input X'60'). The address and command may be obtained by executing an Input X'61' instruction. The control program should not execute an Output X'66' to load the stacked status in the NSC status register. The NSC hardware maintains the stacked status from a Test I/O command in the NSC status register until the host processor has accepted it.

ESC: Initial status byte to a Test I/O addressed to the ESC.

If a Test I/O command is issued to the ESC and the initial select status is stacked, the initial status byte stacked bit is on in the initial selection control register (Input X'60'). The address and command may be obtained by executing an Input X'61'. The control program should treat this the same as a stacked status while in an ESC final status transfer.

##### Stacked Final Status

When a final status or an NSC Channel End is stacked, the CA hardware causes a CA data/status L3 interrupt request. The control program should analyze the contents of the data/status control register (Input X'62') to determine which status was stacked. One of the following actions may be taken:

1. Test the level of the 'suppress out' line at the time the last Input X'62' was executed. If the 'suppress out' line was inactive, the control program can now attempt another transfer sequence. The suppressible status bit must be on for the second transfer.

NSC: Since the NSC stacked status is maintained in the NSC status register the stacked status should not again be placed in the NSC status register with an Output X'66' instruction.

ESC: The status byte along with the ESC address must be loaded by an Output X'63' when preparing an ESC final status transfer.

If the 'suppress out' tag line is active, the control program can queue the stacked status; or if no data services are required, it can set the suppress out monitor as described below.

2. The suppress out monitor may be used to notify the control program when the 'suppress out' line falls and when the status may be presented. To use this feature, the control program should execute an Output X'62' to reset the CA data/status L3 interrupt request and execute an Output X'67' to set a suppress out monitor interrupt. The hardware then causes a CA data/status L3 interrupt request when the 'suppress out' line becomes inactive. When the control program executes an Input X'62', the suppress out monitor interrupt bit is active. The stacked status can be transferred by loading the subchannel address and status (in the case of ESC fmal status) into a general register and executing an Output X'63' with the type of transfer indicated and reset suppress out monitor. The suppressible status bit (byte 1, bit 3) in Output X'62' must also be on.

NSC: When operating in NSC mode only, the control program can initiate another NSC status transfer immediately after being notified of a stacked status by executing an Output X'62' with the type of transfer indicated and the reset data/status L3 interrupt request bit on. The hardware does not attempt to transfer the status until the 'suppress out' line becomes inactive. It then transfers the status across the channel to the host. The suppressible status bit does not have to be on since the channel adapter hardware monitors the channel 'suppress out' tag line.

Programming Note

Whenever the channel adapter is in the ESC mode and the control program is presenting suppressible status, the suppressible status bit (byte 1, bit 3) must be on when an Output X'62' is executed. Status is suppressible if 'stacked status' is received for a particular line or when the line has been issued an interface disconnect. Refer to IBM System/360 and System/370 I/O Interface Channel to Control Unit Original Equipment Manufacturers Information (GA22-6974) for further information on suppressible status.

IBM 3704 and 3705 Communications Controllers Principles of Operation  9-9

Status Transfer Sequences

Upon completion of a command, the control program should present a final status associated with that command. The control program may choose to separate the Channel End and Device End on an NSC command. This status is transferred by executing the following output instructions.

##### ESC Final Status

Output X'63'—Loads the I/O device address that is presenting the status and the status byte that is to be presented.

Output X'62'—Sets the following control bits: (1) ESC final status transfer, (2) reset initial selection (if this status transfer is being initiated because of a stacked initial status), or (3) reset data/status interrupt (for final status transfer).

##### NSC Final Status

Output X'63'—Loads the address of the NSC (must be loaded only once at the beginning of the program if operating in NSC mode only).

Output X'66'—Loads the NSC status byte to be presented.

Output X'62'—Sets the following control bits: (1) NSC final status transfer, (2) reset initial selection (if this status transfer is being initiated because of a stacked initial status), or (3) reset data/status interrupt (for a final status transfer).

##### NSC Channel End Status

Output X'63'—Loads the address of the NSC (can be loaded only once at the beginning of the program if operating in NSC mode only).

Output X'62'—Sets the following control bits: NSC channel end transfer sequence and reset data/status interrupt.

When the NSC channel end transfer sequence bit is active during the execution of an Output X'62', the CA hardware loads Channel End Status (X'08') in the NSC status register.

When the Output X'62' instruction is executed in the above sequence, the CA hardware notifies the I/O channel that it requires service. In a hardware-controlled sequence, the CA hardware indicates that status is being passed to the host and gates the status byte out to the I/O channel. After this sequence is completed, the CA hardware causes a data/status L3 interrupt to inform the control program of the results of the status transfer.

The control program should react to the data/status L3 interrupt request by executing an Input X'62' instruction. If the status byte is accepted by the host processor, the related final status bit is on (that is, ESC or NSC final status transfer or Channel End transfer sequence).

If any unusual condition occurred during this transfer (that is, status stacked, selective reset, or interface disconnect), the related bits are on when the Input X'62' instruction is executed.

Data Transfer Sequences— Non-EB Mode

Outbound or inbound data through a Type 1 CA or through a Type 4 CA that is not in extended-buffer mode is transferred through input and output instructions executed by the control program. As described in the following paragraphs, these I/O instructions must be in the correct sequence for proper transfer operation.

##### Outbound Data Transfer

When reacting to a read type I/O command, which requires data to be passed from the communications controller to the host processor, the control program must execute the following instructions.

Output X'63'—Loads the address of the I/O device sending the data.

NSC: If the CA is operating in NSC mode only, it is necessary to load this address only once. It remains constant for subsequent NSC transfers.

ESC: When operating in ESC mode, this address may change and must be updated when required.

Output X'64'—Places the first and second bytes of outbound data in the CA data buffer byte I/data buffer byte 2 register.

Output X'65'—Places the third and fourth bytes of outbound data in the CA data buffer byte 3/data buffer byte 4 register.

Output X'62'—Sets the following control bits: (1) outbound data transfer sequence, (2) reset initial selection (if this transfer is the first after an initial selection), or (3) reset data/status interrupt and the request byte count. A maximum of four bytes may be transferred during one sequence.

When Output X'62' is executed, the CA hardware notifies the I/O channel that it requires service. In a hardware-controlled sequence, the CA then identifies itself by gating its I/O device address onto the I/O channel and passes the required data. The CA hardware then causes a CA data/ status L3 interrupt to inform the control program of the completion of the transfer.

The control program should react to the CA data/status L3 interrupt by executing an Input X'62'. Assuming a normal data transfer, the outbound data transfer sequence bit

9-10

is on and the transferred byte count field reflects the number of bytes passed across the channel during this transfer sequence.

If any unusual conditions occurred during this transfer (that is, selective reset, interface disconnect or channel stop), their related bits are on when the Input X'62' was executed.

##### Inbound Data Transfer

When reacting to a write type I/O command, which requires data to be passed from the host processor to the communications controller, the control program must execute the following instructions.

Output X'63'—Loads the address of the I/O device that is transferring the data.

Output X'62'—Sets the following control bits: (1) inbound transfer sequence, (2) reset initial selection (if this is the first transfer after an initial select), (3) Reset data/status interrupt, and (4) the transferred byte count. A maximum of four bytes may be transferred during one sequence.

When the Output X'62' instruction is executed, the CA hardware notifies the channel that it needs service. In a defined sequence, the CA then identifies itself by gating its I/O device address onto the I/O channel and accepts the required number of bytes of data. The CA hardware then causes a CA data/status L3 interrupt to inform the control program of the completion of the transfer.

The control program should react to this interrupt by executing an Input X'62' instruction. Assuming normal data transfer, the inbound data transfer sequence bit is on and the transferred byte count field reflects the number of bytes of data passed from the host processor to the controller during this sequence.

If any unusual conditions occurred during this transfer (that is, selective reset, interface disconnect, channel bus out check, or channel stop), their respective bits are active when the Input X'62' is executed.

Data Transfer Sequences-EB Mode (Type 4 CA)

Outbound or inbound data through a Type 4 CA operating in extended-buffer (EB) mode is transferred via input and output instructions executed by the control program in the following sequences.

Outbound Data Transfer: When reacting to a read-type I/O command, which requests data to be passed from the communications controller to the host processor, the control program must execute the following instructions.

Output X'63'—Loads the address of the I/O device sending the data.

NSC: If the CA is operating in NSC mode only, it is necessary to load this address only once. It remains constant for subsequent transfers in NSC mode.

ESC: When operating in ESC mode, this address may change and must therefore be updated when required.

Output X'6C'—Places the CA in EB mode and requests the number of bytes to be transferred. (Byte 0, bit 0 of the instruction must be 1 to place the CA in EB mode; bits 3-7 of byte 1 specifies the number of bytes to be transferred. (00001 = 1 byte, 00010 = 2 bytes,.. . 11111=31 bytes, 00000 = 32 bytes). (A delay of at least one instruction time must precede execution of the Output X'6D' instruction.)

Output X'6D'—Loads two bytes into the EB mode local store. This instruction may be executed up to 16 times, one for each two bytes to be transferred to the CA (maximum: 32 bytes). (A delay of at least one instruction time must elapse between successive Output X'6D' instructions.)

Output X'62'-Sets the following control bits: (1) outbound data transfer sequence, (2) reset initial selection (if this data transfer was the first after an initial selection), or (3) reset data/status interrupt. In addition, bit 5 of byte 1 must be set to indicate a priority outbound sequence to the CA.

The control program should react to the CA data/status L3 interrupt by executing (1) an Input X'62' instruction (assuming a normal data transfer, the outbound data transfer sequence bit is on), and (2) an Input X'6C' instruction to determine the number of bytes transferred. Bits 2-7 of byte 1 indicate the number of bytes (000000 = no bytes, 000001 = 1 byte, 100000 = 32 bytes) transferred.

Finally, the control program should execute an Output X'6C' instruction to reset the EB mode of the CA (bit 0 of byte 0 is 0 to perform this reset), if data transfer in EB mode is no longer desired.

Inbound Data Transfer: When reacting to a write-type I/O command, which requests data to be passed from the host processor to the communications controller, the control program must execute the following instructions.

Output X'63'-Loads the address of the I/O device that is transferring the data.

Output X'6C'—Places the CA in EB mode, requests the number of bytes to be transferred, and activates the BSC control character recognition logic of the CA. (Byte 0, bit 0 of the instruction must be 1 to place the CA in EB mode, bits 3-7

IBM 3704 and 3705 Communications Controllers Principles of Operation  9-11

of byte 1 specify the number of bytes to be transferred (00001 =1 byte, 00010 =2 bytes,... 11111 =31 bytes, 00000 = 32 bytes). Bits 5, 6, and 7 of byte 0 activate the control character recognition logic as follows:

bit 4 — SYN monitor control latch (1 = set, 0 = reset)

bit 5 — DLE remember latch (1 = set, 0 = reset)

bit 6 — monitor for ASCII control characters (1 = yes,  0=no)

bit 7 — monitor for EBCDIC control characters (1 = yes,  0=no)

Output X'62'-Sets the following control bits: (1) inbound data transfer sequence, (2) reset initial selection (if this is the first transfer after an initial selection), and (3) reset data/ status interrupt.

When the Output X'62' instruction is executed, the CA hardware notifies the channel that it needs service. In a hardware-controlled sequence, the CA then identifies itself by gating its I/O device address onto the I/O channel and accepts the required number of bytes of data. The CA hardware then causes a CA data/status L3 interrupt to inform the control program of the completion of the transfer.

The control program should react to this interrupt by executing (1) an Input X'62' instruction (assuming a normal data transfer, the inbound data transfer sequence bit is on), and (2) an Input X'6C' instruction to determine the number of bytes transferred. Bits 2-7 of byte 1 indicate the number of bytes (000000 = no bytes, 000001 = 1 byte,. .. 100000 = 32 bytes) transferred. Bits 5, 6, and 7 of byte 0 indicate the state of the control character recognition latches, as follows:

bit 4 — SYN monitor control latch

bit 5 — DLE remember latch

bit 6 — ASCII monitor latch

bit 7 — EBCDIC monitor latch

(A delay of at least one cycle must precede execution of the instruction.)

Input X'6D'—Reads two bytes from the EB mode local store. This instruction can be executed up to 16 times, once for each two bytes to be read from the local store (maximun 32 bytes). (A delay of at least one instruction time must elapse between successive Input X'6D' instructions.)

Finally, the control program should execute an Output X'6C' instruction to reset the EB mode of the CA (bit 0 of byte 0 is 0 to perform the reset, if data transfer in EB mode is no longer desired.

Input/Output Instructions

The X'60' through X'6F' input/output instructions are used to manipulate data, status, and control information in the Type 1 and Type 4 Channel Adapters. (Not all values of X are used for both types of CA, and Input '6E' and X'6F' and Output X'6E' and X'6F' are reserved for future use.) Refer to Appendix B for the definitions of the bits within the input and output instructions.

##### Input Instructions

Eight input instructions are assigned to the Type 1 Channel Adapter to monitor its operation and data flow; twelve are assigned to the Type 4 CA. Appendix B defines the bits within each input instruction.

Input X'60' (Initial Selection Control): This instruction loads the contents of the initial selection control register into the general register specified by R. The bits of the control register are set as a result of initial selection or system reset and reflect the status of the operation when completed.

This should normally be the first instruction executed after determining that the level 3 interrupt was caused by a channel adapter initial selection or a system reset.

Input X'61' (Initial Selection Address and I/O Command Byte): This instruction loads the initial selection address and the I/O command into byte 0 and byte 1 respectively of the general register specified by R.

The control program should normally execute an Input X'61' instruction when servicing a CA initial L3 interrupt request. If the cause of the interrupt (identified by executing an Input X'60') is found to be an initial selection sequence, the addressed subchannel and channel I/O command byte can be determined by executing an Input X'61' instruction.

Input X'62' (Data/Status Control): This instruction loads the contents of the CA data/status control register into the general register specified by R. An Input X'62' should normally be the first instruction executed by the control program when servicing a CA data/status L3 interrupt. It is used to determine the cause of the interrupt.

Input X'63' (Transfer Address and ESC Status Bytes): This instruction loads byte 0 and byte 1 of the general register specified by R with the I/O device address byte and ESC status byte respectively. These are the address byte and status byte given to the channel when the last Output X'63' was executed. This instruction can be executed for checking

9-12

When servicing a CA data/status L3 interrupt request due to a transfer sequence, the program can execute this instruction either for checking purposes or to obtain the information if not retained elsewhere.

Input X'64' (Data Buffer Bytes 1 and 2): This instruction loads the contents of data buffer bytes 1 and 2 into bytes 0 and 1 of the general register specified by R. The control program normally executes the Input X'64' instruction when servicing a CA data/status L3 interrupt caused by the ending of an inbound data transfer sequence in which one or more data bytes were transferred. (For a Type 4 CA this instruction is used when the CA is not in extended-buffer mode.)

Programming Note

Input X'62' should be executed before the Input X'64' instruction to determine the transferred byte count. Data buffer 1 contains valid information if the transfer count is greater than zero. Data buffer 2 contains valid information if the transfer count is two or more.

Input X'65' (Data Buffer Bytes 3 and 4): This instruction loads the contents of data buffer bytes 3 and 4 into bytes 0 and 1 of the general register specified by R. The control program normally executes the Input X'65' instruction when servicing a CA data/status L3 interrupt caused by the ending of an inbound data transfer sequence in which three or four data bytes were transferred. (For a Type 4 CA this instruction is used when the CA is not in extended buffer mode.)

Programming Note

Input X'62' should be executed before the Input X'65' instruction to determine the transferred byte count. Data buffer 3 contains valid information if the transfer count is three or four. Data buffer 4 contains valid information if the transfer count is four.

Input X'66' (NSC Status Byte): This instruction loads the contents of the NSC status byte register into bytes 0 and 1 of the general register specified by R. These bits reflect the status bits loaded into the NSC status byte register when an Output X'66' instruction is executed. These bits are reset either when the channel accepts an NSC status byte presentation or when the control program executes an Output X'67' instruction with byte 1, bit 3 or byte 0, bit 4 set to one.

Programming Note

Reset of this instruction by the control program (output X'67' byte 0, bit 4) should be only as a diagnostic aid and should not be used during active communication over the channel.

Input X'67' (CA Controls): This instruction loads a general register with the NSC address byte and the state of various check and control latches.

The control program should execute this instruction when (1) an asynchronous status sequence is required (determined by the control program and initiated by executing an Output X'67' with byte 1 bit 1 on) or (2) there is a CA LI interrupt request and the control program wants to determine the cause of the interrupt.

This instruction can be used to determine which of dual Type 4 CAs is selected, as follows:

Bit 7 of byte 1 = 0: First Type 4 CA (in 3705 base module) is selected

Bit 7 of byte 1=1: Second Type 4 CA (in 3705 expansion module) is selected

Input X'6C' (EB Mode Control Register): This instruction loads the content of the CA control character recognition latches, the EB mode control bit, and the transferred byte count into the general register specified by R.

Input X'6D' (EB Mode Data Buffer): This instruction loads the content of the two-byte EB mode buffer into the general register specified by R. The first (even) data byte is placed in byte 0 of the general register; the second (odd) data byte is placed in byte 1.

##### Output Instructions

Seven output instructions are assigned to the Type 1 channel Adapter to control its operation and data flow; eleven are assigned to the Type 4 CA. Appendix B defines the bits within each output instruction.

Programming Note

If the control program executes an Output X'61' instruction,  (an assigned, but unused instruction) the hardware takes no action unless execution occurs during a data/status transfer. During data/status transfer, the hardware sets the in/out instruction accept check and causes a CA level 1 interrupt request.

Output X'60' (Reset Initial Selection): When this instruction is executed, the hardware resets both the initial selection hardware latches and the CA L3 interrupt request resulting from an initial selection sequence. This output does not reset a system reset condition. See Output X'67' to reset a system reset. Since this instruction performs a function instead of an operation, the bit settings of the register are ignored.

Output X'62' (Data/Status Control): This instruction sets the data/status control register according to the contents of bytes 0 and 1 of the general register specified by R.

IBM 3704 and 3705 Communications Controllers Principles of Operation  9-13

Output X'63' (Transfer Address and ESC Status Bytes):  This instruction loads the transfer address and ESC status registers with the contents of byte 0 and 1 respectively of the general register specified by R.

This instruction should be executed before signaling the CA that a data/status sequence is required. The I/O device address provided by the last Output X'63' executed is presented to the channel in all subsequent transfer sequences. The Output X'63' instruction must be executed each time a transfer sequence is required for a different I/O device address. This instruction must also be executed before signaling the channel adapter that an ESC final status transfer is required. This presents the I/O device address in addition to the status byte to the host channel.

Programming Note

For compatibility, the program should ensure that the bits of the ESC status byte that are set are consistent with the bits set under similar conditions in the operation of the IBM 2701/2702/2703 transmission control units.

Output X'64' (Data Buffer Bytes 1 and 2): This instruction loads data buffer bytes 1 and 2 with the contents of the general register specified by R. This instruction should be executed before signaling the CA that an outbound data transfer sequence is required. When an outbound data transfer sequence occurs, data buffer bytes 1 and 2 are the first and second data bytes transferred. (For a Type 4 CA this instruction is used when the CA is not in extended-buffer mode.)

Programming Note

The request byte count (Output X'62', byte a, bits 6-7) must be set with the total number of valid data bytes loaded into data buffers 1-4 (by Outputs X'64' and X'65') to ensure data integrity.

Output X'65' (Data Buffer Bytes 3 and 4): This instruction loads data buffer bytes 3 and 4 with the contents of the general register specified by R. This instruction should be executed before signaling the CA that an outbound data transfer sequence is required. When an outbound data transfer sequence occurs, data buffer bytes 3 and 4 are the third and fourth data bytes transferred. (For a Type 4 CA this instruction is used when the CA is not in extended-buffer mode.)

Programming Note

The request byte count (Output X'62', byte 1, bits 6-7) nust be set with the total number of valid data bytes loaded into data buffers 1-4 (by Outputs X'64' and X'65') to ensure data integrity.

Output X'66' (NSC Status Byte): This instruction sets bits in the NSC status register according to the contents of the general register specified by R. The Output X'66' instruction should be executed before signaling the CA that an NSC final status transfer sequence is required.

When the NSC final status transfer sequence occurs, the status byte provided by this output is presented to the channel. If the status byte has previously been given to the channel adapter (but was stacked by the channel), it need not be given again. This output may be used to present an asynchronous status (for example, Attention) or the final status, ending a channel I/O command in the native mode.

For Type 4 CA only: By executing an Output X'66' instruction in which bit 0.4 is 1 (after requesting and receiving a Type 4 CA program interrupt), the Type 4 CA will respond with a 'busy' status (X'10') to complete initial selection sequence and will not interrupt the control program. This status is presented at the end of a complete normal initial selection sequence. The control program must end this 'long busy' state by requesting and receiving a program interrupt and present 'device end' status via an NSC final status transfer sequence.

Inhibition of level 3 interrupts does not occur in the case of a system reset, an initial selection interface disconnect, or an initial selection selective reset condition. In these cases the Type 4 CA does request a level 3 interrupt.

Programming Note

For compatibility, the control program should ensure that the bits of the NSC status byte that are set are consistent with the bits set under similar conditions in the operation of the Type 2 Channel Adapter.

Output X'67' (CA Controls): This instruction causes various control latches to be set or reset in the channel adapter according to the bits in the general register specified by R.

If a 3705 is equipped with dual Type 4 CAs, the control program uses this instruction to designate which of the two CAs is to be selected. Bits 5 and 7 of byte 0 are used for this purpose, as follows:

00 - Leave the currently selected CA active 01 - Leave the currently selected CA active

10 - Select the first CA (in 3705 base module)

11 - Select the second CA (in 3705 expansion module)

Other functions specified by bit settings in the Output X'67' instruction apply to the CA selected in accordance with the value of bits 5 and 7 above.

9-14

Output X'67' may also be executed to set a program-requested interrupt in a specific one of the two CAs, without regard for whether that CA is the currently selected one. Bits 3 and 7 of byte 0 of the instruction are used for this purpose, thus:

10 - Perform this instruction for first CA (in 3705 base module)

11 - Perform this instruction for second CA (in 3705 expansion module)

Other functions specified by bit settings in the Output X'67' instruction apply to the CA specified, regardless of whether it or the other CA is currently selected. Whichever one is selected remains so.

Programming Note:

Bits 3 and 5 of byte 0 of an Output X'67' instruction must never both be 1.

Programming Note

The CA Diagnostic Reset (byte 0, bit 4) resets the channel adapter only if both interface A and interface B are disabled.

Output X'6C' (EB Mode Control Register): This instruction sets the control character recognition latches, the EB mode control bit, and the byte count bits of the Type 4 CA from the corresponding bits of the general register specified by R.

Output X'6D' (EB Mode Data Register): This instruction loads two data bytes from the register specified by R into the Type 4 CA EP mode data buffer. The first data byte (byte 0) of the general register is placed in the first (even) byte of the buffer; the second data byte (byte 1) of the register is placed in the second (odd) byte of the buffer.

Error Indications

The programmer should decide what specific action to take in attempting to recover from an error condition. The various ways to handle errors depend on the application and installation.

The following indications require error recovery procedures.

##### Channel Bus In Check

When the CA hardware detects bad parity in the byte to be sent across the I/O Channel Interface, it generates good parity for that byte and gates it onto the 'I/O channel interface' bus. The hardware than causes a CA level 1 interrupt. The control program should interrogate the condition register by executing an Input X'67' and record the error conditions that occurred. The control program should then reset the level 1 interrupt and end the command by presenting a CE, DE, UC status.

Channel Bus Out Check in EB Mode: If a bus out check occurs on an inbound data transfer sequence in EB mode, bad parity may have been written into the EB mode local store. The control program must re-establish good parity by executing successive Output X'6D' instructions as described in this chapter under Extended-Buffer Mode Local Store (Type 4 CA Only}. Failure to do so may cause a local store parity check level 1 interrupt to occur when the local store is next accessed.

##### In/Out Instruction Accept Check

An in/out instruction accept check is set when the control program executes an Input or Output X'60', X'61', X'62', X'63', X'64', X'65', or X'66' instruction when the CA is in the process of handling any data/status transfer sequence. A level 1 interrupt request is also set, and the instruction is prohibited from setting any hardware control latches. The control program should interrogate the condition register by executing an Input X'67' and record the error indication. An Output X'67' should be issued to reset the LI interrupt request and the 'in/out instruction accept check' latch.

I/O Check on Input or Output X'6D' Instruction in EB Mode: If an I/O check occurs during execution of an Input or Output X'6D' instruction while the CA is in EB mode, the entire data transfer sequence must be re-executed. It is not sufficient simply to re-execute the instruction on which the error occurred. If the error occurred on an outbound sequence, the Output X'6C' instruction must be re-executed and the EB mode local store re-loaded from the beginning with successive Output X'6D' instructions.

If the error occurred on an inbound sequence, the Input X'6C' instruction must be re-executed and the EB mode local store re-accessed from the beginning with successive Input X'6D' instructions.

##### CCU Outbus Check

When the CA hardware detects bad parity on the CCU Outbus, it sets the 'CCU Outbus check' latch, causes a LI interrupt, and prohibits reselection on the channel interface. The control program should interrogate the condition register by executing an Input X'67' and record the error indication. The control program should then issue an Output X'67' to reset the LI interrupt and the 'CCU Outbus check' latches. If the CA was active on the channel in a data/status state when the error occurred, the hardware terminates the transfer and causes a CA data/status L3 interrupt request. The control program must reissue the output instructions to allow reselection to a channel interface.

IBM 3704 and 3705 Communications Controllers Principels of Operation  9-15

CCU Out Bus Check on Output X'6D' Instruction in EB Mode: If a CCU Out Bus Check level 1 interrupt occurs on an Output X'6D' instruction when the CA is in EB mode, the entire data transfer sequence must be re-executed. It is not sufficient simply to re-execute the instruction on which the error occurred. The Output X'6C' instruction must be re-executed and the EB mode local store reloaded from the beginning with successive Output X'6D' instructions.

##### Local Store Parity Check

When the hardware detects bad parity on data bytes being gated out of local store registers, it causes a level 1 interrupt. The control program should interrogate the condition register by executing an Input X'67' and record the error indication. The control program should then issue an Output X'67' to reset the LI interrupt request and end the command by presenting CE, DE, UC status and equipment check sense.

9-16