### Chapter 6: Type 1 Communication Scanner

This chapter is intended to give the reader a basic understanding of the operation of the Type 1 Communication Scanner and the requirements necessary to program the scanner.

The Type 1 Communication Scanner (Type 1 Scanner) permits the control program to communicate with a line or autocall interface. This communication is done through the use of input and output instructions executed by the control program to the interface addressed by the communication scanner. The Type 1 Scanner makes data, status, and control information pertaining to each of the installed communication lines available to the program.

Most of the scanner functions are under direct control of the level 2 interrupt program and of processing routines that may be in other program levels. This permits increased flexibility by decreasing the number of restrictions caused by requirements of fixed hardware. The control program must assume the responsibility of the assembly and disassembly of characters, control character recognition, translation, and line control. Character assembly and disassembly are required of the program because the Type 1 Scanner transfers only one information bit at a time to or from the interface.

The Type 1 Scanner differs slightly, depending on whether it is installed in a 3704 or a 3705. In a 3705, the scanner supports four LIBs with a maximum of 64 lines. When installed in a 3704, the Type 1 Scanner supports two LIBs with a maximum of 32 lines.

### Operation and Data Flow

The Type 1 Communication Scanner hardware operates asynchronously with the other functional components of the controller. See Figure 6-1 for the Type 1 Scanner general hardware operation.

A scan counter sequentially addresses each interface in search of a service request. If a bit service request is detected, the scanner stops on that interface and requests a level 2 interrupt. All data and information for that interface is then made available to the control program through input instructions. When the program executes the proper output instructions, the scanner hardware passes information to the interface hardware.

When a character service request is detected, the control program is notified by a level 2 interrupt request. The scanner does not stop for a character service interrupt, but continues generating addresses in search of a bit service request.

Programming for the Type 1 Scanner should normally be done in the level 2 interrupt program because

Figure 6-1. Type 1 Communication Scanner Operation

IBM 3704 and 3705 Communications Controllers Principles of Operation  6-1

of the high priority placed on communication lines. However, the transferred bits and characters may be processed in a lower priority program level. See Figure 6-2 for a general flow of the Type 1 Scanner level 2 program.

When a level 2 interrupt occurs, an Input X'41' (interface address) instruction should be the first instruction executed to determine the cause of the interrupt. A branch can then be taken to the bit service or character service routine to handle the request.

Figure 6-2. Level 2 Program Flow (Type 1 Communication Scanner)

The bit service routine determines if a bit has actually been received or must be transmitted. If a bit has been received, the routine places the received bit in the proper storage location allocated for the character to be received. If a bit is to be transmitted, the routine must remove a bit from the character and pass it to the interface. When the last bit of a character has been received or transmitted, the bit service routine must request a character service interrupt from the scanner hardware.

Before exiting the level 2 bit service routine, the program must restart the scanner by executing the proper output instruction (Output X'41' or Output X'46').

### Interface Scanning

All interface addresses (both used and unused) assigned to the LIB positions are scanned sequentially. These lines are physically addressed by stepping a scan counter that generates an interface address. As the scan counter steps through each interface address, a check is made to see if that interface has a request for service. If a service request is present, the scanner checks the service priority and mode of the interface. If interrupts are allowed, the scanner determines whether the request is for bit service or character service. If bit service is required, the scanner stops on the interface and causes a level 2 interrupt request. If the request is for character service, the scanner signals the control program by requesting a level 2 interrupt.

When the bit service routine has obtained all necessary information pertaining to that line, it must execute an Output X'41' (or Output X'46' to request character service). These output instructions reset the level 2 bit service interrupt request and restart the scan counter.

Note: The scan counter, when not stopped to allow a particular interface to be serviced, is stepped at a rate of 400 nanoseconds per interface in a 3705 and at a rate of 600 nanoseconds per interface in a 3704.

Bit-Service Priority

Higher-speed communication lines should be serviced more frequently than lower-speed lines. To do this, the Type 1 Scanner allows the program to assign one of two (high or low) bit service priorities to each interface. An Output X'42' (control A) instruction is used to set the service priority for a particular interface.

When an interface is scanned, its assigned service priority is checked to determine whether it has high or low priority. If the service priority is high, and the interface requires service on this scan, the scanner stops and causes a level 2 bit service interrupt request.

6-2

This permits the program to service that interface. A high-priority interface is serviced each time the line is scanned and bit service is required.

If the service priority is low, and the interface requires service, the scanner stops only if the interface being addressed is the first low-priority line encountered on that cycle of the scan counter. (Refer to Figure 6-3.) To simplify this concept, Figure 6-3 shows only the first ten lines, and all are assumed to have a bit service request each time the scanner addresses them. Also assume for this example that a cycle is one complete scan of all interface addresses.

In a 3705, when the scanner addresses the first low priority interface requiring service and the control program executes an Input X'41' instruction, the priority counter is reset to zero. The priority counter then adds a one to its count for each interface scanned (high or low priority) following the reset. As the scanner encounters the next and any other low priority interface, the priority counter is checked to see if a low priority interrupt is allowed. Low priority interrupts are not allowed until the counter reaches 65. When the scanner stops on the next low-priority interface and the control program executes an Input X'41' instruction, the priority counter is reset to zero and again, low-priority interrupts are not allowed until the counter reaches 65.

In a 3704, when a service request from a low priority line is honored by the scanner, that line address is stored in the address remember register and the 'accept low priority' latch is reset. When the scanner is restarted by an Output X'41' instruction, it will continue to honor service requests from lines assigned high priority but will service no further low priority lines until the address in the scan counter is one greater than that in the address remember register, at which time the 'accept low priority' latch is set again. The scanner has then passed the address that caused the last low priority bit service interrupt and is able to stop on the next low priority line requesting service.

Programming Note

All interfaces are set to high priority by a power-on-reset, IPL, machine reset, or a scanner disable (Output X'45' byte 0, bit 2).

Interrupt Requests

The Type 1 Scanner can initiate interrupt requests for either level 1 or level 2 service. Level 1 requests occur when the scanner detects an error condition affecting interface or scanner operation. Two different types of level 2 interrupt requests can occur for the purpose of handling normal service requests: the level 2 bit service request and the level 2 character service request.

The following paragraphs describe the bit service and character service interrupt requests.

Bit Service Interrupt Request

The Type 1 Scanner level 2 bit service interrupt request occurs when the scanner stops on the address of an interface requesting service. Once the scanner has stopped on an interface, the program can identify the interface by executing an Input X'41' (interface address) instruction. Input X'41' loads a general register with the storage address associated with the interface requesting service. (See Figure 3-5 to determine the interface address and storage address association.) With the scanner stopped and the interrupting interface identified, a bit service routine can gain access to various control latches and status signals in the interface hardware to allow servicing of the bit request.

When enough information has been exchanged between the program and the interface hardware, the program must execute an output instruction to restart the scanner. An Output X'41' instruction is used to reset the interrupt request and start the scanner when the bit received is not the last bit of a character. An Output X'46' instruction sets the character service pending indication in addition to resetting the interrupt request and starting the scanner when character service is required.

Programming Note

The instruction to start the scanner should be executed as early as possible in the bit servicing program so the scanner delay (the time required to address another interface) can be overlapped with bit service processing.

Character Service Interrupt Request

When a bit service routine detects that character service is required, it should execute an Output X'46' instruction to set the 'character service pending' latch, reset the level 2 bit service interrupt request, and start the scanner.

Once the 'character service pending' latch is set, the scanner allows a character service interrupt request to be set when either of the following conditions is satisfied.

1. The scanner passes four enabled high-priority interfaces that do not request bit service.

2. The scanner makes four complete cycles, addressing all interfaces without detecting a bit service request.

When a character service interrupt is set, the address provided by the Input X'41' instruction (interface address) is forced to X'06FO'. This address identifies the interrupt as being caused by a character service request and can be used to direct

IBM 3704 and 3705 Communications Controllers Principles of Operation  6-3

| Line Number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Priority | H | H | L | H | L | H | H | L | L | H |
| 1st Cycle | X | X | X* | X |  | X | X |  |  | X |
| 2nd Cycle | X | X | ** | X | X* | X | X |  |  | X |
| 3rd Cycle | X | X |  | X | ** | X | X | X* |  | X |
| 4th Cycle | X | X |  | X |  | X | X | ** | X* | X |
| 5th Cycle | X | X |  | X |  | X | X |  | ** | X |
| 6th Cycle | X | X | X* | X |  | X | X |  |  | X |

X = Request honored.

* = Priority circuits prevent further low priority interrupts.

** = Priority circuits allow low priority interrupts.

Figure 6-3. Interfaces Serviced According to Priority

6-4

the control program to a character service routine.

The Type 1 Scanner has only one 'character service pending' latch; therefore, the control program should ensure that this latch remains set until the character service routine processes all pending character service requests. The control program should then execute an Output X'44' instruction with byte 1, bit 4 set to 1 to reset the 'character service pending' latch and the level 2 interrupt request.

Programming Note

A character service interrupt request does not stop the scanner; therefore, during the processing of a character, a bit service interrupt request can also be set. However, the request is not honored until an Exit instruction is executed in level 2 character service.

### Scanner and Interface Initialization

Initialization is required to place the Type 1 Scanner and the attached interfaces into the proper mode for operation. This must be done any time the Type 1 Scanner has become disabled so that all interfaces have been reset and no interrupts occur.

Conditions such as 'power on reset' and 'initial program load' cause the Type 1 Scanner and the attached interfaces to be disabled. The control program can also cause the same condition by executing an Output X'45' instruction with byte 0, bit 2 set to 1.

In the reset or disabled state, the scanner and all interfaces are reset to a nonoperational state. The scan counter continues to run, but no interrupts are permitted, and all interfaces are set to high bit service priority. A minimum delay of 30 microseconds is required before the program can enable the scanner. The control program can enable the scanner and all the LIB positions by executing an Output X'45' instruction with byte 0, bit 1 set to 1 and byte 0, bits 4-7 set to 0.

Each interface address to be used for communications must be initialized. Once the scanner has been enabled, an individual interface may be enabled or disabled at any time.

When an interface is disabled, normal service interrupts for that interface are prohibited. To enable an interface, the control program must execute an Output X'47' (force bit service) with the associated storage address set in byte 0, bits 6-7 and byte 1, bits 0-3. This causes the scanner to stop when it reaches that interface address and to set a level 2 interrupt request. With the scanner stopped, the interface can be initialized for operation with an Output X'42' (control A) instruction.

The Output X'42' is a control instruction and includes (1) setting the interface mode, (2) selecting high or low priority, (3) selecting start-stop or

synchronous/business machine or modem clocking, (4) selecting data rate, and (5) selecting an oscillator. (See Appendix B for Output X'42' bit definitions.) When the mode bits (byte 0, bits 6-7) are set to a nonzero value, the interface is enabled to accept interrupts. (See Interface Modes of Operation in this chapter.) The interface is then ready to send or receive data. If the mode is set to 11, an interrupt request is set for the interface at each bit interval when that interface is addressed by the scan counter.

### Programming Considerations

When the control program has a record or message to send to a teleprocessing device, it may request a bit service interrupt by executing an Output X'47' instruction containing the storage address associated with that line interface. This causes that interface to set a level 2 bit service request. The next time the scanner services this interface, it stops, and the level 2 interrupt is requested. The bit service routine must then disassemble the character to be sent and execute a series of output instructions to transmit the bits. For each bit to be transmitted, the program must execute an Output X'43' (control B) instruction with byte 1, bit 4 set to 1 to indicate a transmit operation, and byte 1, bit 7 set to the proper bit value (mark or space). The Output X'43' is executed once each time a level 2 interrupt occurs for the particular interface until the complete character is transmitted.

### Interface Modes of Operation

The Type 1 Scanner provides four modes of operation for the interface. The modes are set by the combination of bits 6 and 7 of byte 0 in the Output X'42' instruction. The setting of these bits are:

| Bit 6 | Bit 7 | Mode |
| --- | --- | --- |
| 0 | 0 | Level 2 interrupts disabled. |
| 0 | 1 | Monitor ring-indicator or |
|  |  | data-set-ready. |
| 1 | 0 | Monitor Receive-Data-Space. |
| 1 | 1 | Allow level 2 interrupts. |

In the disabled mode (00), no interrupts are allowed from the interface until an Output X'47' (force bit service request) is executed or mode bit override (Output X'40') is set. This generates the first bit service interrupt request. When the interrupt occurs, the mode can then be changed to allow interrupts (11) for normal operation. This applies to all transmit and receive operations.

The monitor ring-indicator or data-set-ready mode (01) allows the scanner to monitor for ring-indicator or data-set-ready on a low-priority interface without

IBM 3704 and 3705 Communications Controllers Principles of Operation  6-5

causing an interrupt each bit time. This mode is valid only when the line is set to low priority. When either the ring-indicator lead or the data-set-ready lead becomes active, a bit service interrupt is requested. When the scanner stops on this interface to honor the request, the program should set the mode to allow interrupts (11) for normal operation. When the program has determined that the interface has completed the required action, the mode should be returned to a monitor mode to wait for the next request.

If the interface service priority is high, that interface is disabled from causing a level 2 interrupt in the monitor ring-indicator or data-set-ready mode. A force bit service (Output X'47') or a diagnostic bit service (Output X'44' byte 1, bit 0) is required to override the mode setting and cause a level 2 interrupt.

The monitor receive-data-space mode (10) allows the interface to remain idle in a 'mark' state without causing an interrupt each bit time. However, when the line changes to a 'space' level, signaling the start of activity, a bit service is requested. When the scanner stops on this interface to honor the request, the program should set the allow interrupt mode (11) for normal operation. When the program has determined that the interface has completed the required action, the mode should be returned to a monitor mode to wait for the next request.

The allow interrupt mode (11) permits interrupt requests to be serviced at each bit interval when the scanner addresses that interface.

The mode bit override function is initiated by executing an Output X'40' instruction. Setting the 'mode bit override' latch overrides all mode settings except mode 01 at high-priority. When this latch is set, all interfaces set to any mode other than 01 and to high-priority request a level 2 interrupt each bit-interval regardless of that interface's mode setting. This function allows the control program to force bit service on multiple interfaces without requiring an Output X'47' instruction for each interface address.

Programming Note for a 3705

When in the monitor ring-indicator or data-set-ready mode, the scanner hardware does not service bit service requests. Therefore, a bit overrun/underrun may be indicated when a level 2 interrupt is forced. The bit overrun/underrun indication should be ignored and reset by the control program. When in the monitor for receive data space mode, the hardware services bits while monitoring for a space. Therefore, the control program should always test for a bit overrun/underrun indication.

The Type 1 Scanner hardware services interface bit service requests in accordance with the state of the

'enable scanner' latch and the mode bit setting for each line. When a bit service request is to be ignored, as in the case of mode 01 or 10 conditions not being satisfied, or the line interface or the entire scanner being disabled, the interface service request is reset by the scanner hardware before the next line is scanned. Lines attached through a disabled LIB cannot be serviced by the program or hardware.

Unless its LIB is disabled, a line interface bit overrun indication (Input X'43', byte 1, bit 7) is always valid regardless of the setting of the interface mode bits or the 'enable scanner' latch. When a LIB is disabled, all lines attached through it will overrun until the LIB is reenabled, and indications of overrun should be ignored.

### Business Machine Clocks

The Type 1 Scanner must have at least one business machine clock installed and may have as many as four. If modem clocking is used with any of the lines, a business machine clock must be installed in the scanner with a speed less than one-half that of the lowest speed modem clock. Figure 6-4 lists the business machine clocks available.

| Clock Speed | Power On Warm Up Period (Seconds) |
| --- | --- |
| 45.5 bps | 5 |
| 50.0 bps | 4 |
| 56.89 bps | 20 |
| 74.2 bps | 5 |
| 75.0 bps | 5 |
| 100.0 bps | 4 |
| 110.0 bps | 3 |
| 134.5 bps | 2 |
| 150.0bps | less than 1 |
| 200.0 bps | less than 1 |
| 300.0 bps | less than 1 |
| 600.0 bps | less than 1 |
| 950.0 bps | less than 1 |
| 1200.0bps | less than 1 |
| 2000.0 bps | less than 1 |
| 2400.0 bps | less than 1 |

Figure 6-4. Business Machine Clocks and Warm Up Period

For line speeds greater than 2400 bps, an external modem must provide the clock pulses. Some line sets can operate with a business machine clock or a modem clock and some can operate only with a business machine clock. Refer to the Introduction to the IBM 3704 and 3705 manual for a description

6-6

of the individual LIB and line set types and their clocking requirements.

The installed business machine clock used for a given line is selected under program control by executing an Output X'42' (control A) instruction with byte 1, bits 6 and 7 set to indicate the desired clock. Figure 6-5 shows the proper setting of the oscillator select bits to assign an installed oscillator to a given interface.

| Bit |  |  |  |
| --- | --- | --- | --- |
| 6 | 7 | Selected Business Machine | Clock |
| 0 | 0 | Lowest speed clock | (OSC0) |
| 0 | 1 | Next higher speed clock | (OSC1) |
| 1 | 0 | Next higher speed clock | (OSC2) |
| 1 | 1 | Highest speed clock | (OSC3) |

Figure 6-5. Type 1 Scanner Business Machine Clock Selection

No business machine clock is selected if the Output X'42' bits are set to select an uninstalled oscillator (for example, bits 6 and 7 set to 11 when only two or three oscillators are installed).

Every interface must have a business machine clock assigned whether it is specified to be business machine or modem clocked. For autocall interfaces and for line interfaces that are to use modem clocking, the assigned business machine clock is used to ensure that the interface is periodically accessed. The lowest speed oscillator is always used for an autocall interface.

The oscillator select bits are set to 0 by a reset to the scanner. Therefore, the lowest speed clock is initially selected, and unless an Output X'42' is executed to select another clock for a given interface, the lowest speed clock is used.

After a power-on reset occurs, there is a warm-up period associated with the different clocks. (Refer to Figure 6-4.) During this warm-up period, a business machine clock cannot provide bit service requests.

- Programming Notes

1. The oscillator select bits for a line interface can be changed without causing a switched network connection to be broken, if data terminal ready is up when the Output X'42' is executed.

2. The business machine clock selected for a modem-clocked line interface must be less than one-half the rate of the modem clock.

### Autocall Interface Operation

Programming the Type 1 Scanner for an autocall interface is essentially the same as for a line interface. The major difference is in the bit settings of Input instruc

tions X'42' and X'43' and Output instructions X'42' and X'43'. Appendix B defines the affected instructions and the differences in the bit definitions.

### Input/Output Instructions

The Type 1 Communication Scanner and the line interface bases it supports are controlled through the use of input and output instructions. These instructions allow the program to (1) control the attached interfaces, (2) send and receive data, and (3) monitor the status of the scanner and line interfaces to ensure proper operation. Appendix B defines the bits within each input and output instruction.

Certain input and output instructions should be executed only when the scanner is stopped. If an Input X'41', X'42', or X'43' instruction is executed when the scanner is running, the result in the general register specified in the instruction pertains to the interface addressed at the time of execution. However, there is no way to determine which interface was addressed. If an Output X'41', X'42', X'43' is executed when the scanner is running, the mode or status of an unknown interface may be changed, and an error condition may result. Input X'41' and X'43' may be executed when a character service interrupt is pending with the scanner running.

- Input Instructions

Four functional input instructions (Input X'41', X'42', X'43', and X'44') are used with the Type 1 Communication Scanner. With these instructions, the control program receives data from the lines, monitors the status of the lines, and is informed of any error conditions. Execution of Input instructions X'40', X'45', X'46', and X'47' sets the bits in the general register to zero. (Appendix B defines the bits within each input instruction.)

Input X'41' (Interface Address): This instruction loads a general register with the storage address associated with the interface the scanner is addressing. Each time the scanner stops because of a bit service interrupt, this input may be issued to determine which interface caused the request.

If the level 2 interrupt is the result of a character service request, the address loaded into the register will be X'06FO'. This address identifies the interrupt as being caused by a character service request and can be used to direct the control program to a character service routine.

Programming Note

An Input X'41' instruction should normally be the first scanner instruction executed in program level 2.

IBM 3704 and 3705 Communications Controllers Principles of Operation  6-7

Input X'42' (Control A): This instruction loads a general register with the 'control A' information as set by an Output X'42' instruction. An Input X'42' instruction is used to check the state of the control A bits. It is a direct bit-for-bit reflection of the last Output X'42' instruction to that interface.

The Input X'42' instruction should be executed only when the scanner is stopped.

Input X'43' (Control B/C): This instruction can be executed in either a bit service or a character service routine. However, an Input X'41' instruction must be executed between the level 2 interrupt and execution of the Input X'43' instruction.

Executing an Input X'43' instruction as a result of a bit service interrupt loads a general register with the control B/C information. Data received from the interface, along with error information and line status, is set for the interface that caused the service request. If an Input X'43' is executed as the result of a character service interrupt, the specified general register (R) will contain X'0000'.

An Input X'43' instruction must not be executed if there is not a bit service or character service level 2 interrupt pending. Execution without an interrupt pending may cause a CCU In/Out level 1 check due to incorrect input parity.

- Programming Note

This instruction cannot be executed immediately following an Output X'43' for feedback checking. However, it can be executed after Output X'43' to obtain other status indications.

Input X'44' (Status Register): This instruction loads a general register with the contents of the Type 1 Scanner status register. This register contains (1) indications of level 1 check interrupt requests from the scanner, (2) the scanner enable/disable condition, (3) character service pending information, and (4) mode bit override and override remember indications.

- Output Instructions

Eight output instructions are used with the Type 1 Communication Scanner to control its operation and data transfer. (Appendix B defines the bits within each output instruction.)

Output X'40' (Set Mode Bit Override and Override Remember): This instruction sets the 'mode bit override' latch and the 'override remember' latch. The function of the, 'mode bit override' latch is described in the Interface Modes of Operation section of this chapter. The 'override remember' latch performs no

hardware function but is available for use by the control program. Since this instruction performs a function, the bit settings of the register are ignored.

Output X'41' (Reset Bit Service Level 2 Request and Start Scanner): This instruction resets the level 2 bit service interrupt request and starts the scanner. The bit service routine should issue this output after determining that all the information required to service the interface has been obtained or sent. An Output X'41' instruction should be the last instruction executed in the level 2 routine before exiting to a lower priority level for continued bit service processing. Since this instruction performs a function, the bit settings of the register are ignored.

The Output X'41' instruction should be executed only when the scanner is stopped.

Output X'42' (Control A): This instruction sets the mode of an interface according to the bit setting of the specified general register. In addition to setting the mode, it can also set service priority, clocking, data rate, oscillator selection, and diagnostic mode for the interface.

The Output X'42' instruction should be executed only when the scanner is stopped.

Output X'43' (Control B): This instruction sets the interface into a transmit or receive mode and can activate or deactivate various line and autocall-interface leads as required. When the line is transmitting, byte 1, bit 7 of the general register must be loaded with the mark or space to be sent to the interface terminal.

The Output X'43' instruction should be executed only when the scanner is stopped.

Output X'44' (General Control): This instruction sets or resets the diagnostic bit service request and resets (1) the mode bit override and override remember, (2) character service pending, and (3) outstanding error indications.

Output X'45' (Scanner Control): This instruction enables or disables the scanner and/or the line interface bases (LIBs). Normally, this instruction is used only during initialization or when an error has occurred that requires a LIB to be disabled.

- Programming Note

Output X'45' can disable interrupts from any LIB attached to the Type 1 Scanner. This can be particularly useful when a bit-clock error occurs in a LIB and causes a level 1 interrupt. The failing LIB, including all lines attached to that LIB, can be disabled to prevent further error conditions while the remaining LIBs continue normal operation.

6-8

Output X'46' (Set Character Service Pending, Start Scanner, and Reset Level 2 Bit Service Request): This instruction is normally used at the end of bit service processing when character service is required for that interface- It sets the 'character service pending' latch to signal the scanner that a character service interrupt is required. The instruction then resets the level 2 bit service interrupt request and starts the scanner. Since this instruction performs a function, the bit settings of the register are ignored.

The Output X'46' instruction should be executed only when the scanner is stopped.

Output X'47' (Force Bit Service Request): This instruction forces a bit service interrupt request for the interface address that is specified in the general register. This instruction stops the scanner on an interface and requests a bit service interrupt so that the program can enable the interface or access it to transmit a bit.

###### Programming Notes

1. Forced bit service cannot be stacked. If an Output X'47' is executed before the previous Output X'47' has been serviced, the second address overlays the first.

2. When bit service is forced to a line interface attached to a LIB that has been disabled, the line adapter is not accessed even though the scanner is stopped at that interface address. Also, except for the mode bits and the feedback check bit, all bit settings in Inputs X'42' and X'43' may be invalid. The execution of Output X'42' or X'43' will either be ineffective or cause scanner checks.

### Error Indications

Error conditions detected by the Type 1 Scanner are in one of two groups, depending on the type of error and the impact on the overall system operation. The first and most critical group (level 1 errors) causes a level 1 interrupt request and must be handled with high priority because the error may involve many lines. The second group (interface errors) can be handled at a lower-priority interrupt level because the errors have less system impact. The following paragraphs describe these errors and their detection.

- Level 1 Errors

Failures in the Type 1 Communications Scanner or in a line interface base can affect all communication lines attached to the controller or at least a group of lines within a particular LIB. The detection of one or more of these failures by the hardware check circuits causes a Type 1 Scanner LI interrupt request (Input X'76',

byte 0, bit 1). The level 1 interrupt routine, after determining that the interrupt request came from the Type 1 Scanner, should execute an Input X'44' (status register) instruction to further identify the error.

The error condition that caused a level 1 interrupt is indicated in byte 1, bits 2-7 of the Type 1 Scanner status register (Input X'44'). Bits 2-5 correspond to LIB positions 1-4 and are turned on respectively as a result of a LIB bit clock parity error. Bit 6 is turned on by the detection of a LIB select error. Bit 7 is turned on by a parity error on the CCU outbus, which is an internal interface between the Central Control Unit and the Type 1 Scanner. See Appendix B, Input X'44' for a description of these bits and the error conditions.

If the level 1 interrupt routine can handle the error condition and processing can continue, the routine should then issue an Output X'44' instruction with byte 1, bit 5 on to reset the level 1 request. When the error is permanent, the LIB can be disabled via an Output X'45' so that processing can continue on the remaining LIBs.

- Interface Errors

Line interface errors indicate intermittent or permanent internal logic faults and most problems with communication facilities. The failure is detected at the interface level, but if failures are detected in a group of interfaces, the fault may be in either the (1) LIB logic, (2) Type 1 Scanner logic, (3) CCU input/output mechanism, or (4) program logic.

Interface errors normally are not critical enough to interrupt the entire system. Therefore, the Type 1 Scanner does not generate a level 1 interrupt when this type of error is detected. Instead, certain bits are set in the control B/C register to indicate the failure. By issuing an Input X'43' instruction when an interface requests service, the program can test for error conditions on that line without disrupting normal processing.

Byte 0, bit 2 of the Input X'43' instruction is a partial summary of interface errors. This interface-error summary bit is set on whenever the Type 1 Scanner detects (1) a feedback check, (2) bit overrun or under-run, or (3) that the 'data set ready' line is not up. If the control program checks this bit first, considerable time can be saved in detecting errors.

A feedback check (byte 0, bit 1) is set on when the hardware circuits detect that the bit actually sent to the line set does not compare to the bit as it appears in byte 1, bit 7 of Output X'43'. This error is also set if the interface bit service failed to be reset. A feedback check must be reset before the scanner can be restarted.

Bit overrun/underrun (byte 1, bit 7) is set when the Type 1 Scanner determines that a bit has been lost

IBM 3704 and 3705 Communications Controllers Principles of Operation  6-9

because of improper timings between the control program and the bit rate used by an interface.

Telegraph Echo Check (byte 1, bit 4) is set when the telegraph interface detects that an echo check has occurred.

In addition to monitoring byte 0, bit 2 of Input X'43', which is a summary of several error conditions, the bit servicing routine should also monitor byte 1, bit 3 of the same input. Monitoring these bits provides a higher level of security on switched lines. This only applies for start-stop, half-duplex units that require duplex facilities (for transmission interruption ability).

### Diagnostic Functions

The Type 1 Communication Scanner provides for three internal diagnostic functions: (1) diagnostic bit service, (2) diagnostic wrap mode, and (3) IBM modem wrap test. These tests run under the control of the scanner program and can provide online testing as described in the following sections. Diagnostic bit service can be issued to an autocall interface, but the diagnostic wrap and the modem wrap test cannot.

- Diagnostic Bit Service

The Type 1 Scanner diagnostic bit service provides a means for forcing level 2 bit service interrupt requests. This facility allows the control program, through the use of a diagnostic routine, to exercise program and/or hardware functions in a test environment. The diagnostic routine performed must be part of the control program. Diagnostic bit service causes continuous level 2 bit service requests for all 64 interface addresses whether the interface is used or not. An Output X'44' instruction with byte 1, bit 0 set to 1 indicates the diagnostic bit service function. When an Output X'44' is executed with this bit off (0), the diagnostic requests are terminated.

- Diagnostic Wrap Mode

The Type 1 Scanner diagnostic wrap provides a means of testing and locating defects in the line control logic and in the line-interface transmit and receive logic. Diagnostic wrap can be performed online without affecting normal program operation or the lines not in diagnostic mode. The test requires one line interface to act as a transmit line and one or more line interfaces to act as receive lines. Any line can be a transmit or a receive line; however, only one diagnostic wrap transmit line may be present at any one time.

Diagnostic wrap is initiated by executing an Output X'42' instruction to each line to be tested with the following bits set in the register specified by the R operand.

Byte 0, bits 6-7: (Mode Bits 1 and 2)—These bits select the appropriate bit setting for the desired mode. See Interface Modes of Operation in this chapter for the mode options.

Byte 1, bit 0: (Bit Service Priority)—This bit selects the appropriate service priority. See Output X'42' for service priority options.

Byte 1, bit 1: (Diagnostic Mode)—This bit must be 1.

Byte 1, bit 2: (Data Terminal Ready)—This bit must be 0.

Byte 1, bit 3: (Synchronous Clock)—This bit must be set according to the type of communication line to be tested. A 1 is set in this position for binary synchronous lines, and a 0 for start-stop lines.

Byte 1, bit 4: (External Clock)—This bit must be 0.

Byte 1, bit 5: (Data Rate Select)—This bit may be either 0 or 1. However, the same type line sets must use the same data rate.

Byte 1, bits 6-7: (Oscillator Select 1 & 2)—These bits select an available line oscillator (business machine clock). For bit clock options, see Business Machine Clocks in this chapter. All wrap-test lines must select the same oscillator.

After the Output X'42' instructions are executed, the affected lines can be used through any sequence of point-to-point or multipoint operations.

Diagnostic wrap mode simulates 'data set ready' as active. 'Clear to send' is simulated active if 'request to send' is active.

Programming Notes

1. Only one line may be in a diagnostic wrap transmit state at any given time during the operation.

2. The line used for transmit should be the last line to be issued the Output X'42' instruction.

- Modem Wrap Test

Modem wrap test tests the scrambler circuits of IBM 3872, 3874, and 3875 modems under program control. The modem test can be performed online without affecting the normal operation of other lines. This test may also be performed simultaneously on any or all lines for which the test function is initiated.

Modem wrap test is initiated by executing an Output X'42' (control A) instruction with the following bits set in the register specified by the R operand to each line to be tested.

6-10

Byte 0, bits 6-7: (Mode Bits 1 and 2)—These bits select the appropriate bit setting for the desired mode. See Interface Modes of Operation in this chapter for mode options.

Byte 1, bit 0: (Bit Service Priority)—This bit selects the appropriate service priority. See Output X'42' for service priority options.

Byte 1, bit 1: (Diagnostic Mode)—This bit must be 1.

Byte 1, bit 2: (Data Terminal Ready)—This bit must be 1 to cause the 'data terminal ready' latch to be set in the line interface. When this bit, together with diagnostic mode, is set on, the modem wrap test is performed instead of the diagnostic wrap test.

Byte 1, bit 3: (Synchronous Clock)—This bit must be 0.

Byte 1, bit 4: (External Clock)—This bit must be 1 if the modem provides the clock, and it must be 0 if the modem does not provide the clock.

Byte 1, bit 5: (Data Rate Select)—This bit may be either 0 or 1.

Byte 1, bits 6-7: (Oscillator Select 1 and 2)—These bits must select an internal oscillator whose speed is less then one-half the clock speed if modem clocking is used. If modem clocking is not used, the internal oscillator must match the modem speed. For the proper setting, see Business Machine Clocks in this chapter.

IBM 3704 and 3705 Communications Controllers Principles of Operation  6-11