### Chapter 10: Type 2 and Type 3 Channel Adapters

This chapter describes the basic operation of the Type 2 and Type 3 Channel Adapters and the requirements necessary to program these adapters.

The Type 2 Channel Adapter (Type 2 CA) and the Type 3 Channel Adapter (Type 3 CA) are high-performance, cycle-stealing adapters that support instantaneous channel data transfer rates of up to 276 kilobytes per second.

The Type 2 and Type 3 CAs are minimally dependent on the control program. Data transfer and control word chaining are handled without control program intervention. The Type 2 or Type 3 CA notifies the control program when a data transfer is complete with an "I/O interrupt".

The Type 3 CA has all the capabilities of the Type 2 CA, and appears to the 3705 control program as a Type 2 CA. In addition the Type 3 CA enables the 3705 to be attached to tightly-coupled multiprocessors as a symmetrically shared I/O unit. This means that the 3705 can be attached to two CPUs of a multiprocessor through one Type 3 Channel Adapter. The 3705 appears as the same I/O unit to each processor, and can be sequentially accessed in exactly the same manner by each processor. This feature allows the CPU access methods for the 3705 to run in either CPU without having to perform a "shoulder-tap" interrupt to the alternate CPU in order to perform an I/O operation to the 3705. The Type 3 CA can also provide an alternate path capability for uniprocessors.

Both channel interfaces of the Type 3 CA can be concurrently enabled, but concurrent data transfer is not permitted.

Because the Type 2 and Type 3 CAs are similar in operation, only the differences are identified as pertaining to the Type 2 CA or Type 3 CA. Otherwise, the term channel adapter or CA refers to both the Type 2 and Type 3 CA.

Operation and Data Flow

A host processor channel command is received by the CA, decoded, and placed into the command register. If the operation is a data transfer type (Read or Write), the CA checks the validity of the control word (CW) address contained in the inbound or outbound CWAR (control word address register). (See the Channel Adapter Control Word section in this chapter.) A valid control word address should have previously been placed into the CWARs by an output instruction from the control program.

If the address in the appropriate CWAR is not valid, Unit Exception initial status is returned to the channel, and no data transfer is attempted. If the

CWAR is valid, a cycle-steal operation places the information from the first halfword of the CW into the CA. The information from this cycle-steal operation loads the command and flag fields of the CW into the CW command/flag registers and loads the count register. In addition, the first two bits of the starting address of the storage area where data is to be read from, or written to, are placed into the cycle-steal address register (CSAR). Another cycle steal operation places the information from the second (and final) halfword of the CW into the CA. The second halfword, containing the remainder of the starting data address, is placed into the CSAR.

For 3705-to-channel data transfer, a halfword is transferred via cycle steal from storage to the data 1 and data 2 registers and then sent to the channel, a byte at a time.

For channel-to-3705 data transfer, a byte at a time is received from the channel and alternately loaded into the data 1 and data 2 registers. The contents of both registers (one halfword total) are then transf erred to storage via a cycle steal using the CSAR address.

Each byte tranferred to or from the channel causes the count register to be decremented by one. Each halfword transferred to or from storage causes CSAR to be incremented by two in order to address the next halfword storage location.

When the count register is equal to 0, the data transfer for that CW is complete, and the next sequential CW is transferred to the CA if CW chaining is indicated. Otherwise, the operation ends by causing a level 3 interrupt after presentation of CE status.

On the byte-multiplexer channel, the CA disconnects from the host processor channel after two bytes have been transferred in burst mode across the channel. After a CA-to-storage cycle-steal operation, the CA reconnects to the channel to transfer another two bytes.

On selector and block-multiplexer channels, the total data transfer, from Initial Selection to Channel End status, is in burst mode.

Note: The channel adapter should not be on line if the controller is in a program stop state. The code necessary to handle adapter interrupts is not running when in this state and any interrogation of the adapter across the channel interface will receive Busy if an interrupt request is present.

IBM 3704 and 3705 Communications Controllers Principles of Operation  10-1

Cycle Steal Operation

To relieve the system control program of the responsibility of transferring data and information between the channel adapter and storage, the CA uses cycle steal. Under hardware control, the CA "steals" machine cycles from the Central Control Unit. Cycle steal allows overlap of channel operations with control operations. When the channel adapter needs data from storage or has data to be stored, it requests a cycle steal from the CCU. A CA cycle-steal request has the highest priority in the controller. Therefore, the CCU, at the end of the current machine cycle, permits the CA cycle-steal request to be serviced. Two bytes of information are transferred into or out of storage during each cycle steal operation.

Channel Adapter States

Because of the problems associated with the handling of two asynchronous communications—the Central Control Unit communication and the host processor channel interface—the CA must be in a certain state with respect to one before access is allowed from the other.

A thorough understanding of these states is required to write or modify a CA control program or a host processor access method. More than one of these states may be present at the same time.

• CA Active State - The channel adapter is in the process of transferring data or information across the host processor channel. The active state exists from the time the CA accepts a channel command during initial selection until the channel accepts Device End status for that command.

• CA Level 3 Interrupt State - This state may have been initiated by the control program or by the completion of either a control word or a channel command. If the CA is not in the active state, the control program can gain access to all CA registers except the channel sense and status registers. If the CA is in the active state, all registers are available to the control program. See Output X'57' in Appendix B for an additional restriction on the channel adapter mode register.

• CA Level 1 Interrupt State - The channel adapter enters this state only when an error condition is detected during the execution of an input or output instruction directed at the CA, during a cycle steal operation, or during transfer of information across the channel interface. If the channel adapter is executing a channel command when a hardware detected error occurs, the command is ended with a hardware generated Channel End and/or Device End, and Unit Check status. A level 1 interrupt is

requested by the adapter when the host processor accepts the ending status.

• CA Busy State - This state refers to the 'busy' status generation independent of the adapter's status register. This state exists when the CA is both active and engaged in an L1 or L3 interrupt and the channel attempts an initial selection sequence.

• Diagnostic Wrap State - This state is for diagnostic purposes and is entered when the control program executes an Output X'57' instruction with byte 1, bit 7 set to 1. In the diagnostic wrap state, the CA is forced to an offline status regardless of the position of the control panel Enable/Disable switch. All CA registers, including the IPL portion of Output X'57', are available to the control program to be used for test functions. Output instructions X'58' and X'5B' can be used to simulate the channel interface lines. The diagnostic wrap state remains in effect until Output X'57' is executed with byte 1, bit 7 off (0).

• Hard Stop State - This state is entered when the 'hard stop' latch is set in the Central Control Unit. The hard stop state causes the adapter to go offline, but the channel enable light on the control panel remains on. The channel adapter attempts to complete execution of any outstanding channel command by presenting CE-DE-UC, or DE-UC ending status. If the 3705 hard stops while the CA is active with a level 1 or level 3 interrupt request set, the hardware generated status will not be presented if the channel had 'suppress out' up. With a Type 3 CA, any initial selection attempt by the opposite channel receives a Busy initial status. Once 'suppress out' falls, the adapter raises 'request in' to present status, generated because of hard stop, to the interface over which data transfer was taking place, and goes offline when the channel accepts the status.

Test I/O loops, to clear suppressible status, receive a continuous Busy if the 3705 has hard stopped with the CA in the active state with an interrupt pending. Once a 3705 hard stop occurs, the adapter goes offline as soon as the using channel accepts ending status for an outstanding channel command. If there is a pending Device End due to a previous Busy status offered to the opposite channel (Type 3 CA), it will be presented to that channel. When that channel accepts Device End status, the adapter goes offline to it also.

Channel Adapter Registers

The CA contains 13 external registers that are used by the control program for normal operations. Input/output instructions (X'50' to X'5F')

10-2

are used to gain access to these registers. See Appendix B for I/O instruction bit position assignments.

With Extended Addressing, byte X, bits 6 and 7, are used in the cycle-steal address register to present a full 18-bit address to the Central Control Unit during a cycle-steal operation. This is the only register with more than 16 bits used by the CA. For input instructions, byte X contains zeros; for output instructions, it is ignored.

Inbound Data Control Word Address Register (INCWAR)

The INCWAR (inbound data control word address register) contains the storage address of the control word (CW) to be fetched by the CA cycle-steal hardware when a channel Write, Write Break, or Write IPL command is being executed.

The control program can load this register with an Output X'50' instruction or use it as input by executing an Input X'50' instruction. The CA recognizes this input/output instruction only after a CA has been selected and either has an interrupt pending or is in the diagnostic wrap mode.

The address in INCWAR is incremented by 4 (bytes) each time a CW fetch is executed by the CA cycle-steal hardware.

Programming Note

All CWs must begin on a halfword boundary and reside in the lower 64K bytes of storage. Execution of a CW chaining operation or a TIC (transfer in channel) control word to an address above 64K causes a CW error condition resulting in a CA level 1 interrupt with register X'56', byte 0, bit 0 set to 1.

Outbound Data Control Word Address Register (OUTCWAR)

The OUTCWAR (outbound data control word address register) contains the storage address of the control word (CW) to be fetched by the CA cycle-steal hardware when a channel Read command is being executed.

The control program can load this register with an Output X'51' instruction or use it as input by executing an Input X'51' instruction. The CA recognizes this input/output instruction only after a CA has been selected and either has an interrupt pending or is in the diagnostic wrap mode.

The address in OUTCWAR is incremented by 4 (bytes) each time a CW fetch is executed by the CA cycle-steal hardware.

Programming Note

All CWs must begin on a halfword boundary and reside in the lower 64K bytes of storage. Execution of a

CW chaining operation or a TIC (transfer in channel) control word to an address above 64K causes a CW error condition resulting in a CA level 1 interrupt with register X'56', byte 0, bit 0 set to 1.

Control Word Byte Count Register (CWCNTR)

At the completion of a control word fetch operation, the byte count register (CWCNTR) contains the value of the byte count that was loaded from the control word just fetched. As each byte is transferred across the channel interface, the CWCNTR is decremented by 1. When the count reaches zero, either CW chaining or a CA level 3 interrupt is initiated, depending upon the state of the chain flag and zero count override flag of the control word just ended.

The count contained in CWCNTR is precise and should be used by the control program to determine the location of the end of data in storage when a channel data transfer is complete.

The contents of CWCNTR are available to the control program by executing an Input X'52' instruction (control word byte count). The Input X'52' instruction can be recognized by the CA only after the CA has been selected and either has an interrupt pending or is in the diagnostic wrap mode.

Channel Adapter Sense Register (CASNSR)

The CA sense register (CASNSR) provides data for the channel Sense command. The CASNSR is one byte long and conforms to the System/3 70 standard definition of sense bits 0 through 4. Bits 6 and 7, however, are unique to the 3705. All bits can be used as input to the control program by an Input X'53' instruction (sense register). An Input X'53' or an Output X'53' instruction is recognized by the CA only after the CA has been selected and either is in the active state and has an interrupt pending, or is in diagnostic wrap mode. If Output X'53' is executed while the CA is in a CA interrupt state and the CA is not active, the output is ignored, and no indication is returned to the control program.

Programming Note

The setting of any CASNSR bit causes the Unit Check bit to be set in the CA status register and thus causes the termination of any data transfer that may have been in progress. CASNSR, with the exception of the not-initialized bit (bit 6), is reset during initial selection whenever the CA accepts a command other than Sense, Test I/O, or No-Op.

Channel Adapter Status Register (CASTR)

The channel adapter status register (CASTR) contains the standard System/370 status byte. The control program can load this register by executing an Output

IBM 3704 and 3705 Communications Controllers Principles of Operation  10-3

X'54' instruction (set status register bits) and can examine the register by executing an Input X'54' instruction (status register). This input/output instruction is recognized by the CA only after the CA has been selected, is in the active state, and has an interrupt request set or the adapter has been selected and is in the diagnostic wrap mode.

Channel Adapter Control Register (CACR)

The channel adapter control register (CACR) is a collection of control latches to be used by the control program when initiating or terminating a CA operation. The control program can set three of the latches in this register with an Output X'55' instruction (set control register bits) and can use it as input by executing an Input X'55' instruction (control register). These input/output instructions are recognized by the CA only after the CA has been selected and either has an interrupt request pending or is in the diagnostic wrap mode. The CA need not be in the CA active state.

Channel Adapter Check Register (CACHKR)

The channel adapter check register (CACHKR) is a collection of latches that cause the level 1 CA check interrupt request to be set. By examining this register, the level 1 interrupt program can determine what caused the CA level 1 interrupt. All of the latches in this register are automatically reset when an Output X'57' instruction is executed to reset the LI interrupt request. An exception to this is the bus out check that is reset when a channel command other than Sense, TIO, or No-Op is accepted by the adapter. The control program can use this register as input by executing an Input X'56' instruction (check register). This input instruction can be recognized only following the setting of a CA interrupt request and selecting of the CA.

Channel Adapter Mode Register (CAMR)

The channel adapter mode register (CAMR) can be loaded by the control program via Output X'57' (channel adapter mode register). When the controller is in the not-initialized state, the complete register can be accessed. When initialized, only byte 0, bit 3 and byte 1, bits 0-7 may be accessed.

When not-initialized, an Output X'57' instruction will set or reset IPL Unit Exception (byte 0, bit 6);

when initialized, this bit may be reset only. This register cannot be used as input to the control program.

Channel Bus Out Diagnostic Register (CBODR)

The primary function of the channel bus out diagnostic register (CBODR) is to simulate the channel interface 'bus out' when the CA is in the diagnostic wrap mode. The control program can load this register by execut

ing an Output X'58' instruction (channel bus out diagnostic register) and can use it as input by executing an Input X'58' instruction. Byte 0, bits 0-7 and byte 1, bit 0 of this register are accessible only when the CA is in the diagnostic wrap mode. Byte 1, bits 1-7 are accessible following the setting of a CA interrupt request.

Under normal operation (non-diagnostic mode) this input/output instruction is recognized only after the CA has been selected and has an interrupt pending.

Cycle Steal Address Register (CSAR)

This register is the interface to the 'cycle steal address bus'. It contains the current data address while data transfer is in progress. With each data halfword (two bytes) transferred to and from the channel, this address increases by two. The register is initially loaded with the control word (CW) address at the beginning of a CW-fetch operation and then is loaded with the starting data address when the CW fetch is complete.

The control program can use this register as input by executing an Input X'59' instruction. This input instruction is recognized only after the CA has been selected and has either an interrupt request pending or is in the diagnostic wrap mode. The CSAR is not available for output.

Channel Adapter Data Buffer (CADB)

The channel adapter data buffer register (CADB) forms the CA buffer for all channel data being transferred through the CA for either normal or diagnostic operations. This register physically shares a local store array with the INCWAR and OUTCWAR. The control program can load this register or use it as input by executing an Output or Input X'5A' instruction. The control program should ensure good parity in this register by executing an Output X'5A' instruction before attempting an Input X'5A' instruction. Under normal operation, this register is accessible when the CA has requested an interrupt.

Channel Tag Diagnostic Register (CTDR)

The channel tag diagnostic register (CTDR) contains two bytes used to simulate 'tag out' and interrogate 'tag in' signals for diagnostic purposes. The control program can load byte 0 of this register with an Output X'5B' instruction, and it can use the entire register as input by executing an Input X'5B' instruction. This register is accessible in the diagnostic wrap mode only.

Channel Adapter Command Register (CMDR)

The CA command register (CMDR) indicates the current channel command being executed by the CA. It also indicates the current or last control word executed.

10-4

The control program can use this register as input by executing an Input X'5C' (command register) instruction. The CMDR is not available for output. This input instruction is recognized only after a CA interrupt request and after the adapter select latch has been set.

Channel Adapter Control Word

The channel adapter control word (CW) specifies the operation to be performed by the CA in conjunction with a host processor channel operation. Control words are built by the control program according to the operation to be performed. See Figure 10-1, Type 2 and Type 3 Channel Adapter Control Word.

The following paragraphs describe the various fields of the CA control word.

Command Code: Bits 0-1 specify the operation to be performed. See Control Word Command Codes in this chapter.

Zero Count Override: Bit 2 is used with the command chain flag to determine what action the CA should perform when the current CW count is exhausted. When this bit is on, the CA requests a level 3 interrupt and disconnects from the byte-multiplexer channel without giving Channel End or remains connected to the burst channels without giving Channel End. The CA resumes data transfer when an Output X'57' instruction is executed to reset the channel adapter's level 3 interrupt request. The purpose of this flag is to allow the CA to transfer multiple buffers under a single host processor channel command with a minimum assignment of buffers to the CA.

Chain Flag: Bit 3 can cause multiple CWs to be used for one operation. When this bit is on, the appropriate 'CWAR valid' latch remains set during the CW-fetch operation. When the CW count reaches 0 while the CA is executing a Read, Write, Write Break, or Write IPL channel command, control word chaining to the next sequential CW occurs automatically. This bit must be on for a TIC (transfer in channel) control word.

Count: Bits 4-13 specify the number of bytes to be transferred across the channel interface by the CW. The maximum count is 1023 bytes. This field is ignored for TIC control words.

Data Address: Bits 14-31 contain the starting storage address for a read or a write operation, depending on the control word. For a TIC control word, this field contains the address of the next CW to be executed.

Control Word Command Codes

The following four CW commands are recognized by the CA. The first three are operational and indicate the direction of data movement between-the controller and the host processor. The fourth is used for chaining control words.

Out (B'00'): This control word command is used to control data transfer from the communications controller to the host processor via a channel Read command. If the count register decrements to 0, the zero count override flag is not on, and CW chaining is indicated, the CA immediately chains to the next CW without ending the channel command. If CW chaining is not indicated and the zero count override bit is not on, the current channel command is ended with CE status, and a level 3 interrupt request is set. If the zero count override bit is on, a level 3 interrupt is requested without presenting any status to the channel. If both zero count override and CW chaining are on, a level 3 interrupt is requested and the chaining indication is ignored.

Out Stop (B'0l'): This control word command is also used to control data transfer from the communications controller to the channel. If the count register decrements to 0, the zero count override flag is not on, and CW chaining is indicated, the current channel command ends immediately with CE, DE. A level 3 interrupt is not requested. If CW chaining is not indicated and the zero count override flag is on, the current channel command is ended with CE status, and a level 3 interrupt is requested. If both zero count override and CW chaining are on, a level 3 interrupt is requested before chaining is executed.

In (B'10'): This control word command is used to control data transfer from the channel to the communications controller. If the count register reaches 0, the zero count override flag is not on, and CW chaining is indicated, the CA chains immediately to the next CW without ending the channel command. If CW chaining is not indicated and the zero count override bit is not on, a level 3 interrupt request is set after the current channel command is ended with Channel End status. If the zero count override bit is on, a level 3 interrupt is requested without presenting any status to the channel. If both zero count override and CW chaining are on, a level 3 interrupt is requested and the chaining indication is ignored.

TIC (B'll'): The Transfer-In-Channel control word causes the CA cycle-steal hardware to load the address field of the CW into the appropriate CWAR. This causes a transfer to another string of CWs. The CA

IBM 3704 and 3705 Communications Controllers Principles of Operation  10-5

Figure 10-1. Type 2 and Type 3 Channel Adapter Control Word

10-6

must complete another CW fetch request before resuming data transfer across the channel interface. The zero count override flag is ignored for a TIC command.

Control Word Conventions

All control words must reside in the lower 64K bytes of storage. CW chaining or a TIC (transfer in channel) to an address above 64K causes a CW error condition resulting in a CA level 1 interrupt with register X'56', byte 0, bit 0 set on.

The data address portion of the control word can specify an odd or even address. If an odd address is specified by an Out or Out Stop control word, the data transferred to the host begins with the second byte of the halfword. If an odd address is specified by an In control word, the first byte from the host is stored in the second byte of the target halfword; the first byte remains unchanged.

The address specified by the address field of a TIC control word must be even.

The control word names include "In" or "Out" to denote the direction of data transfer with respect to the communications controller. Thus an In control word must be coupled with a channel Write, Write Break, or Write IPL command, and the Out and Out Stop CWs must be coupled with a channel Read command.

Channel Adapter Interrupt Requests

The CA has two interrupt requests assigned to it: a check interrupt request at level 1, and a normal service interrupt request at level 3. All programming operations for the CA are performed in these two interrupt levels with the use of input/output instructions.

A level 1 interrupt request is caused when a CA hardware or program error is detected. The error condition is set in the channel adapter check register. This register is available to the control program via an Input X'56' instruction, and error recovery may then be attempted.

A level 3 interrupt request occurs whenever the channel adapter requires service. Any of the following conditions cause a level 3 interrupt.

1. The In, Out, or Out Stop control word being executed has ended because the data count has decremented to zero and CW chaining is not indicated.

2. An In, Out, or Out Stop control word is being executed, the count has decremented to zero, and the zero count override flag is on.

3. All zeros initial status is accepted during executon of a Write IPL command.

4. The CA is in the active state, not initialized, no status is pending or stacked, and a program requested interrupt is generated.

5. The CA is in the inactive state, chaining is not

indicated, or there is no stacked or pending status,

and one of the following occurred:

a. The 'program level 3 request' latch was set by an output to register X'57' with byte 1, bit 0 on.

b. Program abort was set with an output to register X'57' with byte 1, bit 1 on.

c. A selective or system reset was recognized on the channel interface and is no longer indicated (Op Out is up again).

d. A channel stop or halt I/O indication was recognized on the channel interface.

6. The channel adapter has received a nonstandard command during an initial selection sequence and has been enabled to accept these commands by an output to register X'56' with byte 1, bit 7 on.

Channel Commands

Because the control program handles most of the functions previously handled by the host access method, the number of commands accepted by the controller has been greatly reduced. Only Test I/O and the following six commands are accepted. All others cause a Unit Check initial status to be returned to the host processor and the Command Reject sense bit to be set in a subsequent Sense command unless the control program has enabled the channel adapter to accept a nonstandard channel command. See Nonstandard Commands in this section.

Write (X'0l')

This channel command executes a data transfer from the host processor channel to the controller. The data may be user data or a control message to inform the control program of a function to be performed. The data transfer is terminated by (1) a Channel Stop indicated in response to a data service request, or (2) the control word count becoming zero and no further CW chaining being indicated.

Write Break (X'09')

This channel command causes the CA to perform the same functions as a Write command (X'Ol'). The only difference is that this command sets byte 1, bit 1 (Write Break command remember) in the CA control register (Input X'55'). This permits the host processor to inform the control program of the point it has reached in the host CCW chain.

IBM 3704 and 3705 Communications Controllers Principles of Operation  10-7

Write IPL (X'05')

This channel command transfers object code from the host processor to controller storage. When all zeros initial status is accepted by the host processor, the CA presents a level 3 interrupt request and causes the Central Control Unit to initiate IPL phase 1 if the controller is currently initialized. When the CCU read-only-storage code resets the interrupt request, the CA initiates a CW fetch and begins data transfer across the channel. Each time two bytes are received from the channel, a CA cycle-steal operation places the data in storage and increments the CSAR by 2.

The communications controller ends this command in the same manner as a normal Write command; that is. Channel End is generated by the CA hardware, and Device End is generated by interrupt-driven code. This implies that the level 3 interrupt program must have been loaded by the Write IPL command just concluded. If a Halt I/O is received while a Write IPL command is being executed, the host processor must reissue the Write IPL command.

Refer to Chapter 5 for further information on IPL.

Read (X'02')

This channel command executes a data transfer from the communications controller to the host processor channel. The operation is normally ended when a Channel Stop is indicated or the CW count register reaches zero, and either of the following occurs:

1. The CA is executing an Out Stop control word that has CW chaining indicated. The CA presents Channel End and Device End status to the channel and does not request a level 3 interrupt.

2. The CA is executing either an Out CW or Out Stop CW with no CW chaining indicated and with the zero count override bit off. With this condition, the CA presents Channel End status to the host processor channel and requests a level 3 interrupt. The control program must then signal the CA hardware to present Device End and any other appropriate status to the channel.

If the Read command is terminated by a Channel Stop or Halt I/O, the CA presents Channel End status immediately. When the host processor accepts this status, the channel adapter requests a CA level 3 interrupt. The control program must signal the CA to present Device End and other appropriate status.

Sense (X'04')

The Sense command transfers one byte of sense data from the controller to the host processor channel. This byte is described in the channel adapter sense register (CASNSR). The normal ending status for the Sense command is Channel End, Device End after the channel has accepted

the sense byte. No interrupt request is generated.

Except for the not-initialized bit, the sense byte is reset during an initial selection when any host processor command, other than Sense, No-Op, or Test I/O, is accepted by the adapter.

No-Op (X'03')

The channel adapter responds to this command with Channel End, Device End initial status. No further action occurs, and no interrupt request is generated.

Test I/O (X'00')

The channel adapter presents the following initial status in response to a Test I/O:

1. Zero status indicates that the CA is command-free, and there is no pending or stacked status.

2. Busy status is indicated from the acceptance of a command until the generation of Device End. If CE or DE is pending. Busy and CE or DE are presented together.

3. Device End (and any other status) is presented if the ending status is pending or stacked from the previous host processor command.

4. Busy status is indicated if the CA has a level 1 or level 3 interrupt request set.

Once the initial status has been accepted, the Test I/O operation is complete. An interrupt request is not set, and no further action is taken.

Nonstandard Commands

The communications controller recognizes only Test I/O and the above six commands as valid. All other commands are considered nonstandard.

The control program can enable the channel adapter to accept nonstandard commands by executing an Output X'56' instruction to set byte 1, bit 7 on. The CA then responds to a nonstandard command by presenting Channel End rather than Unit Check initial status. The CA also requests a level 3 interrupt, with the CA in the active state, to allow the control program to examine the command register (Input X'5C') and the data buffer (Input X'5A'). A nonstandard command is indicated by byte 1, bit 4 being on in the command register and all other command decode bits being off. The channel command byte is stored in byte 0 of the data buffer (register X'5A'). During this level 3 interrupt, the program can set the command reject bit in the sense register by executing an Output X'53' instruction to turn byte 0, bit 0 on. This output instruction also sets Unit Check in the status register (X'54').

This ability of the channel adapter to accept non-standard channel commands is reset whenever:

10-8

• The controller is powered-up.

• The LOAD or RESET buttons are pressed.

• A Write IPL (X'05') channel command is executed.

•  An Output X'56' with byte 1, bit 6 on is executed.

Programming Note

The program must always set Device End in the status register (X'54') during a level 3 interrupt. When the level 3 interrupt is reset by the program, the CA presents the status to the channel.

Status Servicing

The status byte informs the host processor channel if the channel adapter is available to communicate. This status byte is generated by either the CA hardware or the control program and is stored in the channel adapter status register (CASTR).

The four acceptable status configurations for the channel adapter are described below.

Initial Status

Initial status is always generated by the CA hardware without control program intervention. The valid initial status combinations are:

All Zeros: The channel command is accepted.

Channel End, Device End: This is an unconditional response to a No-Op.

Unit Check: An error condition has occurred, and the channel command cannot be executed because:

a. The command is invalid.

b. The channel adapter is not initialized.

c. A bus-out parity error was detected in the command byte.

Further information on the error can be obtained by examining the sense byte.

Unit Exception: When the appropriate control word address register (CWAR) does not contain a valid address, this status is indicated for a Read, Write, or Write Break command. Also, in a controller containing two channel adapters. Unit Exception is returned as initial status to a Sense command if an IPL is in progress on the other channel adapter.

Busy: The controller is already in the process of executing a channel command or the channel adapter level 1 or level 3 interrupt request latch is set. If ending status has been generated for the command currently being executed, this ending status; is presented along with Busy as initial status. The only exception occurs when the channel command indicated in the

initial selection is a Test I/O.

Ending Status

At the termination of a channel command, the CA generates the following status bits to be passed to the host processor.

Channel End: This status is hardware-generated under normal circumstances for host processor Read, Write, Write Break, and Write IPL commands. Whenever the CA generates CE alone, it also requests a level 3 interrupt. The control program is thus informed of the circumstances that caused the Channel End and must complete the channel operation by signaling the CA to present Device End and any other appropriate status:

Channel End, Device End: This status is normally generated as the ending status for a channel Read command if combined with an Out Stop control word. Channel End, Device End is also the normal ending status for a No-Op or Sense command.

Channel End, Device End, Unit Check: This status is generated by the CA when any one of the following three conditions exists.

a. When a sense bit is set during the execution of a Read, Write, Write Break, or Write IPL command. The CA also requests a level 1 interrupt to inform the control program of the error.

b. When a program abort is indicated by the control program before Channel End status is set during any data transfer command. CASNSR bit 6 (abort sense) is also set, and a level 3 interrupt request is generated.

c. When the controller is in a 'hard stop' state due either to a hardware or program detected malfunction.

Device End: This status is presented to the host processor when the CA level 3 interrupt request latch is reset after CE status has been presented. Only those status bits generated by the control program, including DE, are presented. Therefore, the control program must never reset the level 3 interrupt request without having set at least Device End in the status register via Output X'54'.

Stacked Status

Stacked status occurs when the channel adapter has status information to present to the channel, but the channel cannot accept it at that time.

When the channel indicates stacked status, the controller status is retained in the channel adapter status register until accepted by the host processor

IBM 3704 and 3705 Communications Controllers Principles of Operation  10-9

channel. All status except all zeros to a Start I/O is subject to being stacked by the channel.

Stacked ending status is handled in the same manner as stacked initial status. All ending status conditions can be stacked.

Asynchronous Status

The CA presents asynchronous status to the host channel under one of four conditions:

1. The channel adapter receives an Output X'55' instruction from the control program with byte 0, bit 6 set to 1 (set program requested attention), and the CA is not active. Attention status is presented to the host channel when the CA level 3 interrupt request is reset.

2. Execution of an Output X'57' instruction with byte 0, bits 3 and 4 (Set IPL Device End and Unit Check) set to 1 while in IPL Phase 3. This causes the CA to present an asynchronous status of Device End and Unit Check to the host channel and indicates that the controller requires an IPL from the host processor.

3. An asynchronous Device End is presented to the host channel when an interrupt is reset that caused a Busy initial status to be presented in response to an initial selection.

4. An asynchronous Device End is presented to the host channel when the control program executes an Output X'57' instruction with byte 0, bit 3 set to 1.

The Type 3 CA can present asynchronous status to either channel. If both channel interfaces are enabled, the first channel to poll the Type 3 CA is offered the status.

A channel receives a Busy status if it attempts an initial selection sequence while the Type 3 CA is logically connected to the opposite channel. The CA presents an asynchronous Device End to this channel as soon as the opposite interface's connection is terminated.

Multiple Channel Operation

A maximum of two channel adapters can be installed in the 3705, one in the base module and one in the first expansion module. A Type 2 or Type 3 CA in the expansion module can coexist with a Type 1, Type 2, or Type 3 CA in the base module. The control program issues an Output X'57' instruction to select the channel adapter it wants to use. Byte 1, bit 4 of this output determines whether the adapter in the base module (bit 4= 1), or the adapter in the first expansion module (bit 4=0) is selected.

If two type 2 CAs are installed, both can be attached to the same channel or each to a separate chan

nel. Both adapters operate independently and can be enabled concurrently.

If a Type 3 CA is installed, its two interfaces can be attached to a uniprocessor as an I/O unit with an alternate path capability, or they can each be attached to a separate CPU of a tightly-coupled multiprocessor. Both interfaces can be enabled concurrently, but simultaneous operation is not permitted. When a channel I/O operation over one interface is being executed, an initial selection sequence attempt by the channel associated with the other interface will cause a busy status to be presented to that channel.

Either of the Type 3 channel interfaces can be manually enabled or disabled by using the channel enable/disable toggle switches located on the 3705 control panel. These toggle switches may be alternately located on a remote configuration console of a multiprocessing system.

When both interfaces are enabled, the adapter is selected by the first channel to initiate a selection sequence. If both channels simultaneously poll the Type 3 CA, the adapter logic breaks the tie. If the enable/disable switch for either interface is moved to the disable position, that interface can go offline subject to the following conditions:

• The channel adapter is not executing a command on that interface.

•  Command chaining is not being indicated for that interface.

•  A Device End status is not pending on that interface.

• The CPU is in a wait state.

•  An Input X'58' instruction is not being executed to examine the state of the 'enable' latch.

•  'Select Out' is not up on that interface.

If the switch is moved to the enable position, the interface can go online if the CPU is in a wait state and the 3705 is not executing an Input X'58' to examine the state of the 'enable' latch.

Note: A pending asynchronous status, which is available to either channel, does not inhibit manually disabling an interface. If the CA is presenting asynchronous status to the channel when the disabling switch is thrown, that sequence is completed.

Type 3 Channel Adapter-Channel Interface States

The Type 3 CA can be in one of three states in relation to the attached channel interfaces: disabled, neutral, or switched.

Disabled: When an interface of the Type 3 CA is disabled by the use of the enable/disable switch, the 3705 appears not-operational to the associated channel.

10-10

Neutral: In this state the adapter is enabled and available to both attached channels, but is not logically connected to either channel.

Switched: The adapter enters the switched state when it becomes logically connected to a channel when the channel or the channel adapter attempts an initial selection sequence. The adapter remains switched to a channel after an initial selection sequence ends if the channel accepts an all zero status from the adapter in response to a command for which (1) subsequent communication is required (data transfer and/or ending status required to complete the command), or (2) command chaining is indicated for a No-Op.

If the adapter remains switched after the initial selection sequence is completed, it maintains the logical connection to the channel until one of the following occurs:

• The channel accepts Device End status for the last command of a particular Start I/O that does not indicate command chaining,

•  Command chaining is suppressed after the channel's acceptance of Device End status, but before reselection occurs.

•  System or selective reset is received over that interface.

While in the switched state the adapter continually monitors the opposite interface and responds to an initial selection sequence over that interface in one of two ways:

•  The CA temporarily suspends the selection sequence if it is already involved in an initial selection sequence with the first channel. If the CA returns to the neutral state as soon as the initial selection sequence is completed (as would be the case if the adapter had instigated the sequence to present asynchronous status), the opposite channel completes its initial selection sequence, which had been delayed.

•  The adapter responds with a Device Busy status. In this case, the first channel has (1) already accepted an all-zero status from the CA in response to a command for which subsequent communication is required, or (2) sent a No-Op instruction for which command chaining is indicated.

The adapter does not remain in the switched state after the initial selection sequence is completed if the sequence was instigated by the Type 3 CA to present asynchronous status to a channel and the channel has accepted or stacked the status.

Error Condition: If the Type 3 CA ends a channel command with Unit Check status indicating an error condition, the adapter remains switched to that channel until one of the following occurs:

• The adapter decodes a command other than Test I/O or No-Op.

•  A system or selective reset is detected over the channel to which the adapter is switched.

•  A 3705 hard stop occurs.

In this way the Type 3 CA ensures the availability and use of the same channel path to return sense data after an I/O operation has ended with Unit Check status. A halt I/O presented over the channel to which the adapter is logically connected does not cause it to leave the switched state. Under any of these conditions, any selection attempt by the opposite channel will receive a Device Busy status.

Type 3 CA Response to Resets

There are several different reset combinations for the Type 3 CA. The following paragraphs describe those combinations.

System Reset Over the Interface To Which the Adapter Is Switched: A system reset from the channel to which the adapter is switched will always be recognized, will reset the adapter (CA returns to neutral state), and will cause a level 3 interrupt. Only a pending Device End due to a previous Device Busy over the opposite interface is not reset.

Selective Reset over the Interface to Which The Adapter Is Switched: A selective reset over the interface to which the CA is switched performs the same function as a system reset.

System and Selective Resets—Adapter Neutral: The only interface activity which may be reset when the adapter is neutral is a pending asynchronous status. A pending Device End due to a previous Device Busy is reset only if the reset indication is received over the interface for which the Device End is intended.

System Reset over the Opposite Interface: A system reset received over the channel opposite that to which the interface is switched resets only a pending Device End for the channel over which the reset is being signaled. No further action is taken, no level 3 interrupt is requested, and no further resetting of the adapter occurs.

Input/Output Instructions

The channel adapter input/output instructions enable the control program to communicate with the host

IBM 3704 and 3705 Communications Controllers Principles of Operation   10-11

processor I/O channel. These instructions are available to the control program only when the CA is selected and (1) a level 1 or level 3 interrupt has been requested or (2) the CA is in the diagnostic wrap state. Type 2 and Type 3 CA instructions are specified by X'50' to X'5F'.

Input Instructions

Twelve input instructions allow the control program to obtain the status of various channel adapter registers. Listed below are the usable Type 2 and Type 3 CA input instructions and the registers associated with them. The register descriptions are found elsewhere in this chapter. See Appendix B for the input instruction bit definitions.

When the adapter is selected and a CA LI or L3 interrupt request is set, or the CA is in the diagnostic wrap state, execution if Input X'57', X'5D', X'5E', or X'5F', results in loading all zeros into the general register specified by the R operand. If the CA is not selected or an interrupt has not been requested, any attempt to execute an Input X'50' through X'5E' results in an in/out check LI interrupt request.

Input X'50':

Inbound Data Control Word Address Register (INCWAR).

Input X'51':

Outbound Data Control Word Address Register (OUTCWAR).

Input X'52':

Control Word Byte Count Register (CWCNTR).

Input X'53':

Channel Adapter Sense Register (CASNSR).

Input X'54':

Channel Adapter Status Register (CASTR).

Input X'55':

Channel Adapter Control Register (CACR).

Input X'56':

Channel Adapter Check Register (CACHKR).

Input X'58':

Channel Bus Out Diagnostic Register (CBODR).

Input X'59':

Cycle Steal Address Register (CSAR).

Input X'5A':

Channel Adapter Data Buffer (CADB)

Input X'5B':

Channel Tag Diagnostic Register (CTDR)

Input X'5C':

CA Command Register (CMDR)

Output Instructions

Eleven output instructions allow the control program to set the bits of various channel adapter registers. Listed below are the usable CA output instructions and the registers associated with them. The register descriptions are found elsewhere in this chapter.

One of these instructions (Output X'59') is for diagnostic purposes. It allows a diagnostic program to make either interface or both interfaces Busy. See Appendix B for output instruction bit definitions.

When the adapter is selected and a CA LI or L3 interrupt request is set or the CA is in the diagnostic wrap state, the execution of an Output X'52', X'5C', X'5D', X'5E', or X'5F' instruction is ignored. If the CA is not selected or an interrupt has not been requested, any attempt to execute an Output X'50' through X'5F' results in an in/out check LI interrupt request.

Output X'50':

Inbound Data Control Word Address Register (INCWAR).

Output X'51':

Outbound Data Control Word Address Register (OUTCWAR).

Output X'53':

Channel Adapter Sense Register (CASNSR).

Output X'54':

Channel Adapter Status Register (CASTR).

Output X'55':

Channel Adapter Control Register (CACR).

Output X'56':

Channel Adapter Control Register (CACR).

Output X'57':

Channel Adapter Mode Register (CAMR).

Output X'58':

Channel Bus Out Diagnostic Register (CBODR).

Output X'59':

Type 3 CA Diagnostic Control (CADC).

Output X'5A':

Channel Adapter Data Buffer (CADB).

Output X'5B':

Channel Tag Diagnostic Register (CTDR).

Error Indications

When the channel adapter hardware detects an error condition, a level 1 interrupt request is automatically set. With this request, a bit is also set in the channel adapter check register (X'56') to indicate the nature of the error. The level 1 interrupt handling routine, using an Input X'56' instruction, should then analyze the contents of this register to determine the cause of the error and execute appropriate recovery procedures.

10-12

Refer to the channel adapter check register (CACHKR) and Appendix B for the error bit definitions and register reset.

The following are the valid CA error checks:

• Invalid CWAR Address

• Invalid Control Word Format

• Data Address Error Buffer Parity Error

• Outbus Parity Error

• Inbus Parity Error

• Channel Bus In Parity Error

• Channel Bus Out Check

IBM 3704 and 3705 Communications Controllers Principles of Operation—  10-13